CONFIDENTIAL

ATTORNEY WORK PRODUCT  ·  PATENT REVIEW (UNIQUE ENGINES)

AutoVAD

AI Copilot for Civil Engineers

Patent-Only Pack
Unique Takeoff, CAD, and EOQ Engines

Prepared: 2026-08-27    ·    Engine source files: 29    ·    Approximate source lines: 14,371

This pack is for patent counsel. It describes the novel automated civil-quantity methods and reprints only the engine source that implements them. It does not include the website shell (login, projects UI, billing, admin, notifications). Those files remain copyrighted as part of AutoVAD; they are in the separate full copyright deposit. This document is not legal advice and does not file an application.

# 1. Confidentiality and handling

Every page of this document is marked CONFIDENTIAL in red in the header, footer, and as a diagonal watermark. Counsel, inventors, and authorized reviewers only. Do not email to personal accounts, upload to public AI tools, or share with vendors without a written NDA.

Classification: Confidential — trade secret / unpublished source.

Suggested legend: CONFIDENTIAL — AutoVAD — Attorney review only.

Use this pack for patent claims. Use the full source deposit for copyright registration of the whole program.

# 2. What this pack is for

Patent law protects novel methods and systems, not every line of a website. This pack is limited to AutoVAD’s unique quantity-takeoff engines. Owner / inventors: to be completed by counsel.

Included: PDF/CAD takeoff engines, incidental filter, location combining, traffic-control rollup, CSI/EOQ grouping, bid-template matching, Civil 3D stationing, Excel EOQ engine.

Excluded: login, registration, dashboard chrome, billing, notifications, admin UI, generic API glue.

# 3. System overview (engine)

An engineer uploads PDF plan sets and/or CAD (DXF, DWG, LandXML). The engines extract pay items and quantities, build an Estimate of Quantities (EOQ), optionally map them to an agency bid template, and export Excel. Schedule tables are treated as authoritative. Incidental drawing notes are not paid twice. The same item from multiple locations is combined.

| Engine | Role |
| --- | --- |
| Document AI fusion | Text, tables, PDF vision, labels — schedule-first ranking |
| Incidental filter | Drop work marked incidental; do not add it into the parent qty |
| Location combiner | Sum similar pay items from different sheets; keep true variants apart |
| Traffic Control | MUTCD sign rollup to SqFt, or copy schedule companion rows |
| CAD / Civil 3D | Size-aware quantities, trench extras, APS DWG, station-offset lengths |
| EOQ assembly | CSI + agency bid numbers, municipal groups, Mobilization, Excel review |

# 4. Unique methods for patent review

Counsel should assess patentability (novelty, non-obviousness, eligible subject matter) and whether to file a provisional covering the combination of schedule-first extraction, incidental exclusion, location combining, traffic-control rollup, and Civil 3D stationing.

## U-01  Bid-schedule-authoritative pay-item copy

When an Estimate of Quantities / Bid Items table exists, AutoVAD copies those rows as the pay-item list and drops F-sheet device tables, graphic symbol counts, assumed trench extras, and typical-section invents. Agency bid numbers (for example 634.0110) are preserved instead of being overwritten by CSI codes.

Primary source: backend/app/services/ai_analysis.py — _is_bid_schedule_table, _items_from_document_tables, _should_drop_inferred_extra, _finalize_analysis

## U-02  Multi-engine drawing takeoff fusion

PDF plans are read by combining OpenAI text/table extraction, rendered-sheet vision, deterministic utility-label parsing, heuristic patterns, and CAD geometry. Evidence is ranked so a printed schedule quantity is preferred over a drawing measurement of the same item.

Primary source: backend/app/services/ai_analysis.py, pdf_vision.py, openai_client.py, utility_labels.py

## U-03  Incidental-to-bid-item exclusion

Drawing notes that mark work as incidental to / included in / paid under a bid item (bedding, trench, fittings, tracer wire, thrust blocks, testing, and similar) are omitted as separate pay items and are not added into the parent quantity.

Primary source: backend/app/services/incidental.py and wiring in ai_analysis.py, utility_labels.py, civil_estimator.py

## U-04  Similar-item location combining

The same pay item taken off on multiple sheets (example: Fertilizer 1,189 lb and Fertilizer 39 lb) is identified as one item and the quantities are added. True variants stay separate (pipe size, barricade type, fertilizer N-P-K grade, seed vs fertilizer). Duplicate schedule copies are not double-counted; a printed project total is not added on top of its location parts.

Primary source: backend/app/services/item_combine.py — combine_similar_pay_items

## U-05  Traffic Control sign rollup with schedule companions

Individual MUTCD / plan signs are rolled into one Traffic Control SqFt pay item when no bid schedule exists. When a Bid Items table has a Traffic Control section, every printed companion row is copied (SqFt signing, miscellaneous LS, barricades, PCMS, mailbox, gravel access) and F-sheet Project Totals / channelizer graphics are not invented as extra bid items.

Primary source: backend/app/services/traffic_control.py; MUTCD sizes in backend/app/data/mutcd_sign_sizes.json

## U-06  Size-aware CAD quantity engine

CAD entities are converted to civil pay items by network (water / sanitary / storm), detected pipe size, length, hatch area, and named-block counts. Trench excavation / bedding / backfill CY is derived from pipe OD with stated cover assumptions, and those extras are skipped when the CAD text already contains a Bid Items / EOQ table.

Primary source: backend/app/services/cad/quantity_engine.py, civil_estimator.py

## U-07  Civil 3D / DWG station-offset takeoff

Native DWG is processed through Autodesk Platform Services (upload, translation, properties) and an AutoCAD Design Automation plugin that exports alignments, pipes with start/end XY, blocks, and paper-space callouts. Station-offset logic builds utility bid-item lengths along Civil alignments.

Primary source: backend/app/services/cad/utility_stationing.py, aps_client.py, design_automation.py, dwg_aps.py; backend/cad_plugins/AutoVadCivilTakeoff/Commands.cs

## U-08  Municipal EOQ grouping and dual Mobilization display

Pay items are grouped in agency EOQ order (General, Traffic Control, Removals, Grading, utilities, surfacing, and related). General and Traffic Control are separate sections. Mobilization (1 LS) is stored once and displayed under both section headers in the UI and Excel export.

Primary source: backend/app/services/eoq_groups.py; frontend/src/utils/eoqGroups.ts; eoq_service.ensure_mobilization_item

## U-09  CSI mapping that does not erase agency bid numbers

Descriptions are mapped to CSI MasterFormat codes and USA pay units (including SQFT and TON export labels), while agency Standard Bid Item Numbers remain on item_code when they look like bid numbers rather than CSI.

Primary source: backend/app/services/csi_mapper.py

## U-10  Design-only civil estimator

When no bid schedule exists, typical-section dimensions produce pavement CY/TON, prime/tack SY, building/dam quantities, and related estimator lines, with assumptions written into calculation_method for engineer review.

Primary source: backend/app/services/civil_estimator.py

## U-11  Plan-label utility parser

Callouts such as 8" WATER MAIN 245 LF or station-range labels are parsed deterministically into sized water / sanitary / storm quantities. Fitting invents are suppressed when an EOQ schedule is present.

Primary source: backend/app/services/utility_labels.py

## U-12  Bid-template matching without dumping the unused list

An uploaded agency bid list is matched to evidenced takeoff only. Unused template lines are omitted. Multiple location hits on the same template line are summed.

Primary source: backend/app/services/bid_service.py — build_eoq_items_from_template

## U-13  Gold-set EOQ evaluation / Training Lab

Expected EOQ gold cases are compared to engine output (recall, misses by category, quantity error). An internal Training Lab lets reviewers compare original vs AutoVAD analysis vs evaluation.

Primary source: backend/app/services/eoq_eval.py, training_service.py; frontend training views

## U-14  Engineer-review Excel EOQ workbook

Excel export writes municipal section headers, bid numbers, SQFT/TON labels, and conditional formatting so Verified vs Engineer Review changes cell fill and font. Utility stationing can export detail plus rolled summary.

Primary source: backend/app/services/eoq_service.py

# 5. What is not claimed as AutoVAD invention

Generic civil formulas (CY = L × W × D / 27) as such.

MUTCD sign designations as a public standard; AutoVAD’s use/rollup logic is claimed, not the MUTCD catalog.

CSI MasterFormat numbering as a published classification; AutoVAD’s mapping rules and agency-number preservation are claimed.

Autodesk APS / AutoCAD / Civil 3D platforms; AutoVAD’s plugin, property mapping, and stationing pipeline are claimed.

OpenAI models; AutoVAD’s prompts, fusion policy, and post-filters are claimed.

Ordinary website features (login, project CRUD, billing pages) — copyright only, not patent claims in this pack.

# 6. Engine source inventory

Each engine file below is identified by path, line count, and SHA-256. Section 7 reprints those files in full.

| Path | Lines | SHA-256 (prefix) |
| --- | --- | --- |
| backend/app/services/ai_analysis.py | 1716 | bc7b82c0c05fe4b8… |
| backend/app/services/incidental.py | 256 | 9116b7c1e47b79ea… |
| backend/app/services/item_combine.py | 464 | b21dd42b3038d4ee… |
| backend/app/services/traffic_control.py | 651 | 27ff1d0f5363fae0… |
| backend/app/data/mutcd_sign_sizes.json | 149 | 7bcd63567ede8698… |
| backend/app/services/civil_estimator.py | 484 | 597d4d5574445792… |
| backend/app/services/utility_labels.py | 241 | a45c8f0ea5c539b4… |
| backend/app/services/csi_mapper.py | 280 | 3ce1167a945bc4cc… |
| backend/app/services/eoq_groups.py | 497 | fa2396464b6047fc… |
| backend/app/services/eoq_service.py | 990 | 45eecf1e3f64b54c… |
| backend/app/services/bid_service.py | 678 | 667ddcd9a6bbd081… |
| backend/app/services/eoq_eval.py | 539 | 9e30daaa8b78e90f… |
| backend/app/services/pdf_vision.py | 274 | 5a74fc2d9794fa59… |
| backend/app/services/openai_client.py | 318 | 11cbe59cbb254656… |
| backend/app/services/extractors.py | 113 | 2557e02acc3cb1bd… |
| backend/app/services/cad/quantity_engine.py | 769 | 092e3278afd4aa4b… |
| backend/app/services/cad/utility_stationing.py | 1726 | 2d73d1b245346653… |
| backend/app/services/cad/civil_location.py | 769 | 24bb4338e57186fe… |
| backend/app/services/cad/engine.py | 202 | f08bd346bd5c79ce… |
| backend/app/services/cad/dxf_parser.py | 279 | 7e1383f8c0fc7146… |
| backend/app/services/cad/landxml_parser.py | 220 | 81c1d6cc50d729de… |
| backend/app/services/cad/aps_client.py | 1081 | ae5bb6d18c94a5d4… |
| backend/app/services/cad/design_automation.py | 647 | 4961e3dba47672eb… |
| backend/app/services/cad/dwg_aps.py | 5 | 4618904a7159e3e2… |
| backend/app/services/cad/autodesk_aps.py | 158 | 024f4c06bd3eb384… |
| backend/cad_plugins/AutoVadCivilTakeoff/Commands.cs | 512 | b5a76909b7209bd9… |
| backend/cad_plugins/AutoVadCivilTakeoff/PackageContents.xml | 27 | 7bf457c84b2db065… |
| frontend/src/utils/eoqGroups.ts | 319 | fdb95d74b12e27de… |
| backend/app/schemas/units.py | 7 | 20bc039c590d2553… |

Total engine files: 29. Total lines: 14,371.

# 7. Engine source listing

Verbatim listing of unique engine source for patent counsel. Formatting is for identification, not execution. Website shell source is omitted.

### backend/app/services/ai_analysis.py

SHA-256: bc7b82c0c05fe4b87459727175c8e705ca7bc9cc26b788f8d25de5150a89bc4f    ·    1716 line(s)    ·    .py

"""Civil document intelligence: OpenAI when configured, heuristic fallback otherwise.

        PDF plans use text/tables PLUS rendered drawing sheets via OpenAI vision so
engineering drawings (not only OCR text) drive EOQ quantities.
"""

        from __future__ import annotations

        import json
import re
from decimal import Decimal, InvalidOperation
from pathlib import Path
from typing import Any

        from app.services.extractors import ExtractedContent
from app.services.incidental import (
    description_is_incidental_child,
    extraction_should_skip,
    quantity_inflated_by_incidentals,
    should_drop_incidental_item,
)
from app.services.item_combine import combine_similar_pay_items

        CIVIL_PATTERNS: list[tuple[str, str, str, str]] = [
    # description_key, category, unit_hint, regex
    ("GSB", "Pavement", "m3", r"\bGSB\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cu\.?\s*m|cubic\s*meters?|cy|cu\.?\s*yd)"),
    ("WMM", "Pavement", "m3", r"\bWMM\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cu\.?\s*m|cubic\s*meters?|cy|cu\.?\s*yd)"),
    ("DBM", "Pavement", "m3", r"\bDBM\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cu\.?\s*m|cubic\s*meters?|cy|cu\.?\s*yd)"),
    ("Bituminous Concrete", "Pavement", "m3", r"\b(?:BC|Bituminous\s*Concrete)\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|t|ton|tons?|cy)"),
    ("Asphalt", "Pavement", "m3", r"\bAsphalt\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|t|ton|tons?|cy)"),
    ("HMA", "Pavement", "ton", r"\b(?:HMA|Hot\s*Mix\s*Asphalt)\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(t|ton|tons?|cy|m3)"),
    ("Earthwork Cut", "Earthwork", "m3", r"\b(?:Cut|Excavation)\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy|cu\.?\s*yd)"),
    ("Earthwork Fill", "Earthwork", "m3", r"\b(?:Fill|Embankment)\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy|cu\.?\s*yd)"),
    ("Concrete", "Structures", "m3", r"\bConcrete\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Kerb", "Roadside", "m", r"\bKerb(?:ing)?\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m|lm|lin(?:ear)?\s*m|lf|ft)"),
    ("Curb and Gutter", "Roadside", "lf", r"\bCurb(?:ing)?(?:\s*(?:and|&)\s*Gutter)?\b.*?(\d{1,6}(?:,\d{3})*(?:\.\d+)?)\s*(lf|lft|ft|m)"),
    ("Sidewalk", "Roadside", "sf", r"\bSidewalk\b.*?(\d{1,7}(?:,\d{3})*(?:\.\d+)?)\s*(sf|sq\.?\s*ft|m2|m²|sy)"),
    ("Culvert", "Drainage", "nos", r"\bCulvert(?:s)?\b.*?(\d{1,4})\s*(nos?|no\.?|each|ea)"),
    ("Drainage", "Drainage", "m", r"\bDrain(?:age)?\b.*?(\d{1,3}(?:,\d{3})*(?:\.\d+)?)\s*(m|lm|lf|ft)"),
    ("Brickwork", "Building", "cy", r"\bBrick(?:work)?\b.*?(\d{1,6}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Plastering", "Building", "sf", r"\bPlaster(?:ing)?\b.*?(\d{1,7}(?:,\d{3})*(?:\.\d+)?)\s*(m2|m²|sf)"),
    ("Doors", "Building", "ea", r"\bDoors?\b.*?(\d{1,4})\s*(nos?|ea|each)"),
    ("Windows", "Building", "ea", r"\bWindows?\b.*?(\d{1,4})\s*(nos?|ea|each)"),
    ("Dam Embankment Fill", "Dams & Reservoirs", "cy", r"\b(?:Dam\s+)?Embankment\b.*?(\d{1,8}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Spillway Concrete", "Dams & Reservoirs", "cy", r"\bSpillway\b.*?(\d{1,7}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Riprap", "Dams & Reservoirs", "cy", r"\bRiprap\b.*?(\d{1,7}(?:,\d{3})*(?:\.\d+)?)\s*(m3|m³|cy|ton)"),
    ("Reservoir Lining", "Dams & Reservoirs", "sf", r"\b(?:Reservoir|Pond)\s*Lin(?:ing|er)\b.*?(\d{1,8}(?:,\d{3})*(?:\.\d+)?)\s*(m2|m²|sf)"),
    ("Road Width", "Geometry", "m", r"\b(?:Road|Carriageway)\s*Width\b.*?(\d{1,2}(?:\.\d+)?)\s*(m|ft)\b"),
]

        ITEM_ALIASES = {
    "gsb": ("GSB", "Pavement", "m3"),
    "granular sub base": ("GSB", "Pavement", "m3"),
    "wmm": ("WMM", "Pavement", "m3"),
    "wet mix macadam": ("WMM", "Pavement", "m3"),
    "dbm": ("DBM", "Pavement", "m3"),
    "dense bituminous macadam": ("DBM", "Pavement", "m3"),
    "bc": ("Bituminous Concrete", "Pavement", "m3"),
    "bituminous concrete": ("Bituminous Concrete", "Pavement", "m3"),
    "asphalt": ("Asphalt", "Pavement", "m3"),
    "hma": ("HMA", "Pavement", "ton"),
    "kerb": ("Kerb", "Roadside", "m"),
    "curb": ("Curb and Gutter", "Roadside", "lf"),
    "sidewalk": ("Sidewalk", "Roadside", "sf"),
    "culvert": ("Culvert", "Drainage", "nos"),
    "concrete": ("Concrete", "Structures", "m3"),
    "excavation": ("Earthwork Cut", "Earthwork", "m3"),
    "cut": ("Earthwork Cut", "Earthwork", "m3"),
    "fill": ("Earthwork Fill", "Earthwork", "m3"),
    "embankment": ("Earthwork Fill", "Earthwork", "m3"),
    "brickwork": ("Brickwork", "Building", "cy"),
    "plaster": ("Plastering", "Building", "sf"),
    "doors": ("Doors", "Building", "ea"),
    "windows": ("Windows", "Building", "ea"),
    "spillway": ("Spillway Concrete", "Dams & Reservoirs", "cy"),
    "riprap": ("Riprap", "Dams & Reservoirs", "cy"),
    "reservoir lining": ("Reservoir Lining", "Dams & Reservoirs", "sf"),
    "dam embankment": ("Dam Embankment Fill", "Dams & Reservoirs", "cy"),
}

        # Shared accuracy rules for text + vision takeoff (Training Lab Test 1/2 failure modes).
TAKEOFF_ACCURACY_RULES = """
SOURCE OF TRUTH (strict — follow in order):
1. Estimate Of Quantities / bid schedule / proposal quantity tables are AUTHORITATIVE.
   For every item, copy description, STD BID NO / item_code, unit, and quantity from the SAME schedule row.
2. Treat “(Ctd.)”, “continued”, and repeated table headers as ONE continuous schedule.
   Extract ALL rows on continuation pages and keep the parent category (e.g. Water Main, not only “Water Main (Ctd.)”).
3. Include Alternate A / Alternate B / option sections as valid pay items; keep the alternate label in category or description.
4. Include lump-sum and non-physical rows: Mobilization, Tax on City Furnished Materials, Winter Maintenance,
   Traffic Control, temporary items, signals, lighting, removals, erosion/landscaping.
5. When a Bid Items / Estimate Of Quantities table exists, COPY those table rows as the pay-item list.
   Do NOT invent extras from F-sheets, typical sections, trench assumptions, graphic/symbol counts,
   or device “Project Totals” tables. Do NOT decompose a bundled schedule item into fittings
   (elbows, tees, valves, hydrants, plugs, reducers, casing/carrier segments) unless those are separate schedule rows.
6. When a schedule quantity exists, NEVER replace it by counting symbols, reading a detail callout, or measuring geometry.
7. Keep schedule-distinct variants separate (furnish vs install, left vs right flange, diameters, materials, alternates).
8. INCIDENTAL WORK is not a pay item. If a note says work is incidental to / included in / paid under a bid item
   (or "no separate payment/measurement"), do NOT output it as its own quantity AND do NOT add it into the parent
   item quantity. Parent qty = the pay item only (schedule cell or measured pipe/pavement), never parent + incidentals.
   Typical-section details for bedding, trench, backfill, fittings, tracer wire, thrust blocks, polywrap, and testing
   are construction details unless that work is its own Bid Items / EOQ row.
9. Same pay item on multiple sheets/locations (e.g. Fertilizer 1,189 lb and Fertilizer 39 lb) is ONE bid item.
   Keep the same description/unit so location quantities can be combined. Do not invent a second pay item name.

        REQUIRED SEARCH PASSES:
- General / LS / tax / temporary / winter maintenance
- Traffic Control, Traffic Signals, Lighting
- Removals / abandon / sawcut / salvage / clear & grub
- Grading / topsoil
- Erosion Control, Planting & Landscaping (seed, fertilizer, silt fence, wattles, riprap, blankets, weed control)
- Surfacing / curb / pavement
- Water Main (+ continuation pages), Storm Sewer, Sanitary Sewer
- Structures / bridges / retaining walls
- Dams, spillways, reservoirs / pond lining
- Building: brickwork, RCC, doors, windows, plaster, flooring, roofing
- Alternate A / Alternate B sections

        UNITS (normalize on output):
LS, Each, Ft, LFt, SqFt, SqYd, CuYd, Ton, Lb, Acre, Hour, MGal, Ac-Ft.
Map ea→Each, lf→Ft when the schedule does not specify otherwise. Preserve decimals (e.g. 4.2 Acre, 7.3 Ton).

        TRAFFIC CONTROL:
IF a Bid Items / Estimate Of Quantities table has a Traffic Control section:
- Copy EVERY row under that heading (BID ITEM number, description, unit, EST. QTY). Do not collapse the section to one line.
- "Traffic Control" with unit SqFt uses the schedule EST. QTY as-is. Never recompute it from MUTCD 30×30 or plan symbol counts.
- Keep companion pay items as separate rows when listed: Traffic Control Miscellaneous (LS), barricades (Each),
  temporary business signs (Each), portable changeable message signs (Each), temporary mailbox (Each),
  temporary gravel access (LS), winter maintenance (LS), and any other printed Traffic Control bid rows.
- Copy BID ITEM / Standard Bid Item Number (e.g. 634.0110, 9.0010, Special).
- Do NOT add extra bid items from F-sheet device tables (“Project Totals”), graphic channelizer counts, or individual MUTCD signs.
  Those faces are already inside the schedule Traffic Control SqFt quantity.
ELSE (no bid schedule for signing):
- Do NOT list individual STOP/YIELD/Speed Limit/MUTCD signs as separate Each pay items.
- Roll those sign faces into ONE pay item: description "Traffic Control", unit SqFt (width×height in inches ÷ 144).
- Keep barricades, drums, PCMS, temporary business signs, and true TTC LS items as their own pay items when evidenced.

        EVIDENCE:
Every output item should cite a schedule/pay-item row in source_reference / calculation_method
(e.g. "EOQ schedule p.3 row"). Omit invents without schedule evidence.
"""

        DESIGN_TAKEOFF_RULES = """
NO BID SCHEDULE — CIVIL ESTIMATOR MODE (design / drawings only):
This PDF/drawing is a design, not an agency bid schedule. You MUST generate a complete Estimate of Quantities
from the design evidence (typical sections, dimensions, callouts, hatch areas, counts, tables of quantities
printed on the drawing). Cover whichever of these the design actually shows:

        ROADS / HIGHWAYS:
- Pavement layers from typical section: width × thickness × length / 27 = CY (HMA also tons @ 145 pcf).
- Prime/tack coat SY = width × length / 9. Curb/gutter LF, sidewalk SF, shoulders, markings.
- Earthwork cut/fill CY when cross-sections or mass-haul notes exist. Do not invent corridor volumes without numbers.

        UTILITIES:
- Pipe LF by size and network (water / sanitary / storm). Count valves, hydrants, manholes, and inlets as EA when they are proposed pay items.
- Do NOT take off trench excavation, bedding, backfill, tracer wire, polywrap, thrust blocks, testing, or fittings as separate quantities unless they appear as their own Bid Items / EOQ / table-of-quantities row. Those are incidental to the pipe.
- Do NOT add incidental fitting counts, bedding volumes, or trench CY into the pipe LF (or any parent bid item).

        DAMS & RESERVOIRS:
- Embankment fill, foundation excavation, cutoff, filter/drain, riprap, spillway/stilling-basin concrete.
- Reservoir / pond lining SF, capacity MGAL or Ac-Ft when stated. Do not fake 3D dam volumes from a 2D outline alone.

        BUILDINGS / HOUSES:
- Brickwork/blockwork CY, RCC/foundation CY, plaster/flooring/roofing/formwork SF, doors/windows EA.
- Floor/roof areas from stated dimensions (L×W) when no schedule exists.

        RULES:
- Use numbers printed on the design (station range, width, thickness, counts). Show the formula in calculation_method.
- If a value is assumed (cover, trench width, HMA density), say so and set confidence 70-80.
- Do NOT invent items the design does not support. Do NOT omit a pay item that the typical section or schedule-like table shows.
- Do NOT output incidental/included work as extra pay items, and do not add those amounts into a parent quantity.
- Keep Traffic Control sign rollup (one SqFt item) as in the shared rules below.

        """ + TAKEOFF_ACCURACY_RULES.split("REQUIRED SEARCH PASSES:")[-1].split("EVIDENCE:")[0] + """
EVIDENCE:
Cite the typical section, dimension, sheet, or callout in source_reference / calculation_method.
"""

        _SCHEDULE_METHOD_HINTS = (
    "quantity sheet",
    "estimate of quantities",
    "bid items",
    "bid item",
    "std bid",
    "standard bid",
    "eoq",
    "pay item",
    "proposal quantity",
    "est. qty",
    "est qty",
    "approx. quantity",
    "approx quantity",
    "for bidding purposes",
    "extracted table total",
    "extracted from quantity table",
    "extracted from eoq",
    "extracted from tabular quantity",
    "eoq schedule",
)
_PLAN_INVENT_HINTS = (
    "graphic count",
    "channelizer symbol",
    "from drawing symbol",
    "from symbol",
    "typical section",
    "assumed ",
    "cover assumed",
    "trench width",
    "estimator takeoff",
    "estimator allowance",
    "civil estimator",
    "visible labeled",
    "unique labeled",
    "one table row counted",
    "counted as one proposed",
    "measured from",
    "inferred from station",
    "mutcd",
    "consolidated",
    "project total",
    "itemized table",
    "f-sheet",
    "sheets f",
    "incidental",
    "no separate pay",
    "no separate measurement",
    "subsidiary to",
)
_DERIVED_METHOD_HINTS = (
    "geometry",
    "hatch",
    "typical section",
    "inferred",
    "assumed",
    "from drawing symbol",
    "from symbol",
    "plan label",
    "callout",
)
# Plan-derived water / utility components that dominate false extras (Test 2).
_GENERIC_INFERRED_DESC = re.compile(
    r"(?:"
    r"(?:\d+[-\s]?inch\s+)?(?:gravity\s+)?(?:sanitary\s+)?sewer\s+main|"
    r"(?:\d+[-\s]?inch\s+)?(?:c900|dr\s*\d+\s+)?(?:pvc\s+)?water\s+main|"
    r"(?:\d+[-\s]?inch\s+)?restrained\s+joint|"
    r"(?:\d+[-\s]?inch\s+)?(?:steel\s+)?casing\s+pipe|"
    r"(?:\d+[-\s]?inch\s+)?(?:rj\s+)?(?:pvc\s+)?carrier\s+pipe|"
    r"sanitary\s+sewer\s+manhole|"
    r"(?:\d+[-\s]?inch\s+)?fire\s+hydrant|"
    r"(?:\d+[-\s]?inch\s+)?(?:salvaged\s+)?(?:gate\s+)?valve|"
    r"water\s+(?:valve|tee|bend|elbow)|"
    r"(?:\d+[-\s]?inch\s+)?(?:x\s*\d+[-\s]?inch\s+)?mj\s+(?:reducer|elbow|tee|bend)|"
    r"(?:\d+[-\s]?inch\s+)?(?:long\s+)?sleeve|"
    r"(?:\d+[-\s]?inch\s+)?plug\b|"
    r"utility\s+locate|"
    r"aggregate\s+base|"
    r"geotextile|"
    r"asphalt\s+concrete\s+pavement|"
    r"concrete\s+curb\s+and\s+gutter"
    r")",
    re.I,
)

        _CONTRACT_UNIT_MAP = {
    "ea": "Each",
    "each": "Each",
    "nos": "Each",
    "no": "Each",
    "no.": "Each",
    "lf": "Ft",
    "l.f.": "Ft",
    "l.f": "Ft",
    "lin ft": "Ft",
    "linear ft": "Ft",
    "linear feet": "Ft",
    "ft": "Ft",
    "feet": "Ft",
    "sy": "SqYd",
    "sqyd": "SqYd",
    "sq yd": "SqYd",
    "sq.yd": "SqYd",
    "sf": "SqFt",
    "sqft": "SqFt",
    "sq ft": "SqFt",
    "cy": "CuYd",
    "cuyd": "CuYd",
    "cu yd": "CuYd",
    "ls": "LS",
    "lump sum": "LS",
    "ton": "Ton",
    "tons": "Ton",
    "lb": "Lb",
    "lbs": "Lb",
    "acre": "Acre",
    "acres": "Acre",
    "hour": "Hour",
    "hr": "Hour",
    "mgal": "MGal",
    "ac-ft": "Ac-Ft",
    "acre-ft": "Ac-Ft",
    "acre ft": "Ac-Ft",
}

        
        def analyze_content(
    *,
    filename: str,
    content: ExtractedContent,
    document_id: int,
    bid_catalog: list[dict[str, Any]] | None = None,
    file_path: Path | str | None = None,
) -> dict[str, Any]:
    from app.config import get_settings
    from app.services.openai_client import openai_configured

            settings = get_settings()
    path = Path(file_path) if file_path else None

            if openai_configured():
        text_result: dict[str, Any] | None = None
        vision_result: dict[str, Any] | None = None
        errors: list[str] = []

                try:
            text_result = _analyze_with_openai(
                filename=filename,
                content=content,
                document_id=document_id,
                bid_catalog=bid_catalog,
            )
        except Exception as exc:
            errors.append(f"text AI: {exc}")

                # Deterministic label pass — skip when EOQ/schedule text is present so plan
        # fittings/valves do not flood extras (Training Test 2).
        label_result = None
        if not _content_has_eoq_schedule(content):
            label_result = _analyze_utility_labels(
                filename=filename,
                content=content,
                document_id=document_id,
            )
        else:
            # Still allow sized water-main LF labels only (no fittings).
            label_result = _analyze_utility_labels(
                filename=filename,
                content=content,
                document_id=document_id,
                mains_only=True,
            )

                if (
            settings.openai_pdf_vision_enabled
            and path
            and path.exists()
            and path.suffix.lower() == ".pdf"
        ):
            try:
                vision_result = _analyze_pdf_drawings_with_vision(
                    filename=filename,
                    content=content,
                    document_id=document_id,
                    pdf_path=path,
                    bid_catalog=bid_catalog,
                    max_pages=settings.openai_vision_max_pages,
                    dpi=settings.openai_vision_dpi,
                    min_score=settings.openai_vision_min_score,
                    force_utility_pages=settings.openai_vision_force_utility_pages,
                    scan_all_pages=settings.openai_vision_scan_all_pages,
                    batch_pages=settings.openai_vision_batch_pages,
                )
            except Exception as exc:
                errors.append(f"drawing vision: {exc}")

                merged_parts = [r for r in (text_result, label_result, vision_result) if r]
        if len(merged_parts) >= 2:
            merged = merged_parts[0]
            for part in merged_parts[1:]:
                merged = _merge_analysis_results(merged, part)
            if errors:
                merged["notes"] = (merged.get("notes") or "") + " | ".join(errors)
            return _finalize_analysis(merged, content=content)
        if vision_result:
            return _finalize_analysis(vision_result, content=content)
        if label_result and label_result.get("items"):
            return _finalize_analysis(label_result, content=content)
        if text_result and text_result.get("items"):
            return _finalize_analysis(text_result, content=content)
        if text_result:
            return _finalize_analysis(text_result, content=content)

                heuristic = _analyze_heuristic(filename=filename, content=content, document_id=document_id)
        if errors:
            heuristic["notes"] = f"OpenAI failed ({'; '.join(errors)}); used heuristic fallback."
        return _finalize_analysis(heuristic, content=content)

            return _finalize_analysis(
        _analyze_heuristic(filename=filename, content=content, document_id=document_id),
        content=content,
    )

        
        def _analyze_heuristic(*, filename: str, content: ExtractedContent, document_id: int) -> dict[str, Any]:
    items: list[dict[str, Any]] = []
    seen: set[str] = set()

            # 1) Structured tables (CSV/Excel/PDF). Bid Items / EOQ tables are copied as-is;
    # F-sheet “Project Totals” / device tables are not pay items.
    for it in _items_from_document_tables(content, filename=filename, document_id=document_id):
        key = str(it.get("description") or "").lower()
        if key and key not in seen:
            seen.add(key)
            items.append(it)

            schedule_present = _content_has_eoq_schedule(content)

            # 2) Regex over free text — skip when a Bid Items / EOQ table is the pay-item source
    text = content.text or ""
    pattern_rows = () if schedule_present else CIVIL_PATTERNS
    for desc, category, unit_hint, pattern in pattern_rows:
        if desc.lower() in seen:
            continue
        chosen = None
        for match in re.finditer(pattern, text, flags=re.IGNORECASE | re.DOTALL):
            qty = _parse_number(match.group(1))
            if qty is None:
                continue
            if extraction_should_skip(desc, text, match.start()):
                continue
            chosen = match
            break
        if chosen is None:
            continue
        qty = _parse_number(chosen.group(1))
        if qty is None:
            continue
        unit = (chosen.group(2) if chosen.lastindex and chosen.lastindex >= 2 else unit_hint) or unit_hint
        unit = _normalize_unit(unit)
        page = _guess_page(text, chosen.start(), content)
        seen.add(desc.lower())
        items.append(
            _item(
                description=desc,
                category=category,
                unit=unit,
                quantity=qty,
                document_id=document_id,
                page=page,
                source=f"{filename}" + (f" - Page {page}" if page else ""),
                method="Pattern match from document text",
                confidence=78,
            )
        )

            # Water main / utility labels & callouts (often the only size/LF source on plans)
    label_pack = _analyze_utility_labels(
        filename=filename,
        content=content,
        document_id=document_id,
        mains_only=_content_has_eoq_schedule(content),
    )
    for it in (label_pack.get("items") or []):
        key = str(it.get("description") or "").lower()
        if key and key not in seen:
            seen.add(key)
            items.append(it)

            # Geometry notes
    facts: list[dict[str, Any]] = []
    from app.services.civil_estimator import detect_project_types, items_from_design_text

            project_types = detect_project_types(filename, text)
    if project_types:
        facts.append({"key": "project_types", "value": ", ".join(project_types)})

            if not _content_has_eoq_schedule(content):
        for it in items_from_design_text(text, filename=filename):
            key = str(it.get("description") or "").lower()
            if not key or key in seen:
                continue
            seen.add(key)
            qty = _parse_number(it.get("quantity"))
            if qty is None:
                continue
            items.append(
                _item(
                    description=str(it.get("description")),
                    category=str(it.get("category") or "General"),
                    unit=str(it.get("unit") or "unit"),
                    quantity=qty,
                    document_id=document_id,
                    page=None,
                    source=f"{filename} - design takeoff",
                    method=str(it.get("calculation_method") or "Civil estimator from design text"),
                    confidence=float(it.get("confidence") or 80),
                )
            )

            width_match = re.search(r"\b(?:Road|Carriageway)\s*Width\b[^\d]{0,20}(\d{1,2}(?:\.\d+)?)\s*(m|ft)\b", text, re.I)
    if width_match:
        facts.append({"key": "road_width_m", "value": width_match.group(1), "source_page": _guess_page(text, width_match.start(), content)})

            chainage_match = re.search(r"\b(?:Chainage|Ch\.?)\s*[:\-]?\s*([\d\+\.]+(?:\s*(?:to|\-)\s*[\d\+\.]+)?)", text, re.I)
    if chainage_match:
        facts.append({"key": "chainage", "value": chainage_match.group(1), "source_page": _guess_page(text, chainage_match.start(), content)})

            summary = (
        f"Analyzed '{filename}' with heuristic civil extractor. "
        f"Found {len(items)} quantity item(s)"
        + (f" and {len(facts)} geometry/note fact(s)." if facts else ".")
    )
    if not items:
        summary += " No explicit quantities found — flagging for engineer review."

            return {
        "engine": "heuristic",
        "summary": summary,
        "facts": facts,
        "items": items,
        "needs_review": len(items) == 0,
    }

        
        def _catalog_prompt_bits(
    bid_catalog: list[dict[str, Any]] | None,
    *,
    design_takeoff: bool = False,
) -> tuple[str, str, str]:
    catalog = bid_catalog or []
    catalog_preview = json.dumps(catalog[:100], ensure_ascii=True)[:12000] if catalog else "[]"
    takeoff_rules = DESIGN_TAKEOFF_RULES if design_takeoff else TAKEOFF_ACCURACY_RULES
    if catalog:
        system = (
            "You are a USA civil/highway quantity surveyor AI for AutoVAD. "
            "An agency bid template is active. Extract ONLY items needed for THIS project "
            "with evidence in schedules, quantity tables, drawings, or explicit callouts. "
            "Prefer bid/EOQ schedule quantities over geometry. Return STRICT JSON only."
        )
        catalog_rules = f"""
ACTIVE BID TEMPLATE (Standard Bid Item Number / description / unit):
{catalog_preview}

        Template rules:
- Do NOT dump the entire bid list.
- Only include bid lines evidenced in schedules/tables/plans/callouts.
- When matched, use the template description EXACTLY, its unit, and Standard Bid Item Number as item_code.
- Unmatched but evidenced schedule/callout work may still be included with empty item_code.
- Never invent extras that are not on the template or clearly called out.
{takeoff_rules}
"""
        code_hint = "Use Standard Bid Item Numbers from the active template when matched."
    else:
        role = (
            "civil estimator AI for AutoVAD. Produce a complete Estimate of Quantities from the design "
            "(roads, utilities, dams, reservoirs, buildings) when no bid schedule exists."
            if design_takeoff
            else "USA civil/highway quantity surveyor AI for AutoVAD. Extract Estimate Of Quantities / EOQ pay items. "
            "Prefer quantity schedules and explicit callouts over inferred geometry."
        )
        system = f"You are a {role} Return STRICT JSON only."
        catalog_rules = f"""
No agency bid template — use exact schedule/pay-item wording when an EOQ table is present;
otherwise use clear USA civil/CSI descriptions
(earthwork, pavement, curb/gutter, sidewalk, drainage, utilities, dams, reservoirs, buildings, removals).
{takeoff_rules}
"""
        code_hint = "Prefer STD BID NO / USA CSI codes when identifiable."
    return system, catalog_rules, code_hint

        
        def _items_from_openai_payload(
    data: dict[str, Any],
    *,
    filename: str,
    document_id: int,
    default_method: str,
) -> list[dict[str, Any]]:
    items: list[dict[str, Any]] = []
    for raw_item in data.get("items") or []:
        qty = _parse_number(raw_item.get("quantity"))
        if qty is None or not raw_item.get("description"):
            continue
        conf = _parse_number(raw_item.get("confidence")) or Decimal("80")
        built = _item(
                description=str(raw_item.get("description")).strip(),
                category=str(raw_item.get("category") or "General"),
                unit=str(raw_item.get("unit") or "unit"),
                quantity=qty,
                item_code=(str(raw_item["item_code"]) if raw_item.get("item_code") else None),
                document_id=document_id,
                page=raw_item.get("source_page"),
                source=f"{filename}"
                + (f" - Page {raw_item.get('source_page')}" if raw_item.get("source_page") else "")
                + (f" - {raw_item.get('source_reference')}" if raw_item.get("source_reference") else ""),
                method=str(raw_item.get("calculation_method") or default_method),
                confidence=float(conf),
                status=str(raw_item.get("status") or "needs_review"),
                source_reference=raw_item.get("source_reference"),
            )
        if _should_drop_incidental_item(built):
            continue
        items.append(built)
    return items

        
        def _analyze_with_openai(
    *,
    filename: str,
    content: ExtractedContent,
    document_id: int,
    bid_catalog: list[dict[str, Any]] | None = None,
) -> dict[str, Any]:
    from app.services.openai_client import ask_openai_json

            clipped = (content.text or "")[:50000]
    tables_preview = json.dumps(content.tables[:12], ensure_ascii=True)[:18000]
    design_takeoff = not _content_has_eoq_schedule(content)
    system, catalog_rules, code_hint = _catalog_prompt_bits(bid_catalog, design_takeoff=design_takeoff)

            design_or_schedule = (
        "No EOQ/bid schedule was detected — act as a civil estimator and generate pay items from "
        "typical sections, dimensions, and callouts (roads, utilities, dams, reservoirs, buildings)."
        if design_takeoff
        else "Harvest EVERY row from EOQ / Bid Items / quantity tables (ITEM NUMBER, BID ITEM, DESCRIPTION, UNITS, EST. QTY or ITEM NO, STD BID NO, APPROX. QUANTITY). For a Traffic Control heading, copy every row in that section, including LS and Each companions, and the SqFt EST. QTY as printed."
    )

            user = f"""
Extract Estimate Of Quantities / EOQ pay items from the document TEXT and TABLES.
(Drawing sheets are analyzed separately via vision.)

        Rules:
- {design_or_schedule}
- Merge continuation tables: “(Ctd.)”, “continued”, repeated headers → same category; do not stop at page breaks.
- Include Alternates (A/B), LS items (Mobilization, Tax…), Traffic Control/Signals/Lighting, Removals, Erosion/Landscaping.
- Copy the schedule quantity cell — do not substitute a detail/callout count — WHEN a schedule exists.
- Do NOT invent water fittings/valves/hydrants/pipe segments from free text unless they are schedule rows OR this is design-only takeoff with printed sizes/counts.
- Skip incidental/included work. Do not list it separately and do not add it into the parent bid-item quantity.
- If unsure, omit inventing values and mark needs_review.
{catalog_rules}

        Document filename: {filename}
Document id: {document_id}

        TEXT:
{clipped}

        TABLES_JSON:
{tables_preview}

        Return JSON shape:
{{
  "summary": "string",
  "facts": [{{"key":"eoq_table_found","value":"true","source_page":1}}],
  "items": [
    {{
      "item_code": "optional Standard Bid Item Number or CSI",
      "description": "exact schedule description",
      "category": "Water Main",
      "unit": "Ft",
      "quantity": 245,
      "source_page": 4,
      "source_reference": "EOQ schedule table",
      "calculation_method": "Extracted from Estimate Of Quantities schedule",
      "confidence": 95,
      "status": "needs_review"
    }}
  ],
  "needs_review": false
}}
"""
    data = ask_openai_json(system + " " + code_hint, user)
    items = _items_from_openai_payload(
        data, filename=filename, document_id=document_id, default_method="OpenAI text/table extraction"
    )
    return {
        "engine": "openai",
        "summary": data.get("summary") or f"OpenAI text-analyzed '{filename}'.",
        "facts": data.get("facts") or [],
        "items": items,
        "needs_review": bool(data.get("needs_review")) or len(items) == 0,
    }

        
        def _analyze_utility_labels(
    *,
    filename: str,
    content: ExtractedContent,
    document_id: int,
    mains_only: bool = False,
) -> dict[str, Any]:
    """Deterministic extraction of water main / utility quantities from label text."""
    from app.services.utility_labels import extract_utility_label_items

            raw_items = extract_utility_label_items(
        content.text or "",
        filename=filename,
        document_id=document_id,
        mains_only=mains_only,
    )
    items: list[dict[str, Any]] = []
    for raw in raw_items:
        qty = _parse_number(raw.get("quantity"))
        if qty is None or not raw.get("description"):
            continue
        items.append(
            _item(
                description=str(raw["description"]),
                category=str(raw.get("category") or "Utilities"),
                unit=str(raw.get("unit") or "LF"),
                quantity=qty,
                document_id=document_id,
                page=raw.get("source_page"),
                source=f"{filename} - {raw.get('source_reference') or 'label'}",
                method=str(raw.get("calculation_method") or "Plan label/callout"),
                confidence=float(raw.get("confidence") or 85),
                status=str(raw.get("status") or "needs_review"),
                source_reference=raw.get("source_reference"),
            )
        )
    return {
        "engine": "label-parser",
        "summary": (
            f"Label/callout parser found {len(items)} utility item(s) "
            f"(water main / fittings / related) in '{filename}'."
        ),
        "facts": [],
        "items": items,
        "needs_review": True,
    }

        
        def _analyze_pdf_drawings_with_vision(
    *,
    filename: str,
    document_id: int,
    pdf_path: Path,
    bid_catalog: list[dict[str, Any]] | None = None,
    max_pages: int = 0,
    dpi: int = 150,
    min_score: float = 18.0,
    force_utility_pages: bool = True,
    scan_all_pages: bool = True,
    batch_pages: int = 8,
    content: ExtractedContent | None = None,
) -> dict[str, Any]:
    from app.services.openai_client import ask_openai_vision_json
    from app.services.pdf_vision import iter_rendered_pdf_batches, plan_pdf_vision_pages

            plan = plan_pdf_vision_pages(
        pdf_path,
        max_pages=max_pages,
        min_score=min_score,
        force_utility_pages=force_utility_pages,
        scan_all_pages=scan_all_pages,
        batch_pages=batch_pages,
    )
    if not plan.selected_pages:
        raise RuntimeError("No PDF pages could be rendered for vision")

            design_takeoff = not _content_has_eoq_schedule(content)
    system, catalog_rules, code_hint = _catalog_prompt_bits(
        bid_catalog, design_takeoff=design_takeoff
    )

            # Hint model with OCR snippets that look like water-main labels
    label_hints = ""
    if content and content.text:
        snippets = []
        for line in (content.text or "").splitlines():
            low = line.lower()
            if any(k in low for k in ("water main", "watermain", " wm", "wm ", 'water"', "dip wm", "prop. wm", "proposed wm")):
                snippets.append(line.strip()[:160])
            if len(snippets) >= 40:
                break
        if snippets:
            label_hints = "OCR/label snippets that may appear on sheets:\n" + "\n".join(f"- {s}" for s in snippets)

            vision_system = (
        system
        + " "
        + code_hint
        + " Prefer EOQ/bid schedules on sheets when present. Merge (Ctd.) pages. "
        + (
            "No schedule: generate civil-estimator EOQ from typical sections, dimensions, and counts. "
            "Do not take off incidental trench/bedding/fittings or add them into pipe quantities."
            if design_takeoff
            else "Do not invent fittings/valves from plan details when a schedule exists. "
            "Do not take off incidental work or add it into a parent bid-item quantity."
        )
    )
    all_items: list[dict[str, Any]] = []
    all_facts: list[Any] = []
    summaries: list[str] = []
    vision_pages_meta: list[dict[str, Any]] = []
    batch_errors: list[str] = []
    batch_index = 0

            for batch in iter_rendered_pdf_batches(pdf_path, plan, dpi=dpi, batch_pages=batch_pages):
        batch_index += 1
        page_meta = ", ".join(f"p{p.page} ({p.reason})" for p in batch)
        images = [{"page": p.page, "png_b64": p.png_b64} for p in batch]
        vision_pages_meta.extend({"page": p.page, "reason": p.reason} for p in batch)
        coverage_note = (
            f"This is batch {batch_index} of the PDF. "
            f"Document has {plan.page_count} page(s); this request covers pages "
            f"{[p.page for p in batch]}. Extract ALL bid/takeoff items visible on THESE sheets only."
        )
        sheet_job = (
            """
Primary job:
- If an Estimate Of Quantities / bid quantity table appears, extract EVERY row (LEFT then RIGHT).
- Continuation “(Ctd.)” keeps the parent category. Include Alternates, LS, Traffic Control, Removals, landscaping.
- Copy APPROX. QUANTITY from the schedule cell. Do not count symbols when the schedule shows a project total.
- On sheets WITHOUT an EOQ table: take off pay items as a civil estimator from typical sections, dimensions,
  hatch areas, and printed counts (roads, utilities, dams/reservoirs, buildings). Show formulas in calculation_method.
  Assumed HMA 145 pcf — write assumptions down. Do not invent trench/bedding/backfill or fittings unless they
  are printed as pay items. Never add incidental work into a parent quantity.
"""
            if design_takeoff
            else """
Primary job:
- If an Estimate Of Quantities / Bid Items table appears (ITEM NUMBER, BID ITEM, DESCRIPTION, UNITS, EST. QTY
  or ITEM NO / STD BID NO / APPROX. QUANTITY), extract EVERY row from LEFT then RIGHT. That schedule is the
  ONLY source for pay items on that sheet.
- Traffic Control section: copy EVERY row under the heading — not only the SqFt signing line. Typical companions:
  Traffic Control (SqFt = EST. QTY), Traffic Control Miscellaneous (LS), barricades (Each), temporary business
  signs, portable changeable message signs, temporary mailbox, temporary gravel access (LS), winter maintenance (LS).
  Copy the BID ITEM number (e.g. 634.0110 or Special). Do not recompute Traffic Control SqFt from MUTCD 30×30.
  Do not add channelizers or MUTCD signs from F-sheet graphics or device “Project Totals” tables.
- Continuation: if the header says “(Ctd.)” / Continued, keep extracting rows into the parent category.
- Include Alternates, LS/general, Traffic Control/Signals/Lighting, Removals, Erosion/Landscaping.
- Copy the EST. QTY / APPROX. QUANTITY from the schedule cell. Do not count symbols when the schedule shows a total.
- On pure plan/profile/detail sheets WITHOUT an EOQ/Bid Items table: do not invent water fittings, valves,
  hydrants, casing/carrier segments, pavement layers, or extra traffic devices from symbols.
- Incidental notes (incidental to / included in / no separate payment): omit those items entirely. Do not add
  their counts or volumes into the parent bid item.
"""
        )
        user = f"""
You are looking at RENDERED ENGINEERING PLAN SHEETS from a civil PDF (not just OCR text).

        Document: {filename}
Rendered sheets: {page_meta}
{coverage_note}
{sheet_job}
{catalog_rules}

        {label_hints}

        Return JSON:
{{
  "summary": "EOQ tables / continuation / alternates found on these sheets",
  "facts": [{{"key":"eoq_table_found","value":"true","source_page":1}}],
  "items": [
    {{
      "item_code": "optional STD BID NO",
      "description": "exact schedule description",
      "category": "Water Main",
      "unit": "Ft",
      "quantity": 245,
      "source_page": 4,
      "source_reference": "EOQ schedule table",
      "calculation_method": "Extracted from Estimate Of Quantities schedule",
      "confidence": 95,
      "status": "needs_review"
    }}
  ],
  "needs_review": true
}}
"""
        try:
            data = ask_openai_vision_json(vision_system, user, images)
        except Exception as exc:
            batch_errors.append(f"batch {batch_index} pages {[p.page for p in batch]}: {exc}")
            continue
        batch_items = _items_from_openai_payload(
            data,
            filename=filename,
            document_id=document_id,
            default_method="OpenAI vision — engineering drawing sheet",
        )
        all_items.extend(batch_items)
        all_facts.extend(data.get("facts") or [])
        if data.get("summary"):
            summaries.append(str(data["summary"]))

            # Merge duplicate keys across batches (same desc/unit)
    merged_pack = _merge_analysis_results(
        {"items": [], "facts": [], "summary": "", "needs_review": False},
        {
            "items": all_items,
            "facts": all_facts,
            "summary": " ".join(summaries).strip(),
            "needs_review": len(all_items) == 0,
            "vision_pages": vision_pages_meta,
        },
    )
    items = merged_pack.get("items") or all_items
    scanned = len(plan.selected_pages)
    summary = (
        f"Vision-analyzed {scanned}/{plan.page_count} page(s) from '{filename}' "
        f"in {batch_index} batch(es)."
    )
    if summaries:
        summary = f"{summary} {' '.join(summaries[:3])}"
    if plan.truncated:
        summary += (
            f" Safety cap skipped {len(plan.skipped_pages)} page(s) "
            f"(set OPENAI_VISION_MAX_PAGES=0 and OPENAI_VISION_SCAN_ALL_PAGES=true for full scan)."
        )
    if batch_errors:
        summary += " Batch errors: " + " | ".join(batch_errors[:5])

            return {
        "engine": "openai+vision",
        "summary": summary,
        "facts": merged_pack.get("facts") or all_facts,
        "items": items,
        "needs_review": bool(merged_pack.get("needs_review"))
        or len(items) == 0
        or plan.truncated
        or bool(batch_errors),
        "vision_pages": vision_pages_meta,
        "notes": (" | ".join(batch_errors) if batch_errors else None),
        "vision_coverage": {
            "page_count": plan.page_count,
            "selected_pages": plan.selected_pages,
            "skipped_pages": plan.skipped_pages,
            "truncated": plan.truncated,
            "scan_all": plan.scan_all,
            "batch_pages": batch_pages,
            "batches": batch_index,
            "forced_utility_pages": plan.forced_utility_pages,
        },
    }

        
        def _normalize_contract_unit(unit: str | None) -> str:
    raw = str(unit or "UNIT").strip()
    if not raw:
        return "UNIT"
    key = re.sub(r"\s+", " ", raw.lower().replace("³", "3"))
    return _CONTRACT_UNIT_MAP.get(key, raw)

        
        def _item_evidence_blob(item: dict[str, Any]) -> str:
    return (
        f"{item.get('calculation_method') or ''} "
        f"{item.get('source_reference') or ''} "
        f"{item.get('source') or ''}"
    ).lower()

        
        def _is_schedule_pay_item(item: dict[str, Any]) -> bool:
    """True when the row came from a Bid Items / EOQ / quantity-schedule table."""
    from app.services.traffic_control import is_plan_device_takeoff, looks_like_agency_bid_number

            if is_plan_device_takeoff(item):
        return False
    if looks_like_agency_bid_number(item.get("item_code")):
        return True
    blob = _item_evidence_blob(item)
    return any(h in blob for h in _SCHEDULE_METHOD_HINTS)

        
        def _should_drop_incidental_item(item: dict[str, Any]) -> bool:
    """Incidental children are omitted; their quantities are never added to a parent."""
    return should_drop_incidental_item(
        item,
        scheduled=_is_schedule_pay_item(item),
        drop_default_extras=True,
    )

        
        def _is_plan_invent_extra(item: dict[str, Any]) -> bool:
    """F-sheet devices, symbol counts, MUTCD rollups, assumed trench, typical sections."""
    from app.services.traffic_control import is_plan_device_takeoff, looks_like_agency_bid_number

            if looks_like_agency_bid_number(item.get("item_code")):
        return False
    if is_plan_device_takeoff(item):
        return True
    blob = _item_evidence_blob(item)
    if any(h in blob for h in _PLAN_INVENT_HINTS):
        # Vision may still cite a bid table in the same blob — that is a schedule row.
        if any(h in blob for h in _SCHEDULE_METHOD_HINTS):
            return False
        return True
    entity = str(item.get("entity_type") or "").upper()
    if entity == "ESTIMATOR":
        return True
    if re.search(r"\bsheet f\d", blob) and any(
        h in blob for h in ("graphic", "symbol", "project total", "itemized", "measured", "approximate")
    ):
        return True
    return False

        
        def _method_rank(item: dict[str, Any]) -> int:
    """Higher = more trustworthy evidence for EOQ accuracy."""
    if _is_plan_invent_extra(item):
        return 0
    if _is_schedule_pay_item(item):
        return 3
    method = _item_evidence_blob(item)
    desc = str(item.get("description") or "")
    if _GENERIC_INFERRED_DESC.search(desc) and any(
        h in method for h in ("label", "callout", "drawing", "symbol", "geometry")
    ):
        return 0
    if any(h in method for h in ("label", "callout")):
        return 1
    if any(h in method for h in _DERIVED_METHOD_HINTS):
        return 0
    return 1

        
        def _looks_like_schedule_item(item: dict[str, Any]) -> bool:
    return _is_schedule_pay_item(item)

        
        def _is_plan_derived(item: dict[str, Any]) -> bool:
    if _is_schedule_pay_item(item):
        return False
    if _is_plan_invent_extra(item):
        return True
    method = _item_evidence_blob(item)
    return any(
        h in method
        for h in (
            "plan label",
            "callout",
            "drawing label",
            "geometry",
            "from symbol",
            "graphic count",
            "project total",
            "itemized table",
            "typical section",
            "inferred",
        )
    )

        
        def _table_header_text(table: dict[str, Any]) -> str:
    rows = table.get("rows") or []
    if not rows:
        return ""
    return " ".join(str(c or "") for c in rows[0]).lower()

        
        def _is_plan_device_table(table: dict[str, Any]) -> bool:
    header = _table_header_text(table)
    return any(k in header for k in ("project total", "project totals", "itemized"))

        
        def _is_bid_schedule_table(table: dict[str, Any]) -> bool:
    """True for Bid Items / EOQ / quantity-schedule tables — not F-sheet device tables."""
    if _is_plan_device_table(table):
        return False
    header = _table_header_text(table)
    if any(
        k in header
        for k in (
            "std bid",
            "standard bid",
            "est. qty",
            "est qty",
            "approx. quantity",
            "approx quantity",
            "bid item",
            "item description",
            "for bidding",
            "proposal quantity",
        )
    ):
        return True
    if (
        ("description" in header or "particular" in header)
        and ("quantity" in header or "qty" in header)
        and ("unit" in header)
        and not any(k in header for k in ("tjb", "fjb", "fitting", "locator", "channelizer"))
    ):
        return True
    return False

        
        _QTY_COL_NAMES = (
    "approx. quantity",
    "approx quantity",
    "est. qty",
    "est qty",
    "quantity",
    "quantities",
    "qty",
    "qnty",
)
_DESC_COL_NAMES = (
    "item description",
    "description",
    "particular",
    "material",
    "desc",
    "item",
)
_UNIT_COL_NAMES = ("units", "unit", "uom")
_CODE_COL_NAMES = (
    "std bid no",
    "standard bid",
    "std bid",
    "bid item",
    "item code",
    "item_code",
    "bid no",
    "code",
)
_SKIP_TABLE_DESC_RE = re.compile(
    r"^(project\s+)?totals?$|"
    r"^(item(\s*(no|number|#))?|bid item|description|item description|"
    r"units?|est\.?\s*qty|approx\.?\s*quantity|quantity|std bid.*)$",
    re.I,
)

        
        def _items_from_document_tables(
    content: ExtractedContent,
    *,
    filename: str,
    document_id: int,
) -> list[dict[str, Any]]:
    """Copy Bid Items / EOQ tables as pay items. Skip F-sheet Project Totals tables."""
    items: list[dict[str, Any]] = []
    seen: set[str] = set()
    tables = list(content.tables or [])
    schedule_tables = [t for t in tables if _is_bid_schedule_table(t)]
    if schedule_tables:
        chosen = schedule_tables
        schedule_copy = True
    else:
        chosen = [t for t in tables if not _is_plan_device_table(t)]
        schedule_copy = False

            for table in chosen:
        page = table.get("page")
        rows = table.get("rows") or []
        if not rows:
            continue
        header = [str(c or "").lower() for c in rows[0]]
        qty_idx = _find_col(header, list(_QTY_COL_NAMES))
        desc_idx = _find_col(header, list(_DESC_COL_NAMES))
        unit_idx = _find_col(header, list(_UNIT_COL_NAMES))
        code_idx = _find_col(header, list(_CODE_COL_NAMES))

                if qty_idx is None and len(header) >= 3:
            for i, h in enumerate(header):
                if "unit" in h:
                    unit_idx = i
                if any(k in h for k in ("item", "desc", "material")):
                    desc_idx = i
                if any(k in h for k in ("qty", "quantity")):
                    qty_idx = i
            if qty_idx is None and len(header) >= 3:
                desc_idx, unit_idx, qty_idx = 0, 1, 2

                method = (
            "Extracted from quantity table"
            if schedule_copy
            else "Extracted from tabular quantity sheet"
        )
        source = (
            f"{filename} - Bid Items / EOQ table" + (f" p.{page}" if page else "")
            if schedule_copy
            else f"{filename}" + (f" - Table p.{page}" if page else " - Table")
        )
        source_reference = "Bid Items / EOQ table" if schedule_copy else source

                data_rows = rows[1:] if desc_idx is not None else rows
        for row in data_rows:
            if not row or desc_idx is None or qty_idx is None:
                joined = " ".join(str(c or "") for c in row).strip()
                if not joined:
                    continue
                mapped = _map_alias(joined)
                qty = _parse_number(row[-1] if row else None)
                if mapped and qty is not None:
                    key = mapped[0].lower()
                    if key in seen:
                        continue
                    seen.add(key)
                    items.append(
                        _item(
                            description=mapped[0],
                            category=mapped[1],
                            unit=(
                                row[unit_idx]
                                if unit_idx is not None and unit_idx < len(row) and row[unit_idx]
                                else mapped[2]
                            ),
                            quantity=qty,
                            document_id=document_id,
                            page=page,
                            source=source,
                            method=method,
                            source_reference=source_reference,
                            confidence=92,
                        )
                    )
                continue

                    if desc_idx >= len(row) or qty_idx >= len(row):
                continue
            desc = str(row[desc_idx] or "").strip()
            if not desc or _SKIP_TABLE_DESC_RE.match(desc):
                continue
            row_joined = " ".join(str(c or "") for c in row)
            if description_is_incidental_child(desc) or (
                not schedule_copy and description_is_incidental_child(row_joined)
            ):
                continue
            qty = _parse_number(row[qty_idx])
            if qty is None:
                continue
            mapped = _map_alias(desc)
            description = mapped[0] if mapped else desc
            category = mapped[1] if mapped else "General"
            unit = (
                str(row[unit_idx]).strip()
                if unit_idx is not None and unit_idx < len(row) and str(row[unit_idx] or "").strip()
                else (mapped[2] if mapped else "unit")
            )
            code = (
                str(row[code_idx]).strip()
                if code_idx is not None and code_idx < len(row) and row[code_idx]
                else None
            )
            key = description.lower()
            if key in seen:
                continue
            seen.add(key)
            items.append(
                _item(
                    description=description,
                    category=category,
                    unit=unit,
                    quantity=qty,
                    item_code=code,
                    document_id=document_id,
                    page=page,
                    source=source,
                    method=method,
                    source_reference=source_reference,
                    confidence=94,
                )
            )
    return items

        
        def _content_has_eoq_schedule(content: ExtractedContent | None) -> bool:
    if not content:
        return False
    blob = (content.text or "").lower()
    if "estimate of quantities" in blob:
        return True
    if re.search(r"\bbid items\b", blob) and any(
        k in blob
        for k in (
            "est. qty",
            "est qty",
            "std bid",
            "item description",
            "approx. quantity",
            "approx quantity",
            "bid item",
        )
    ):
        return True
    if any(
        k in blob
        for k in (
            "item description",
            "approx. quantity",
            "approx quantity",
            "std bid",
            "standard bid",
            "for bidding purposes only",
            "est. qty",
            "est qty",
        )
    ) and any(k in blob for k in ("quantity", "unit", "item")):
        return True
    return any(_is_bid_schedule_table(t) for t in (content.tables or []) if t.get("rows"))

        
        def _should_drop_inferred_extra(
    item: dict[str, Any],
    *,
    schedule_present: bool,
) -> bool:
    """When a Bid Items / EOQ table exists, keep schedule rows and drop F-sheet invents."""
    if not schedule_present:
        return False
    if _looks_like_schedule_item(item):
        return False
    desc = str(item.get("description") or "").strip()
    if not desc:
        return True
    if _is_plan_invent_extra(item) or _is_plan_derived(item):
        return True
    if _GENERIC_INFERRED_DESC.search(desc) and _method_rank(item) <= 1:
        return True
    return False

        
        def _prefer_schedule_quantity(
    existing: dict[str, Any],
    candidate: dict[str, Any],
) -> dict[str, Any]:
    """When merging duplicates, keep schedule wording/qty over derived geometry.

            Never replace a clean parent quantity with one that folded in incidental work.
    """
    e_blob = _item_evidence_blob(existing)
    c_blob = _item_evidence_blob(candidate)
    e_inflated = quantity_inflated_by_incidentals(e_blob)
    c_inflated = quantity_inflated_by_incidentals(c_blob)
    if c_inflated and not e_inflated:
        return dict(existing)
    if e_inflated and not c_inflated:
        return dict(candidate)

            er = _method_rank(existing)
    cr = _method_rank(candidate)
    if cr > er:
        return dict(candidate)
    if er > cr:
        return dict(existing)
    try:
        old_c = float(existing.get("confidence") or 0)
        new_c = float(candidate.get("confidence") or 0)
        old_q = float(existing.get("quantity") or 0)
        new_q = float(candidate.get("quantity") or 0)
    except (TypeError, ValueError):
        return dict(existing)
    # Prefer schedule-coded item_code
    if candidate.get("item_code") and not existing.get("item_code"):
        return dict(candidate)
    if existing.get("item_code") and not candidate.get("item_code"):
        return dict(existing)
    if new_c > old_c + 2:
        if c_inflated:
            return dict(existing)
        return dict(candidate)
    # When both schedule-grade, prefer the larger qty (avoids detail undercounts)
    # unless the larger figure mixed in incidental work.
    if er >= 2 and cr >= 2 and new_q > old_q * 1.15 and not c_inflated:
        return dict(candidate)
    if abs(new_c - old_c) <= 2 and new_q > old_q and er == 0 and not c_inflated:
        return dict(candidate)
    if abs(new_c - old_c) <= 2 and er >= 2:
        return dict(existing)
    if abs(new_c - old_c) <= 2 and new_q > old_q and not c_inflated:
        return dict(candidate)
    return dict(existing)

        
        def _finalize_analysis(result: dict[str, Any], *, content: ExtractedContent | None = None) -> dict[str, Any]:
    """Copy Bid Items / EOQ tables as pay items; drop F-sheet and estimator invents."""
    items = [dict(i) for i in (result.get("items") or []) if i.get("description")]
    if content is not None:
        doc_id = 0
        for existing in items:
            try:
                doc_id = int(existing.get("source_document_id") or 0)
            except (TypeError, ValueError):
                doc_id = 0
            if doc_id:
                break
        copied = _items_from_document_tables(content, filename="plan", document_id=doc_id)
        if copied:
            items = copied + items

            if not items:
        return result

            for item in items:
        item["unit"] = _normalize_contract_unit(item.get("unit"))
        # Normalize continuation category labels
        cat = str(item.get("category") or "")
        if cat:
            item["category"] = re.sub(
                r"\s*\(ctd\.?\)\s*$",
                "",
                cat,
                flags=re.I,
            ).strip() or cat

            before_incidental = len(items)
    items = [item for item in items if not _should_drop_incidental_item(item)]
    incidental_dropped = before_incidental - len(items)
    if not items:
        out = dict(result)
        out["items"] = []
        note = "Dropped incidental-to-bid-item work (not separately paid)."
        out["notes"] = ((out.get("notes") or "") + " | " + note).strip(" |")
        return out

            schedule_present = _content_has_eoq_schedule(content) or any(
        _looks_like_schedule_item(i) for i in items
    )

            cleaned: list[dict[str, Any]] = []
    dropped = 0
    for item in items:
        if _should_drop_inferred_extra(item, schedule_present=schedule_present):
            dropped += 1
            continue
        cleaned.append(item)

            before_combine = len(cleaned)
    cleaned = combine_similar_pay_items(cleaned)
    combined_groups = before_combine - len(cleaned)

            # Roll individual traffic signs → one SqFt "Traffic Control" item
    from app.services.traffic_control import consolidate_traffic_control_signs

            cleaned, tc_meta = consolidate_traffic_control_signs(
        cleaned,
        allow_online_refresh=not schedule_present,
        schedule_present=schedule_present,
    )
    for item in cleaned:
        item["unit"] = _normalize_contract_unit(item.get("unit"))

            out = dict(result)
    out["items"] = cleaned
    if incidental_dropped:
        inc_note = (
            f"Omitted {incidental_dropped} incidental item(s); "
            "not separately paid and not added to parent quantities."
        )
        out["notes"] = ((out.get("notes") or "") + " | " + inc_note).strip(" |")
        out["summary"] = f"{(out.get('summary') or '')} {inc_note}".strip()
    if combined_groups:
        comb_note = (
            f"Combined {combined_groups} similar pay item(s) from multiple locations into one quantity."
        )
        out["notes"] = ((out.get("notes") or "") + " | " + comb_note).strip(" |")
        out["summary"] = f"{(out.get('summary') or '')} {comb_note}".strip()
    if dropped:
        note = f"Filtered {dropped} low-evidence inferred item(s) (schedule-first policy)."
        out["notes"] = ((out.get("notes") or "") + " | " + note).strip(" |")
        summary = out.get("summary") or ""
        out["summary"] = f"{summary} {note}".strip()
    if tc_meta.get("sign_rows"):
        tc_note = (
            f"Rolled {tc_meta['sign_rows']} traffic sign(s) into Traffic Control "
            f"({tc_meta.get('total_sqft')} SqFt; plan sizes or MUTCD)."
        )
        out["notes"] = ((out.get("notes") or "") + " | " + tc_note).strip(" |")
        out["summary"] = f"{(out.get('summary') or '')} {tc_note}".strip()
        out["traffic_control"] = tc_meta
    out["needs_review"] = bool(out.get("needs_review")) or len(cleaned) == 0
    return out

        
        def _merge_analysis_results(text_result: dict[str, Any], vision_result: dict[str, Any]) -> dict[str, Any]:
    """Union text/table items with drawing-vision items.

            Keep every location takeoff; similar rows are summed later in finalize.
    """
    items: list[dict[str, Any]] = []
    for source in (text_result.get("items") or [], vision_result.get("items") or []):
        for item in source:
            if not item.get("description"):
                continue
            normalized = dict(item)
            normalized["unit"] = _normalize_contract_unit(normalized.get("unit"))
            if _should_drop_incidental_item(normalized):
                continue
            items.append(normalized)

            facts = list(text_result.get("facts") or []) + list(vision_result.get("facts") or [])
    summary = (
        f"{vision_result.get('summary') or ''} "
        f"Also merged text/table takeoff ({len(text_result.get('items') or [])} text items, "
        f"{len(vision_result.get('items') or [])} drawing items → {len(items)} unique)."
    ).strip()
    return {
        "engine": "openai+vision",
        "summary": summary,
        "facts": facts,
        "items": items,
        "needs_review": bool(text_result.get("needs_review") or vision_result.get("needs_review"))
        or len(items) == 0,
        "vision_pages": vision_result.get("vision_pages") or [],
        "vision_coverage": vision_result.get("vision_coverage")
        or text_result.get("vision_coverage"),
    }

        
        def answer_engineering_question(*, question: str, context: str) -> dict[str, Any]:
    from app.services.openai_client import openai_configured

            if openai_configured():
        try:
            return _answer_with_openai(question=question, context=context)
        except Exception as exc:
            fallback = _answer_heuristic(question=question, context=context)
            fallback["answer"] = f"{fallback['answer']}\n\n(Note: OpenAI unavailable: {exc})"
            return fallback
    return _answer_heuristic(question=question, context=context)

        
        def _answer_with_openai(*, question: str, context: str) -> dict[str, Any]:
    from app.services.openai_client import ask_openai_json

            system = (
        "You are AutoVAD, an assistant for civil engineers working on USA road projects. "
        "Answer using ONLY the project context. If unknown, say you don't have enough information. "
        "Return STRICT JSON only."
    )
    user = f"""
CONTEXT:
{context[:50000]}

        QUESTION:
{question}

        Return JSON:
{{
  "answer": "markdown-friendly answer",
  "sources": [{{"label":"Page 18 – Table 4.2","document_id":1,"page":18}}]
}}
"""
    data = ask_openai_json(system, user, temperature=0.2)
    return {
        "answer": data.get("answer") or "No answer returned.",
        "sources": data.get("sources") or [],
        "engine": "openai",
    }

        
        def _answer_heuristic(*, question: str, context: str) -> dict[str, Any]:
    q = question.lower()
    sources: list[dict[str, Any]] = []
    lines = [ln.strip() for ln in context.splitlines() if ln.strip()]

            def find_lines(keywords: list[str]) -> list[str]:
        hits = []
        for ln in lines:
            low = ln.lower()
            if any(k in low for k in keywords):
                hits.append(ln)
            if len(hits) >= 8:
                break
        return hits

            if "road width" in q or "carriageway" in q:
        hits = find_lines(["road width", "carriageway width", "width"])
        if hits:
            return {"answer": "From project documents:\n- " + "\n- ".join(hits[:5]), "sources": sources, "engine": "heuristic"}

            if "gsb" in q:
        hits = find_lines(["gsb"])
        if hits:
            return {"answer": "GSB references found:\n- " + "\n- ".join(hits[:6]), "sources": sources, "engine": "heuristic"}

            if "wmm" in q:
        hits = find_lines(["wmm"])
        if hits:
            return {"answer": "WMM references found:\n- " + "\n- ".join(hits[:6]), "sources": sources, "engine": "heuristic"}

            if "culvert" in q:
        hits = find_lines(["culvert"])
        if hits:
            return {"answer": "Culvert references found:\n- " + "\n- ".join(hits[:6]), "sources": sources, "engine": "heuristic"}

            if "pavement" in q or "layer" in q:
        hits = find_lines(["gsb", "wmm", "dbm", "bituminous", "asphalt", "pavement"])
        if hits:
            return {"answer": "Pavement-related findings:\n- " + "\n- ".join(hits[:8]), "sources": sources, "engine": "heuristic"}

            if "eoq" in q or "boq" in q or "quantity" in q or "how much" in q:
        hits = find_lines(["quantity", "qty", "m3", "gsb", "wmm", "eoq", "boq"])
        if hits:
            return {"answer": "Quantity-related findings:\n- " + "\n- ".join(hits[:8]), "sources": sources, "engine": "heuristic"}

            # generic keyword search
    keywords = [w for w in re.findall(r"[a-zA-Z]{3,}", q) if w not in {"what", "where", "show", "find", "this", "that", "from", "with", "have", "many", "much"}]
    hits = find_lines(keywords[:4]) if keywords else []
    if hits:
        return {
            "answer": "I found these related excerpts in the project documents:\n- " + "\n- ".join(hits[:8]),
            "sources": sources,
            "engine": "heuristic",
        }

            return {
        "answer": (
            "I don't have enough extracted information to answer confidently yet. "
            "Run AI analysis on the uploaded documents, then ask again."
        ),
        "sources": [],
        "engine": "heuristic",
    }

        
        def _item(
    *,
    description: str,
    category: str,
    unit: str,
    quantity: Decimal,
    document_id: int,
    page: int | None,
    source: str,
    method: str,
    confidence: float,
    item_code: str | None = None,
    status: str = "needs_review",
    source_reference: str | None = None,
) -> dict[str, Any]:
    from app.services.csi_mapper import enrich_quantity_item

            raw = {
        "item_code": item_code,
        "description": description,
        "category": category,
        "unit": _normalize_contract_unit(unit),
        "quantity": float(quantity),
        "source_document_id": document_id,
        "source_page": page,
        "source_reference": source_reference or source,
        "calculation_method": method,
        "confidence": confidence,
        "status": status,
    }
    return enrich_quantity_item(raw)

        
        def _map_alias(text: str) -> tuple[str, str, str] | None:
    low = text.lower()
    for key, value in ITEM_ALIASES.items():
        if key in low:
            return value
    return None

        
        def _find_col(header: list[str], names: list[str]) -> int | None:
    """Pick the column whose header best matches (longest / exact name wins)."""
    best: tuple[int, int] | None = None
    for i, raw in enumerate(header):
        h = str(raw or "").strip().lower()
        if not h:
            continue
        for n in names:
            key = str(n or "").strip().lower()
            if not key:
                continue
            if key == h:
                score = 1000 + len(key)
            elif key in h:
                score = len(key)
            else:
                continue
            if best is None or score > best[0]:
                best = (score, i)
    return best[1] if best else None

        
        def _parse_number(value: Any) -> Decimal | None:
    if value is None:
        return None
    if isinstance(value, (int, float, Decimal)):
        return Decimal(str(value))
    text = str(value).strip().replace(",", "")
    match = re.search(r"-?\d+(?:\.\d+)?", text)
    if not match:
        return None
    try:
        return Decimal(match.group(0))
    except InvalidOperation:
        return None

        
        def _normalize_unit(unit: str) -> str:
    u = unit.strip().lower().replace("³", "3")
    mapping = {
        "m3": "m3",
        "cu.m": "m3",
        "cu m": "m3",
        "cubic meter": "m3",
        "cubic meters": "m3",
        "m": "m",
        "lm": "m",
        "lin m": "m",
        "linear m": "m",
        "nos": "nos",
        "no": "nos",
        "no.": "nos",
        "each": "nos",
        "ea": "nos",
        "t": "t",
        "ton": "t",
        "tons": "t",
    }
    return mapping.get(u, unit.strip())

        
        def _guess_page(full_text: str, pos: int, content: ExtractedContent) -> int | None:
    prefix = full_text[:pos]
    pages = re.findall(r"--- Page (\d+) ---", prefix)
    if pages:
        return int(pages[-1])
    if content.pages:
        return content.pages[0].page
    return None

### backend/app/services/incidental.py

SHA-256: 9116b7c1e47b79eaa93ab861e74bb19ab196840e18f5fb6420c234b77c31a590    ·    256 line(s)    ·    .py

"""Detect work that is incidental to a bid item and must not be taken off.

        Municipal plans routinely note bedding, trench, fittings, tracer wire, etc. as
incidental to a parent pay item. Those notes must not become extra EOQ rows
and must not be added into the parent quantity.
"""

        from __future__ import annotations

        import re
from typing import Any

        # Pay items that happen to contain the word "incidental" but are bid rows.
_KEEP_PAY_ITEM_RE = re.compile(
    r"incidental\s+construction|"
    r"contract\s+incidentals?|"
    r"miscellaneous\s+incidentals?|"
    r"^incidentals?\s*$",
    re.I,
)

        # Child work pointed at a parent bid item.
_INCIDENTAL_TO_RE = re.compile(
    r"\bincidental(?:ly)?\s+to\b|"
    r"\bsubsidiary\s+to\b|"
    r"\bpaid\s+(?:for\s+)?(?:under|as\s+part\s+of)\b|"
    r"\bincluded\s+in\s+(?:the\s+)?(?:bid|pay|contract|parent)\s+items?\b|"
    r"\bconsidered\s+(?:incidental|included)\b",
    re.I,
)

        _NO_SEPARATE_PAY_RE = re.compile(
    r"\bno\s+separate\s+(?:pay(?:ment|s|item)?|measurement|compensation|bid)\b|"
    r"\bnot\s+(?:a\s+)?(?:separate\s+)?pay\s+items?\b|"
    r"\bno\s+additional\s+(?:payment|compensation|pay)\b|"
    r"\bdo\s+not\s+(?:include|pay|measure)\s+(?:as\s+)?(?:a\s+)?separate\b",
    re.I,
)

        _INCIDENTAL_MARK_RE = re.compile(
    r"\(\s*incidental(?:ly)?\s*\)|"
    r"\bincidental(?:ly)?\b|"
    r"\bincidentals\b",
    re.I,
)

        _UNIT_INCIDENTAL_RE = re.compile(
    r"^(?:inc|incidental|incidentals|nsp|incl|included|n/?a)$",
    re.I,
)

        # Customarily incidental to pipe / pavement unless listed as its own schedule row.
_DEFAULT_EXTRA_RE = re.compile(
    r"\b(?:"
    r"trench\s+excavation|"
    r"(?:pipe|sand|gravel|granular)?\s*bedding|"
    r"haunch(?:ing|es)?|"
    r"pipe(?:\s+zone)?\s+backfill|"
    r"trench\s+backfill|"
    r"tracer\s+wires?|"
    r"locate\s+wires?|"
    r"(?:warning|marking|detectable)\s+tapes?|"
    r"poly(?:ethylene)?\s*(?:wrap|encasement)|"
    r"polywrap|"
    r"thrust\s+blocks?|"
    r"sheeting|"
    r"shoring|"
    r"trench\s+box(?:es)?|"
    r"hydrostatic\s+tests?|"
    r"disinfection|"
    r"chlorination|"
    r"flushing|"
    r"pipe\s+(?:lubricants?|gaskets?)"
    r")\b",
    re.I,
)

        _FITTING_RE = re.compile(
    r"\b(?:fittings?|bends?|elbows?|tees?|reducers?|plugs?|couplings?|"
    r"(?:long\s+)?sleeves?|cross(?:es)?|caps?|wye[s]?|wyes)\b",
    re.I,
)

        _QTY_INFLATED_RE = re.compile(
    r"(?:"
    r"(?:plus|added|add(?:ing)?|including|include[sd]?|and)\s+"
    r"(?:incidental|\d+\s+(?:fittings?|bends?|elbows?|tees?|bedding)|"
    r"fittings?|bedding|backfill|trench|tracer)|"
    r"including\s+incidentals?|"
    r"pipe\s+(?:lf|ft|length)\s+plus|"
    r"added\s+to\s+(?:the\s+)?(?:pipe|parent|bid\s+item)|"
    r"rolled\s+(?:in|into)"
    r")",
    re.I,
)

        _TRENCH_INCIDENTAL_NOTES_RE = re.compile(
    r"(?:trench|bedding|backfill|haunch).{0,60}incidental|"
    r"incidental.{0,60}(?:trench|bedding|backfill|haunch)|"
    r"incidental(?:ly)?\s+to.{0,40}(?:pipe|watermain|water\s*main|sewer)",
    re.I,
)

        _PARENT_PAY_RE = re.compile(
    r"\b(?:"
    r"water\s*mains?|watermains?|"
    r"sanitary|storm|"
    r"sewer\s+pipe|"
    r"pavement|asphalt|hma|curb|gutter|sidewalk|"
    r"mobilization|traffic\s+control|"
    r"riprap|embankment|spillway"
    r")\b",
    re.I,
)

        
        def item_evidence_text(item: dict[str, Any]) -> str:
    return (
        f"{item.get('description') or ''} "
        f"{item.get('calculation_method') or ''} "
        f"{item.get('source_reference') or ''} "
        f"{item.get('source') or ''}"
    )

        
        def is_kept_incidental_pay_item(description: str | None) -> bool:
    text = str(description or "").strip()
    return bool(text and _KEEP_PAY_ITEM_RE.search(text))

        
        def unit_is_incidental(unit: str | None) -> bool:
    raw = re.sub(r"\s+", " ", str(unit or "").strip())
    return bool(raw and _UNIT_INCIDENTAL_RE.match(raw))

        
        def has_incidental_language(text: str | None) -> bool:
    blob = str(text or "")
    if not blob.strip():
        return False
    return bool(
        _INCIDENTAL_TO_RE.search(blob)
        or _NO_SEPARATE_PAY_RE.search(blob)
        or _INCIDENTAL_MARK_RE.search(blob)
    )

        
        def description_is_incidental_child(text: str | None) -> bool:
    """True when the text describes work that is not separately paid."""
    blob = str(text or "")
    if not blob.strip() or is_kept_incidental_pay_item(blob):
        return False
    if _INCIDENTAL_TO_RE.search(blob) or _NO_SEPARATE_PAY_RE.search(blob):
        return True
    if re.search(r"\(\s*incidental(?:ly)?\s*\)", blob, re.I):
        return True
    return False

        
        def is_default_incidental_extra(description: str | None) -> bool:
    text = str(description or "")
    if not text.strip() or is_kept_incidental_pay_item(text):
        return False
    return bool(_DEFAULT_EXTRA_RE.search(text))

        
        def is_fitting_like(description: str | None) -> bool:
    text = str(description or "")
    if not text.strip():
        return False
    if re.search(r"\b(?:valve|hydrant|manhole|inlet|catch\s*basin)\b", text, re.I):
        return False
    return bool(_FITTING_RE.search(text))

        
        def quantity_inflated_by_incidentals(text: str | None) -> bool:
    return bool(_QTY_INFLATED_RE.search(str(text or "")))

        
        def cad_notes_make_trench_incidental(text: str | None) -> bool:
    return bool(_TRENCH_INCIDENTAL_NOTES_RE.search(str(text or "")))

        
        def clause_around(text: str, index: int) -> tuple[str, int]:
    """Return the clause containing index and its start offset in text."""
    if not text:
        return "", 0
    index = max(0, min(index, len(text)))
    start = 0
    end = len(text)
    for match in re.finditer(r"[\n;]|(?<!\d)\.(?!\d)\s+", text):
        if match.end() <= index:
            start = match.end()
        elif match.start() > index:
            end = match.start()
            break
    return text[start:end].strip(), start

        
        def skip_text_extraction(
    description: str,
    line: str,
    *,
    match_start: int = 0,
) -> bool:
    """True when a regex/label hit is incidental work or only names the parent item."""
    if is_kept_incidental_pay_item(description):
        return False
    if description_is_incidental_child(description):
        return True
    if is_default_incidental_extra(description):
        return True
    inc_to = _INCIDENTAL_TO_RE.search(line)
    if inc_to and match_start >= inc_to.start():
        return True
    if has_incidental_language(line) and (
        is_fitting_like(description) or not _PARENT_PAY_RE.search(description)
    ):
        # Subject of an incidental note (excavation, collars, …) — not a parent pay item.
        if inc_to and match_start < inc_to.start():
            return True
        if not inc_to:
            return True
    return False

        
        def extraction_should_skip(description: str, full_text: str, match_start: int) -> bool:
    clause, clause_start = clause_around(full_text, match_start)
    return skip_text_extraction(
        description,
        clause or full_text,
        match_start=max(0, match_start - clause_start),
    )

        
        def should_drop_incidental_item(
    item: dict[str, Any],
    *,
    scheduled: bool = False,
    drop_default_extras: bool = True,
) -> bool:
    """Drop incidental children. Never use this to add their qty onto a parent."""
    desc = str(item.get("description") or "")
    if is_kept_incidental_pay_item(desc):
        return False
    if unit_is_incidental(item.get("unit")):
        return True
    if description_is_incidental_child(desc):
        return True
    evidence = item_evidence_text(item)
    if description_is_incidental_child(evidence) and not scheduled:
        return True
    if drop_default_extras and not scheduled and is_default_incidental_extra(desc):
        return True
    if not scheduled and is_fitting_like(desc) and has_incidental_language(evidence):
        return True
    return False

### backend/app/services/item_combine.py

SHA-256: b21dd42b3038d4eefdf84e0e26d1b73383ea278679b98ba05dfcfa1ceedf0245    ·    464 line(s)    ·    .py

"""Combine similar pay items from different locations into one quantity.

        Plan sheets often take off the same bid item in more than one place
(Fertilizer 1,189 lb on one sheet and 39 lb on another). Those rows should
become a single EOQ line with the quantities added — without merging true
variants (8\" vs 12\", Type II vs Type III, furnish vs remove, etc.).
"""

        from __future__ import annotations

        import re
from typing import Any

        from app.services.csi_mapper import normalize_unit
from app.services.traffic_control import looks_like_agency_bid_number

        _LOCATION_NOISE_RE = re.compile(
    r"\bsta(?:tion)?\.?\s*\d+\+\d+(?:\.\d+)?(?:\s*(?:to|-|through|–|—)\s*"
    r"(?:sta(?:tion)?\.?\s*)?\d+\+\d+(?:\.\d+)?)?|"
    r"\bsheets?\s+[a-z]?\-?\d+[a-z0-9]*|"
    r"\bpages?\s+\d+|"
    r"\bp\.\s*\d+|"
    r"\barea\s+[a-z0-9\-]+|"
    r"\blocation\s+\d+|"
    r"\bfrom\s+sta\b|"
    r"\b(?:left|right)\s+(?:side|end)\b|"
    r"\bprop(?:osed)?\.?\b|"
    r"\bctd\.?\b|\bcontinued\b",
    re.I,
)

        _NPK_RE = re.compile(r"\b(\d{1,2}-\d{1,2}-\d{1,2})\b")
_TYPE_RE = re.compile(r"\btype\s*([ivxl]+|\d+)\b", re.I)
_SIZE_RE = re.compile(
    r"(\d{1,2}(?:\.\d+)?)\s*-?\s*(?:inch|in|\"|'')",
    re.I,
)

        _MATERIAL_TOKENS = frozenset(
    {"dip", "pvc", "hdpe", "rcp", "cpp", "di", "ci", "hma", "pcc", "copper", "steel"}
)
_REMOVE_HINTS = ("remove", "removal", "salvage", "abandon", "demolish", "demolition")
_WEAK_MODIFIERS = frozenset(
    {
        "commercial",
        "agricultural",
        "chemical",
        "standard",
        "municipal",
        "complete",
        "specified",
        "new",
        "temp",
        "temporary",
        "permanent",
        "lawn",
        "turf",
        "restoration",
        "roadside",
        "native",
        "mix",
        "mixed",
        "material",
        "materials",
        "application",
        "applied",
        "grade",
        "graded",
        "in",
        "place",
        "as",
        "per",
        "plan",
        "plans",
        "detail",
        "typical",
    }
)
_STOP = frozenset(
    {
        "the",
        "and",
        "for",
        "with",
        "of",
        "a",
        "an",
        "to",
        "on",
        "or",
        "item",
        "per",
        "at",
        "by",
        "from",
        "incl",
        "including",
        "all",
        "work",
        "provide",
        "sta",
        "station",
        "sheet",
        "page",
        "area",
        "location",
        "side",
        "prop",
        "proposed",
        "ctd",
        "continued",
    }
)
_SYNONYM = {
    "fertilising": "fertiliz",
    "fertilizing": "fertiliz",
    "fertiliser": "fertiliz",
    "fertilizer": "fertiliz",
    "fertilizers": "fertiliz",
    "seeding": "seed",
    "seeds": "seed",
    "mulching": "mulch",
    "watermain": "watermain",
    "watermains": "watermain",
    "temp": "temporary",
    "barricades": "barricade",
    "signs": "sign",
    "pipes": "pipe",
    "valves": "valve",
    "hydrants": "hydrant",
}
_SCHEDULE_HINTS = (
    "quantity sheet",
    "estimate of quantities",
    "bid items",
    "bid item",
    "std bid",
    "standard bid",
    "eoq",
    "pay item",
    "proposal quantity",
    "est. qty",
    "est qty",
    "approx. quantity",
    "approx quantity",
    "for bidding purposes",
    "extracted table total",
    "extracted from quantity table",
    "extracted from eoq",
    "extracted from tabular quantity",
    "eoq schedule",
)

        
        def _norm_token(token: str) -> str:
    t = token.lower().strip()
    if t in _SYNONYM:
        t = _SYNONYM[t]
    if t.startswith("fertiliz"):
        return "fertiliz"
    roman = {"i": "1", "ii": "2", "iii": "3", "iv": "4", "v": "5"}
    if t in roman:
        t = roman[t]
    if len(t) > 4 and t.endswith("ies"):
        t = t[:-3] + "y"
    elif len(t) > 3 and t.endswith("s") and not t.endswith("ss"):
        t = t[:-1]
    return t

        
        def _tokens(description: str) -> set[str]:
    cleaned = _LOCATION_NOISE_RE.sub(" ", description or "")
    cleaned = re.sub(r"[^a-z0-9]+", " ", cleaned.lower())
    out = set()
    for raw in cleaned.split():
        if len(raw) <= 1 or raw in _STOP:
            continue
        tok = _norm_token(raw)
        if tok and tok not in _STOP:
            out.add(tok)
    return out

        
        def _size_token(description: str) -> str | None:
    m = _SIZE_RE.search(description or "")
    if not m:
        return None
    try:
        val = float(m.group(1))
    except ValueError:
        return None
    if abs(val - int(val)) < 0.01:
        return str(int(val))
    return f"{val:g}"

        
        def _npk_token(description: str) -> str | None:
    m = _NPK_RE.search(description or "")
    return m.group(1) if m else None

        
        def _type_token(description: str) -> str | None:
    m = _TYPE_RE.search(description or "")
    if not m:
        return None
    raw = m.group(1).lower()
    roman = {"i": "1", "ii": "2", "iii": "3", "iv": "4", "v": "5", "vi": "6"}
    return roman.get(raw, raw)

        
        def _materials(description: str) -> set[str]:
    toks = _tokens(description)
    return toks & _MATERIAL_TOKENS

        
        def _action(description: str) -> str | None:
    low = (description or "").lower()
    if any(h in low for h in _REMOVE_HINTS):
        return "remove"
    furnish = "furnish" in low
    install = "install" in low
    if furnish and install:
        return "furnish_install"
    if furnish:
        return "furnish"
    if install:
        return "install"
    return None

        
        def _alternate(description: str) -> str | None:
    m = re.search(r"\b(?:alternate|alt\.?)\s*([ab])\b", description or "", re.I)
    return m.group(1).lower() if m else None

        
        def _agency_code(item: dict[str, Any]) -> str:
    raw = str(item.get("item_code") or "").strip()
    if looks_like_agency_bid_number(raw):
        return raw.lower()
    return ""

        
        def _unit_key(item: dict[str, Any]) -> str:
    return normalize_unit(str(item.get("unit") or ""))

        
        def _is_ls(item: dict[str, Any]) -> bool:
    return _unit_key(item) == "ls"

        
        def item_is_schedule(item: dict[str, Any]) -> bool:
    if _agency_code(item):
        return True
    blob = (
        f"{item.get('calculation_method') or ''} "
        f"{item.get('source_reference') or ''} "
        f"{item.get('source') or ''}"
    ).lower()
    return any(h in blob for h in _SCHEDULE_HINTS)

        
        def _qty(item: dict[str, Any]) -> float:
    try:
        return float(item.get("quantity") or 0)
    except (TypeError, ValueError):
        return 0.0

        
        def _near_equal(a: float, b: float, rel: float = 0.08) -> bool:
    mx = max(abs(a), abs(b), 1e-9)
    return abs(a - b) / mx <= rel

        
        def pay_items_similar(a: dict[str, Any], b: dict[str, Any]) -> bool:
    """True when two rows are the same pay item taken off in different places."""
    if _unit_key(a) != _unit_key(b):
        return False
    if _unit_key(a) in {"unit", ""}:
        return False

            da = str(a.get("description") or "")
    db = str(b.get("description") or "")
    if not da.strip() or not db.strip():
        return False

            sa, sb = _size_token(da), _size_token(db)
    if sa and sb and sa != sb:
        return False
    na, nb = _npk_token(da), _npk_token(db)
    if na and nb and na != nb:
        return False
    ta, tb = _type_token(da), _type_token(db)
    if ta and tb and ta != tb:
        return False
    ma, mb = _materials(da), _materials(db)
    if ma and mb and ma != mb:
        return False
    aa, ab = _action(da), _action(db)
    if aa and ab and aa != ab:
        return False
    alta, altb = _alternate(da), _alternate(db)
    if alta and altb and alta != altb:
        return False
    ca, cb = _agency_code(a), _agency_code(b)
    if ca and cb and ca != cb:
        return False

            tok_a, tok_b = _tokens(da), _tokens(db)
    if not tok_a or not tok_b:
        return False
    if tok_a == tok_b:
        return True

            extra_a = tok_a - tok_b
    extra_b = tok_b - tok_a
    if tok_a <= tok_b and extra_b <= _WEAK_MODIFIERS:
        return True
    if tok_b <= tok_a and extra_a <= _WEAK_MODIFIERS:
        return True

            inter = len(tok_a & tok_b)
    if inter <= 0:
        return False
    dice = 2.0 * inter / (len(tok_a) + len(tok_b))
    return dice >= 0.86 and not (extra_a - _WEAK_MODIFIERS) and not (extra_b - _WEAK_MODIFIERS)

        
        def _pick_best(items: list[dict[str, Any]]) -> dict[str, Any]:
    def score(item: dict[str, Any]) -> tuple:
        desc = str(item.get("description") or "")
        clean = 0 if _LOCATION_NOISE_RE.search(desc) else 1
        conf = 0.0
        try:
            conf = float(item.get("confidence") or 0)
        except (TypeError, ValueError):
            conf = 0.0
        return (
            1 if item_is_schedule(item) else 0,
            1 if _agency_code(item) else 0,
            clean,
            conf,
            -len(desc),
        )

            return dict(max(items, key=score))

        
        def _fmt_qty(value: float) -> str:
    if abs(value - round(value)) < 1e-9:
        return str(int(round(value)))
    return f"{value:.4g}"

        
        def _collapse_group(items: list[dict[str, Any]]) -> dict[str, Any]:
    if len(items) == 1:
        return dict(items[0])

            if any(_is_ls(i) for i in items):
        best = _pick_best(items)
        best["quantity"] = 1.0
        return best

            schedule = [i for i in items if item_is_schedule(i)]
    work = schedule if schedule else list(items)
    qtys = [_qty(i) for i in work]
    total = sum(qtys)
    mx = max(qtys) if qtys else 0.0
    rest = total - mx

            # One row is already the project total of the others — keep that total.
    if len(work) >= 3 and rest > 0 and mx > 0 and abs(mx - rest) / mx <= 0.08:
        keeper = next(i for i in work if _near_equal(_qty(i), mx))
        return dict(keeper)

            # Duplicate extracts of the same quantity (schedule copies / continuation pages).
    refs = {str(i.get("source_reference") or "").strip() for i in work}
    refs.discard("")
    pages = {str(i.get("source_page") or "").strip() for i in work}
    pages.discard("")
    same_ref = bool(refs) and len(refs) == 1 and all(
        str(i.get("source_reference") or "").strip() for i in work
    )
    different_pages = len(pages) > 1
    if len(work) > 1 and all(_near_equal(q, mx) for q in qtys):
        if all(item_is_schedule(i) for i in work) or (same_ref and not different_pages):
            best = _pick_best(work)
            best["quantity"] = mx
            return best

            best = _pick_best(work)
    out = dict(best)
    combined = round(total, 4)
    out["quantity"] = combined
    out["calculation_method"] = (
        f"Combined {len(work)} similar location/takeoff rows: "
        + " + ".join(_fmt_qty(q) for q in qtys)
        + f" = {_fmt_qty(combined)}"
    )
    pages = sorted(
        {
            str(i.get("source_page") or "").strip()
            for i in work
            if str(i.get("source_page") or "").strip()
        }
    )
    refs = []
    for item in work:
        ref = str(item.get("source_reference") or item.get("source") or "").strip()
        if ref and ref not in refs:
            refs.append(ref)
        if len(refs) >= 4:
            break
    if pages or refs:
        loc = []
        if pages:
            loc.append("pages " + ", ".join(pages))
        if refs:
            loc.append("; ".join(refs[:3]))
        out["source_reference"] = "Combined locations — " + " · ".join(loc)
    try:
        out["confidence"] = max(float(i.get("confidence") or 0) for i in work)
    except (TypeError, ValueError):
        pass
    return out

        
        def combine_similar_pay_items(items: list[dict[str, Any]]) -> list[dict[str, Any]]:
    """Cluster similar pay items and add location quantities into one row each."""
    rows = [dict(item) for item in items if str(item.get("description") or "").strip()]
    n = len(rows)
    if n <= 1:
        return rows

            parent = list(range(n))

            def find(i: int) -> int:
        while parent[i] != i:
            parent[i] = parent[parent[i]]
            i = parent[i]
        return i

            def union(a: int, b: int) -> None:
        ra, rb = find(a), find(b)
        if ra != rb:
            parent[rb] = ra

            for i in range(n):
        for j in range(i + 1, n):
            if pay_items_similar(rows[i], rows[j]):
                union(i, j)

            groups: dict[int, list[dict[str, Any]]] = {}
    for i, row in enumerate(rows):
        groups.setdefault(find(i), []).append(row)

            combined = 0
    out: list[dict[str, Any]] = []
    for group in groups.values():
        collapsed = _collapse_group(group)
        if len(group) > 1:
            combined += 1
        out.append(collapsed)
    # Preserve a stable order: original first-seen group order
    return out

### backend/app/services/traffic_control.py

SHA-256: 27ff1d0f5363fae0d371ab73519081b1926cd9cbe083b4d2be9d42f0b1a39648    ·    651 line(s)    ·    .py

"""Traffic Control signing: consolidate individual signs into one SqFt pay item.

        Size priority:
1. Explicit dimensions on the plan / DWG / item text (inches or feet)
2. MUTCD designation code lookup (local table mirrored from FHWA MUTCD)
3. Name alias → MUTCD code (STOP → R1-1, etc.)
4. Default conventional-road size when clearly a sign but size unknown

        Optional online refresh: FHWA MUTCD / Standard Highway Signs pages when reachable.
"""

        from __future__ import annotations

        import json
import logging
import re
from pathlib import Path
from typing import Any

        logger = logging.getLogger(__name__)

        _DATA_PATH = Path(__file__).resolve().parent.parent / "data" / "mutcd_sign_sizes.json"
_MUTCD_CACHE: dict[str, Any] | None = None

        # MUTCD designation: R1-1, W20-7a, S5-1, G20-2, M4-5, OM1-1, etc.
_MUTCD_CODE_RE = re.compile(
    r"\b([RWMGSODP]|OM|I)[A-Z]?\d{1,2}(?:[A-Z])?(?:-\d{1,2}[A-Za-z]{0,2})(?:P)?\b",
    re.I,
)

        # Explicit size callouts: 30x30, 24" x 36", 48-Inch x 30-Inch, 2 ft x 2.5 ft
_UNIT = r"(?:\"|''|in(?:ch(?:es)?)?|ft|feet|')"
_SIZE_PATTERNS = [
    re.compile(
        rf"(?P<w>\d+(?:\.\d+)?)\s*-?\s*(?P<wu>{_UNIT})?\s*[x×]\s*"
        rf"(?P<h>\d+(?:\.\d+)?)\s*-?\s*(?P<hu>{_UNIT})?",
        re.I,
    ),
]

        _SIGN_POSITIVE = re.compile(
    r"(?:"
    r"\b(?:traffic\s+)?(?:control\s+)?signs?\b|"
    r"\bsigning\b|"
    r"\bsignage\b|"
    r"\broad\s+sign\b|"
    r"\bwarning\s+sign\b|"
    r"\bregulatory\s+sign\b|"
    r"\bguide\s+sign\b|"
    r"\btemp(?:orary)?\s+sign\b|"
    r"\bstop\s+sign\b|"
    r"\byield\s+sign\b|"
    r"\bspeed\s+limit\b|"
    r"\bdo\s+not\s+enter\b|"
    r"\bwrong\s+way\b|"
    r"\bone\s+way\b|"
    r"\bno\s+parking\b|"
    r"\broad\s+work\s+ahead\b|"
    r"\bend\s+road\s+work\b|"
    r"\bflagger\b|"
    r"\bchevron\b|"
    r"\bmutcd\b|"
    r"\b[rwmgsodp]\d{1,2}-\d{1,2}[a-z]?\b"
    r")",
    re.I,
)

        _SIGN_NEGATIVE = re.compile(
    r"(?:"
    r"\btraffic\s+signal\b|"
    r"\bsignal\s+(?:head|pole|cabinet|controller|mast)\b|"
    r"\bpavement\s+mark(?:ing)?\b|"
    r"\bstrip(?:e|ing)\b|"
    r"\bthermoplastic\b|"
    r"\bchanneliz(?:er|ation)\b|"
    r"\bbarricade\b|"
    r"\bdrum\b|"
    r"\bcone\b|"
    r"\bdelineator\b|"
    r"\bflagger\s+(?:station|hours?)\b|"
    r"\bmobilization\b|"
    r"\btemporary\s+business\s+sign\b|"
    r"\b(?:portable\s+)?changeable\s+message\s+sign\b|"
    r"\bpcms\b|"
    r"\bmailbox\b|"
    r"\bgravel\s+access\b|"
    r"\bwinter\s+maintenance\b|"
    r"\btraffic\s+control\s*,?\s*misc|"
    r"\btemporary\s+traffic\s+control\b(?!.*\bsign)|"
    r"\bttc\b(?!.*\bsign)"
    r")",
    re.I,
)

        _GENERIC_TC_RE = re.compile(
    r"^\s*traffic\s+control(?:\s*\(signing\))?\s*$",
    re.I,
)

        # Bid-schedule companions under a Traffic Control heading — never roll into SqFt.
_DISTINCT_TC_PAY_RE = re.compile(
    r"(?:"
    r"\bbarricade\b|"
    r"\bchanneliz(?:er|ation)\b|"
    r"\bdrum\b|"
    r"\bcone\b|"
    r"\bpcms\b|"
    r"\bchangeable\s+message\b|"
    r"\bbusiness\s+sign\b|"
    r"\bmailbox\b|"
    r"\bgravel\s+access\b|"
    r"\bwinter\s+maintenance\b|"
    r"\btraffic\s+control\s*,?\s*misc|"
    r"\bflagging\b|"
    r"\bpilot\s+car\b"
    r")",
    re.I,
)

        _AGENCY_BID_NO_RE = re.compile(r"^\s*(?:special|\d{1,4}\.\d{2,4}[a-z]?)\s*$", re.I)

        _PLAN_DEVICE_HINTS = (
    "graphic count",
    "symbol",
    "symbols",
    "project total",
    "itemized table",
    "mutcd ref",
    "consolidated",
    "in²",
    "in2",
    "÷144",
    "/144",
    "width(in)",
    "default_conventional",
)

        
        def _load_mutcd() -> dict[str, Any]:
    global _MUTCD_CACHE
    if _MUTCD_CACHE is not None:
        return _MUTCD_CACHE
    try:
        _MUTCD_CACHE = json.loads(_DATA_PATH.read_text(encoding="utf-8"))
    except Exception as exc:
        logger.warning("MUTCD size table missing/unreadable (%s); using empty fallback", exc)
        _MUTCD_CACHE = {
            "signs": {},
            "name_aliases": {},
            "default_unknown_sign_in": {"width_in": 30, "height_in": 30},
            "source_url": "https://mutcd.fhwa.dot.gov/",
        }
    return _MUTCD_CACHE

        
        def refresh_mutcd_from_online(*, timeout_s: float = 8.0) -> dict[str, Any]:
    """Best-effort online check that MUTCD resources are reachable; keep local sizes.

            Full FHWA PDF table parsing is brittle; we verify the official pages and keep the
    curated local table (MUTCD 11th Edition conventional-road defaults) as authority.
    """
    catalog = _load_mutcd()
    urls = [
        str(catalog.get("source_url") or "https://mutcd.fhwa.dot.gov/kno_11th_Editionr1.htm"),
        str(
            catalog.get("standard_highway_signs_url")
            or "https://mutcd.fhwa.dot.gov/shsm_interim/index.htm"
        ),
        "https://mutcd.fhwa.dot.gov/",
    ]
    checked: list[dict[str, Any]] = []
    try:
        import httpx

                with httpx.Client(timeout=timeout_s, follow_redirects=True) as client:
            for url in urls:
                try:
                    resp = client.head(url)
                    if resp.status_code >= 400:
                        resp = client.get(url)
                    checked.append(
                        {
                            "url": url,
                            "status": resp.status_code,
                            "ok": resp.status_code < 400,
                        }
                    )
                except Exception as exc:  # noqa: BLE001
                    checked.append({"url": url, "ok": False, "error": str(exc)[:160]})
    except Exception as exc:  # noqa: BLE001
        checked.append({"url": "httpx", "ok": False, "error": str(exc)[:160]})

            catalog = dict(catalog)
    catalog["online_check"] = {
        "checked_at": __import__("datetime").datetime.utcnow().isoformat() + "Z",
        "results": checked,
        "using_local_table": True,
        "sign_count": len(catalog.get("signs") or {}),
    }
    global _MUTCD_CACHE
    _MUTCD_CACHE = catalog
    return catalog

        
        def inches_to_sqft(width_in: float, height_in: float) -> float:
    return round((float(width_in) * float(height_in)) / 144.0, 4)

        
        def _to_inches(value: float, unit: str | None) -> float:
    u = (unit or "").strip().lower()
    if u in {"ft", "feet", "'", "’"}:
        return value * 12.0
    # bare number next to another inch unit, or empty → inches (MUTCD convention)
    return value

        
        def parse_sign_size_inches(*texts: Any) -> tuple[float, float] | None:
    blob = " ".join(str(t) for t in texts if t)
    if not blob.strip():
        return None
    for pattern in _SIZE_PATTERNS:
        m = pattern.search(blob)
        if not m:
            continue
        w = _to_inches(float(m.group("w")), m.group("wu"))
        h = _to_inches(float(m.group("h")), m.groupdict().get("hu") or m.group("wu"))
        # Sanity: sign faces are rarely < 6" or > 20'
        if 6 <= w <= 240 and 6 <= h <= 240:
            return w, h
    return None

        
        def extract_mutcd_code(*texts: Any) -> str | None:
    blob = " ".join(str(t) for t in texts if t)
    m = _MUTCD_CODE_RE.search(blob)
    if not m:
        return None
    code = m.group(0).upper().replace(" ", "")
    # Normalize OM1-1 style already matched; ensure hyphen form for R11 etc.
    return code

        
        def lookup_mutcd_size(
    *,
    code: str | None = None,
    description: str | None = None,
    allow_online_refresh: bool = False,
) -> dict[str, Any] | None:
    catalog = _load_mutcd()
    if allow_online_refresh:
        try:
            catalog = refresh_mutcd_from_online()
        except Exception:  # noqa: BLE001
            pass

            signs: dict[str, Any] = catalog.get("signs") or {}
    aliases: dict[str, str] = {
        str(k).lower(): str(v).upper() for k, v in (catalog.get("name_aliases") or {}).items()
    }

            resolved = (code or "").upper().strip() or None
    if resolved and resolved not in signs:
        # Try without trailing plaque letter variants already in table
        alt = resolved.rstrip("P")
        if alt in signs:
            resolved = alt
        else:
            resolved = None

            if not resolved and description:
        low = re.sub(r"[^a-z0-9]+", " ", description.lower()).strip()
        # Longest alias first
        for alias in sorted(aliases.keys(), key=len, reverse=True):
            if alias in low or low == alias:
                resolved = aliases[alias]
                break

            if resolved and resolved in signs:
        entry = signs[resolved]
        return {
            "code": resolved,
            "name": entry.get("name"),
            "width_in": float(entry["width_in"]),
            "height_in": float(entry["height_in"]),
            "source": "mutcd_table",
            "source_url": catalog.get("source_url"),
        }

            return None

        
        def _evidence_blob(item: dict[str, Any]) -> str:
    return (
        f"{item.get('calculation_method') or ''} "
        f"{item.get('source_reference') or ''} "
        f"{item.get('source') or ''} "
        f"{item.get('description') or ''}"
    ).lower()

        
        def looks_like_agency_bid_number(code: Any) -> bool:
    raw = str(code or "").strip()
    if not raw:
        return False
    # CSI MasterFormat uses spaces: "01 71 13" — not an agency bid number
    if re.match(r"^\d{2}\s+\d{2}\s+\d{2}", raw):
        return False
    return bool(_AGENCY_BID_NO_RE.match(raw))

        
        def is_distinct_traffic_pay_item(item: dict[str, Any]) -> bool:
    """True for bid-schedule Traffic Control companions (barricade, PCMS, mailbox, …)."""
    desc = str(item.get("description") or "")
    if _GENERIC_TC_RE.match(desc.strip()):
        return False
    if _DISTINCT_TC_PAY_RE.search(desc):
        return True
    unit = str(item.get("unit") or "").lower()
    if "traffic control" in desc.lower() and unit in {"ls", "lump sum"}:
        return True
    return False

        
        def is_plan_device_takeoff(item: dict[str, Any]) -> bool:
    """F-sheet device tables, graphic symbol counts, MUTCD 30×30 rollups — not bid rows."""
    blob = _evidence_blob(item)
    return any(h in blob for h in _PLAN_DEVICE_HINTS)

        
        def is_generic_traffic_control_signing(item: dict[str, Any]) -> bool:
    desc = str(item.get("description") or "").strip()
    if not _GENERIC_TC_RE.match(desc):
        return False
    if is_distinct_traffic_pay_item(item):
        return False
    return True

        
        def _unit_is_sqft(unit: Any) -> bool:
    u = str(unit or "").lower().replace(".", "").replace(" ", "")
    return u in {"sqft", "sf", "squarefoot", "squarefeet", "sqft"}

        
        def is_traffic_sign_item(item: dict[str, Any]) -> bool:
    """True for individual MUTCD/plan signs (not schedule pay items or signals)."""
    desc = str(item.get("description") or "")
    cat = str(item.get("category") or "")
    method = str(item.get("calculation_method") or "")
    ref = str(item.get("source_reference") or "")
    blob = f"{desc} {cat} {method} {ref}"

            if is_distinct_traffic_pay_item(item):
        return False
    if is_generic_traffic_control_signing(item):
        return False
    if _SIGN_NEGATIVE.search(blob) and not _SIGN_POSITIVE.search(desc):
        return False
    if _SIGN_NEGATIVE.search(desc):
        return False
    if extract_mutcd_code(desc, ref):
        return True
    if _SIGN_POSITIVE.search(desc):
        return True
    unit = str(item.get("unit") or "").lower()
    if unit in {"each", "ea", "nos"} and re.search(r"\bsign", desc, re.I):
        return True
    if parse_sign_size_inches(desc) and re.search(
        r"sign|traffic|mutcd|warning|regulatory|guide|signing", blob, re.I
    ):
        return True
    return False

        
        def resolve_sign_area_sqft(
    item: dict[str, Any],
    *,
    allow_online_refresh: bool = False,
) -> dict[str, Any]:
    """Resolve one sign face area in SqFt with size provenance."""
    desc = str(item.get("description") or "")
    extras = [
        item.get("size"),
        item.get("source_reference"),
        item.get("calculation_method"),
        item.get("notes"),
    ]
    qty = float(item.get("quantity") or 1) or 1.0
    # If already SqFt and description is a single sign type, trust quantity as total SF for that row
    unit = str(item.get("unit") or "").lower().replace(".", "").replace(" ", "")
    if unit in {"sqft", "sf", "squarefoot", "squarefeet"} and qty > 0:
        return {
            "sqft": round(qty, 4),
            "count": 1.0,
            "width_in": None,
            "height_in": None,
            "size_source": "item_sqft",
            "code": extract_mutcd_code(desc),
            "description": desc,
        }

            plan_size = parse_sign_size_inches(desc, *extras)
    code = extract_mutcd_code(desc, *extras)
    mutcd = lookup_mutcd_size(
        code=code,
        description=desc,
        allow_online_refresh=allow_online_refresh,
    )

            width_in: float | None = None
    height_in: float | None = None
    size_source = "default_mutcd_30x30"

            if plan_size:
        width_in, height_in = plan_size
        size_source = "plan_or_dwg_callout"
    elif mutcd:
        width_in = float(mutcd["width_in"])
        height_in = float(mutcd["height_in"])
        size_source = f"mutcd:{mutcd.get('code')}"
        code = mutcd.get("code") or code
    else:
        catalog = _load_mutcd()
        default = catalog.get("default_unknown_sign_in") or {"width_in": 30, "height_in": 30}
        width_in = float(default["width_in"])
        height_in = float(default["height_in"])
        size_source = "default_conventional_30x30"

            # Count: Each/EA quantity = number of faces. Inches are the face size, never the pay unit.
    unit_l = str(item.get("unit") or "").lower().replace(".", "")
    if unit_l in {"each", "ea", "nos", "no", "unit", ""}:
        count = qty
    else:
        count = 1.0
    if count <= 0:
        count = 1.0
    face_sf = inches_to_sqft(width_in, height_in)
    formula = f"{width_in:g} in × {height_in:g} in = {face_sf:g} SqFt (in²÷144)"
    if count != 1.0:
        formula = f"{count:g} × {formula}"
    return {
        "sqft": round(face_sf * count, 4),
        "count": count,
        "width_in": width_in,
        "height_in": height_in,
        "face_sqft": face_sf,
        "formula": formula,
        "size_source": size_source,
        "code": code,
        "description": desc,
    }

        
        def _prefer_schedule_tc_row(existing: dict[str, Any], candidate: dict[str, Any]) -> dict[str, Any]:
    """Keep the bid-schedule Traffic Control SqFt row over a MUTCD-computed one."""
    def score(row: dict[str, Any]) -> tuple[int, float]:
        pts = 0
        if looks_like_agency_bid_number(row.get("item_code")):
            pts += 8
        blob = _evidence_blob(row)
        if any(h in blob for h in ("estimate of quantities", "bid item", "est. qty", "eoq schedule", "std bid")):
            pts += 6
        if is_plan_device_takeoff(row):
            pts -= 8
        try:
            conf = float(row.get("confidence") or 0)
        except (TypeError, ValueError):
            conf = 0.0
        return pts, conf

            return candidate if score(candidate) > score(existing) else existing

        
        def _filter_plan_invented_when_schedule(
    kept: list[dict[str, Any]],
) -> list[dict[str, Any]]:
    """Drop F-sheet extras when a real Traffic Control bid section was captured."""
    has_gravel_ls = any(
        re.search(r"gravel\s+access", str(i.get("description") or ""), re.I)
        and str(i.get("unit") or "").lower() in {"ls", "lump sum"}
        for i in kept
    )
    out: list[dict[str, Any]] = []
    for item in kept:
        desc = str(item.get("description") or "")
        unit = str(item.get("unit") or "").lower()
        planish = is_plan_device_takeoff(item) and not looks_like_agency_bid_number(item.get("item_code"))
        if planish and re.search(r"channeliz", desc, re.I):
            continue
        if has_gravel_ls and re.search(r"gravel\s+access", desc, re.I) and unit not in {"ls", "lump sum"}:
            continue
        out.append(item)

            barricades = [i for i in out if re.search(r"barricade", str(i.get("description") or ""), re.I)]
    if len(barricades) > 1:
        best = barricades[0]
        for b in barricades[1:]:
            best = _prefer_schedule_tc_row(best, b)
        out = [
            i
            for i in out
            if i is best or not re.search(r"barricade", str(i.get("description") or ""), re.I)
        ]
    return out

        
        def consolidate_traffic_control_signs(
    items: list[dict[str, Any]],
    *,
    allow_online_refresh: bool = True,
    schedule_present: bool = False,
) -> tuple[list[dict[str, Any]], dict[str, Any]]:
    """Schedule Traffic Control pay items stay intact; MUTCD plan signs roll to SqFt only if no schedule."""
    if not items:
        return items, {"sign_rows": 0, "total_sqft": 0.0}

            if allow_online_refresh and not schedule_present:
        try:
            refresh_mutcd_from_online()
        except Exception:  # noqa: BLE001
            pass

            kept: list[dict[str, Any]] = []
    existing_tc: dict[str, Any] | None = None
    sign_details: list[dict[str, Any]] = []
    dropped_plan_signs = 0

            for raw in items:
        item = dict(raw)
        if is_distinct_traffic_pay_item(item):
            kept.append(item)
            continue

                if is_generic_traffic_control_signing(item):
            if _unit_is_sqft(item.get("unit")):
                existing_tc = _prefer_schedule_tc_row(existing_tc, item) if existing_tc else item
            elif schedule_present:
                # LS/Each "Traffic Control" without "Miscellaneous" — keep if it has a bid number
                if looks_like_agency_bid_number(item.get("item_code")):
                    kept.append(item)
                elif existing_tc is None:
                    existing_tc = item
            else:
                existing_tc = item
            continue

                if is_traffic_sign_item(item):
            if schedule_present:
                dropped_plan_signs += 1
                continue
            detail = resolve_sign_area_sqft(item, allow_online_refresh=False)
            sign_details.append(detail)
            continue

                kept.append(item)

            meta = {
        "sign_rows": len(sign_details) + dropped_plan_signs,
        "dropped_plan_signs": dropped_plan_signs,
        "total_sqft": 0.0,
        "details": sign_details[:80],
        "mutcd_source": (_load_mutcd() or {}).get("source_url"),
        "schedule_present": schedule_present,
    }

            if schedule_present:
        kept = _filter_plan_invented_when_schedule(kept)
        if existing_tc is not None:
            if _unit_is_sqft(existing_tc.get("unit")):
                existing_tc = dict(existing_tc)
                existing_tc["unit"] = "SqFt"
                existing_tc["category"] = existing_tc.get("category") or "Traffic Control"
                kept.append(existing_tc)
                try:
                    meta["total_sqft"] = round(float(existing_tc.get("quantity") or 0), 2)
                except (TypeError, ValueError):
                    meta["total_sqft"] = 0.0
            else:
                kept.append(existing_tc)
        return kept, meta

            total_sqft = 0.0
    if existing_tc is not None and _unit_is_sqft(existing_tc.get("unit")):
        try:
            total_sqft += float(existing_tc.get("quantity") or 0)
        except (TypeError, ValueError):
            pass
    for detail in sign_details:
        total_sqft += float(detail.get("sqft") or 0)
    meta["total_sqft"] = round(total_sqft, 2)

            if not sign_details and existing_tc is None:
        return items, meta
    if total_sqft <= 0 and not sign_details:
        if existing_tc:
            kept.append(existing_tc)
        return kept, meta

            if sign_details:
        sources = sorted({d.get("size_source") or "" for d in sign_details if d.get("size_source")})
        formulas = [str(d.get("formula")) for d in sign_details if d.get("formula")]
        method = (
            f"Consolidated {len(sign_details)} traffic sign(s) into SqFt of sign face. "
            f"USA pay unit is square feet, not inches: width(in)×height(in)÷144. "
            f"Size sources: {', '.join(s for s in sources if s) or 'n/a'}."
        )
        if formulas:
            method += " " + "; ".join(formulas[:8])
            if len(formulas) > 8:
                method += f" (+{len(formulas) - 8} more)"
        codes = [d.get("code") for d in sign_details if d.get("code")]
        ref_bits = []
        if codes:
            ref_bits.append("codes " + ", ".join(sorted(set(str(c) for c in codes))[:12]))
        if meta.get("mutcd_source"):
            ref_bits.append(f"MUTCD ref {meta['mutcd_source']}")

                from app.services.csi_mapper import enrich_quantity_item

                src_doc = (existing_tc or {}).get("source_document_id")
        if src_doc is None:
            src_doc = next((i.get("source_document_id") for i in items if i.get("source_document_id")), None)

                rolled = {
            "item_code": (existing_tc or {}).get("item_code")
            if looks_like_agency_bid_number((existing_tc or {}).get("item_code"))
            else (existing_tc or {}).get("item_code"),
            "description": "Traffic Control",
            "category": "Traffic Control",
            "unit": "SqFt",
            "quantity": round(total_sqft, 2),
            "source_document_id": src_doc,
            "source_page": (existing_tc or {}).get("source_page"),
            "source_reference": "; ".join(ref_bits) or "Traffic signing takeoff",
            "calculation_method": method
            if not (existing_tc and _unit_is_sqft(existing_tc.get("unit")) and not is_plan_device_takeoff(existing_tc))
            else str(existing_tc.get("calculation_method") or method),
            "confidence": 90.0
            if any((d.get("size_source") or "").startswith(("plan", "mutcd")) for d in sign_details)
            else 82.0,
            "status": "needs_review",
            "traffic_control_breakdown": sign_details,
        }
        rolled = enrich_quantity_item(rolled)
        rolled["unit"] = "SqFt"
        kept.append(rolled)
        meta["rolled_into"] = "Traffic Control"
        return kept, meta

            if existing_tc:
        kept.append(existing_tc)
    return kept, meta

### backend/app/data/mutcd_sign_sizes.json

SHA-256: 7bcd63567ede8698b5aa1433331d0c1163ae8f606d19be63420d3d297f389399    ·    149 line(s)    ·    .json

{
  "source": "FHWA MUTCD 11th Edition (conventional road / single-lane defaults)",
  "source_url": "https://mutcd.fhwa.dot.gov/kno_11th_Editionr1.htm",
  "standard_highway_signs_url": "https://mutcd.fhwa.dot.gov/shsm_interim/index.htm",
  "unit": "inches",
  "roadway_column": "conventional_single_lane",
  "notes": "Sizes are width x height in inches. Yield triangles use equal side length; area uses W×H panel envelope / 144 for SqFt pay quantities.",
  "signs": {
    "R1-1": {"name": "STOP", "width_in": 30, "height_in": 30},
    "R1-2": {"name": "Yield", "width_in": 36, "height_in": 36, "shape": "triangle"},
    "R1-3P": {"name": "ALL WAY plaque", "width_in": 18, "height_in": 6},
    "R2-1": {"name": "Speed Limit", "width_in": 24, "height_in": 30},
    "R2-4": {"name": "Minimum Speed Limit", "width_in": 24, "height_in": 30},
    "R2-5P": {"name": "FINES HIGHER plaque", "width_in": 24, "height_in": 18},
    "R3-1": {"name": "No Right Turn", "width_in": 24, "height_in": 24},
    "R3-2": {"name": "No Left Turn", "width_in": 24, "height_in": 24},
    "R3-4": {"name": "No U-Turn", "width_in": 24, "height_in": 24},
    "R3-5": {"name": "Mandatory Movement Lane Control", "width_in": 30, "height_in": 36},
    "R3-7": {"name": "Lane Control", "width_in": 30, "height_in": 30},
    "R3-8": {"name": "Advance Intersection Lane Control", "width_in": 30, "height_in": 30},
    "R4-1": {"name": "Do Not Pass", "width_in": 24, "height_in": 30},
    "R4-7": {"name": "Keep Right", "width_in": 24, "height_in": 30},
    "R5-1": {"name": "Do Not Enter", "width_in": 30, "height_in": 30},
    "R5-1a": {"name": "WRONG WAY", "width_in": 36, "height_in": 24},
    "R6-1": {"name": "One Way", "width_in": 36, "height_in": 12},
    "R6-2": {"name": "One Way", "width_in": 18, "height_in": 24},
    "R7-1": {"name": "No Parking", "width_in": 12, "height_in": 18},
    "R7-8": {"name": "Reserved Parking", "width_in": 12, "height_in": 18},
    "R8-3": {"name": "No Parking Symbol", "width_in": 24, "height_in": 24},
    "R9-3a": {"name": "No Pedestrian Crossing", "width_in": 18, "height_in": 18},
    "R10-1": {"name": "Pedestrian Signal Instructions", "width_in": 12, "height_in": 18},
    "R10-3": {"name": "Pedestrian Push Button", "width_in": 9, "height_in": 12},
    "R10-6": {"name": "STOP HERE ON RED", "width_in": 24, "height_in": 30},
    "R10-11": {"name": "NO TURN ON RED", "width_in": 24, "height_in": 30},
    "R10-12": {"name": "LEFT TURN YIELD ON GREEN", "width_in": 30, "height_in": 36},
    "R11-1": {"name": "Road Closed", "width_in": 48, "height_in": 30},
    "R11-2": {"name": "Road Closed — Local Traffic Only", "width_in": 60, "height_in": 30},
    "R11-3": {"name": "Road Closed Ahead", "width_in": 60, "height_in": 30},
    "R11-4": {"name": "Road Closed — Detour", "width_in": 60, "height_in": 30},
    "W1-1": {"name": "Turn", "width_in": 30, "height_in": 30},
    "W1-2": {"name": "Curve", "width_in": 30, "height_in": 30},
    "W1-3": {"name": "Reverse Turn", "width_in": 30, "height_in": 30},
    "W1-4": {"name": "Reverse Curve", "width_in": 30, "height_in": 30},
    "W1-5": {"name": "Winding Road", "width_in": 30, "height_in": 30},
    "W1-6": {"name": "Large Arrow", "width_in": 48, "height_in": 24},
    "W1-7": {"name": "Two-Direction Large Arrow", "width_in": 48, "height_in": 24},
    "W1-8": {"name": "Chevron Alignment", "width_in": 18, "height_in": 24},
    "W1-10": {"name": "Combination Horizontal Alignment / Intersection", "width_in": 36, "height_in": 36},
    "W1-11": {"name": "Hairpin Curve", "width_in": 30, "height_in": 30},
    "W1-13": {"name": "Truck Rollover Warning", "width_in": 36, "height_in": 36},
    "W2-1": {"name": "Cross Road", "width_in": 30, "height_in": 30},
    "W2-2": {"name": "Side Road", "width_in": 30, "height_in": 30},
    "W2-3": {"name": "T Intersection", "width_in": 30, "height_in": 30},
    "W2-4": {"name": "Y Intersection", "width_in": 30, "height_in": 30},
    "W2-5": {"name": "Circular Intersection", "width_in": 30, "height_in": 30},
    "W3-1": {"name": "Stop Ahead", "width_in": 30, "height_in": 30},
    "W3-2": {"name": "Yield Ahead", "width_in": 30, "height_in": 30},
    "W3-3": {"name": "Signal Ahead", "width_in": 30, "height_in": 30},
    "W3-5": {"name": "Speed Reduction", "width_in": 36, "height_in": 36},
    "W4-1": {"name": "Merge", "width_in": 36, "height_in": 36},
    "W4-2": {"name": "Lane Ends", "width_in": 36, "height_in": 36},
    "W4-3": {"name": "Added Lane", "width_in": 36, "height_in": 36},
    "W5-1": {"name": "Road Narrows", "width_in": 36, "height_in": 36},
    "W5-2": {"name": "Narrow Bridge", "width_in": 36, "height_in": 36},
    "W5-3": {"name": "One Lane Bridge", "width_in": 36, "height_in": 36},
    "W6-1": {"name": "Divided Highway", "width_in": 36, "height_in": 36},
    "W6-2": {"name": "Divided Highway Ends", "width_in": 36, "height_in": 36},
    "W6-3": {"name": "Two-Way Traffic", "width_in": 30, "height_in": 30},
    "W7-1": {"name": "Hill", "width_in": 30, "height_in": 30},
    "W8-1": {"name": "Bump", "width_in": 30, "height_in": 30},
    "W8-2": {"name": "Dip", "width_in": 30, "height_in": 30},
    "W8-3": {"name": "Pavement Ends", "width_in": 30, "height_in": 30},
    "W8-5": {"name": "Slippery When Wet", "width_in": 30, "height_in": 30},
    "W8-9": {"name": "Low Shoulder", "width_in": 30, "height_in": 30},
    "W8-17": {"name": "Shoulder Drop Off", "width_in": 30, "height_in": 30},
    "W9-1": {"name": "Right Lane Ends", "width_in": 36, "height_in": 36},
    "W9-2": {"name": "Lane Ends Merge Left", "width_in": 36, "height_in": 36},
    "W10-1": {"name": "Railroad Crossing Advance", "width_in": 36, "height_in": 36},
    "W11-1": {"name": "Bicycle", "width_in": 30, "height_in": 30},
    "W11-2": {"name": "Pedestrian", "width_in": 30, "height_in": 30},
    "W11-3": {"name": "Deer Crossing", "width_in": 30, "height_in": 30},
    "W14-1": {"name": "Dead End", "width_in": 30, "height_in": 30},
    "W14-2": {"name": "No Outlet", "width_in": 30, "height_in": 30},
    "W16-7P": {"name": "Diagonal Arrow plaque", "width_in": 24, "height_in": 12},
    "W16-9P": {"name": "Ahead plaque", "width_in": 24, "height_in": 12},
    "W20-1": {"name": "Road Work Ahead", "width_in": 36, "height_in": 36},
    "W20-2": {"name": "Bridge Work Ahead", "width_in": 36, "height_in": 36},
    "W20-3": {"name": "Shoulder Work Ahead", "width_in": 36, "height_in": 36},
    "W20-4": {"name": "One Lane Road Ahead", "width_in": 36, "height_in": 36},
    "W20-5": {"name": "Lane Closed Ahead", "width_in": 36, "height_in": 36},
    "W20-7": {"name": "Flagger Ahead", "width_in": 36, "height_in": 36},
    "W21-1": {"name": "Workers", "width_in": 36, "height_in": 36},
    "W21-5": {"name": "Shoulder Work", "width_in": 36, "height_in": 36},
    "G20-1": {"name": "Road Work Next XX Miles", "width_in": 36, "height_in": 18},
    "G20-2": {"name": "End Road Work", "width_in": 36, "height_in": 18},
    "S1-1": {"name": "School", "width_in": 36, "height_in": 36},
    "S3-1": {"name": "School Bus Stop Ahead", "width_in": 36, "height_in": 36},
    "S4-3P": {"name": "SCHOOL plaque", "width_in": 24, "height_in": 8},
    "S4-5": {"name": "Reduced School Speed Limit Ahead", "width_in": 36, "height_in": 36},
    "S5-1": {"name": "School Speed Limit When Flashing", "width_in": 24, "height_in": 48},
    "S5-2": {"name": "End School Zone", "width_in": 24, "height_in": 30},
    "M1-1": {"name": "Interstate Route Marker", "width_in": 24, "height_in": 24},
    "M1-4": {"name": "US Route Marker", "width_in": 24, "height_in": 24},
    "M1-5": {"name": "State Route Marker", "width_in": 24, "height_in": 24},
    "M3-1": {"name": "NORTH", "width_in": 24, "height_in": 12},
    "M4-5": {"name": "TO", "width_in": 24, "height_in": 12},
    "M6-1": {"name": "Directional Arrow", "width_in": 21, "height_in": 15},
    "D1-1": {"name": "Destination", "width_in": 48, "height_in": 18},
    "D3-1": {"name": "Street Name", "width_in": 36, "height_in": 9}
  },
  "name_aliases": {
    "stop": "R1-1",
    "stop sign": "R1-1",
    "yield": "R1-2",
    "yield sign": "R1-2",
    "speed limit": "R2-1",
    "speed limit sign": "R2-1",
    "do not enter": "R5-1",
    "wrong way": "R5-1a",
    "one way": "R6-1",
    "no parking": "R8-3",
    "no right turn": "R3-1",
    "no left turn": "R3-2",
    "no u-turn": "R3-4",
    "no u turn": "R3-4",
    "keep right": "R4-7",
    "do not pass": "R4-1",
    "road closed": "R11-1",
    "road work ahead": "W20-1",
    "end road work": "G20-2",
    "flagger": "W20-7",
    "flagger ahead": "W20-7",
    "workers": "W21-1",
    "worker": "W21-1",
    "merge": "W4-1",
    "lane ends": "W4-2",
    "pedestrian": "W11-2",
    "pedestrian crossing": "W11-2",
    "school": "S1-1",
    "school crossing": "S1-1",
    "curve": "W1-2",
    "turn": "W1-1",
    "chevron": "W1-8",
    "stop ahead": "W3-1",
    "yield ahead": "W3-2",
    "signal ahead": "W3-3"
  },
  "default_unknown_sign_in": {"width_in": 30, "height_in": 30}
}

### backend/app/services/civil_estimator.py

SHA-256: 597d4d5574445792968326d77c3659e0cd19d41feb4f341bba724acca5513e48    ·    484 line(s)    ·    .py

"""General civil estimator — EOQ from any design (road, utility, dam, reservoir, building).

        Used when drawings/CAD exist. Formulas follow common QS practice:

        * Trench excavation CY = trench_width × trench_depth × length / 27
* Bedding CY = trench_width × bedding_thickness × length / 27
* Pavement layer CY = width × thickness × length / 27  (HMA also as tons @ 145 pcf)
* Areas from hatch/closed polyline layers
* Counts from named blocks (doors, windows, lights, trees, …)

        Assumptions are written into calculation_method so an engineer can review them.
"""

        from __future__ import annotations

        import math
import re
from collections import defaultdict
from typing import Any

        from app.services.incidental import cad_notes_make_trench_incidental, extraction_should_skip

        # --- Project type -----------------------------------------------------------

        _TYPE_HINTS: list[tuple[str, tuple[str, ...]]] = [
    ("dam", ("dam", "spillway", "gallery", "abutment", "crest", "cofferdam", "stilling basin")),
    ("reservoir", ("reservoir", "impoundment", "pond lining", "tank farm", "clearwell", "storage tank")),
    ("building", ("building", "house", "residence", "dwelling", "apartment", "floor plan", "architectural")),
    ("road", ("roadway", "highway", "pavement", "typical section", "carriageway", "asphalt", "hma")),
    ("utility", ("watermain", "water main", "sanitary", "storm sewer", "force main")),
]

        
        def detect_project_types(*blobs: Any) -> list[str]:
    text = " ".join(str(b or "") for b in blobs).lower()
    found: list[str] = []
    for kind, keys in _TYPE_HINTS:
        if any(k in text for k in keys):
            found.append(kind)
    return found or ["civil"]

        
        # --- Typical-section / text takeoff ----------------------------------------

        _THICK_IN = re.compile(
    r"(?P<name>GSB|WMM|DBM|BC|HMA|PCC|sub[- ]?base|base\s*course|binder|surface\s*course|"
    r"aggregate\s*base|asphalt|concrete\s*pavement|sidewalk|curb)"
    r"[^\n]{0,40}?(?P<th>\d+(?:\.\d+)?)\s*(?P<u>mm|cm|in(?:ch(?:es)?)?|\"|ft|m)\b",
    re.I,
)
_WIDTH = re.compile(
    r"(?:road|carriageway|pavement|roadway|crest|dam)\s*width[^\d]{0,16}(?P<w>\d+(?:\.\d+)?)\s*(?P<u>m|ft|')?",
    re.I,
)
_LENGTH = re.compile(
    r"(?:length|alignment\s*length|road\s*length|crest\s*length)[^\d]{0,16}"
    r"(?P<l>\d{2,6}(?:\.\d+)?)\s*(?P<u>m|ft|km|mi)?",
    re.I,
)
_STA_RANGE = re.compile(
    r"(?P<a>\d{1,4}\+\d{2}(?:\.\d+)?)\s*(?:to|[-–—])\s*(?P<b>\d{1,4}\+\d{2}(?:\.\d+)?)",
    re.I,
)

        _BUILDING = [
    ("Brickwork", "Building", "CY", r"\bbrick(?:work)?\b[^\n.;\d]{0,40}(\d{1,6}(?:\.\d+)?)\s*(m3|m³|cy|cu\.?\s*yd)"),
    ("RCC Concrete", "Building", "CY", r"\b(?:rcc|reinforced\s*concrete)\b[^\n.;\d]{0,40}(\d{1,6}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Foundation Concrete", "Building", "CY", r"\bfoundation(?:s)?\b[^\n.;\d]{0,40}(\d{1,6}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Doors", "Building", "EA", r"\bdoors?\b[^\n.;\d]{0,40}(\d{1,4})\s*(nos?|ea|each)"),
    ("Windows", "Building", "EA", r"\bwindows?\b[^\n.;\d]{0,40}(\d{1,4})\s*(nos?|ea|each)"),
    ("Plastering", "Building", "SF", r"\bplaster(?:ing)?\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m2|m²|sf|sft)"),
    ("Flooring", "Building", "SF", r"\bfloor(?:ing)?\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m2|m²|sf|sft)"),
    ("Roofing", "Building", "SF", r"\broof(?:ing)?\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m2|m²|sf|sft)"),
    ("Formwork", "Building", "SF", r"\bformwork\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m2|m²|sf|sft)"),
    ("Painting", "Building", "SF", r"\bpaint(?:ing)?\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m2|m²|sf|sft)"),
]
_DAM = [
    ("Dam Embankment Fill", "Dams & Reservoirs", "CY", r"\b(?:dam\s+)?embankment\b[^\n.;\d]{0,40}(\d{1,8}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Dam Excavation", "Dams & Reservoirs", "CY", r"\b(?:dam|foundation)\s*excavation\b[^\n.;\d]{0,40}(\d{1,8}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Spillway Concrete", "Dams & Reservoirs", "CY", r"\bspillway\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Riprap", "Dams & Reservoirs", "CY", r"\briprap\b[^\n.;\d]{0,40}(\d{1,7}(?:\.\d+)?)\s*(m3|m³|cy|ton)"),
    ("Reservoir Lining", "Dams & Reservoirs", "SF", r"\b(?:reservoir|pond|tank)\s*lin(?:ing|er)\b[^\n.;\d]{0,40}(\d{1,8}(?:\.\d+)?)\s*(m2|m²|sf)"),
    ("Cutoff Trench", "Dams & Reservoirs", "CY", r"\bcutoff(?:\s+trench)?\b[^\n.;\d]{0,40}(\d{1,8}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Filter / Drain Material", "Dams & Reservoirs", "CY", r"\b(?:filter\s*(?:material|zone)|chimney\s*drain)\b[^\n.;\d]{0,40}(\d{1,8}(?:\.\d+)?)\s*(m3|m³|cy)"),
    ("Reservoir Capacity", "Dams & Reservoirs", "MGAL", r"\bcapacit(?:y|ies)\b[^\n.;\d]{0,40}(\d{1,6}(?:\.\d+)?)\s*(mgal|million\s*gal|acre[- ]?ft|ml)"),
]
_SHOULDER = re.compile(
    r"shoulder[^\d]{0,20}(?P<w>\d+(?:\.\d+)?)\s*(?P<u>m|ft|'|in)?",
    re.I,
)

        
        def _to_feet(val: float, unit: str | None) -> float:
    u = (unit or "ft").lower().replace("'", "ft")
    if u in {"mm"}:
        return val / 304.8
    if u in {"cm"}:
        return val / 30.48
    if u in {"m", "meter", "metre"}:
        return val * 3.280839895
    if u in {"km"}:
        return val * 3280.839895
    if u in {"mi", "mile"}:
        return val * 5280.0
    if u in {"in", "inch", "inches", '"'}:
        return val / 12.0
    return val

        
        def _sta_feet(sta: str) -> float | None:
    m = re.match(r"(\d+)\+(\d+(?:\.\d+)?)", sta.strip())
    if not m:
        return None
    return float(m.group(1)) * 100.0 + float(m.group(2))

        
        def items_from_design_text(text: str, *, filename: str = "plan") -> list[dict[str, Any]]:
    """Quantity lines from typical sections, BOQ-like sentences, and civil callouts."""
    if not (text or "").strip():
        return []
    items: list[dict[str, Any]] = []
    width_ft = None
    wm = _WIDTH.search(text)
    if wm:
        width_ft = _to_feet(float(wm.group("w")), wm.group("u"))

            length_ft = None
    lm = _LENGTH.search(text)
    if lm:
        length_ft = _to_feet(float(lm.group("l")), lm.group("u"))
    sm = _STA_RANGE.search(text)
    if sm and length_ft is None:
        a, b = _sta_feet(sm.group("a")), _sta_feet(sm.group("b"))
        if a is not None and b is not None:
            length_ft = abs(b - a)

            # Pavement layers from typical section
    if width_ft and length_ft and width_ft > 2 and length_ft > 20:
        seen_layer: set[str] = set()
        for m in _THICK_IN.finditer(text):
            name = m.group("name").strip()
            key = name.lower()
            if key in seen_layer:
                continue
            seen_layer.add(key)
            th_ft = _to_feet(float(m.group("th")), m.group("u"))
            if th_ft <= 0 or th_ft > 4:
                continue
            cy = width_ft * th_ft * length_ft / 27.0
            label = _layer_label(name)
            items.append(
                _qty(
                    description=label,
                    category="Surfacing",
                    unit="CY",
                    quantity=round(cy, 2),
                    method=(
                        f"Typical section: {width_ft:.1f}' wide × {th_ft * 12:.1f}\" thick × "
                        f"{length_ft:.0f}' long / 27 (from '{filename}')"
                    ),
                    confidence=82.0,
                )
            )
            if any(k in key for k in ("hma", "asphalt", "dbm", "bc", "bituminous", "binder", "surface")):
                tons = cy * 145.0 / 2000.0  # ~145 pcf HMA
                items.append(
                    _qty(
                        description=f"{label} (HMA tons)",
                        category="Surfacing",
                        unit="TON",
                        quantity=round(tons, 2),
                        method=f"HMA tons from CY × 145 pcf / 2000 ({filename})",
                        confidence=78.0,
                    )
                )
        sy = width_ft * length_ft / 9.0
        low = text.lower()
        if "prime" in low:
            items.append(
                _qty(
                    "Prime Coat",
                    "Surfacing",
                    "SY",
                    round(sy, 2),
                    f"Typical section area {width_ft:.1f}' × {length_ft:.0f}' / 9 ({filename})",
                    74.0,
                )
            )
        if "tack" in low:
            items.append(
                _qty(
                    "Tack Coat",
                    "Surfacing",
                    "SY",
                    round(sy, 2),
                    f"Typical section area {width_ft:.1f}' × {length_ft:.0f}' / 9 ({filename})",
                    74.0,
                )
            )
        sm = _SHOULDER.search(text)
        if sm:
            sh_ft = _to_feet(float(sm.group("w")), sm.group("u"))
            if 1.0 < sh_ft < 20.0:
                items.append(
                    _qty(
                        "Shoulder Area",
                        "Surfacing",
                        "SF",
                        round(sh_ft * length_ft * 2.0, 2),
                        f"2 shoulders × {sh_ft:.1f}' × {length_ft:.0f}' ({filename})",
                        72.0,
                    )
                )

            combined = _BUILDING + _DAM
    for desc, cat, unit, pat in combined:
        qty_val: float | None = None
        raw_u = unit
        for m in re.finditer(pat, text, re.I | re.S):
            if extraction_should_skip(desc, text, m.start()):
                continue
            try:
                qty_val = float(str(m.group(1)).replace(",", ""))
            except (TypeError, ValueError):
                continue
            raw_u = (m.group(2) if m.lastindex and m.lastindex >= 2 else unit) or unit
            break
        if qty_val is None:
            continue
        items.append(
            _qty(
                description=desc,
                category=cat,
                unit=_norm_unit(raw_u, unit),
                quantity=qty_val,
                method=f"Design text quantity on '{filename}'",
                confidence=80.0,
            )
        )
    return items

        
        def _layer_label(name: str) -> str:
    n = name.lower()
    mapping = {
        "gsb": "Granular Sub-Base (GSB)",
        "wmm": "Wet Mix Macadam (WMM)",
        "dbm": "Dense Bituminous Macadam (DBM)",
        "bc": "Bituminous Concrete (BC)",
        "hma": "Hot Mix Asphalt",
        "pcc": "PCC Pavement",
        "subbase": "Aggregate Subbase",
        "sub-base": "Aggregate Subbase",
        "base course": "Aggregate Base Course",
        "aggregate base": "Aggregate Base Course",
        "binder": "Asphalt Binder Course",
        "surface course": "Asphalt Surface Course",
        "asphalt": "Asphalt Pavement",
        "concrete pavement": "Concrete Pavement",
        "sidewalk": "Concrete Sidewalk",
        "curb": "Concrete Curb",
    }
    for k, v in mapping.items():
        if k in n:
            return v
    return name.title()

        
        def _norm_unit(raw: str, fallback: str) -> str:
    r = (raw or fallback or "unit").lower()
    if r in {"m3", "m³", "cy", "cu.yd", "cu yd"}:
        return "CY"
    if r in {"m2", "m²", "sf", "sft"}:
        return "SF"
    if r in {"ton", "tons", "t"}:
        return "TON"
    if r in {"nos", "no", "ea", "each"}:
        return "EA"
    if r in {"mgal", "million gal"}:
        return "MGAL"
    if r in {"acre-ft", "acre ft"}:
        return "AC-FT"
    return fallback.upper()

        
        # --- Trench from pipes (CAD) ------------------------------------------------

        def trench_items_from_pipes(extraction: dict[str, Any]) -> list[dict[str, Any]]:
    """Standard trench pay items from pipe length + diameter.

            Width = max(OD + 2.0 ft, 2.5 ft). Cover defaults to 4.0 ft if unknown.
    Bedding = 6 inches. Pipe displacement subtracted from backfill.
    """
    groups: dict[tuple[str, str], dict[str, float]] = defaultdict(lambda: {"lf": 0.0, "od": 0.0, "n": 0})
    pipes = list(extraction.get("pipes") or [])
    # Prefer Civil pipe objects so centerline polylines do not double trench CY.
    pipe_rows = pipes if pipes else list(extraction.get("polylines") or [])
    for pipe in pipe_rows:
        layer = str(pipe.get("layer") or "")
        name = str(pipe.get("name") or "")
        blob = f"{name} {layer} {pipe.get('network') or ''} {pipe.get('description') or ''}".lower()
        if not any(k in blob for k in ("pipe", "water", "sewer", "storm", "san", "main", "casing")):
            continue
        try:
            length = float(pipe.get("length") or 0)
        except (TypeError, ValueError):
            length = 0.0
        if length < 1:
            continue
        od_ft = _pipe_od_ft(pipe)
        net = "utility"
        if "water" in blob:
            net = "water"
        elif "sanitary" in blob or ("sewer" in blob and "storm" not in blob):
            net = "sanitary"
        elif "storm" in blob:
            net = "storm"
        elif "casing" in blob:
            net = "casing"
        key = (net, f"{od_ft:.2f}")
        groups[key]["lf"] += length
        groups[key]["od"] += od_ft
        groups[key]["n"] += 1

            items: list[dict[str, Any]] = []
    cover_ft = 4.0
    bedding_ft = 0.5
    for (net, _odk), g in groups.items():
        lf = g["lf"]
        od = g["od"] / max(g["n"], 1)
        width = max(od + 2.0, 2.5)
        depth = cover_ft + od + bedding_ft
        excav_cy = width * depth * lf / 27.0
        bed_cy = width * bedding_ft * lf / 27.0
        pipe_cy = math.pi * (od / 2.0) ** 2 * lf / 27.0
        backfill_cy = max(0.0, excav_cy - bed_cy - pipe_cy)
        label = {
            "water": "Watermain",
            "sanitary": "Sanitary Sewer",
            "storm": "Storm Sewer",
            "casing": "Casing",
        }.get(net, "Utility")
        method = (
            f"Trench from {label} pipes: width {width:.1f}' (= OD+2' min 2.5'), "
            f"cover assumed {cover_ft:.0f}', bedding {bedding_ft * 12:.0f}\", L={lf:.0f} LF"
        )
        items.append(_qty(f"{label} Trench Excavation", "Grading", "CY", round(excav_cy, 2), method, 76.0))
        items.append(_qty(f"{label} Pipe Bedding", "Grading", "CY", round(bed_cy, 2), method, 76.0))
        items.append(_qty(f"{label} Trench Backfill", "Grading", "CY", round(backfill_cy, 2), method, 76.0))
    return items

        
        def _pipe_od_ft(pipe: dict[str, Any]) -> float:
    for key in ("diameter", "inner_diameter", "outer_diameter", "size"):
        try:
            d = float(pipe.get(key) or 0)
        except (TypeError, ValueError):
            d = 0.0
        if d <= 0:
            continue
        if d > 20:  # already inches-ish large; treat as inches if < 80
            if d < 80:
                return d / 12.0
            return d  # feet
        if d < 8:  # feet (Civil often stores 0.67 ft = 8")
            return d
        return d / 12.0  # inches
    # parse from name
    m = re.search(r"(\d{1,2}(?:\.\d+)?)\s*(?:\"|in)", str(pipe.get("name") or pipe.get("layer") or ""), re.I)
    if m:
        return float(m.group(1)) / 12.0
    return 1.0  # 12" default

        
        # --- Extra CAD layers (buildings, dams, roads) ------------------------------

        _LAYER_LENGTH = (
    (("curb", "kerb", "gutter"), "Concrete Curb and Gutter", "Curb, Gutter & Sidewalk", "LF"),
    (("sidewalk", "walkway", "footpath"), "Concrete Sidewalk", "Curb, Gutter & Sidewalk", "LF"),
    (("guardrail", "barrier"), "Guardrail", "Miscellaneous", "LF"),
    (("fence",), "Fence", "Miscellaneous", "LF"),
    (("a-wall", "partition", "wall-"), "Wall", "Building", "LF"),
    (("crest",), "Dam Crest", "Dams & Reservoirs", "LF"),
    (("spillway",), "Spillway", "Dams & Reservoirs", "LF"),
    (("ditch", "swale"), "Roadside Ditch", "Storm Sewer", "LF"),
)
_BLOCK_COUNTS = (
    (("door",), "Doors", "Building"),
    (("window", "glaz"), "Windows", "Building"),
    (("column", "pillar"), "Columns", "Building"),
    (("pile",), "Piles", "Structures"),
    (("luminaire", "lightpole", "light pole", "streetlight", "street light"), "Light Poles", "Lighting & Electrical"),
    (("tree", "shrub"), "Trees / Shrubs", "Landscaping & Irrigation"),
    (("bollard",), "Bollards", "Miscellaneous"),
    (("catch", "inlet"), "Storm Drainage Inlet", "Drainage"),
)

        
        def items_from_civil_layers(extraction: dict[str, Any]) -> list[dict[str, Any]]:
    scale = 1.0
    items: list[dict[str, Any]] = []
    length_acc: dict[str, float] = defaultdict(float)

            for ent in list(extraction.get("lines") or []) + list(extraction.get("polylines") or []):
        layer = str(ent.get("layer") or "")
        low = layer.lower()
        try:
            length = float(ent.get("length") or 0)
        except (TypeError, ValueError):
            length = 0.0
        for keys, desc, cat, unit in _LAYER_LENGTH:
            if any(k in low for k in keys) and length > 0.5:
                length_acc[f"{desc}|{cat}|{unit}"] += length * scale
                break

            counts: dict[str, int] = defaultdict(int)
    for block in extraction.get("blocks") or []:
        blob = f"{block.get('name') or ''} {block.get('layer') or ''} {block.get('type') or ''}".lower()
        for keys, desc, cat in _BLOCK_COUNTS:
            if any(k in blob for k in keys):
                counts[f"{desc}|{cat}|EA"] += 1
                break

            for key, qty in length_acc.items():
        desc, cat, unit = key.split("|")
        items.append(_qty(desc, cat, unit, round(qty, 2), f"Sum of CAD lengths on matching {desc.lower()} layers", 84.0))
    for key, qty in counts.items():
        desc, cat, unit = key.split("|")
        items.append(_qty(desc, cat, unit, float(qty), f"Count of CAD blocks classified as {desc}", 88.0))
    return items

        
        def _qty(description: str, category: str, unit: str, quantity: float, method: str, confidence: float) -> dict[str, Any]:
    return {
        "description": description,
        "category": category,
        "unit": unit.upper(),
        "quantity": float(quantity),
        "layer": "",
        "entity_type": "ESTIMATOR",
        "calculation_method": method,
        "confidence": confidence,
    }

        
        def extraction_has_bid_schedule(extraction: dict[str, Any]) -> bool:
    """True when CAD text includes a Bid Items / EOQ table (don't invent trench extras)."""
    parts = [str(extraction.get("filename") or "")]
    for t in extraction.get("texts") or []:
        parts.append(str(t.get("text") or ""))
    blob = " ".join(parts).lower()
    return any(
        k in blob
        for k in (
            "estimate of quantities",
            "bid items",
            "std bid no",
            "approx. quantity",
            "for bidding purposes",
        )
    )

        
        def merge_estimator_items(base: list[dict[str, Any]], extra: list[dict[str, Any]]) -> list[dict[str, Any]]:
    """Append extras unless the same description+unit already exists (don't double-count)."""
    seen = {(str(i.get("description") or "").strip().lower(), str(i.get("unit") or "").upper()) for i in base}
    out = list(base)
    for item in extra:
        key = (str(item.get("description") or "").strip().lower(), str(item.get("unit") or "").upper())
        if key in seen or float(item.get("quantity") or 0) <= 0:
            continue
        seen.add(key)
        out.append(item)
    return out

        
        def expand_cad_takeoff(extraction: dict[str, Any], items: list[dict[str, Any]]) -> list[dict[str, Any]]:
    texts = " ".join(str(t.get("text") or "") for t in (extraction.get("texts") or [])[:400])
    extra: list[dict[str, Any]] = []
    if not cad_notes_make_trench_incidental(texts):
        extra.extend(trench_items_from_pipes(extraction))
    extra.extend(items_from_civil_layers(extraction))
    extra.extend(items_from_design_text(texts, filename="CAD text"))
    return merge_estimator_items(items, extra)

### backend/app/services/utility_labels.py

SHA-256: a45c8f0ea5c539b45032428ed70e87d3f52f89a014b32b9f0de403341f8ab21f    ·    241 line(s)    ·    .py

"""Extract water/sanitary/storm utility quantities from plan labels & callouts.

        On civil plan PDFs, pipe sizes and lengths often live in callout text like:
  8" WATER MAIN, PROP. WM 12", 8" DIP WM (STA 10+00 TO 12+50), WATERMAIN 245 LF
"""

        from __future__ import annotations

        import re
from typing import Any

        from app.services.incidental import skip_text_extraction

        # Use horizontal whitespace only so labels on the next line are not glued on.
_S = r"[^\S\n]*"
_SIZE = rf"(?P<size>\d{{1,2}}(?:\.\d+)?){_S}(?:\"|''|in(?:ch(?:es)?)?)"
_QTY_LF = rf"(?P<qty>\d{{2,5}}(?:\.\d+)?){_S}(?:lf|lin(?:ear)?{_S}(?:ft|feet)|l\.?f\.?)\b"
_STA = (
    rf"(?:sta(?:tion)?\.?{_S})?(?P<sta1>\d{{1,3}}\+\d{{2}}(?:\.\d+)?)"
    rf"{_S}(?:to|[-–]){_S}"
    rf"(?:sta(?:tion)?\.?{_S})?(?P<sta2>\d{{1,3}}\+\d{{2}}(?:\.\d+)?)"
)

        # Ordered patterns: (regex, network) — each match stays on one line
_LABEL_PATTERNS: list[tuple[re.Pattern[str], str]] = [
    # 8" WATER MAIN … 245 LF
    (
        re.compile(
            rf"{_SIZE}{_S}"
            rf"(?:(?P<mat>dip|pvc|hdpe|di|ci|rcp|cpp){_S})?"
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>water\s*mains?|watermains?|w\.?\s*m\.?|wm)\b"
            rf"(?:{_S}[-:]?{_S}{_QTY_LF})?",
            re.I,
        ),
        "water",
    ),
    # WATER MAIN 8" … LF
    (
        re.compile(
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>water\s*mains?|watermains?)\b"
            rf"{_S}{_SIZE}"
            rf"(?:{_S}[-:]?{_S}{_QTY_LF})?",
            re.I,
        ),
        "water",
    ),
    # PROP. WM 12" STA 10+00 TO 12+50  (same line)
    (
        re.compile(
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>water\s*mains?|watermains?|wm)\b"
            rf"{_S}{_SIZE}{_S}{_STA}",
            re.I,
        ),
        "water",
    ),
    # 8" WM STA … TO … / 8" DIP WM STA …
    (
        re.compile(
            rf"{_SIZE}{_S}"
            rf"(?:(?:dip|pvc|hdpe){_S})?"
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>wm|w\.?\s*m\.?|water\s*mains?|watermains?)\b"
            rf"{_S}{_STA}",
            re.I,
        ),
        "water",
    ),
    # WATERMAIN 245 LF  (size unknown)
    (
        re.compile(
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>water\s*mains?|watermains?)\b"
            rf"{_S}[-:]?{_S}{_QTY_LF}",
            re.I,
        ),
        "water",
    ),
    # Fittings (same line)
    (
        re.compile(
            rf"(?:{_SIZE}{_S})?"
            r"(?P<kind>gate\s*valve|butterfly\s*valve|water\s*valve|\bvalve\b|"
            r"fire\s*hydrant|\bhydrant\b|\bbend\b|\belbow\b|\btee\b)\b"
            rf"(?:{_S}(?P<qty>\d{{1,3}}){_S}(?:ea|each|nos?))?",
            re.I,
        ),
        "water_fitting",
    ),
    # Sanitary / storm mains with size + optional LF
    (
        re.compile(
            rf"{_SIZE}{_S}"
            rf"(?:prop(?:osed)?\.?{_S})?"
            r"(?P<kind>sanitary\s*(?:sewer)?(?:\s*main)?|ss\s*main|"
            r"storm\s*(?:drain|sewer)?(?:\s*main)?)\b"
            rf"(?:{_S}{_QTY_LF})?",
            re.I,
        ),
        "other_pipe",
    ),
]

        
        def _station_to_feet(sta: str) -> float | None:
    m = re.match(r"(\d+)\+(\d+(?:\.\d+)?)", sta.strip())
    if not m:
        return None
    return float(m.group(1)) * 100.0 + float(m.group(2))

        
        def _size_label(raw: str | None) -> str | None:
    if not raw:
        return None
    m = re.search(r"(\d{1,2}(?:\.\d+)?)", raw)
    if not m:
        return None
    val = float(m.group(1))
    if val < 1 or val > 96:
        return None
    if abs(val - int(val)) < 0.01:
        return f"{int(val)}-Inch"
    return f"{val:g}-Inch"

        
        def _kind_description(kind: str, network: str, size: str | None) -> tuple[str, str, str]:
    """Return description, category, unit."""
    k = (kind or "").lower().strip()
    size_bit = f"{size} " if size else ""
    if network == "water_fitting":
        if "hydrant" in k:
            return f"{size_bit}Fire Hydrant".strip(), "Utilities", "EA"
        if "bend" in k or "elbow" in k:
            return f"{size_bit}Water Bend / Elbow".strip(), "Utilities", "EA"
        if "tee" in k:
            return f"{size_bit}Water Tee".strip(), "Utilities", "EA"
        return f"{size_bit}Water Valve".strip(), "Utilities", "EA"
    if network == "water" or "water" in k or k in {"wm", "w.m.", "w.m", "w m"}:
        return f"{size_bit}Water Main".strip(), "Utilities", "LF"
    if "sanitary" in k or k.startswith("ss"):
        return f"{size_bit}Sanitary Sewer Pipe".strip(), "Drainage", "LF"
    if "storm" in k:
        return f"{size_bit}Storm Drain Pipe".strip(), "Drainage", "LF"
    return f"{size_bit}Utility Pipe".strip(), "Utilities", "LF"

        
        def extract_utility_label_items(
    text: str,
    *,
    filename: str = "",
    document_id: int | None = None,
    mains_only: bool = False,
) -> list[dict[str, Any]]:
    """Parse plan text for utility label/callout quantities.

            When mains_only=True (EOQ/schedule documents), skip fitting/valve/hydrant invents
    that inflate false extras vs bid schedules.
    """
    if not text or len(text.strip()) < 8:
        return []

            cleaned = (
        text.replace("″", '"')
        .replace("”", '"')
        .replace("’", "'")
        .replace("–", "-")
        .replace("—", "-")
    )
    found: dict[str, dict[str, Any]] = {}

            for pattern, network in _LABEL_PATTERNS:
        if mains_only and network == "water_fitting":
            continue
        for match in pattern.finditer(cleaned):
            gd = match.groupdict()
            size = _size_label(gd.get("size"))
            kind = gd.get("kind") or ""
            desc, category, unit = _kind_description(kind, network, size)

                    qty: float | None = None
            if gd.get("qty"):
                try:
                    qty = float(gd["qty"])
                except ValueError:
                    qty = None
            if qty is None and gd.get("sta1") and gd.get("sta2"):
                a = _station_to_feet(gd["sta1"])
                b = _station_to_feet(gd["sta2"])
                if a is not None and b is not None:
                    qty = abs(b - a)

                    if qty is None and unit == "EA":
                qty = 1.0

                    # Sized water main label without length: keep as EA? No — skip LF without qty
            if qty is None or qty <= 0:
                continue
            line_start = cleaned.rfind("\n", 0, match.start()) + 1
            line_end = cleaned.find("\n", match.end())
            line = cleaned[line_start : line_end if line_end >= 0 else len(cleaned)]
            if skip_text_extraction(
                desc,
                line,
                match_start=match.start() - line_start,
            ):
                continue
            if unit == "LF" and (qty < 1 or qty > 200000):
                continue
            if unit == "EA" and qty > 500:
                continue

                    key = f"{desc.lower()}|{unit.lower()}"
            snippet = re.sub(r"\s+", " ", match.group(0)).strip()[:120]
            conf = 92.0 if size and unit == "LF" else 88.0 if size else 84.0
            if key not in found:
                found[key] = {
                    "description": desc,
                    "category": category,
                    "unit": unit,
                    "quantity": qty,
                    "confidence": conf,
                    "status": "needs_review",
                    "calculation_method": f"Plan label/callout: '{snippet}'",
                    "source_reference": f"Drawing label — {snippet}",
                    "source_document_id": document_id,
                    "item_code": None,
                }
            else:
                existing = float(found[key]["quantity"])
                if unit == "LF":
                    if abs(qty - existing) / max(existing, 1) < 0.08:
                        found[key]["confidence"] = max(float(found[key]["confidence"]), conf)
                    else:
                        found[key]["quantity"] = existing + qty
                        found[key]["calculation_method"] += f" + '{snippet}'"
                else:
                    found[key]["quantity"] = existing + qty

            return list(found.values())

### backend/app/services/csi_mapper.py

SHA-256: 3ce1167a945bc4cc90ec6d70e535da0fdb083dccbc9d2072ebde775d2ae96414    ·    280 line(s)    ·    .py

"""USA CSI MasterFormat mapping for civil / roadway EOQ items.

        Maps common civil descriptions to Division 31/32/33 (and related) CSI codes.
Also normalizes units used in USA highway takeoffs and assigns EOQ groups.
"""

        from __future__ import annotations

        import re
from typing import Any

        from app.services.eoq_groups import assign_group_category

        # keywords (lower) → (csi_code, default_category, preferred_unit_hint or None)
CSI_RULES: list[tuple[list[str], str, str, str | None]] = [
    # Division 02 – Existing Conditions
    (["demolition", "remove pavement", "sawcut", "remove "], "02 41 13", "Removals", None),
    (["clearing", "grubbing"], "31 11 00", "Clearing & Grubbing", "acre"),
    # Division 31 – Earthwork
    (["trench excavation", "trench backfill", "pipe bedding"], "31 23 16.13", "Grading", "cy"),
    (["earthwork cut", "excavation", "cut to fill", "roadway excavation", "unclassified"], "31 23 16", "Grading", "cy"),
    (["earthwork fill", "embankment", "borrow", "fill"], "31 23 23", "Grading", "cy"),
    (["subgrade", "proof roll", "topsoil"], "31 22 13", "Grading", "sy"),
    (["geotextile", "geogrid", "stabilization fabric", "trench stabilization"], "31 05 19", "Grading", "sy"),
    # Division 32 – Exterior Improvements / Pavement
    (["gsb", "granular sub base", "granular subbase", "subbase"], "32 11 23", "Surfacing", "cy"),
    (["wmm", "wet mix macadam", "aggregate base", "crushed aggregate base", "cab"], "32 11 23", "Surfacing", "cy"),
    (["dbm", "dense bituminous", "bituminous base"], "32 12 16", "Surfacing", "ton"),
    (["bituminous concrete", "asphalt concrete", "hma", "hot mix", "ac paving", "bc "], "32 12 16", "Surfacing", "ton"),
    (["asphalt", "paving", "pavement"], "32 12 16", "Surfacing", "ton"),
    (["prime coat", "tack coat"], "32 12 13", "Surfacing", "sy"),
    (["concrete pavement", "pcc pavement", "rigid pavement"], "32 13 13", "Surfacing", "sy"),
    (["kerb", "curb", "curbing", "kerbing", "gutter"], "32 16 13", "Curb, Gutter & Sidewalk", "lf"),
    (["sidewalk", "footpath", "walkway", "pedestrian", "driveway"], "32 16 23", "Curb, Gutter & Sidewalk", "sf"),
    (["guardrail", "barrier", "guiderail"], "32 17 23", "Miscellaneous", "lf"),
    (["fence", "fencing"], "32 31 13", "Miscellaneous", "lf"),
    (["road marking", "pavement marking", "striping", "thermoplastic"], "32 17 23", "Traffic Signals & Signing", "lf"),
    (["traffic sign", "road sign", "signage", "signing"], "10 14 53", "Traffic Signals & Signing", "sf"),
    (["signal", "traffic signal"], "34 41 13", "Traffic Signals & Signing", "ea"),
    (["landscaping", "sodding", "seeding", "turf", "fertiliz", "mulch", "erosion", "silt fence"], "32 92 00", "Erosion Control / Restoration", "sy"),
    (["mobilization"], "01 71 13", "General", "ls"),
    (["traffic control"], "01 71 13", "Traffic Control", "sf"),
    # Division 33 – Utilities / Drainage
    (["culvert", "box culvert"], "33 42 13", "Storm Sewer", "ea"),
    (["storm drain", "storm sewer", "drainage pipe", "drain pipe", "catch basin", "inlet"], "33 41 00", "Storm Sewer", "lf"),
    (["sanitary sewer", "sanitary pipe", "force main", "forcemain"], "33 31 00", "Sanitary Sewer", "lf"),
    (["sanitary manhole", "ssmh"], "33 05 61", "Sanitary Sewer", "ea"),
    (["manhole", "junction box", "headwall"], "33 05 61", "Storm Sewer", "ea"),
    (["watermain", "water main", "waterline", "hydrant", "gate valve"], "33 11 00", "Watermain", "lf"),
    (["pipe", "sewer"], "33 31 00", "Sanitary Sewer", "lf"),
    # Structures / Concrete
    (["reinforced concrete", "rcc", "structural concrete", "retaining wall"], "03 30 00", "Structures", "cy"),
    (["concrete"], "03 30 00", "Structures", "cy"),
    (["rebar", "reinforcement", "steel reinforcement"], "03 20 00", "Structures", "lb"),
    (["formwork"], "03 11 00", "Structures", "sf"),
    (["dam embankment", "earthfill dam", "cofferdam"], "31 24 13", "Dams & Reservoirs", "cy"),
    (["spillway", "stilling basin"], "03 30 00", "Dams & Reservoirs", "cy"),
    (["reservoir lining", "pond lining", "geomembrane"], "33 56 13", "Dams & Reservoirs", "sf"),
    (["impoundment", "storage tank", "reservoir capacity"], "33 16 00", "Dams & Reservoirs", "mgal"),
    (["brickwork", "masonry", "blockwork"], "04 20 00", "Building", "cy"),
    (["plaster"], "09 24 00", "Building", "sf"),
    (["flooring", "floor tile"], "09 30 00", "Building", "sf"),
    (["roofing"], "07 50 00", "Building", "sf"),
    (["doors", "hollow metal door", "wood door"], "08 11 00", "Building", "ea"),
    (["windows", "window "], "08 50 00", "Building", "ea"),
    (["road width", "carriageway", "alignment", "centerline"], "01 71 23", "Miscellaneous", "lf"),
]

        CSI_CODE_RE = re.compile(r"\b(\d{2})\s*[-\s]?\s*(\d{2})\s*[-\s]?\s*(\d{2})(?:\.\d+)?\b")

        UNIT_ALIASES = {
    "m3": "m3",
    "m³": "m3",
    "cu.m": "m3",
    "cu m": "m3",
    "cum": "m3",
    "cubic meter": "m3",
    "cubic meters": "m3",
    "cy": "cy",
    "cu.yd": "cy",
    "cubic yard": "cy",
    "cubic yards": "cy",
    "m2": "m2",
    "m²": "m2",
    "sq.m": "m2",
    "sq m": "m2",
    "sm": "m2",
    "square meter": "m2",
    "square meters": "m2",
    "sy": "sy",
    "sq.yd": "sy",
    "sqyd": "sy",
    "sq yd": "sy",
    "sq.yd.": "sy",
    "square yard": "sy",
    "square yards": "sy",
    "cuyd": "cy",
    "cu yd": "cy",
    "cu.yd.": "cy",
    "sf": "sf",
    "sqft": "sf",
    "sq ft": "sf",
    "sq.ft": "sf",
    "sq.ft.": "sf",
    "square foot": "sf",
    "square feet": "sf",
    "m": "m",
    "lm": "m",
    "lin m": "m",
    "linear m": "m",
    "ft": "lf",
    "lf": "lf",
    "lft": "lf",
    "lin ft": "lf",
    "linear foot": "lf",
    "linear feet": "lf",
    "nos": "nos",
    "no": "nos",
    "no.": "nos",
    "each": "ea",
    "ea": "ea",
    "ls": "ls",
    "lump sum": "ls",
    "t": "t",
    "ton": "t",
    "tons": "t",
    "tonne": "t",
    "lb": "lb",
    "lbs": "lb",
    "pound": "lb",
    "pounds": "lb",
    "mgal": "mgal",
    "ac-ft": "ac-ft",
    "acre-ft": "ac-ft",
    "acre ft": "ac-ft",
    "kg": "kg",
    "acre": "acre",
    "ac": "acre",
}

        
        def normalize_unit(unit: str | None) -> str:
    if not unit:
        return "unit"
    raw = str(unit).strip().lower().replace("³", "3").replace("²", "2")
    raw = re.sub(r"\s+", " ", raw)
    if raw in UNIT_ALIASES:
        return UNIT_ALIASES[raw]
    compact = raw.replace(".", "").replace(" ", "")
    return UNIT_ALIASES.get(compact, str(unit).strip())

        
        _EXPORT_UNIT_LABELS = {
    "sf": "SQFT",
    "sqft": "SQFT",
    "sq ft": "SQFT",
    "square foot": "SQFT",
    "square feet": "SQFT",
    "squarefoot": "SQFT",
    "squarefeet": "SQFT",
    "t": "TON",
    "ton": "TON",
    "tons": "TON",
    "tonne": "TON",
    "tonnes": "TON",
}

        
        def format_export_unit(unit: str | None) -> str:
    """USA pay-item labels for Excel/CSV: SQFT not SF, TON not T."""
    raw = str(unit or "UNIT").strip()
    if not raw:
        return "UNIT"
    key = re.sub(r"\s+", " ", raw.lower().replace("³", "3"))
    if key in _EXPORT_UNIT_LABELS:
        return _EXPORT_UNIT_LABELS[key]
    compact = key.replace(".", "").replace(" ", "")
    if compact in _EXPORT_UNIT_LABELS:
        return _EXPORT_UNIT_LABELS[compact]
    return raw.upper()

        
        def looks_like_csi(code: str | None) -> bool:
    if not code:
        return False
    return bool(CSI_CODE_RE.search(str(code)))

        
        def normalize_csi_code(code: str | None) -> str | None:
    if not code:
        return None
    match = CSI_CODE_RE.search(str(code))
    if not match:
        return str(code).strip() or None
    return f"{match.group(1)} {match.group(2)} {match.group(3)}"

        
        def map_csi(
    *,
    description: str,
    category: str | None = None,
    unit: str | None = None,
    existing_code: str | None = None,
) -> dict[str, Any]:
    """Return CSI code + normalized unit/category for a civil quantity item."""
    unit_n = normalize_unit(unit)
    if looks_like_csi(existing_code):
        return {
            "csi_code": normalize_csi_code(existing_code),
            "item_code": normalize_csi_code(existing_code),
            "category": category,
            "unit": unit_n,
            "csi_match": "provided",
            "csi_confidence": 98.0,
        }

            text = f"{description or ''} {category or ''}".lower()
    for keys, csi, cat, unit_hint in CSI_RULES:
        if any(k in text for k in keys):
            return {
                "csi_code": csi,
                "item_code": existing_code or csi,
                "category": category or cat,
                "unit": unit_n if unit else (normalize_unit(unit_hint) if unit_hint else unit_n),
                "csi_match": "mapped",
                "csi_confidence": 90.0,
            }

            return {
        "csi_code": None,
        "item_code": existing_code,
        "category": category or "General",
        "unit": unit_n,
        "csi_match": "unmapped",
        "csi_confidence": 40.0,
    }

        
        def enrich_quantity_item(item: dict[str, Any]) -> dict[str, Any]:
    """Attach CSI code / normalized unit / EOQ group onto a quantity dict."""
    from app.services.traffic_control import looks_like_agency_bid_number

            out = dict(item)
    mapped = map_csi(
        description=str(out.get("description") or ""),
        category=out.get("category"),
        unit=out.get("unit"),
        existing_code=out.get("csi_code") or out.get("item_code"),
    )
    out["unit"] = mapped["unit"]
    out["category"] = mapped["category"]
    out["csi_code"] = mapped["csi_code"]
    if looks_like_agency_bid_number(out.get("item_code")):
        pass  # keep Bid Items / STD BID NO on item_code
    elif mapped["csi_code"] and (not out.get("item_code") or mapped["csi_match"] == "mapped"):
        if not looks_like_csi(out.get("item_code")):
            out["item_code"] = mapped["csi_code"]
    elif mapped["item_code"]:
        out["item_code"] = mapped["item_code"]
    try:
        conf = float(out.get("confidence") or 0)
    except (TypeError, ValueError):
        conf = 0.0
    if mapped["csi_match"] in {"provided", "mapped"} and conf < 95:
        out["confidence"] = min(95.0, max(conf, mapped["csi_confidence"] - 5))
    out["csi_match"] = mapped["csi_match"]
    return assign_group_category(out)

        
        def list_csi_catalog() -> list[dict[str, str]]:
    seen: dict[str, dict[str, str]] = {}
    for keys, csi, cat, unit in CSI_RULES:
        if csi not in seen:
            seen[csi] = {
                "csi_code": csi,
                "category": cat,
                "unit_hint": unit or "",
                "keywords": ", ".join(keys[:4]),
            }
    return list(seen.values())

### backend/app/services/eoq_groups.py

SHA-256: fa2396464b6047fc03f4f22925178b966dda4647fc6c746aaa03db3abe27671c    ·    497 line(s)    ·    .py

"""Municipal Estimate Of Quantities section grouping.

        Groups EOQ lines the way agency bid schedules / EOQ sheets do
(Removals, Grading, Watermain, Sanitary Sewer, …), so UI and exports
show bold section headers with items underneath.
"""

        from __future__ import annotations

        import re
from typing import Any, Iterable, TypeVar

        T = TypeVar("T")

        # Display order matches typical municipal EOQ / bid schedule flow.
EOQ_GROUP_ORDER: list[str] = [
    "General",
    "Traffic Control",
    "Removals",
    "Clearing & Grubbing",
    "Grading",
    "Erosion Control / Restoration",
    "Surfacing",
    "Curb, Gutter & Sidewalk",
    "Storm Sewer",
    "Watermain",
    "Sanitary Sewer",
    "Water & Sewer Services",
    "Traffic Signals & Signing",
    "Lighting & Electrical",
    "Gas & Dry Utilities",
    "Structures",
    "Dams & Reservoirs",
    "Building",
    "Landscaping & Irrigation",
    "Miscellaneous",
    "Unmapped Takeoff",
]

        # keyword (lower) → group. First match wins — order matters (specific before generic).
_GROUP_RULES: list[tuple[list[str], str]] = [
    # Removals / demolition first
    (
        [
            "remove ",
            "removal",
            "demolition",
            "sawcut",
            "saw cut",
            "abandon",
            "salvage and remove",
            "mill and remove",
        ],
        "Removals",
    ),
    # General (LS / project setup) vs Traffic Control (signing / TTC devices)
    (
        [
            "mobilization",
            "demobilization",
            "tax on city",
            "city furnished",
            "winter maintenance",
            "construction entrance",
            "field office",
            "bonds and insurance",
            "survey",
            "staking",
            "audiovisual",
            "video record",
            "permit",
            "allowance",
            "contingency",
        ],
        "General",
    ),
    (
        [
            "traffic control",
            "temporary traffic",
            "flagging",
            "barricade",
            "mailbox",
            "changeable message",
            "pcms",
            "business sign",
            "gravel access",
            "channeliz",
        ],
        "Traffic Control",
    ),
    (
        ["clearing", "grubbing", "tree removal", "stump", "brush"],
        "Clearing & Grubbing",
    ),
    (
        [
            "dam embankment",
            "earthfill dam",
            "dam excavation",
            "cofferdam",
            "spillway",
            "reservoir lining",
            "pond lining",
            "reservoir",
            "impoundment",
            "stilling basin",
            "intake tower",
            "dam crest",
        ],
        "Dams & Reservoirs",
    ),
    # Grading / earthwork
    (
        [
            "unclassified excavation",
            "excavation",
            "earthwork",
            "embankment",
            "borrow",
            "cut ",
            "fill ",
            "grading",
            "subgrade",
            "topsoil",
            "proof roll",
            "scarify",
            "compaction",
            "trench stabilization",
            "select backfill",
            "imported fill",
            "rock excavation",
        ],
        "Grading",
    ),
    # Erosion / restoration
    (
        [
            "erosion",
            "silt fence",
            "inlet protection",
            "sediment",
            "seeding",
            "sodding",
            "mulch",
            "fertiliz",
            "hydroseed",
            "weed control",
            "water for vegetation",
            "turf establishment",
            "soil blanket",
            "wattle",
            "swppp",
            "temporary seeding",
        ],
        "Erosion Control / Restoration",
    ),
    # Surfacing / pavement
    (
        [
            "aggregate base",
            "crushed aggregate",
            "asphalt",
            "hma",
            "hot mix",
            "bituminous",
            "pavement",
            "paving",
            "gsb",
            "wmm",
            "dbm",
            "prime coat",
            "tack coat",
            "milling",
            "overlay",
            "chip seal",
            "surface course",
            "binder course",
            "pcc pavement",
            "concrete pavement",
        ],
        "Surfacing",
    ),
    (
        [
            "curb and gutter",
            "curb",
            "gutter",
            "sidewalk",
            "driveway",
            "crosswalk",
            "ramp",
            "ada",
            "detectable warning",
            "kerb",
        ],
        "Curb, Gutter & Sidewalk",
    ),
    # Watermain (before generic pipe/water)
    (
        [
            "watermain",
            "water main",
            "waterline",
            "water line",
            "pvc watermain",
            "dip wm",
            "c900",
            "fire hydrant",
            "hydrant",
            "gate valve",
            "butterfly valve",
            "water valve",
            "blowoff",
            "blow-off",
            "air release",
            "water meter",
            "thrust block",
            "trenchless",
            "directional drill",
            "casing pipe",
            "potable",
        ],
        "Watermain",
    ),
    # Sanitary
    (
        [
            "sanitary sewer",
            "sanitary pipe",
            "sanitary manhole",
            "ssmh",
            "force main",
            "forcemain",
            "sewer service",
            "sanitary service",
            "cleanout",
            "lift station",
        ],
        "Sanitary Sewer",
    ),
    # Storm
    (
        [
            "storm sewer",
            "storm drain",
            "storm pipe",
            "catch basin",
            "inlet",
            "culvert",
            "headwall",
            "endwall",
            "junction box",
            "retention",
            "detention",
            "rcp ",
            "storm manhole",
            "drainage structure",
            "underdrain",
            "french drain",
        ],
        "Storm Sewer",
    ),
    (
        [
            "water service",
            "service connection",
            "corporation stop",
            "curb stop",
            "meter pit",
            "tap ",
            "house connection",
            "lateral",
        ],
        "Water & Sewer Services",
    ),
    (
        [
            "traffic signal",
            "signal pole",
            "pavement marking",
            "striping",
            "thermoplastic",
            "road sign",
            "traffic sign",
            "signage",
            "delineator",
            "object marker",
        ],
        "Traffic Signals & Signing",
    ),
    (
        [
            "street light",
            "lighting",
            "luminaire",
            "electrical conduit",
            "pull box",
            "transformer",
            "electric",
        ],
        "Lighting & Electrical",
    ),
    (
        ["gas main", "gas service", "telecom", "fiber", "telephone", "cable tv", "joint trench"],
        "Gas & Dry Utilities",
    ),
    (
        [
            "brickwork",
            "plaster",
            "flooring",
            "roofing",
            "doors",
            "window",
            "house",
            "building",
            "dwelling",
            "apartment",
            "masonry",
            "blockwork",
            "tile",
        ],
        "Building",
    ),
    (
        [
            "bridge",
            "retaining wall",
            "reinforced concrete",
            "structural concrete",
            "rebar",
            "formwork",
            "pile",
            "footing",
            "abutment",
            "box culvert",
        ],
        "Structures",
    ),
    (
        [
            "landscape",
            "irrigation",
            "planting",
            "tree ",
            "shrub",
            "groundcover",
            "sod ",
        ],
        "Landscaping & Irrigation",
    ),
    # Broad utility fallbacks
    (["sewer"], "Sanitary Sewer"),
    (["drain", "drainage"], "Storm Sewer"),
    (["water", "valve", "pipe", "fitting", "bend", "tee", "reducer"], "Watermain"),
    (["fence", "guardrail", "barrier"], "Miscellaneous"),
]

        _CATEGORY_ALIASES: dict[str, str] = {
    "utilities": "Watermain",
    "utility": "Watermain",
    "drainage": "Storm Sewer",
    "earthwork": "Grading",
    "pavement": "Surfacing",
    "roadside": "Curb, Gutter & Sidewalk",
    "landscaping": "Landscaping & Irrigation",
    "traffic": "Traffic Signals & Signing",
    "markings": "Traffic Signals & Signing",
    "structures": "Structures",
    "dams & reservoirs": "Dams & Reservoirs",
    "building": "Building",
    "architecture": "Building",
    "demolition": "Removals",
    "site clearing": "Clearing & Grubbing",
    "geometry": "Miscellaneous",
    "general": "General",
    "traffic control": "Traffic Control",
    "general / traffic control": "General",
    "bid schedule": "Miscellaneous",
    "unmapped takeoff": "Unmapped Takeoff",
}

        
        def resolve_eoq_group(
    *,
    description: str | None = None,
    category: str | None = None,
) -> str:
    """Return canonical EOQ section name for an EOQ line."""
    cat = (category or "").strip()
    cat_low = cat.lower()
    if cat_low in {"general / traffic control", "general/traffic control"}:
        matched = _match_description(description or "")
        return matched or "General"
    if cat:
        alias = _CATEGORY_ALIASES.get(cat_low)
        if alias:
            # Still refine Utilities/Drainage/General using description when possible
            if alias in {"Watermain", "Storm Sewer", "Sanitary Sewer", "Miscellaneous", "General"}:
                refined = _match_description(description or "")
                if refined:
                    return refined
            return alias
        if cat in EOQ_GROUP_ORDER:
            return cat

            matched = _match_description(f"{description or ''} {category or ''}")
    if matched:
        return matched
    if cat and cat.lower() == "unmapped takeoff":
        return "Unmapped Takeoff"
    return "Miscellaneous"

        
        def _match_description(text: str) -> str | None:
    low = text.lower()
    if not low.strip():
        return None
    for keys, group in _GROUP_RULES:
        if any(k in low for k in keys):
            return group
    return None

        
        def looks_like_mobilization(description: str | None) -> bool:
    text = str(description or "").strip().lower()
    if not text or "demobilization" in text:
        return False
    return bool(re.search(r"\bmobilization\b", text))

        
        def group_sort_key(group: str) -> tuple[int, str]:
    try:
        return (EOQ_GROUP_ORDER.index(group), group)
    except ValueError:
        return (len(EOQ_GROUP_ORDER), group)

        
        def group_items(
    items: Iterable[T],
    *,
    get_description,
    get_category,
    repeat_mobilization: bool = False,
) -> list[tuple[str, list[T]]]:
    """Partition items into ordered (group_name, items) sections. Empty groups omitted.

            When repeat_mobilization is True (UI/Excel), Mobilization is listed under both
    General and Traffic Control. Persistence should leave this False so the pay item
    is stored once.
    """
    buckets: dict[str, list[T]] = {}
    for item in items:
        group = resolve_eoq_group(
            description=get_description(item),
            category=get_category(item),
        )
        buckets.setdefault(group, []).append(item)
    if repeat_mobilization:
        mobs = [
            item
            for rows in buckets.values()
            for item in rows
            if looks_like_mobilization(get_description(item))
        ]
        if mobs:
            gen = buckets.setdefault("General", [])
            for mob in reversed(mobs):
                if mob not in gen:
                    gen.insert(0, mob)
            tc = buckets.setdefault("Traffic Control", [])
            for mob in reversed(mobs):
                if mob not in tc:
                    tc.insert(0, mob)
    ordered: list[tuple[str, list[T]]] = []
    for name in EOQ_GROUP_ORDER:
        if name in buckets and buckets[name]:
            ordered.append((name, buckets.pop(name)))
    for name in sorted(buckets.keys()):
        if buckets[name]:
            ordered.append((name, buckets[name]))
    return ordered

        
        def assign_group_category(item: dict[str, Any]) -> dict[str, Any]:
    """Set item['category'] to the canonical EOQ group (copy)."""
    out = dict(item)
    # Preserve unmapped marker as group
    if str(out.get("bid_match_method") or "") == "unmapped" or str(out.get("category") or "").lower() == "unmapped takeoff":
        out["category"] = "Unmapped Takeoff"
        out["eoq_group"] = "Unmapped Takeoff"
        return out
    group = resolve_eoq_group(description=str(out.get("description") or ""), category=out.get("category"))
    out["category"] = group
    out["eoq_group"] = group
    return out

### backend/app/services/eoq_service.py

SHA-256: 45eecf1e3f64b54c61f0f9656042d6990fab1d65d51dae91efa6dbc220c5ec6c    ·    990 line(s)    ·    .py

"""Build EOQ records from document analyses and export Excel."""

        from __future__ import annotations

        from decimal import Decimal, ROUND_HALF_UP
from io import BytesIO, StringIO
import csv
from typing import Any

        from openpyxl import Workbook
from openpyxl.formatting.rule import CellIsRule
from openpyxl.styles import Alignment, Border, Font, PatternFill, Side
from openpyxl.utils import get_column_letter
from openpyxl.worksheet.datavalidation import DataValidation
from sqlalchemy import func, select
from sqlalchemy.orm import Session, selectinload

        from app.models.analysis import DocumentAnalysis
from app.models.eoq import EOQ, EOQItem, EOQItemStatus, EOQStatus
from app.models.cad import CadModel
from app.models.project import Project
from app.services.bid_service import build_eoq_items_from_template, get_active_template
from app.services.eoq_groups import assign_group_category, group_items, looks_like_mobilization
from app.services.csi_mapper import enrich_quantity_item, format_export_unit
from app.services.item_combine import combine_similar_pay_items
from app.services.processing import load_findings

        # AutoVAD standard: confidence below this → Engineer Review
CONFIDENCE_VERIFIED_THRESHOLD = 97

        EOQ_EXPORT_HEADERS = [
    "Item Number",
    "Standard Bid Item Number",
    "Item Description",
    "Unit",
    "Quantity",
    "Cost",
    "Total Cost",
    "AI Confidence",
    "Status",
    "Source",
    "Calculation Method",
]

        
        def _money2(value: Decimal | float | None) -> Decimal | None:
    if value is None:
        return None
    return Decimal(str(value)).quantize(Decimal("0.01"), rounding=ROUND_HALF_UP)

        
        def _qty2(value: Decimal | float | None) -> Decimal:
    return Decimal(str(value or 0)).quantize(Decimal("0.01"), rounding=ROUND_HALF_UP)

        
        def ensure_mobilization_item(
    items: list[dict[str, Any]],
    *,
    template_lines: list | None = None,
) -> list[dict[str, Any]]:
    """Every project EOQ includes Mobilization as 1 LS under General items."""
    from app.services.traffic_control import looks_like_agency_bid_number

            found: list[dict[str, Any]] = []
    rest: list[dict[str, Any]] = []
    for item in items:
        if looks_like_mobilization(str(item.get("description") or "")):
            found.append(dict(item))
        else:
            rest.append(item)

            base: dict[str, Any] = dict(found[0]) if found else {}
    for item in found:
        if looks_like_agency_bid_number(item.get("item_code")):
            base = dict(item)
            break

            if not found and template_lines:
        for line in template_lines:
            if looks_like_mobilization(getattr(line, "description", None)):
                base = {
                    "item_code": getattr(line, "item_code", None),
                    "csi_code": getattr(line, "csi_code", None),
                    "description": getattr(line, "description", None) or "Mobilization",
                    "bid_template_line_id": getattr(line, "id", None),
                    "bid_match_confidence": 100.0,
                    "bid_match_method": "standard_pay_item",
                    "rate": getattr(line, "default_rate", None),
                    "source_reference": f"Bid line {getattr(line, 'line_number', '')}".strip(),
                }
                break

            mob = enrich_quantity_item(
        {
            **base,
            "description": "Mobilization",
            "unit": "LS",
            "quantity": 1,
            "category": "General",
            "calculation_method": base.get("calculation_method")
            or "Standard lump-sum pay item — 1 LS on every project",
            "source_reference": base.get("source_reference") or "General items",
            "confidence": max(float(base.get("confidence") or 0), 97.0),
        }
    )
    mob["description"] = "Mobilization"
    mob["unit"] = format_export_unit("LS")
    mob["quantity"] = 1
    mob["category"] = "General"
    return [assign_group_category(mob), *rest]

        
        def standard_bid_item_number(item: EOQItem) -> str:
    """Agency/state bid code from template; empty when AutoVAD default / unmapped."""
    if item.bid_template_line_id and item.item_code:
        return str(item.item_code)
    return ""

        
        def status_label_for_item(item: EOQItem) -> str:
    """Display status for the AutoVAD EOQ standard."""
    if item.status == EOQItemStatus.VERIFIED:
        return "Verified"
    if item.status in {EOQItemStatus.NEEDS_REVIEW, EOQItemStatus.DRAFT}:
        return "Engineer Review"
    if item.status == EOQItemStatus.APPROVED:
        return "Verified"
    return "Engineer Review"

        
        def resolve_item_status(confidence: float | None, *, force_review: bool = False) -> EOQItemStatus:
    if force_review:
        return EOQItemStatus.NEEDS_REVIEW
    if confidence is not None and float(confidence) >= CONFIDENCE_VERIFIED_THRESHOLD:
        return EOQItemStatus.VERIFIED
    return EOQItemStatus.NEEDS_REVIEW

        
        def generate_eoq_for_project(
    db: Session,
    project: Project,
    user_id: int,
    *,
    document_ids: list[int] | None = None,
) -> EOQ:
    """Build an EOQ from all analyses/CAD, or only the given document IDs."""
    analyses = list(
        db.scalars(select(DocumentAnalysis).where(DocumentAnalysis.project_id == project.id)).all()
    )
    cad_models = list(db.scalars(select(CadModel).where(CadModel.project_id == project.id)).all())

            scope_ids = {int(i) for i in document_ids} if document_ids else None
    if scope_ids is not None:
        analyses = [a for a in analyses if a.document_id in scope_ids]
        cad_models = [c for c in cad_models if c.document_id in scope_ids]

            active = get_active_template(db, project.id)
    has_template = bool(active and active.lines)

            if not analyses and not cad_models:
        if scope_ids:
            raise ValueError(
                "No analysis for the selected file(s). Run Analyze (PDF) or Process CAD (DWG/DXF) on that file first."
            )
        raise ValueError("No analyzed documents or CAD models. Upload design plans and run Analyze first.")

            # Prefer CadModel quantities when both CAD analysis mirror and CadModel exist
    # for the same document (avoids near-duplicate EOQ lines).
    import json

            cad_doc_ids = {
        c.document_id
        for c in cad_models
        if c.quantities_json and str(c.quantities_json).strip() not in {"", "[]", "null"}
    }

            collected: list[dict] = []
    for analysis in analyses:
        if analysis.document_id in cad_doc_ids:
            continue
        findings = load_findings(analysis)
        for item in findings.get("items") or []:
            payload = dict(item)
            payload.setdefault("source_document_id", analysis.document_id)
            collected.append(payload)

            for cad in cad_models:
        if not cad.quantities_json:
            continue
        try:
            qty_items = json.loads(cad.quantities_json)
        except json.JSONDecodeError:
            qty_items = []
        for item in qty_items:
            payload = dict(item)
            payload["source_document_id"] = cad.document_id
            payload["calculation_method"] = payload.get("calculation_method") or "CAD geometry takeoff"
            collected.append(payload)

            extracted = combine_similar_pay_items(collected)

            if not extracted:
        raise ValueError(
            "No quantities found from the selected file(s). "
            "Run Analyze / Process CAD first, then Generate Estimate Of Quantities again."
            if scope_ids
            else "No quantities found from design plans. Upload PDF/DWG and run Analyze / Process CAD first."
        )

            if has_template:
        # Only bid items evidenced in the plans, aligned to the active agency template.
        items_list = build_eoq_items_from_template(extracted, list(active.lines))
        matched = sum(
            1
            for i in items_list
            if i.get("bid_template_line_id") and float(i.get("bid_match_confidence") or 0) > 0
        )
        unmapped = sum(1 for i in items_list if i.get("bid_match_method") == "unmapped")
        notes_extra = (
            f" Matched plan takeoff to bid template '{active.name}' "
            f"({matched} template item(s); {unmapped} unmapped takeoff item(s)). "
            "Unused bid-list lines were omitted."
        )
        if not items_list:
            raise ValueError(
                "Could not match any plan quantities to the active bid template. "
                "Re-analyze plans after uploading the template, or check descriptions/units."
            )
    else:
        # AutoVAD default: CSI-enriched takeoff EOQ
        items_list = [enrich_quantity_item(dict(item)) for item in extracted]
        notes_extra = (
            " Generated with AutoVAD default CSI schedule (no bid template uploaded)."
            " Upload a bid list so Generate Estimate Of Quantities maps only the bid items needed for this project."
        )

            items_list = ensure_mobilization_item(
        items_list,
        template_lines=list(active.lines) if has_template else None,
    )

            from app.models.document import Document

            scope_label = ""
    if scope_ids:
        docs = list(db.scalars(select(Document).where(Document.id.in_(scope_ids))).all())
        names = [d.original_filename for d in docs]
        if len(names) == 1:
            scope_label = f" · {names[0]}"
        elif names:
            scope_label = f" · {len(names)} files"
        notes_extra += f" Scope: selected document_id(s) {sorted(scope_ids)}."

            latest_version = db.scalar(select(func.max(EOQ.version)).where(EOQ.project_id == project.id)) or 0
    eoq = EOQ(
        project_id=project.id,
        title=f"{project.name} - Estimate Of Quantities v{latest_version + 1}{scope_label}",
        version=latest_version + 1,
        status=EOQStatus.AI_GENERATED,
        currency="USD",
        notes=(
            "Generated from document AI + CAD with CSI codes, units, and confidence scores."
            + notes_extra
        ),
        created_by=user_id,
    )
    db.add(eoq)
    db.flush()

            # Persist in municipal group order with continuous Item No. 1, 2, 3, …
    ordered_items: list[dict] = []
    for _group_name, group_rows in group_items(
        items_list,
        get_description=lambda i: str(i.get("description") or ""),
        get_category=lambda i: i.get("category"),
    ):
        ordered_items.extend(group_rows)

            for idx, item in enumerate(ordered_items, start=1):
        conf = item.get("confidence")
        conf_f = float(conf) if conf is not None else None
        bid_match_conf = item.get("bid_match_confidence")
        try:
            bid_match_f = float(bid_match_conf) if bid_match_conf is not None else None
        except (TypeError, ValueError):
            bid_match_f = None
        # Ensure category is the EOQ group label
        grouped = assign_group_category(dict(item))
        force_review = (
            grouped.get("bid_match_method") in {"unmatched", "unmapped", "fuzzy_description"}
            or (bool(grouped.get("bid_template_line_id")) and float(grouped.get("quantity") or 0) == 0)
            or (
                bool(grouped.get("bid_template_line_id"))
                and bid_match_f is not None
                and bid_match_f < 85.0
            )
            or str(grouped.get("category") or "").lower() == "unmapped takeoff"
            or (
                not grouped.get("source_reference")
                and not grouped.get("source_document_id")
                and not grouped.get("calculation_method")
            )
        )
        status = resolve_item_status(conf_f, force_review=force_review)
        qty = _qty2(grouped.get("quantity"))
        rate = _money2(grouped["rate"]) if grouped.get("rate") is not None else None
        amount = _money2(qty * rate) if rate is not None else None
        unit = format_export_unit(grouped.get("unit"))
        db.add(
            EOQItem(
                eoq_id=eoq.id,
                item_number=str(idx),
                item_code=grouped.get("item_code"),
                csi_code=grouped.get("csi_code"),
                description=str(grouped.get("description")),
                category=grouped.get("category"),
                unit=unit,
                quantity=qty,
                rate=rate,
                amount=amount,
                source_document_id=grouped.get("source_document_id"),
                source_page=grouped.get("source_page"),
                source_reference=grouped.get("source_reference"),
                calculation_method=grouped.get("calculation_method"),
                confidence=Decimal(str(conf)) if conf is not None else None,
                bid_template_line_id=grouped.get("bid_template_line_id"),
                bid_match_confidence=grouped.get("bid_match_confidence"),
                status=status,
            )
        )

            db.commit()
    return db.scalar(
        select(EOQ).options(selectinload(EOQ.items)).where(EOQ.id == eoq.id)
    )  # type: ignore[return-value]

        
        def list_project_eoqs(db: Session, project_id: int) -> list[EOQ]:
    return list(
        db.scalars(
            select(EOQ)
            .options(selectinload(EOQ.items))
            .where(EOQ.project_id == project_id)
            .order_by(EOQ.version.desc())
        ).all()
    )

        
        def get_eoq_item(db: Session, item_id: int) -> EOQItem | None:
    return db.scalar(select(EOQItem).where(EOQItem.id == item_id))

        
        def update_eoq_item(
    db: Session,
    item: EOQItem,
    *,
    status: EOQItemStatus | None = None,
    quantity: Decimal | None = None,
    description: str | None = None,
    unit: str | None = None,
    item_code: str | None = None,
    review_note: str | None = None,
) -> EOQItem:
    """Apply engineer review edits to one EOQ line."""
    if description is not None:
        cleaned = description.strip()
        if cleaned:
            item.description = cleaned
    if unit is not None:
        item.unit = format_export_unit(unit) or item.unit
    if item_code is not None:
        item.item_code = item_code.strip() or None
    if quantity is not None:
        item.quantity = _qty2(quantity)
        if item.rate is not None:
            item.amount = _money2(item.quantity * item.rate)
    if status is not None:
        item.status = status
    if review_note:
        method = item.calculation_method or ""
        note = f"[review] {review_note.strip()}"
        item.calculation_method = f"{method} | {note}".strip(" |") if method else note
    db.commit()
    db.refresh(item)
    return item

        
        def get_eoq(db: Session, eoq_id: int) -> EOQ | None:
    return db.scalar(select(EOQ).options(selectinload(EOQ.items)).where(EOQ.id == eoq_id))

        
        def _sorted_eoq_items(eoq: EOQ) -> list[EOQItem]:
    return sorted(
        eoq.items,
        key=lambda x: int(x.item_number) if str(x.item_number).isdigit() else 0,
    )

        
        def _grouped_eoq_items(eoq: EOQ) -> list[tuple[str, list[EOQItem]]]:
    """Return EOQ sections with items (item number order preserved within each group)."""
    items = _sorted_eoq_items(eoq)
    return group_items(
        items,
        get_description=lambda i: i.description,
        get_category=lambda i: i.category,
        repeat_mobilization=True,
    )

        
        def export_eoq_csv(eoq: EOQ) -> bytes:
    """Full EOQ columns + Group column, section separators, Item No. 1, 2, 3..."""
    buffer = StringIO()
    writer = csv.writer(buffer)

            # Keep Group as its own column so Excel/Sheets always show grouping clearly
    headers = [
        "Item Number",
        "Group",
        "Standard Bid Item Number",
        "Item Description",
        "Unit",
        "Quantity",
        "Cost",
        "Total Cost",
        "AI Confidence",
        "Status",
        "Source",
        "Calculation Method",
    ]
    writer.writerow(headers)

            serial = 1
    first_group = True
    for group_name, items in _grouped_eoq_items(eoq):
        if not first_group:
            writer.writerow([])  # blank line between groups
        first_group = False

                # Explicit section banner (shows even without using the Group column)
        writer.writerow(
            [
                "",
                group_name,
                "",
                f"===== {group_name} =====",
                "",
                "",
                "",
                "",
                "",
                "",
                "",
                "",
            ]
        )

                for item in items:
            qty = _qty2(item.quantity)
            cost = _money2(item.rate)
            total = _money2(qty * cost) if cost is not None else None
            writer.writerow(
                [
                    str(serial),
                    group_name,
                    standard_bid_item_number(item),
                    item.description,
                    format_export_unit(item.unit),
                    f"{qty:.2f}",
                    f"{cost:.2f}" if cost is not None else "",
                    f"{total:.2f}" if total is not None else "",
                    f"{float(item.confidence):.2f}" if item.confidence is not None else "",
                    status_label_for_item(item),
                    item.source_reference or "",
                    item.calculation_method or "",
                ]
            )
            serial += 1

            # UTF-8 BOM so Excel on Windows detects encoding / columns correctly
    return ("\ufeff" + buffer.getvalue()).encode("utf-8")

        
        def export_eoq_excel(eoq: EOQ, *, utilities_detail: dict | None = None) -> bytes:
    """Full AutoVAD EOQ columns with municipal-style section grouping.

            When utilities_detail is provided (from CAD DWG/DXF takeoff), adds sheets:
    - Bid Quantity Summary (rolled up FROM detail)
    - Linear Quantity Breakdown (CL station / LT-RT offset / length)
    - Fittings Bends Connections
    - Quantity QAQC
    """
    wb = Workbook()
    ws = wb.active
    ws.title = "Estimate Of Quantities"

            headers = EOQ_EXPORT_HEADERS
    col_count = len(headers)

            title_font = Font(name="Calibri", size=14, bold=True, color="D9FF43")
    header_fill = PatternFill("solid", fgColor="0D1F19")
    header_font = Font(color="D9FF43", bold=True)
    section_fill = PatternFill("solid", fgColor="D9E2F3")
    section_font = Font(name="Calibri", bold=True, size=11, color="1F4E79")
    thin = Side(style="thin", color="000000")
    border = Border(left=thin, right=thin, top=thin, bottom=thin)

            ws.merge_cells(start_row=1, start_column=1, end_row=1, end_column=col_count)
    title_cell = ws.cell(1, 1, "ESTIMATE OF QUANTITIES")
    title_cell.font = title_font
    title_cell.fill = PatternFill("solid", fgColor="0D1F19")
    title_cell.alignment = Alignment(horizontal="center", vertical="center")
    ws.row_dimensions[1].height = 26

            for col, h in enumerate(headers, start=1):
        cell = ws.cell(2, col, h)
        cell.fill = header_fill
        cell.font = header_font
        cell.alignment = Alignment(horizontal="center", wrap_text=True)
        cell.border = border

            row_idx = 3
    item_rows: list[int] = []
    serial = 1

            for group_name, items in _grouped_eoq_items(eoq):
        ws.merge_cells(start_row=row_idx, start_column=1, end_row=row_idx, end_column=col_count)
        sec = ws.cell(row_idx, 1, group_name)
        sec.font = section_font
        sec.fill = section_fill
        sec.alignment = Alignment(horizontal="left", vertical="center")
        for c in range(1, col_count + 1):
            cell = ws.cell(row_idx, c)
            cell.fill = section_fill
            cell.border = border
        row_idx += 1

                for item in items:
            qty = float(_qty2(item.quantity))
            cost = _money2(item.rate)
            status = status_label_for_item(item)

                    ws.cell(row_idx, 1, serial).alignment = Alignment(horizontal="center")
            ws.cell(row_idx, 2, standard_bid_item_number(item)).alignment = Alignment(horizontal="center")
            ws.cell(row_idx, 3, item.description).alignment = Alignment(horizontal="left", wrap_text=True)
            ws.cell(row_idx, 4, format_export_unit(item.unit)).alignment = Alignment(horizontal="center")

                    qty_cell = ws.cell(row_idx, 5, qty)
            qty_cell.number_format = "0.00"
            qty_cell.alignment = Alignment(horizontal="right")

                    if cost is not None:
                cost_cell = ws.cell(row_idx, 6, float(cost))
                cost_cell.number_format = "0.00"
                total_cell = ws.cell(row_idx, 7, f"=E{row_idx}*F{row_idx}")
                total_cell.number_format = "0.00"
            else:
                ws.cell(row_idx, 6, "")
                ws.cell(row_idx, 7, "")

                    if item.confidence is not None:
                conf_cell = ws.cell(row_idx, 8, round(float(item.confidence), 2))
                conf_cell.number_format = "0.00"
            else:
                ws.cell(row_idx, 8, "")

                    status_cell = ws.cell(row_idx, 9, status)
            status_cell.alignment = Alignment(horizontal="center")
            # Do not bake fill/font here. Direct fill stays put when the dropdown
            # changes; Excel only swaps both colors if they come from CF.

                    ws.cell(row_idx, 10, item.source_reference or "")
            ws.cell(row_idx, 11, item.calculation_method or "")

                    for c in range(1, col_count + 1):
                ws.cell(row_idx, c).border = border

                    item_rows.append(row_idx)
            serial += 1
            row_idx += 1

            last_row = max(3, row_idx - 1)
    if item_rows:
        status_dv = DataValidation(
            type="list",
            formula1='"Verified,Engineer Review"',
            allow_blank=False,
            showDropDown=False,
            showErrorMessage=True,
            errorTitle="Invalid status",
            error="Choose Verified or Engineer Review",
        )
        status_dv.add(f"I3:I{last_row}")
        ws.add_data_validation(status_dv)

                # CF owns both fill and font so changing Verified ↔ Engineer Review updates the box.
        green_fill = PatternFill(start_color="C6EFCE", end_color="C6EFCE", fill_type="solid")
        green_font = Font(color="006100", bold=True)
        red_fill = PatternFill(start_color="FFC7CE", end_color="FFC7CE", fill_type="solid")
        red_font = Font(color="9C0006", bold=True)
        status_range = f"I3:I{last_row}"
        ws.conditional_formatting.add(
            status_range,
            CellIsRule(
                operator="equal",
                formula=['"Verified"'],
                fill=green_fill,
                font=green_font,
                stopIfTrue=True,
            ),
        )
        ws.conditional_formatting.add(
            status_range,
            CellIsRule(
                operator="equal",
                formula=['"Engineer Review"'],
                fill=red_fill,
                font=red_font,
                stopIfTrue=True,
            ),
        )

            widths = [12, 22, 42, 10, 12, 12, 12, 14, 16, 28, 28]
    for i, width in enumerate(widths, start=1):
        ws.column_dimensions[get_column_letter(i)].width = width
    ws.freeze_panes = "A3"

            _append_utility_stationing_sheets(wb, utilities_detail, header_fill=header_fill, header_font=header_font, border=border)

            meta = wb.create_sheet("Meta")
    meta.append(["Estimate Of Quantities Title", eoq.title])
    meta.append(["Version", eoq.version])
    meta.append(["Status", eoq.status.value])
    meta.append(["Currency", eoq.currency])
    meta.append(["Notes", eoq.notes or ""])
    meta.append(["Generated by", "AutoVAD"])
    meta.append(
        [
            "Layout",
            "Full Estimate Of Quantities columns with municipal section grouping "
            "(Removals, Grading, Watermain, Sanitary Sewer, …)",
        ]
    )
    meta.append(
        [
            "Utility sheets",
            "Bid Quantity Summary is rolled up FROM Linear Quantity Breakdown + "
            "Fittings Bends Connections. Stationing is always on the project CENTERLINE (CL); "
            "LT/RT from alignment geometry. Quantity QAQC flags unassociated / overlapping / low-confidence items.",
        ]
    )
    meta.append(
        [
            "Template rule",
            "User bid template when active; otherwise AutoVAD default CSI schedule",
        ]
    )
    meta.append(
        [
            "Status rule",
            f"Verified when AI confidence >= {CONFIDENCE_VERIFIED_THRESHOLD}; else Engineer Review",
        ]
    )
    meta.append(["Columns", ", ".join(EOQ_EXPORT_HEADERS)])

            buffer = BytesIO()
    wb.save(buffer)
    return buffer.getvalue()

        
        def load_project_utilities_detail(db: Session, project_id: int) -> dict | None:
    """Merge utilities_detail_json from all CAD models on a project."""
    import json

            models = list(db.scalars(select(CadModel).where(CadModel.project_id == project_id)).all())
    segments: list[dict] = []
    connections: list[dict] = []
    bid_summary: list[dict] = []
    qa_flags: list[dict] = []
    alignments: list[dict] = []
    for cad in models:
        raw = cad.utilities_detail_json
        detail = None
        if raw:
            try:
                detail = json.loads(raw)
            except json.JSONDecodeError:
                detail = None

                # Rebuild when missing, or when station/offset columns are blank (common APS/CL miss)
        needs_rebuild = detail is None
        if isinstance(detail, dict):
            segs = detail.get("segments") or []
            conns = detail.get("connections") or []
            if segs and not any(str(s.get("from_station") or "").strip() for s in segs):
                needs_rebuild = True
            # Fittings often have Alignment filled but blank Station/Side/Offset — rebuild
            if conns and any(
                not str(c.get("station") or "").strip()
                or not str(c.get("side") or "").strip()
                or (
                    c.get("offset") in (None, "")
                    and c.get("offset_ft") is None
                )
                for c in conns
            ):
                needs_rebuild = True

                if needs_rebuild:
            try:
                entities = json.loads(cad.entities_json or "{}")
                texts = json.loads(cad.texts_json or "[]")
                blocks = json.loads(cad.blocks_json or "[]")
                extraction = {
                    **(entities if isinstance(entities, dict) else {}),
                    "texts": texts if isinstance(texts, list) else [],
                    "blocks": blocks if isinstance(blocks, list) else [],
                }
                from app.services.cad.utility_stationing import build_utilities_detail

                        detail = build_utilities_detail(extraction)
            except Exception:
                if detail is None:
                    continue
        if not isinstance(detail, dict):
            continue
        for s in detail.get("segments") or []:
            row = dict(s)
            row.setdefault("cad_document_id", cad.document_id)
            segments.append(row)
        for c in detail.get("connections") or []:
            row = dict(c)
            row.setdefault("cad_document_id", cad.document_id)
            connections.append(row)
        for b in detail.get("bid_summary") or []:
            bid_summary.append(dict(b))
        for q in detail.get("qa_flags") or []:
            qa_flags.append(dict(q))
        if detail.get("alignment"):
            alignments.append(dict(detail["alignment"]))

            if not segments and not connections and not bid_summary:
        return None

            # Re-rollup bid summary from merged detail so Summary always matches Detail
    if segments or connections:
        from app.services.cad.utility_stationing import build_bid_summary_from_detail

                bid_summary = build_bid_summary_from_detail(segments, connections)

            return {
        "segments": segments,
        "connections": connections,
        "bid_summary": bid_summary,
        "qa_flags": qa_flags,
        "alignments": alignments,
        "summary": {
            "segment_count": len(segments),
            "connection_count": len(connections),
            "bid_item_count": len(bid_summary),
            "qa_flag_count": len(qa_flags),
            "total_lf": round(sum(float(s.get("quantity_lf") or 0) for s in segments), 2),
        },
    }

        
        def _style_header_row(
    ws,
    headers: list[str],
    *,
    header_fill: PatternFill,
    header_font: Font,
    border: Border,
) -> None:
    for col, h in enumerate(headers, start=1):
        cell = ws.cell(1, col, h)
        cell.fill = header_fill
        cell.font = header_font
        cell.border = border
        cell.alignment = Alignment(horizontal="center", wrap_text=True)

        
        def _append_utility_stationing_sheets(
    wb: Workbook,
    utilities_detail: dict | None,
    *,
    header_fill: PatternFill,
    header_font: Font,
    border: Border,
) -> None:
    """Civil takeoff sheets: Bid Summary ← Linear Detail + Fittings + QA/QC.

            Stationing is always relative to the project CENTERLINE (CL). LT/RT from
    alignment geometry. Bid Quantity Summary is rolled up FROM the detail sheets.
    """
    detail = utilities_detail or {}
    segments = list(detail.get("segments") or [])
    connections = list(detail.get("connections") or [])
    bid_summary = list(detail.get("bid_summary") or [])
    qa_flags = list(detail.get("qa_flags") or [])
    alignments = list(detail.get("alignments") or [])
    # Always derive summary from detail so Bid Summary matches Linear + Fittings
    if segments or connections:
        from app.services.cad.utility_stationing import build_bid_summary_from_detail

                bid_summary = build_bid_summary_from_detail(segments, connections)
    cl_name = ""
    if alignments:
        cl_name = str(alignments[0].get("name") or "")
    elif detail.get("alignment"):
        cl_name = str((detail.get("alignment") or {}).get("name") or "")

            # --- Sheet: Bid Quantity Summary (from detail) ---
    sum_ws = wb.create_sheet("Bid Quantity Summary")
    sum_headers = ["Bid Item", "Description", "Unit", "Quantity", "Detail Source", "Rollup Note"]
    _style_header_row(sum_ws, sum_headers, header_fill=header_fill, header_font=header_font, border=border)
    if cl_name:
        sum_ws.cell(1, 7, f"Centerline: {cl_name}")
    if not bid_summary:
        sum_ws.cell(
            2,
            1,
            "No CAD utility detail yet. Process DWG/DXF, then re-export — "
            "summary quantities are generated from Linear + Fittings detail sheets.",
        )
    else:
        for r_i, row in enumerate(bid_summary, start=2):
            vals = [
                row.get("bid_item") or "",
                row.get("description") or "",
                format_export_unit(row.get("unit")) if row.get("unit") else "",
                float(row.get("quantity") or 0),
                row.get("detail_source") or "",
                row.get("rollup_note") or "",
            ]
            for c_i, val in enumerate(vals, start=1):
                cell = sum_ws.cell(r_i, c_i, val)
                cell.border = border
                if c_i == 4:
                    cell.number_format = "#,##0.00" if str(row.get("unit")).upper() == "LF" else "#,##0"
                    cell.alignment = Alignment(horizontal="right")
    for i, w in enumerate([14, 36, 8, 12, 28, 40], start=1):
        sum_ws.column_dimensions[get_column_letter(i)].width = w
    sum_ws.freeze_panes = "A2"

            # --- Sheet: Linear Quantity Breakdown ---
    lin_ws = wb.create_sheet("Linear Quantity Breakdown")
    lin_headers = [
        "Item",
        "Size",
        "From Sta.",
        "To Sta.",
        "Side",
        "Offset",
        "From Offset",
        "To Offset",
        "Length (LF)",
        "Non-Parallel",
        "Alignment (CL)",
        "Layer",
        "Source",
    ]
    _style_header_row(lin_ws, lin_headers, header_fill=header_fill, header_font=header_font, border=border)
    if not segments:
        lin_ws.cell(
            2,
            1,
            "No linear utility runs found. Process CAD with pipes/polylines or station callouts. "
            "Stationing is always on the project CENTERLINE.",
        )
    else:
        for r_i, row in enumerate(segments, start=2):
            nonpar = bool(row.get("nonparallel"))
            vals = [
                row.get("item") or row.get("utility") or "",
                row.get("size") or "",
                row.get("from_station") or "",
                row.get("to_station") or "",
                row.get("side") or row.get("side_of_alignment") or "",
                row.get("offset") or "",
                row.get("from_offset") if nonpar else "",
                row.get("to_offset") if nonpar else "",
                float(row.get("length") or row.get("quantity_lf") or 0),
                "Yes" if nonpar else "No",
                row.get("alignment") or cl_name or "CL",
                row.get("layer") or "",
                row.get("source") or "",
            ]
            for c_i, val in enumerate(vals, start=1):
                cell = lin_ws.cell(r_i, c_i, val)
                cell.border = border
                if c_i == 9:
                    cell.number_format = "#,##0.00"
                    cell.alignment = Alignment(horizontal="right")
    for i, w in enumerate([16, 8, 12, 12, 8, 12, 14, 14, 12, 12, 22, 16, 22], start=1):
        lin_ws.column_dimensions[get_column_letter(i)].width = w
    lin_ws.freeze_panes = "A2"

            # --- Sheet: Fittings / Bends / Connections ---
    fit_ws = wb.create_sheet("Fittings Bends Connections")
    fit_headers = [
        "Type",
        "Size",
        "Station",
        "Side",
        "Offset",
        "Angle/Type",
        "Connects To",
        "Utility",
        "Qty",
        "Alignment (CL)",
        "Layer",
        "Source",
    ]
    _style_header_row(fit_ws, fit_headers, header_fill=header_fill, header_font=header_font, border=border)
    if not connections:
        fit_ws.cell(
            2,
            1,
            "No bends/fittings/connections located. Process CAD with fitting blocks or "
            "pipe polylines that deflect at vertices.",
        )
    else:
        for r_i, row in enumerate(connections, start=2):
            side_val = row.get("side") or row.get("direction_from_alignment") or ""
            off_val = row.get("offset")
            if off_val in (None, ""):
                oft = row.get("offset_ft")
                if oft is not None:
                    try:
                        from app.services.cad.utility_stationing import format_offset

                                off_val = format_offset(float(oft), str(side_val) or None)
                    except (TypeError, ValueError):
                        off_val = oft
            vals = [
                row.get("type") or row.get("connection_type") or "",
                row.get("size") or "",
                row.get("station") or "",
                side_val,
                "" if off_val is None else off_val,
                row.get("angle") or row.get("angle_type") or "",
                row.get("connects_to") or "",
                row.get("utility") or "",
                float(row.get("quantity") or 1),
                row.get("alignment") or cl_name or "CL",
                row.get("layer") or "",
                row.get("source") or "",
            ]
            for c_i, val in enumerate(vals, start=1):
                cell = fit_ws.cell(r_i, c_i, val)
                cell.border = border
    for i, w in enumerate([14, 8, 12, 8, 12, 12, 22, 16, 6, 22, 16, 20], start=1):
        fit_ws.column_dimensions[get_column_letter(i)].width = w
    fit_ws.freeze_panes = "A2"

            # --- Sheet: Quantity QA/QC ---
    qa_ws = wb.create_sheet("Quantity QAQC")
    qa_headers = ["Severity", "Issue", "Message", "Object", "Station", "Suggestion"]
    _style_header_row(qa_ws, qa_headers, header_fill=header_fill, header_font=header_font, border=border)
    if not qa_flags:
        qa_ws.cell(
            2,
            1,
            "No QA flags. Process CAD to populate stationing QA "
            "(unidentified fittings, unassociated lines, overlaps, low confidence).",
        )
    else:
        sev_fill = {
            "high": PatternFill("solid", fgColor="FFC7CE"),
            "medium": PatternFill("solid", fgColor="FFEB9C"),
            "low": PatternFill("solid", fgColor="C6EFCE"),
        }
        for r_i, row in enumerate(qa_flags, start=2):
            sev = str(row.get("severity") or "medium").lower()
            vals = [
                sev.upper(),
                row.get("issue") or "",
                row.get("message") or "",
                row.get("object") or "",
                row.get("station") or "",
                row.get("suggestion") or "",
            ]
            for c_i, val in enumerate(vals, start=1):
                cell = qa_ws.cell(r_i, c_i, val)
                cell.border = border
                if c_i == 1 and sev in sev_fill:
                    cell.fill = sev_fill[sev]
    for i, w in enumerate([10, 28, 48, 24, 18, 40], start=1):
        qa_ws.column_dimensions[get_column_letter(i)].width = w
    qa_ws.freeze_panes = "A2"

### backend/app/services/bid_service.py

SHA-256: 667ddcd9a6bbd081a2df9578fe352cd538d7ea23042e00ad4538ee30cee62f37    ·    678 line(s)    ·    .py

"""Bid templates: import PDF/Excel/CSV bid lists and map EOQ items onto them."""

        from __future__ import annotations

        import csv
import io
import re
from decimal import Decimal
from pathlib import Path
from typing import Any

        from openpyxl import load_workbook
from sqlalchemy import select
from sqlalchemy.orm import Session, selectinload

        from app.models.bid import BidTemplate, BidTemplateLine
from app.models.eoq import EOQ
from app.services.csi_mapper import enrich_quantity_item, looks_like_csi, normalize_csi_code, normalize_unit

        
        def list_templates(db: Session, project_id: int) -> list[BidTemplate]:
    return list(
        db.scalars(
            select(BidTemplate)
            .options(selectinload(BidTemplate.lines))
            .where(BidTemplate.project_id == project_id)
            .order_by(BidTemplate.created_at.desc())
        ).all()
    )

        
        def get_active_template(db: Session, project_id: int) -> BidTemplate | None:
    return db.scalar(
        select(BidTemplate)
        .options(selectinload(BidTemplate.lines))
        .where(BidTemplate.project_id == project_id, BidTemplate.is_active.is_(True))
        .order_by(BidTemplate.created_at.desc())
    )

        
        def set_active_template(db: Session, project_id: int, template_id: int) -> BidTemplate:
    templates = list(db.scalars(select(BidTemplate).where(BidTemplate.project_id == project_id)).all())
    found = None
    for t in templates:
        t.is_active = t.id == template_id
        if t.id == template_id:
            found = t
    if not found:
        raise ValueError("Bid template not found")
    db.commit()
    return db.scalar(
        select(BidTemplate).options(selectinload(BidTemplate.lines)).where(BidTemplate.id == template_id)
    )  # type: ignore[return-value]

        
        def delete_template(db: Session, project_id: int, template_id: int) -> None:
    tmpl = db.scalar(select(BidTemplate).where(BidTemplate.id == template_id, BidTemplate.project_id == project_id))
    if not tmpl:
        raise ValueError("Bid template not found")
    db.delete(tmpl)
    db.commit()

        
        def import_bid_template(
    db: Session,
    *,
    project_id: int,
    user_id: int,
    path: Path,
    filename: str,
    name: str | None = None,
) -> BidTemplate:
    rows = _parse_bid_file(path, filename)
    if not rows:
        raise ValueError("No bid line items found in file. Expect columns like CSI/Code, Description, Unit.")

            # Deactivate previous templates; new upload becomes active
    existing = list(db.scalars(select(BidTemplate).where(BidTemplate.project_id == project_id)).all())
    for t in existing:
        t.is_active = False

            template = BidTemplate(
        project_id=project_id,
        name=name or f"Bid list — {filename}",
        source_filename=filename,
        is_active=True,
        notes=f"Imported {len(rows)} line(s) from {filename}",
        created_by=user_id,
    )
    db.add(template)
    db.flush()

            for idx, row in enumerate(rows, start=1):
        csi = normalize_csi_code(row.get("csi_code") or row.get("item_code"))
        if csi and not looks_like_csi(csi):
            # keep as item_code only
            item_code = csi
            csi_val = None
        else:
            item_code = row.get("item_code") or csi
            csi_val = csi if looks_like_csi(csi) else None
        db.add(
            BidTemplateLine(
                template_id=template.id,
                line_number=str(row.get("line_number") or idx),
                csi_code=csi_val,
                item_code=item_code,
                description=str(row["description"]),
                unit=normalize_unit(row.get("unit")),
                default_rate=row.get("default_rate"),
                sort_order=idx,
            )
        )
    db.commit()
    return db.scalar(
        select(BidTemplate).options(selectinload(BidTemplate.lines)).where(BidTemplate.id == template.id)
    )  # type: ignore[return-value]

        
        def map_eoq_to_template(db: Session, *, eoq_id: int, template_id: int | None = None) -> dict[str, Any]:
    eoq = db.scalar(select(EOQ).options(selectinload(EOQ.items)).where(EOQ.id == eoq_id))
    if not eoq:
        raise ValueError("Estimate Of Quantities not found")

            if template_id:
        template = db.scalar(
            select(BidTemplate)
            .options(selectinload(BidTemplate.lines))
            .where(BidTemplate.id == template_id, BidTemplate.project_id == eoq.project_id)
        )
    else:
        template = get_active_template(db, eoq.project_id)
    if not template:
        raise ValueError("No active bid template. Upload a bid list (PDF/Excel/CSV) first.")

            lines = list(template.lines)
    matched = 0
    unmatched = 0
    details: list[dict[str, Any]] = []

            for item in eoq.items:
        # Ensure CSI on EOQ item first
        enriched = enrich_quantity_item(
            {
                "description": item.description,
                "category": item.category,
                "unit": item.unit,
                "item_code": item.item_code,
                "csi_code": getattr(item, "csi_code", None),
                "confidence": float(item.confidence) if item.confidence is not None else None,
            }
        )
        item.csi_code = enriched.get("csi_code")
        if enriched.get("item_code") and not item.item_code:
            item.item_code = enriched["item_code"]
        item.unit = enriched.get("unit") or item.unit

                hit, score, method = _match_line(
            lines,
            description=item.description,
            unit=item.unit,
            csi_code=item.csi_code,
            item_code=item.item_code,
        )
        if hit:
            matched += 1
            item.bid_template_line_id = hit.id
            item.bid_match_confidence = score
            # Align to bidder's schedule (description, codes, unit)
            item.description = hit.description
            if hit.csi_code:
                item.csi_code = hit.csi_code
            if hit.item_code:
                item.item_code = hit.item_code
            item.unit = (hit.unit or item.unit or "UNIT").upper()
            # Leave the Unmapped takeoff bucket once a bid line is linked
            if (item.category or "").strip().lower() == "unmapped takeoff":
                item.category = "Bid schedule"
            if hit.default_rate is not None and item.rate is None:
                item.rate = Decimal(str(hit.default_rate))
                item.amount = Decimal(str(item.quantity)) * item.rate
            details.append(
                {
                    "eoq_item_id": item.id,
                    "description": item.description,
                    "matched_line": hit.line_number,
                    "matched_description": hit.description,
                    "csi_code": item.csi_code,
                    "confidence": score,
                    "method": method,
                }
            )
        else:
            unmatched += 1
            item.bid_template_line_id = None
            item.bid_match_confidence = None
            item.unit = (item.unit or "UNIT").upper()
            if (item.category or "").strip().lower() in {"", "bid schedule"}:
                item.category = "Unmapped takeoff"
            details.append(
                {
                    "eoq_item_id": item.id,
                    "description": item.description,
                    "matched_line": None,
                    "csi_code": item.csi_code,
                    "confidence": 0,
                    "method": "unmatched",
                }
            )

            db.commit()
    return {
        "template_id": template.id,
        "template_name": template.name,
        "eoq_id": eoq.id,
        "matched": matched,
        "unmatched": unmatched,
        "total": matched + unmatched,
        "details": details,
    }

        
        def apply_template_to_items(items: list[dict[str, Any]], lines: list[BidTemplateLine]) -> list[dict[str, Any]]:
    """Legacy: enrich extracted items with matching template metadata (item-first)."""
    out: list[dict[str, Any]] = []
    for item in items:
        enriched = enrich_quantity_item(item)
        hit, score, method = _match_line(
            lines,
            description=str(enriched.get("description") or ""),
            unit=str(enriched.get("unit") or ""),
            csi_code=enriched.get("csi_code"),
            item_code=enriched.get("item_code"),
        )
        if hit:
            enriched["bid_template_line_id"] = hit.id
            enriched["bid_match_confidence"] = score
            enriched["bid_match_method"] = method
            if hit.csi_code:
                enriched["csi_code"] = hit.csi_code
            if hit.item_code:
                enriched["item_code"] = hit.item_code
            if hit.default_rate is not None and enriched.get("rate") is None:
                enriched["rate"] = hit.default_rate
        out.append(enriched)
    return out

        
        def build_eoq_items_from_template(
    extracted: list[dict[str, Any]],
    lines: list[BidTemplateLine],
) -> list[dict[str, Any]]:
    """Match plan takeoff to the bid template — only include items needed for this project.

            - Matched template lines (with evidence from plans/CAD) become EOQ rows using
      the template's Standard Bid Item Number, description, and unit.
    - Unmatched takeoff rows are kept as Unmapped takeoff for engineer review.
    - Template lines with no plan evidence are NOT dumped into the EOQ.
    """
    sorted_lines = sorted(lines, key=lambda line: (line.sort_order, line.id))
    enriched_extracted = [enrich_quantity_item(dict(item)) for item in extracted]

            line_hits: dict[int, list[tuple[dict[str, Any], float, str]]] = {line.id: [] for line in sorted_lines}
    unmatched: list[dict[str, Any]] = []

            for item in enriched_extracted:
        hit, score, method = _match_line(
            sorted_lines,
            description=str(item.get("description") or ""),
            unit=str(item.get("unit") or ""),
            csi_code=item.get("csi_code"),
            item_code=item.get("item_code"),
        )
        if hit and score > 0:
            line_hits[hit.id].append((item, score, method))
        else:
            unmatched.append(item)

            out: list[dict[str, Any]] = []
    for line in sorted_lines:
        hits = line_hits.get(line.id) or []
        if not hits:
            continue  # skip unused bid items — not needed for this project
        total_qty = 0.0
        for payload, _score, _method in hits:
            try:
                total_qty += float(payload.get("quantity") or 0)
            except (TypeError, ValueError):
                pass
        if total_qty <= 0:
            continue
        best_item, best_score, best_method = max(hits, key=lambda row: row[1])
        rate = line.default_rate if line.default_rate is not None else best_item.get("rate")
        out.append(
            {
                "item_code": line.item_code,
                "csi_code": line.csi_code,
                "description": line.description,
                "category": best_item.get("category") or "Bid schedule",
                "unit": (line.unit or best_item.get("unit") or "UNIT").upper(),
                "quantity": round(total_qty, 4),
                "rate": rate,
                "source_document_id": best_item.get("source_document_id"),
                "source_page": best_item.get("source_page"),
                "source_reference": best_item.get("source_reference")
                or f"Bid line {line.line_number}",
                "calculation_method": (
                    best_item.get("calculation_method")
                    or f"Matched to bid template line {line.line_number} ({best_method})"
                ),
                "confidence": best_item.get("confidence"),
                "bid_template_line_id": line.id,
                "bid_match_confidence": best_score,
                "bid_match_method": best_method,
            }
        )

            for item in unmatched:
        out.append(
            {
                **item,
                "unit": str(item.get("unit") or "UNIT").upper(),
                "category": item.get("category") or "Unmapped takeoff",
                "bid_template_line_id": None,
                "bid_match_confidence": 0,
                "bid_match_method": "unmapped",
                "calculation_method": (
                    (item.get("calculation_method") or "Takeoff")
                    + " — not matched to active bid template"
                ),
            }
        )
    return out

        
        def _normalize_token(token: str) -> str:
    """Light normalize so barricades↔barricade, signs↔sign, temp↔temporary."""
    t = token.lower().strip()
    synonyms = {
        "temp": "temporary",
        "tmp": "temporary",
        "misc": "miscellaneous",
        "sig": "sign",
        "signs": "sign",
        "barricades": "barricade",
        "barricading": "barricade",
        "cones": "cone",
        "drums": "drum",
        "markers": "marker",
        "markings": "marking",
        "pavements": "pavement",
        "pipes": "pipe",
        "valves": "valve",
        "hydrants": "hydrant",
        "elbows": "elbow",
        "bends": "bend",
        "tees": "tee",
        "inlets": "inlet",
        "manholes": "manhole",
        "each": "ea",
        "lump": "ls",
        "sum": "ls",
        "lumpsum": "ls",
        "linear": "lf",
        "lin": "lf",
        "sqyd": "sy",
        "sqy": "sy",
        "cuyd": "cy",
        "cuy": "cy",
    }
    if t in synonyms:
        t = synonyms[t]
    roman = {"i": "1", "ii": "2", "iii": "3", "iv": "4", "v": "5", "vi": "6"}
    if t in roman:
        t = roman[t]
    if len(t) > 4 and t.endswith("ies"):
        t = t[:-3] + "y"
    elif len(t) > 3 and t.endswith("s") and not t.endswith("ss"):
        t = t[:-1]
    return t

        
        def _token_set(text: str) -> set[str]:
    stop = {
        "the", "and", "for", "with", "of", "a", "an", "to", "in", "on", "or",
        "item", "type", "per", "as", "at", "by", "from", "incl", "including",
        "complete", "all", "work", "provide", "furnish", "install",
    }
    tokens = re.findall(r"[a-z0-9.]+", text.lower())
    out = {_normalize_token(t) for t in tokens if len(t) > 1 and t not in stop}
    return {t for t in out if t and t not in stop}

        
        def _size_token(text: str) -> str | None:
    m = re.search(r"(\d{1,2}(?:\.\d+)?)\s*-?\s*(?:inch|in|\"|'')", text or "", re.I)
    if not m:
        m = re.search(r"\b(\d{1,2}(?:\.\d+)?)\s*\"", text or "")
    if not m:
        return None
    try:
        val = float(m.group(1))
    except ValueError:
        return None
    if abs(val - int(val)) < 0.01:
        return f"{int(val)}"
    return f"{val:g}"

        
        def _units_compatible(a: str, b: str) -> bool:
    na, nb = normalize_unit(a), normalize_unit(b)
    if na == nb:
        return True
    # Only allow soft match when one side is unknown — never LF↔EA
    if na == "unit" or nb == "unit":
        return True
    return False

        
        def _description_overlap_score(desc_tokens: set[str], line_tokens: set[str]) -> float:
    """Dice coefficient overlap 0–100 (more forgiving than one-sided ratio)."""
    if not desc_tokens or not line_tokens:
        return 0.0
    inter = len(desc_tokens & line_tokens)
    if inter <= 0:
        return 0.0
    return (2.0 * inter / (len(desc_tokens) + len(line_tokens))) * 100.0

        
        def _match_line(
    lines: list[BidTemplateLine],
    *,
    description: str,
    unit: str,
    csi_code: str | None,
    item_code: str | None,
) -> tuple[BidTemplateLine | None, float, str]:
    desc = description.lower().strip()
    unit_n = normalize_unit(unit)
    desc_tokens = _token_set(desc)
    desc_size = _size_token(description)

            if csi_code:
        for line in lines:
            if line.csi_code and normalize_csi_code(line.csi_code) == normalize_csi_code(csi_code):
                if _units_compatible(line.unit, unit_n):
                    return line, 96.0, "csi_code"
    if item_code:
        code = re.sub(r"\s+", "", item_code.lower().strip())
        for line in lines:
            for candidate in (line.item_code, line.csi_code):
                if candidate and re.sub(r"\s+", "", candidate.lower().strip()) == code:
                    if _units_compatible(line.unit, unit_n):
                        return line, 94.0, "item_code"

            for line in lines:
        if line.description.lower().strip() == desc and _units_compatible(line.unit, unit_n):
            return line, 92.0, "exact_description_unit"

            for line in lines:
        ld = line.description.lower().strip()
        if desc and ld and (desc in ld or ld in desc) and _units_compatible(line.unit, unit_n):
            line_size = _size_token(line.description)
            if desc_size and line_size and desc_size != line_size:
                continue
            return line, 82.0, "fuzzy_description"

            # Token overlap (e.g. "Temporary Traffic Control Signs" ↔ "TEMP TRAFFIC CONTROL SIGN")
    best: BidTemplateLine | None = None
    best_score = 0.0
    for line in lines:
        if not _units_compatible(line.unit, unit_n):
            continue
        line_size = _size_token(line.description)
        if desc_size and line_size and desc_size != line_size:
            continue
        line_tokens = _token_set(line.description)
        score = _description_overlap_score(desc_tokens, line_tokens)
        if desc_size and line_size and desc_size == line_size:
            score = min(100.0, score + 12.0)
        # Require at least 2 meaningful shared tokens when descriptions are long
        shared = len(desc_tokens & line_tokens)
        if shared < 2 and max(len(desc_tokens), len(line_tokens)) >= 4:
            continue
        # 48% Dice is enough for near-paraphrases after stemming/synonyms
        if score >= 48 and score > best_score:
            best = line
            best_score = score
    if best:
        return best, round(min(best_score, 88.0), 1), "token_overlap"

            return None, 0.0, "unmatched"

        
        def _parse_bid_file(path: Path, filename: str) -> list[dict[str, Any]]:
    ext = path.suffix.lower()
    if ext in {".xlsx", ".xls"}:
        return _parse_excel(path)
    if ext == ".csv":
        return _parse_csv(path)
    if ext == ".pdf":
        return _parse_pdf(path)
    # try text extract for other docs
    if ext in {".txt"}:
        text = path.read_text(encoding="utf-8", errors="ignore")
        return _parse_free_text(text)
    raise ValueError("Bid template must be PDF, Excel (.xlsx/.xls), or CSV")

        
        def _parse_csv(path: Path) -> list[dict[str, Any]]:
    text = path.read_text(encoding="utf-8", errors="ignore")
    reader = csv.DictReader(io.StringIO(text))
    rows: list[dict[str, Any]] = []
    for i, r in enumerate(reader, start=1):
        mapped = _row_from_dict(r, fallback_line=i)
        if mapped:
            rows.append(mapped)
    return rows

        
        def _parse_excel(path: Path) -> list[dict[str, Any]]:
    wb = load_workbook(path, data_only=True, read_only=True)
    ws = wb.active
    headers = [str(c.value).strip().lower() if c.value is not None else "" for c in next(ws.iter_rows(min_row=1, max_row=1))]
    idx = {h: i for i, h in enumerate(headers)}

            def find(*names: str) -> int | None:
        for name in names:
            for h, i in idx.items():
                if name == h or name in h:
                    return i
        return None

            line_i = find("line", "item no", "item#", "no.", "#")
    code_i = find("csi", "item code", "code", "bid item")
    desc_i = find("description", "desc", "item", "particular")
    unit_i = find("unit", "uom")
    rate_i = find("rate", "unit price", "price")
    if desc_i is None:
        desc_i = 0

            rows: list[dict[str, Any]] = []
    for n, row in enumerate(ws.iter_rows(min_row=2, values_only=True), start=1):
        values = list(row)
        if not values or desc_i >= len(values) or not values[desc_i]:
            continue
        code = str(values[code_i]).strip() if code_i is not None and code_i < len(values) and values[code_i] else None
        unit = str(values[unit_i]).strip() if unit_i is not None and unit_i < len(values) and values[unit_i] else "unit"
        rate = None
        if rate_i is not None and rate_i < len(values) and values[rate_i] is not None:
            try:
                rate = float(str(values[rate_i]).replace(",", ""))
            except ValueError:
                rate = None
        line_no = str(values[line_i]).strip() if line_i is not None and line_i < len(values) and values[line_i] else str(n)
        rows.append(
            {
                "line_number": line_no,
                "csi_code": code if code and looks_like_csi(code) else None,
                "item_code": code,
                "description": str(values[desc_i]).strip(),
                "unit": unit,
                "default_rate": rate,
            }
        )
    wb.close()
    return rows

        
        def _parse_pdf(path: Path) -> list[dict[str, Any]]:
    from app.models.document import DocumentType
    from app.services.extractors import extract_file

            content = extract_file(path, DocumentType.PDF)
    rows: list[dict[str, Any]] = []
    for table in content.tables or []:
        table_rows = table.get("rows") or []
        if len(table_rows) < 2:
            continue
        header = [c.lower() for c in table_rows[0]]
        code_i = _find_col(header, ["csi", "code", "item code", "bid"])
        desc_i = _find_col(header, ["description", "desc", "item", "particular"])
        unit_i = _find_col(header, ["unit", "uom"])
        rate_i = _find_col(header, ["rate", "price", "unit price"])
        line_i = _find_col(header, ["line", "no", "#", "item no"])
        if desc_i is None:
            continue
        for n, row in enumerate(table_rows[1:], start=1):
            if desc_i >= len(row) or not str(row[desc_i]).strip():
                continue
            code = str(row[code_i]).strip() if code_i is not None and code_i < len(row) and row[code_i] else None
            unit = str(row[unit_i]).strip() if unit_i is not None and unit_i < len(row) and row[unit_i] else "unit"
            rate = None
            if rate_i is not None and rate_i < len(row) and row[rate_i]:
                try:
                    rate = float(re.sub(r"[^\d.]", "", str(row[rate_i])) or "0") or None
                except ValueError:
                    rate = None
            rows.append(
                {
                    "line_number": str(row[line_i]).strip() if line_i is not None and line_i < len(row) and row[line_i] else str(n),
                    "csi_code": code if code and looks_like_csi(code) else None,
                    "item_code": code,
                    "description": str(row[desc_i]).strip(),
                    "unit": unit,
                    "default_rate": rate,
                }
            )
    if rows:
        return rows
    return _parse_free_text(content.text or "")

        
        def _parse_free_text(text: str) -> list[dict[str, Any]]:
    rows: list[dict[str, Any]] = []
    for i, line in enumerate(text.splitlines(), start=1):
        line = line.strip()
        if len(line) < 8:
            continue
        # Pattern: CSI CODE  Description  UNIT  [rate]
        m = re.match(
            r"^(?:(\d+)\s+)?(\d{2}\s*\d{2}\s*\d{2}|\w[\w\-]{1,20})?\s*[-:]?\s*(.+?)\s+(m3|m2|m|cy|sy|lf|ea|nos|ls|t|kg)\b(?:\s+([\d,\.]+))?",
            line,
            flags=re.I,
        )
        if not m:
            continue
        code = (m.group(2) or "").strip() or None
        rows.append(
            {
                "line_number": m.group(1) or str(i),
                "csi_code": code if code and looks_like_csi(code) else None,
                "item_code": code,
                "description": m.group(3).strip(),
                "unit": m.group(4),
                "default_rate": float(m.group(5).replace(",", "")) if m.group(5) else None,
            }
        )
    return rows

        
        def _row_from_dict(r: dict[str, Any], fallback_line: int) -> dict[str, Any] | None:
    lower = {str(k).lower().strip(): v for k, v in r.items() if k is not None}

            def get(*names: str) -> Any:
        for n in names:
            for k, v in lower.items():
                if n == k or n in k:
                    return v
        return None

            desc = get("description", "desc", "item", "particular")
    if not desc or not str(desc).strip():
        return None
    code = get("csi", "csi_code", "item_code", "code", "bid item")
    code_s = str(code).strip() if code else None
    rate_raw = get("rate", "unit price", "price")
    rate = None
    if rate_raw is not None and str(rate_raw).strip():
        try:
            rate = float(str(rate_raw).replace(",", ""))
        except ValueError:
            rate = None
    return {
        "line_number": str(get("line", "item no", "no") or fallback_line),
        "csi_code": code_s if code_s and looks_like_csi(code_s) else None,
        "item_code": code_s,
        "description": str(desc).strip(),
        "unit": str(get("unit", "uom") or "unit"),
        "default_rate": rate,
    }

        
        def _find_col(header: list[str], names: list[str]) -> int | None:
    for i, h in enumerate(header):
        for n in names:
            if n == h or n in h:
                return i
    return None

### backend/app/services/eoq_eval.py

SHA-256: 9e30daaa8b78e90f55e52f8f86bce259358403215909a1744fee40437bb9dd93    ·    539 line(s)    ·    .py

"""Compare expected vs actual EOQ takeoff for gold-set / regression checks.

        Used by tests and `scripts/run_gold_set.py`. No OpenAI/APS required for
pure quantity_engine cases; full PDF cases can feed precomputed actual items.
"""

        from __future__ import annotations

        import re
from dataclasses import asdict, dataclass, field
from typing import Any

        
        CATEGORY_ALIASES = {
    "utilities": "Utilities",
    "utility": "Utilities",
    "drainage": "Drainage",
    "earthwork": "Earthwork",
    "pavement": "Pavement",
    "roadside": "Roadside",
    "structures": "Structures",
    "geometry": "Geometry",
    "bid schedule": "Bid schedule",
    "unmapped takeoff": "Unmapped takeoff",
}

        
        def _norm_text(value: str | None) -> str:
    return re.sub(r"\s+", " ", (value or "").strip().lower())

        
        def _norm_unit(value: str | None) -> str:
    u = _norm_text(value).replace(".", "")
    aliases = {
        "lf": "lf",
        "lin ft": "lf",
        "linear feet": "lf",
        "linear foot": "lf",
        "ft": "lf",
        "feet": "lf",
        "foot": "lf",
        "m": "m",
        "lm": "m",
        "ea": "ea",
        "each": "ea",
        "nos": "ea",
        "no": "ea",
        "cy": "cy",
        "cu yd": "cy",
        "cuyd": "cy",
        "m3": "m3",
        "cum": "m3",
        "sf": "sf",
        "sq ft": "sf",
        "sqft": "sf",
        "sy": "sy",
        "sq yd": "sy",
        "sqyd": "sy",
        "ls": "ls",
        "lump sum": "ls",
        "ton": "ton",
        "tons": "ton",
        "lb": "lb",
        "lbs": "lb",
        "acre": "acre",
        "hour": "hour",
        "hr": "hour",
    }
    return aliases.get(u, u or "unit")

        
        def _norm_category(value: str | None) -> str:
    key = _norm_text(value)
    return CATEGORY_ALIASES.get(key, (value or "Other").strip() or "Other")

        
        def _size_token(text: str) -> str | None:
    m = re.search(r"(\d{1,2}(?:\.\d+)?)\s*-?\s*(?:inch|in|\"|'')", text, re.I)
    if not m:
        return None
    try:
        val = float(m.group(1))
    except ValueError:
        return None
    if abs(val - int(val)) < 0.01:
        return f"{int(val)}-inch"
    return f"{val:g}-inch"

        
        def _tokens(text: str) -> set[str]:
    stop = {"the", "and", "for", "with", "of", "a", "an", "to", "in", "on", "or", "type", "size"}
    return {t for t in re.findall(r"[a-z0-9.]+", text.lower()) if len(t) > 1 and t not in stop}

        
        @dataclass
class ExpectedItem:
    description: str
    unit: str
    category: str | None = None
    quantity: float | None = None
    quantity_tolerance: float = 0.05  # relative
    quantity_abs_tolerance: float = 1.0
    item_code: str | None = None
    required: bool = True

        
        @dataclass
class MatchHit:
    expected: dict[str, Any]
    actual: dict[str, Any]
    qty_ok: bool
    qty_delta: float | None
    method: str

        
        @dataclass
class CompareReport:
    expected_count: int = 0
    actual_count: int = 0
    hits: list[MatchHit] = field(default_factory=list)
    misses: list[dict[str, Any]] = field(default_factory=list)
    extras: list[dict[str, Any]] = field(default_factory=list)
    misses_by_category: dict[str, int] = field(default_factory=dict)
    qty_errors: list[dict[str, Any]] = field(default_factory=list)
    unmapped_actual: list[dict[str, Any]] = field(default_factory=list)
    recall: float = 0.0
    precision_proxy: float = 0.0

            def to_dict(self) -> dict[str, Any]:
        visual = getattr(self, "_visual", None) or {}
        return {
            "expected_count": self.expected_count,
            "actual_count": self.actual_count,
            "hits": [
                {
                    "expected": h.expected,
                    "actual": {
                        "description": h.actual.get("description"),
                        "unit": h.actual.get("unit"),
                        "quantity": h.actual.get("quantity"),
                        "item_code": h.actual.get("item_code"),
                        "category": h.actual.get("category"),
                    },
                    "actual_description": h.actual.get("description"),
                    "qty_ok": h.qty_ok,
                    "qty_delta": h.qty_delta,
                    "method": h.method,
                }
                for h in self.hits
            ],
            "misses": self.misses,
            "extras": [
                {
                    "description": e.get("description"),
                    "unit": e.get("unit"),
                    "category": e.get("category"),
                    "quantity": e.get("quantity"),
                    "item_code": e.get("item_code"),
                    "calculation_method": e.get("calculation_method"),
                }
                for e in self.extras
            ],
            "misses_by_category": self.misses_by_category,
            "qty_errors": self.qty_errors,
            "unmapped_actual": [
                {"description": u.get("description"), "unit": u.get("unit")}
                for u in self.unmapped_actual
            ],
            "recall": self.recall,
            "precision_proxy": self.precision_proxy,
            "visual": visual,
        }

        
        def parse_expected_items(raw: list[dict[str, Any]] | dict[str, Any]) -> list[ExpectedItem]:
    rows = raw.get("items") if isinstance(raw, dict) else raw
    out: list[ExpectedItem] = []
    for row in rows or []:
        out.append(
            ExpectedItem(
                description=str(row.get("description") or ""),
                unit=str(row.get("unit") or "UNIT"),
                category=row.get("category"),
                quantity=float(row["quantity"]) if row.get("quantity") is not None else None,
                quantity_tolerance=float(row.get("quantity_tolerance", 0.05)),
                quantity_abs_tolerance=float(row.get("quantity_abs_tolerance", 1.0)),
                item_code=row.get("item_code"),
                required=bool(row.get("required", True)),
            )
        )
    return out

        
        def _item_match_score(expected: ExpectedItem, actual: dict[str, Any]) -> tuple[float, str]:
    ed = _norm_text(expected.description)
    ad = _norm_text(str(actual.get("description") or ""))
    # Normalize compound utility words so "watermain" ≈ "water main"
    ed = ed.replace("watermain", "water main").replace("stormsewer", "storm sewer")
    ad = ad.replace("watermain", "water main").replace("stormsewer", "storm sewer")
    eu = _norm_unit(expected.unit)
    au = _norm_unit(str(actual.get("unit") or ""))
    if not ed or not ad:
        return 0.0, "empty"

            e_size = _size_token(ed)
    a_size = _size_token(ad)
    size_conflict = bool(e_size and a_size and e_size != a_size)
    unit_conflict = bool(eu != au and eu != "unit" and au != "unit")

            if ed == ad:
        base, method = 100.0, "exact"
    elif ed in ad or ad in ed:
        base, method = 90.0, "substring"
    else:
        et, at = _tokens(ed), _tokens(ad)
        if not et or not at:
            return 0.0, "no_tokens"
        # Prefer recall-friendly overlap vs expected tokens (original gold-set behavior),
        # but also consider Jaccard so long AutoVAD strings don't dominate.
        overlap_exp = len(et & at) / max(len(et), 1)
        overlap_jac = len(et & at) / max(len(et | at), 1)
        overlap = max(overlap_exp, overlap_jac)
        base = overlap * 100.0
        method = "token_overlap"
        if e_size and a_size and e_size == a_size:
            base = min(100.0, base + 15.0)
        # Soft boost when most expected tokens are present
        if overlap_exp >= 0.7:
            base = max(base, 70.0)

            # Soft near-miss path for unit/size conflicts (do not hard-match these)
    if size_conflict:
        return min(base, 48.0), "size_mismatch"
    if unit_conflict:
        return min(base, 52.0), "unit_mismatch"
    if base >= 55:
        return base, method
    return base, "no_match" if base < 20 else "weak_overlap"

        
        def _qty_ok(expected: ExpectedItem, actual_qty: float) -> tuple[bool, float | None]:
    if expected.quantity is None:
        return True, None
    delta = abs(actual_qty - expected.quantity)
    rel = delta / max(abs(expected.quantity), 1e-9)
    ok = delta <= expected.quantity_abs_tolerance or rel <= expected.quantity_tolerance
    return ok, delta

        
        def _reason_for_miss(*, method: str, best_score: float, best_actual: dict[str, Any] | None) -> str:
    if not best_actual or best_score < 15:
        return "Not found in AutoVAD EOQ — missing schedule/pay-item extraction"
    if method == "unit_mismatch":
        return (
            f"Looks similar to “{best_actual.get('description')}” but unit differs "
            f"({best_actual.get('unit')}) — treat as unmatched"
        )
    if method == "size_mismatch":
        return (
            f"Similar wording to “{best_actual.get('description')}” but pipe/size differs"
        )
    if method in {"weak_overlap", "token_overlap", "no_match"} and best_score >= 20:
        return (
            f"Near wording to “{best_actual.get('description')}” "
            f"(similarity {best_score:.0f}%) but below match threshold"
        )
    return "Not confidently matched to any AutoVAD line"

        
        def _reason_for_extra(act: dict[str, Any]) -> str:
    desc = str(act.get("description") or "")
    method = str(act.get("calculation_method") or act.get("source_reference") or "").lower()
    if any(k in method for k in ("label", "callout", "drawing", "symbol", "geometry")):
        return "Plan/detail-derived item not listed on original EOQ schedule"
    if re.search(r"valve|hydrant|elbow|tee|bend|reducer|plug|casing|carrier", desc, re.I):
        return "Likely fitting/appurtenance invent — not a matching original pay item"
    return "Extra AutoVAD line with no matching original EOQ item"

        
        def compare_eoq(
    expected: list[ExpectedItem] | list[dict[str, Any]] | dict[str, Any],
    actual: list[dict[str, Any]],
) -> CompareReport:
    """Match expected gold items to actual takeoff/EOQ rows; report misses by category."""
    if isinstance(expected, list) and expected and isinstance(expected[0], ExpectedItem):
        exp_items = expected  # type: ignore[assignment]
    else:
        exp_items = parse_expected_items(expected)  # type: ignore[arg-type]
    report = CompareReport(expected_count=len(exp_items), actual_count=len(actual))
    used_actual: set[int] = set()

            matched_rows: list[dict[str, Any]] = []
    near_miss_rows: list[dict[str, Any]] = []
    line_audit: list[dict[str, Any]] = []
    by_category: dict[str, dict[str, float]] = {}

            def cat_bucket(cat: str | None) -> dict[str, float]:
        key = _norm_category(cat)
        if key not in by_category:
            by_category[key] = {
                "expected": 0,
                "matched": 0,
                "qty_error": 0,
                "missed": 0,
                "near_miss": 0,
                "extras": 0,
            }
        return by_category[key]

            for exp in exp_items:
        cat = _norm_category(exp.category)
        cat_bucket(cat)["expected"] += 1

                best_idx = -1
        best_score = 0.0
        best_method = "no_match"
        for idx, act in enumerate(actual):
            if idx in used_actual:
                continue
            score, method = _item_match_score(exp, act)
            if score > best_score:
                best_score = score
                best_idx = idx
                best_method = method

                best_act = actual[best_idx] if best_idx >= 0 else None

                # Hard match
        if best_idx >= 0 and best_score >= 55 and best_method not in {"unit_mismatch", "size_mismatch"}:
            act = actual[best_idx]
            used_actual.add(best_idx)
            try:
                aqty = float(act.get("quantity") or 0)
            except (TypeError, ValueError):
                aqty = 0.0
            ok, delta = _qty_ok(exp, aqty)
            hit = MatchHit(
                expected=asdict(exp),
                actual=act,
                qty_ok=ok,
                qty_delta=delta,
                method=best_method,
            )
            report.hits.append(hit)
            status = "matched" if ok else "qty_error"
            reason = (
                "Matched description/unit; quantities within tolerance"
                if ok
                else (
                    f"Matched item but quantity differs — original {exp.quantity} vs AutoVAD {aqty} "
                    f"(Δ {delta})"
                )
            )
            row = {
                "status": status,
                "category": cat,
                "item_code": exp.item_code or act.get("item_code"),
                "original_description": exp.description,
                "original_unit": exp.unit,
                "original_qty": exp.quantity,
                "autovad_description": act.get("description"),
                "autovad_unit": act.get("unit"),
                "autovad_qty": aqty,
                "qty_ok": ok,
                "qty_delta": delta,
                "similarity": round(best_score, 1),
                "match_method": best_method,
                "reason": reason,
            }
            matched_rows.append(row)
            line_audit.append(row)
            cat_bucket(cat)["matched"] += 1
            if not ok:
                cat_bucket(cat)["qty_error"] += 1
                report.qty_errors.append(
                    {
                        "description": exp.description,
                        "expected_qty": exp.quantity,
                        "actual_qty": aqty,
                        "delta": delta,
                        "unit": exp.unit,
                        "category": cat,
                        "autovad_description": act.get("description"),
                        "reason": reason,
                    }
                )
            continue

                # Near-miss: looks similar but not accepted
        if best_act is not None and best_score >= 20:
            try:
                aqty = float(best_act.get("quantity") or 0)
            except (TypeError, ValueError):
                aqty = None
            reason = _reason_for_miss(method=best_method, best_score=best_score, best_actual=best_act)
            near = {
                "status": "near_miss",
                "category": cat,
                "item_code": exp.item_code,
                "original_description": exp.description,
                "original_unit": exp.unit,
                "original_qty": exp.quantity,
                "autovad_description": best_act.get("description"),
                "autovad_unit": best_act.get("unit"),
                "autovad_qty": aqty,
                "similarity": round(best_score, 1),
                "match_method": best_method,
                "reason": reason,
            }
            near_miss_rows.append(near)
            cat_bucket(cat)["near_miss"] += 1
            # Still count as miss for recall if required
            if exp.required:
                miss = {
                    "description": exp.description,
                    "unit": exp.unit,
                    "category": cat,
                    "quantity": exp.quantity,
                    "item_code": exp.item_code,
                    "reason": reason,
                    "nearest_autovad": best_act.get("description"),
                    "similarity": round(best_score, 1),
                }
                report.misses.append(miss)
                report.misses_by_category[cat] = report.misses_by_category.get(cat, 0) + 1
                cat_bucket(cat)["missed"] += 1
                line_audit.append({**near, "status": "miss_near"})
            continue

                if exp.required:
            reason = _reason_for_miss(method=best_method, best_score=best_score, best_actual=best_act)
            miss = {
                "description": exp.description,
                "unit": exp.unit,
                "category": cat,
                "quantity": exp.quantity,
                "item_code": exp.item_code,
                "reason": reason,
                "nearest_autovad": (best_act or {}).get("description"),
                "similarity": round(best_score, 1) if best_act else 0,
            }
            report.misses.append(miss)
            report.misses_by_category[cat] = report.misses_by_category.get(cat, 0) + 1
            cat_bucket(cat)["missed"] += 1
            line_audit.append(
                {
                    "status": "miss",
                    "category": cat,
                    "item_code": exp.item_code,
                    "original_description": exp.description,
                    "original_unit": exp.unit,
                    "original_qty": exp.quantity,
                    "autovad_description": (best_act or {}).get("description"),
                    "autovad_unit": (best_act or {}).get("unit"),
                    "autovad_qty": (best_act or {}).get("quantity"),
                    "similarity": round(best_score, 1) if best_act else 0,
                    "match_method": best_method,
                    "reason": reason,
                }
            )

            extras_rows: list[dict[str, Any]] = []
    for idx, act in enumerate(actual):
        if idx not in used_actual:
            report.extras.append(act)
            reason = _reason_for_extra(act)
            cat = _norm_category(str(act.get("category") or "Other"))
            cat_bucket(cat)["extras"] += 1
            extras_rows.append(
                {
                    "status": "extra",
                    "category": cat,
                    "item_code": act.get("item_code"),
                    "autovad_description": act.get("description"),
                    "autovad_unit": act.get("unit"),
                    "autovad_qty": act.get("quantity"),
                    "reason": reason,
                    "source": act.get("calculation_method") or act.get("source_reference"),
                }
            )
        method = str(act.get("bid_match_method") or "")
        if method == "unmapped" or (
            act.get("bid_template_line_id") is None
            and str(act.get("category") or "").lower() == "unmapped takeoff"
        ):
            report.unmapped_actual.append(act)

            required = sum(1 for e in exp_items if e.required) or len(exp_items) or 1
    report.recall = round(len(report.hits) / required, 4)
    report.precision_proxy = round(
        len(report.hits) / max(len(actual), 1),
        4,
    )

            # Attach engineer visual payload on the report object via to_dict extension
    report._visual = {  # type: ignore[attr-defined]
        "matched": matched_rows,
        "near_misses": near_miss_rows,
        "misses": list(report.misses),
        "qty_errors": report.qty_errors,
        "extras": extras_rows,
        "line_audit": line_audit,
        "by_category": [
            {"category": k, **v}
            for k, v in sorted(by_category.items(), key=lambda kv: (-kv[1]["missed"], -kv[1]["expected"], kv[0]))
        ],
        "summary": {
            "expected": report.expected_count,
            "autovad": report.actual_count,
            # All hard matches (qty OK + qty error) — what the Matched table shows
            "matched": len(matched_rows),
            "matched_qty_ok": len([r for r in matched_rows if r["status"] == "matched"]),
            "qty_errors": len(report.qty_errors),
            "near_misses": len(near_miss_rows),
            "misses": len(report.misses),
            "extras": len(extras_rows),
            "recall": report.recall,
            "precision_proxy": report.precision_proxy,
        },
    }
    return report

        
        def summarize_report(report: CompareReport) -> str:
    lines = [
        f"Recall: {report.recall:.0%}  ({len(report.hits)}/{report.expected_count} expected)",
        f"Actual rows: {report.actual_count}  |  extras: {len(report.extras)}  |  qty errors: {len(report.qty_errors)}",
    ]
    if report.misses_by_category:
        cats = ", ".join(f"{k}={v}" for k, v in sorted(report.misses_by_category.items()))
        lines.append(f"Misses by category: {cats}")
    for m in report.misses[:12]:
        lines.append(f"  MISS [{m.get('category')}] {m.get('description')} ({m.get('unit')})")
    for q in report.qty_errors[:8]:
        lines.append(
            f"  QTY  {q.get('description')}: expected {q.get('expected_qty')} "
            f"got {q.get('actual_qty')} (delta {q.get('delta')})"
        )
    return "\n".join(lines)

### backend/app/services/pdf_vision.py

SHA-256: 5a74fc2d9794fa59b0b6a18d46e4a595b10ea8a9dda62484b1d303e72411e943    ·    274 line(s)    ·    .py

"""Render PDF plan sheets for OpenAI vision takeoff.

        Engineering PDFs are often drawing-heavy with little extractable text.
PyMuPDF renders pages so vision models can read plans, details,
profiles, and callouts — not just text/tables.

        By default every page is scanned in batches (upload size/page count is
not limited here). Batch size only controls OpenAI request chunking.
"""

        from __future__ import annotations

        import base64
import re
from dataclasses import dataclass, field
from pathlib import Path
from typing import Iterator

        import fitz

        DRAWING_KEYWORDS = re.compile(
    r"\b(plan|profile|section|detail|typical|cross[\s-]?section|alignment|"
    r"pavement|utility|sewer|water|watermain|water\s*main|\bwm\b|storm|grading|earthwork|quantity|"
    r"schedule|laying|station|curb|gutter|manhole|pipe|valve|hydrant|inlet|"
    r"summary|bid\s*item|pay\s*item|tabulation)\b",
    re.I,
)

        UTILITY_LABEL_HINTS = re.compile(
    r"(\d{1,2}\s*(?:\"|''|in)\s*(?:water|wm|sanitary|storm)|"
    r"water\s*mains?|watermains?|\bwm\b|prop(?:osed)?\.?\s*wm|"
    r"gate\s*valve|fire\s*hydrant|dip\s+wm|pvc\s+wm|"
    r"sanitary\s*sewer|storm\s*drain|manhole|catch\s*basin)",
    re.I,
)

        SCHEDULE_HINTS = re.compile(
    r"\b(quantity|qty|schedule|bid\s*item|pay\s*item|estimate|summary\s*of\s*quantit|"
    r"pipe\s*schedule|structure\s*schedule|tabulation)\b",
    re.I,
)

        PROFILE_HINTS = re.compile(r"\b(profile|sta\.?|stationing|invert|grade\s*line)\b", re.I)

        
        @dataclass
class PageImage:
    page: int
    png_b64: str
    width: int
    height: int
    reason: str

        
        @dataclass
class VisionPagePlan:
    """Which pages will be scanned (does not hold image bytes)."""

            page_count: int
    selected_pages: list[int] = field(default_factory=list)  # 1-based
    skipped_pages: list[int] = field(default_factory=list)
    truncated: bool = False
    forced_utility_pages: list[int] = field(default_factory=list)
    scan_all: bool = True
    batch_size: int = 8
    reasons: dict[int, str] = field(default_factory=dict)  # 1-based page -> reason

        
        @dataclass
class VisionPageSelection:
    """Legacy-compatible selection that may include rendered images."""

            images: list[PageImage]
    page_count: int
    selected_pages: list[int] = field(default_factory=list)
    skipped_pages: list[int] = field(default_factory=list)
    truncated: bool = False
    forced_utility_pages: list[int] = field(default_factory=list)
    scan_all: bool = False
    batch_size: int = 0

        
        def plan_pdf_vision_pages(
    path: Path,
    *,
    max_pages: int = 0,
    min_score: float = 18.0,
    force_utility_pages: bool = True,
    scan_all_pages: bool = True,
    batch_pages: int = 8,
) -> VisionPagePlan:
    """Decide which pages to scan. Default: all pages."""
    batch_pages = max(1, batch_pages)
    with fitz.open(path) as doc:
        page_count = doc.page_count
        if page_count == 0:
            return VisionPagePlan(page_count=0, batch_size=batch_pages, scan_all=scan_all_pages)

                scored: list[tuple[float, int, str, bool]] = []
        for i in range(page_count):
            page = doc.load_page(i)
            text = page.get_text("text") or ""
            score, reason = _score_page(text=text, page_index=i, page_count=page_count)
            has_utility = bool(UTILITY_LABEL_HINTS.search(text)) or bool(SCHEDULE_HINTS.search(text))
            scored.append((score, i, reason, has_utility))

                reasons = {i + 1: reason for _s, i, reason, _u in scored}

                if scan_all_pages:
            chosen_idxs = list(range(page_count))
            if max_pages and max_pages > 0:
                chosen_idxs = chosen_idxs[:max_pages]
            forced: list[int] = []
        else:
            cap = max_pages if max_pages and max_pages > 0 else 24
            scored_sorted = sorted(scored, key=lambda row: (-row[0], row[1]))
            limit = max(1, min(cap, page_count))
            eligible = [row for row in scored_sorted if row[0] >= min_score] or scored_sorted[:]
            chosen_map: dict[int, tuple[float, int, str, bool]] = {
                row[1]: row for row in eligible[:limit]
            }
            forced = []
            if force_utility_pages:
                for row in scored_sorted:
                    if not row[3] or row[1] in chosen_map:
                        continue
                    if len(chosen_map) < limit:
                        chosen_map[row[1]] = row
                        forced.append(row[1] + 1)
                        continue
                    replaceable = sorted(
                        (r for r in chosen_map.values() if not r[3]),
                        key=lambda r: (r[0], -r[1]),
                    )
                    if not replaceable:
                        break
                    victim = replaceable[0]
                    del chosen_map[victim[1]]
                    chosen_map[row[1]] = row
                    forced.append(row[1] + 1)
            chosen_idxs = sorted(chosen_map.keys())

                selected_pages = [i + 1 for i in chosen_idxs]
        skipped_pages = [i + 1 for i in range(page_count) if (i + 1) not in selected_pages]
        return VisionPagePlan(
            page_count=page_count,
            selected_pages=selected_pages,
            skipped_pages=skipped_pages,
            truncated=len(skipped_pages) > 0,
            forced_utility_pages=forced,
            scan_all=scan_all_pages,
            batch_size=batch_pages,
            reasons=reasons,
        )

        
        def iter_rendered_pdf_batches(
    path: Path,
    plan: VisionPagePlan,
    *,
    dpi: int = 150,
    batch_pages: int | None = None,
) -> Iterator[list[PageImage]]:
    """Render selected pages in batches so large PDFs are not held fully in memory."""
    size = max(1, batch_pages or plan.batch_size or 8)
    zoom = dpi / 72.0
    matrix = fitz.Matrix(zoom, zoom)
    pages = plan.selected_pages
    with fitz.open(path) as doc:
        for start in range(0, len(pages), size):
            chunk: list[PageImage] = []
            for page_no in pages[start : start + size]:
                idx = page_no - 1
                page = doc.load_page(idx)
                pix = page.get_pixmap(matrix=matrix, alpha=False)
                if pix.width > 1800:
                    scale = 1800 / pix.width
                    pix = page.get_pixmap(matrix=fitz.Matrix(zoom * scale, zoom * scale), alpha=False)
                png_b64 = base64.b64encode(pix.tobytes("png")).decode("ascii")
                chunk.append(
                    PageImage(
                        page=page_no,
                        png_b64=png_b64,
                        width=pix.width,
                        height=pix.height,
                        reason=plan.reasons.get(page_no, "full scan"),
                    )
                )
            yield chunk

        
        def select_and_render_pdf_pages(
    path: Path,
    *,
    max_pages: int = 0,
    dpi: int = 150,
    min_score: float = 18.0,
    force_utility_pages: bool = True,
    scan_all_pages: bool = True,
    batch_pages: int = 8,
) -> VisionPageSelection:
    """Compatibility helper: plan + render all selected pages (prefer iter_rendered_pdf_batches)."""
    plan = plan_pdf_vision_pages(
        path,
        max_pages=max_pages,
        min_score=min_score,
        force_utility_pages=force_utility_pages,
        scan_all_pages=scan_all_pages,
        batch_pages=batch_pages,
    )
    images: list[PageImage] = []
    for batch in iter_rendered_pdf_batches(path, plan, dpi=dpi, batch_pages=batch_pages):
        images.extend(batch)
    return VisionPageSelection(
        images=images,
        page_count=plan.page_count,
        selected_pages=plan.selected_pages,
        skipped_pages=plan.skipped_pages,
        truncated=plan.truncated,
        forced_utility_pages=plan.forced_utility_pages,
        scan_all=plan.scan_all,
        batch_size=plan.batch_size,
    )

        
        def _score_page(*, text: str, page_index: int, page_count: int) -> tuple[float, str]:
    """Higher score = more likely a useful engineering drawing / qty sheet."""
    stripped = text.strip()
    char_count = len(stripped)
    reasons: list[str] = []
    score = 0.0

            if char_count < 80:
        score += 40
        reasons.append("sparse-text drawing")
    elif char_count < 400:
        score += 28
        reasons.append("light-text sheet")
    elif char_count < 1200:
        score += 12
        reasons.append("mixed sheet")
    else:
        score += 4
        reasons.append("text-heavy")

            if DRAWING_KEYWORDS.search(stripped):
        score += 22
        reasons.append("civil keywords")

            if page_index == 0:
        score += 10
        reasons.append("title/cover")

            if page_count > 8 and 0.15 <= (page_index / max(page_count - 1, 1)) <= 0.9:
        score += 6

            if SCHEDULE_HINTS.search(stripped):
        score += 28
        reasons.append("schedule/qty sheet")

            if PROFILE_HINTS.search(stripped):
        score += 16
        reasons.append("profile/stationing")

            if re.search(r"\b(cu\.?\s*yd|sq\.?\s*yd|linear\s*feet|l\.?f\.?)\b", stripped, re.I):
        score += 12
        reasons.append("unit signals")

            utility_hits = len(UTILITY_LABEL_HINTS.findall(stripped))
    if utility_hits:
        score += min(40, 16 + utility_hits * 4)
        reasons.append(f"utility labels×{utility_hits}")

            return score, ", ".join(reasons) or "selected"

### backend/app/services/openai_client.py

SHA-256: 11cbe59cbb25465631116621210cee4ffe62ec7b3d6aa12ed96e4c99f2e3c127    ·    318 line(s)    ·    .py

"""Shared OpenAI client for document AI, chat, and CAD enrichment.

        Uses OPENAI_MODEL from settings (default: gpt-5.6-terra).
Prefer Responses API when available; fall back to Chat Completions.
"""

        from __future__ import annotations

        import json
import re
from typing import Any

        from app.config import get_settings

        
        def openai_configured() -> bool:
    key = (get_settings().openai_api_key or "").strip()
    return bool(key)

        
        def openai_status() -> dict[str, Any]:
    settings = get_settings()
    configured = openai_configured()
    return {
        "configured": configured,
        "model": settings.openai_model,
        "mode": "openai" if configured else "heuristic",
    }

        
        def get_openai_client():
    if not openai_configured():
        raise RuntimeError("OPENAI_API_KEY is not configured")
    from openai import OpenAI

            # Long timeout per call: large vision batches / takeoff prompts can run many minutes.
    # Retries stay low so a stuck call does not multiply wait time endlessly.
    return OpenAI(
        api_key=get_settings().openai_api_key.strip(),
        timeout=3600.0,  # 60 minutes per OpenAI request
        max_retries=2,
    )

        
        def _model_name() -> str:
    return (get_settings().openai_model or "gpt-5.6-terra").strip()

        
        def _supports_custom_temperature(model: str) -> bool:
    """Some GPT-5 family models only allow the default temperature."""
    low = model.lower()
    if "gpt-5" in low or "terra" in low or "o1" in low or "o3" in low or "o4" in low:
        return False
    return True

        
        def ask_openai_json(system: str, user: str, *, temperature: float = 0.1) -> dict[str, Any]:
    """Ask OpenAI and parse a JSON object from the response."""
    raw = ask_openai_text(system=system, user=user, temperature=temperature)
    return _parse_json_object(raw)

        
        def ask_openai_text(system: str, user: str, *, temperature: float = 0.1) -> str:
    return _ask_openai(
        system=system,
        user_text=user,
        images=None,
        temperature=temperature,
    )

        
        def ask_openai_vision_json(
    system: str,
    user: str,
    images: list[dict[str, Any]],
    *,
    temperature: float = 0.1,
) -> dict[str, Any]:
    """Multimodal JSON ask — images are {page, png_b64} dicts."""
    raw = _ask_openai(
        system=system,
        user_text=user,
        images=images,
        temperature=temperature,
    )
    return _parse_json_object(raw)

        
        def _ask_openai(
    *,
    system: str,
    user_text: str,
    images: list[dict[str, Any]] | None,
    temperature: float = 0.1,
) -> str:
    client = get_openai_client()
    model = _model_name()
    use_temp = _supports_custom_temperature(model)
    errors: list[str] = []
    image_parts = images or []

            # 1) Responses API (preferred) — supports input_image
    if hasattr(client, "responses"):
        try:
            user_content: list[dict[str, Any]] = [{"type": "input_text", "text": user_text}]
            for img in image_parts:
                b64 = img.get("png_b64") or img.get("b64")
                if not b64:
                    continue
                page = img.get("page")
                if page:
                    user_content.append({"type": "input_text", "text": f"[Plan sheet page {page}]"})
                user_content.append(
                    {
                        "type": "input_image",
                        "image_url": f"data:image/png;base64,{b64}",
                    }
                )
            kwargs: dict[str, Any] = {
                "model": model,
                "input": [
                    {"role": "system", "content": [{"type": "input_text", "text": system}]},
                    {"role": "user", "content": user_content},
                ],
            }
            if use_temp:
                kwargs["temperature"] = temperature
            response = client.responses.create(**kwargs)
            text = getattr(response, "output_text", None) or _responses_text(response)
            if text and text.strip():
                return text
            errors.append("Responses API returned empty output_text")
        except Exception as exc:
            errors.append(f"Responses API: {exc}")

                if not image_parts:
            try:
                response = client.responses.create(
                    model=model,
                    input=f"{system}\n\n{user_text}",
                )
                text = getattr(response, "output_text", None) or _responses_text(response)
                if text and text.strip():
                    return text
                errors.append("Simple Responses API returned empty output_text")
            except Exception as exc:
                errors.append(f"Simple Responses API: {exc}")

            # 2) Chat Completions fallback (vision via image_url)
    try:
        user_content_chat: list[dict[str, Any]] | str
        if image_parts:
            user_content_chat = [{"type": "text", "text": user_text}]
            for img in image_parts:
                b64 = img.get("png_b64") or img.get("b64")
                if not b64:
                    continue
                page = img.get("page")
                if page:
                    user_content_chat.append({"type": "text", "text": f"[Plan sheet page {page}]"})
                user_content_chat.append(
                    {
                        "type": "image_url",
                        "image_url": {"url": f"data:image/png;base64,{b64}"},
                    }
                )
        else:
            user_content_chat = user_text

                kwargs = {
            "model": model,
            "messages": [
                {"role": "system", "content": system},
                {"role": "user", "content": user_content_chat},
            ],
        }
        if use_temp:
            kwargs["temperature"] = temperature
        if "json" in system.lower() or "JSON" in user_text:
            kwargs["response_format"] = {"type": "json_object"}
        completion = client.chat.completions.create(**kwargs)
        content = completion.choices[0].message.content or ""
        if content.strip():
            return content
        errors.append("Chat Completions returned empty content")
    except Exception as exc:
        errors.append(f"Chat Completions: {exc}")

            raise RuntimeError(
        f"OpenAI call failed for model '{model}'. "
        + " | ".join(errors[:3])
        + " Check OPENAI_MODEL against models available on your OpenAI account "
        + "(vision requires a multimodal model)."
    )

        
        def apply_cad_label_enrichment(
    original: list[dict[str, Any]],
    refined: list[dict[str, Any]] | None,
) -> list[dict[str, Any]]:
    """Overlay AI labels only. Never add, drop, split, or change quantity/unit."""
    if not original:
        return []
    refined = list(refined or [])
    out: list[dict[str, Any]] = []
    for idx, src in enumerate(original):
        row = dict(src)
        if idx < len(refined) and isinstance(refined[idx], dict):
            cand = refined[idx]
            desc = str(cand.get("description") or "").strip()
            if desc:
                row["description"] = desc
            cat = str(cand.get("category") or "").strip()
            if cat:
                row["category"] = cat
            code = cand.get("item_code") or cand.get("csi_code")
            if code:
                row["item_code"] = str(code).strip()
            try:
                conf = float(cand.get("confidence") or 0)
                row["confidence"] = max(float(row.get("confidence") or 0), min(conf, 95.0))
            except (TypeError, ValueError):
                pass
        out.append(row)
    from app.services.traffic_control import consolidate_traffic_control_signs

            out, _ = consolidate_traffic_control_signs(out, allow_online_refresh=False)
    return out

        
        def enrich_cad_quantities_with_openai(
    *,
    filename: str,
    extraction_summary: str,
    stats: dict[str, Any],
    quantities: list[dict[str, Any]],
    layers: list[Any],
    blocks: list[Any],
    pipes: list[Any] | None = None,
    texts: list[Any] | None = None,
) -> list[dict[str, Any]]:
    """Ask OpenAI to rename/classify CAD quantity candidates. Item set and quantities stay fixed."""
    if not openai_configured() or not quantities:
        return quantities

            payload = {
        "filename": filename,
        "summary": extraction_summary,
        "stats": stats,
        "layers": layers[:100],
        "blocks": blocks[:150],
        "pipes": (pipes or [])[:120],
        "texts": (texts or [])[:80],
        "quantities": quantities[:200],
    }
    n = min(len(quantities), 200)
    system = (
        "You are a USA civil estimator labeling CAD takeoff lines. "
        "You may ONLY improve description, category, and item_code. "
        "Return exactly one output item per input item, same order. JSON only."
    )
    user = f"""
Relabel these CAD quantities for a civil EOQ (roads, utilities, dams, reservoirs, buildings).

        Hard rules:
- Return EXACTLY {n} items in the SAME order as INPUT quantities.
- Do NOT add, drop, merge, or split lines.
- Do NOT change quantity or unit.
- KEEP pipe SIZE in descriptions (e.g. "8-Inch Water Main").
- Prefer network-specific names: Water / Sanitary / Storm.
- TRAFFIC SIGNS: description "Traffic Control" is already rolled up — do not split it.
- Preserve calculation_method and source_reference.

        INPUT_JSON:
{json.dumps(payload, ensure_ascii=True)[:50000]}

        Return JSON:
{{
  "items": [
    {{
      "description": "8-Inch Water Main",
      "category": "Watermain",
      "item_code": null,
      "confidence": 92
    }}
  ]
}}
"""
    try:
        data = ask_openai_json(system, user)
        items = data.get("items") or []
        return apply_cad_label_enrichment(quantities, items)
    except Exception:
        return quantities

        
        def _responses_text(response: Any) -> str:
    chunks: list[str] = []
    for item in getattr(response, "output", []) or []:
        for content in getattr(item, "content", []) or []:
            text = getattr(content, "text", None)
            if text:
                chunks.append(text)
    return "\n".join(chunks)

        
        def _parse_json_object(raw: str) -> dict[str, Any]:
    text = raw.strip()
    if text.startswith("```"):
        text = re.sub(r"^```(?:json)?\s*", "", text)
        text = re.sub(r"\s*```$", "", text)
    try:
        return json.loads(text)
    except json.JSONDecodeError:
        match = re.search(r"\{.*\}", text, flags=re.DOTALL)
        if not match:
            raise
        return json.loads(match.group(0))

### backend/app/services/extractors.py

SHA-256: 2557e02acc3cb1bd1ba708134b65590771d7c24d091f54e1a6d46e831e72ec48    ·    113 line(s)    ·    .py

"""Raw text / table extraction from uploaded civil documents."""

        from __future__ import annotations

        import csv
import io
import zipfile
from dataclasses import dataclass, field
from pathlib import Path

        import fitz  # PyMuPDF
import pdfplumber
from openpyxl import load_workbook

        from app.models.document import DocumentType

        
        @dataclass
class PageText:
    page: int
    text: str

        
        @dataclass
class ExtractedContent:
    text: str
    page_count: int | None = None
    pages: list[PageText] = field(default_factory=list)
    tables: list[dict] = field(default_factory=list)

        
        def extract_file(path: Path, document_type: DocumentType) -> ExtractedContent:
    if document_type == DocumentType.PDF:
        return _extract_pdf(path)
    if document_type == DocumentType.EXCEL:
        return _extract_excel(path)
    if document_type == DocumentType.CSV:
        return _extract_csv(path)
    if document_type == DocumentType.IMAGE:
        return ExtractedContent(
            text=f"[Image file: {path.name}] OCR/vision extraction will use AI when OpenAI key is configured.",
            page_count=1,
            pages=[PageText(page=1, text=f"Image document: {path.name}")],
        )
    if document_type == DocumentType.ZIP:
        return _extract_zip(path)
    return ExtractedContent(text=f"Unsupported or unknown file type: {path.name}")

        
        def _extract_pdf(path: Path) -> ExtractedContent:
    pages: list[PageText] = []
    tables: list[dict] = []

            with fitz.open(path) as doc:
        for i, page in enumerate(doc, start=1):
            text = page.get_text("text") or ""
            pages.append(PageText(page=i, text=text))

            try:
        with pdfplumber.open(path) as pdf:
            for i, page in enumerate(pdf.pages, start=1):
                for table in page.extract_tables() or []:
                    if not table:
                        continue
                    tables.append({"page": i, "rows": table[:80]})
    except Exception:
        pass

            full_text = "\n\n".join(f"--- Page {p.page} ---\n{p.text}" for p in pages if p.text.strip())
    return ExtractedContent(text=full_text, page_count=len(pages), pages=pages, tables=tables)

        
        def _extract_excel(path: Path) -> ExtractedContent:
    wb = load_workbook(path, data_only=True, read_only=True)
    chunks: list[str] = []
    tables: list[dict] = []
    for sheet in wb.worksheets:
        rows: list[list[str]] = []
        for row in sheet.iter_rows(values_only=True, max_row=200, max_col=20):
            values = [("" if c is None else str(c)).strip() for c in row]
            if any(values):
                rows.append(values)
        if rows:
            tables.append({"page": 1, "sheet": sheet.title, "rows": rows[:80]})
            lines = ["\t".join(r) for r in rows[:80]]
            chunks.append(f"--- Sheet {sheet.title} ---\n" + "\n".join(lines))
    wb.close()
    return ExtractedContent(text="\n\n".join(chunks), page_count=1, pages=[PageText(page=1, text="\n\n".join(chunks))], tables=tables)

        
        def _extract_csv(path: Path) -> ExtractedContent:
    text = path.read_text(encoding="utf-8", errors="ignore")
    rows: list[list[str]] = []
    reader = csv.reader(io.StringIO(text))
    for i, row in enumerate(reader):
        if i >= 200:
            break
        rows.append([c.strip() for c in row])
    return ExtractedContent(
        text=text[:50000],
        page_count=1,
        pages=[PageText(page=1, text=text[:20000])],
        tables=[{"page": 1, "rows": rows[:80]}],
    )

        
        def _extract_zip(path: Path) -> ExtractedContent:
    names: list[str] = []
    with zipfile.ZipFile(path, "r") as zf:
        names = zf.namelist()[:200]
    listing = "\n".join(names)
    text = f"ZIP archive contains {len(names)} entries:\n{listing}"
    return ExtractedContent(text=text, page_count=1, pages=[PageText(page=1, text=text)])

### backend/app/services/cad/quantity_engine.py

SHA-256: 092e3278afd4aa4b445ce3f8d13f30163f93e96e545f3940a13f671333fb382f    ·    769 line(s)    ·    .py

"""Map CAD extractions to size-aware civil quantity takeoff candidates.

        Goals vs generic layer sums:
- Pipe lengths split by size + network (water / sanitary / storm)
- Count bends, valves, tees, reducers, hydrants by type (+ size when named)
- Storm/sanitary structures: manholes, inlets, catch basins, junction boxes
- Earthwork cut / fill volumes when surfaces expose them
"""

        from __future__ import annotations

        import re
from typing import Any

        from app.services.csi_mapper import enrich_quantity_item

        # inch sizes: 8", 8 in, 8-inch, 8IN, Ø8, DIA 8, DN200 (approx mm→in when large)
_SIZE_RE = re.compile(
    r"(?:"
    r"(?:Ø|ø|dia\.?|diam\.?|diameter|dn|nps)?\s*"
    r"(\d{1,2}(?:\.\d+)?)\s*(?:\"|''|in(?:ch(?:es)?)?|mm)?\b"
    r"|"
    r"\b(\d{1,2}(?:\.\d+)?)(?:\"|''|in)\b"
    r")",
    re.I,
)

        _MM_SIZE_RE = re.compile(r"\b(\d{2,4})\s*mm\b", re.I)

        _NOISE_LAYERS = {
    "0",
    "defpoints",
    "view",
    "textm",
    "psplan_vport",
    "psprof_vport",
    "viewport",
    "vport",
    "title",
    "border",
    "legend",
}

        _NOISE_NAME_BITS = (
    "mview",
    "viewport",
    "logo",
    "title",
    "border",
    "legend",
    "model_space",
    "paper_space",
    "crowsfoot",
    "*model",
    "*paper",
    "north arrow",
    "barscale",
    "sheet",
)

        
        def _is_noise_layer(name: str) -> bool:
    low = (name or "").strip().lower()
    if not low or low in _NOISE_LAYERS:
        return True
    return any(token in low for token in ("vport", "viewport", "defpoints", "title block"))

        
        def _is_noise_name(name: str) -> bool:
    low = (name or "").lower()
    return any(bit in low for bit in _NOISE_NAME_BITS)

        
        def _is_paper_space(entity: dict[str, Any]) -> bool:
    space = str(entity.get("space") or entity.get("layout") or "").lower()
    layer = str(entity.get("layer") or "").lower()
    return "paper" in space or layer.startswith("ps") or "paper_space" in layer

        
        def _length_scale_to_feet(units: Any) -> float:
    """Convert drawing length units to feet for LF takeoff."""
    if units is None:
        return 1.0
    raw = str(units).strip().lower()
    # ezdxf / INSUNITS common values
    mapping = {
        "0": 1.0,  # unitless — assume feet for US civil
        "1": 1.0 / 12.0,  # inches
        "2": 1.0,  # feet
        "4": 1.0 / 25.4 / 12.0,  # mm
        "5": 1.0 / 2.54 / 12.0,  # cm
        "6": 3.280839895,  # meters
        "inch": 1.0 / 12.0,
        "inches": 1.0 / 12.0,
        "in": 1.0 / 12.0,
        "ft": 1.0,
        "feet": 1.0,
        "foot": 1.0,
        "mm": 1.0 / 25.4 / 12.0,
        "cm": 1.0 / 2.54 / 12.0,
        "m": 3.280839895,
        "meter": 3.280839895,
        "meters": 3.280839895,
        "metre": 3.280839895,
        "metres": 3.280839895,
    }
    if raw in mapping:
        return mapping[raw]
    # strings like "InsertionUnits.Feet"
    for key, scale in mapping.items():
        if key.isalpha() and key in raw:
            return scale
    return 1.0

        
        def _size_hints_from_texts(extraction: dict[str, Any]) -> dict[str, str]:
    """Map layer → most common size label found in TEXT/MTEXT on that layer."""
    counts: dict[str, dict[str, int]] = {}
    for text in extraction.get("texts") or []:
        layer = str(text.get("layer") or "").strip()
        body = str(text.get("text") or text.get("content") or "")
        size = extract_size_label(body, layer)
        if not layer or not size:
            continue
        bucket = counts.setdefault(layer.lower(), {})
        bucket[size] = bucket.get(size, 0) + 1
    # Also scan dimension text-like measurement if present as inches
    for dim in extraction.get("dimensions") or []:
        layer = str(dim.get("layer") or "").strip()
        measurement = dim.get("measurement")
        size = extract_size_label(measurement, layer, dim.get("text"))
        if not layer or not size:
            continue
        # Only trust small civil pipe diameters from dimensions
        try:
            inches = float(re.sub(r"[^0-9.]", "", size.split("-")[0]))
        except ValueError:
            continue
        if inches < 1 or inches > 72:
            continue
        bucket = counts.setdefault(layer.lower(), {})
        bucket[size] = bucket.get(size, 0) + 1

            out: dict[str, str] = {}
    for layer, bucket in counts.items():
        best = max(bucket.items(), key=lambda kv: kv[1])
        out[layer] = best[0]
    return out

        
        def extract_size_label(*parts: Any) -> str | None:
    """Return normalized size like '8-Inch' from layer/name/props text."""
    text = " ".join(str(p) for p in parts if p)
    if not text:
        return None

            mm = _MM_SIZE_RE.search(text)
    if mm:
        try:
            inches = float(mm.group(1)) / 25.4
            # Civil pipe sizes are usually whole/half inches
            if inches >= 2:
                rounded = round(inches * 2) / 2
                if abs(rounded - int(rounded)) < 0.01:
                    return f"{int(rounded)}-Inch"
                return f"{rounded:g}-Inch"
        except ValueError:
            pass

            # Prefer patterns near pipe/water keywords
    low_all = text.lower()
    # Structure tags like SSMH-2 / CB-12 are indices, not diameters (unless "12\"")
    structure_index = bool(
        re.search(r"\b(?:ssmh|smh|mh|cb|inlet|str|jbox|jh|dmh)[-_ ]?\d+\b", low_all)
        and not re.search(r"\d+\s*(?:\"|''|in(?:ch)?|mm)\b", low_all)
    )
    for match in _SIZE_RE.finditer(text):
        raw = match.group(1) or match.group(2)
        if not raw:
            continue
        try:
            val = float(raw)
        except ValueError:
            continue
        # Skip stationing-like numbers and tiny/huge junk
        if val < 1 or val > 120:
            continue
        matched = match.group(0)
        span = text[max(0, match.start() - 12) : match.end() + 12].lower()
        common = {2, 3, 4, 6, 8, 10, 12, 14, 15, 16, 18, 20, 21, 24, 27, 30, 36, 42, 48, 54, 60}
        has_unit = bool(re.search(r"[\"'']|in(?:ch(?:es)?)?|mm", matched, re.I))
        has_cue = any(c in span for c in ('"', "in", "dia", "ø", "dn", "nps", "pipe", "casing", "wm"))
        civil_context = any(
            c in low_all
            for c in ("pipe", "water", "main", "sewer", "storm", "valve", "hydrant", "casing", "ductile", "pvc", "wm")
        )
        if structure_index and not has_unit and not has_cue:
            continue
        if has_unit or has_cue or (val in common and civil_context):
            if abs(val - int(val)) < 0.01:
                return f"{int(val)}-Inch"
            return f"{val:g}-Inch"
    return None

        
        def detect_network(*parts: Any) -> str | None:
    text = " ".join(str(p) for p in parts if p).lower()
    if "casing" in text:
        return "casing"
    if any(k in text for k in ("storm", "st-", "p_storm", "sd-", "inlet", "catch basin", "catchbasin", "cb-")):
        return "storm"
    if any(k in text for k in ("sanitary", "san-", "p_san", "e_san", "ss-", "ssmh", "smh")):
        return "sanitary"
    if "sewer" in text and "storm" not in text:
        return "sanitary"
    if any(k in text for k in ("water", "watermain", "wm-", "p_water", "e_water", "potable", "hydrant")):
        return "water"
    # Cautious short tokens
    if re.search(r"(^|[^a-z])w[-_]", text) or " water " in f" {text} ":
        return "water"
    return None

        
        def classify_fitting(name: str, layer: str = "") -> tuple[str, str, str] | None:
    """Return (description_prefix, category, unit) for countable fittings/structures."""
    text = f"{name} {layer}".lower()
    if _is_noise_name(name) or _is_noise_layer(layer):
        return None

            # Structures first (storm/sanitary)
    structure_map = [
        (["smh", "sanitary manhole", "sanhole", "ss mh", "ssmh"], "Sanitary Sewer Manhole", "Drainage"),
        (["manhole", " mh", "mh_", "mh-", "junc"], "Manhole", "Drainage"),
        (["inlet", "catch basin", "catchbasin", "cb-", "cb_", "sf_b", "curb inlet", "grate"], "Storm Drainage Inlet", "Drainage"),
        (["junction box", "jbox", "j-box", "drop structure"], "Storm Drainage Junction Box", "Drainage"),
        (["headwall", "endwall", "outlet structure"], "Drainage Headwall / Outlet", "Drainage"),
        (["structure"], "Drainage Structure", "Drainage"),
        (["hydrant", "fh-", "fh_"], "Fire Hydrant", "Utilities"),
        (["valve", "gv-", "bv-", "wv-", "gate valve", "butterfly"], "Valve", "Utilities"),
        (["bend", "elbow", "ell-", "90deg", "45deg", "22.5"], "Bend / Elbow", "Utilities"),
        (["tee", "wye", "branch"], "Tee / Wye Fitting", "Utilities"),
        (["reducer", "increaser", "coupling", "adapter"], "Reducer / Coupling", "Utilities"),
        (["cross fitting", "cross-"], "Cross Fitting", "Utilities"),
        (["cap", "plug", "blind"], "Cap / Plug", "Utilities"),
        (["sleeve", "casing"], "Casing / Sleeve", "Utilities"),
        (["meter"], "Water Meter", "Utilities"),
        (["blowoff", "blow-off", "air release", "arv"], "Blowoff / Air Release", "Utilities"),
        (["a-door", "doors", "door-"], "Doors", "Building"),
        (["window", "glaz"], "Windows", "Building"),
        (["column", "pillar"], "Columns", "Building"),
        (["pile"], "Piles", "Structures"),
        (["luminaire", "lightpole", "light pole", "streetlight", "street light"], "Light Poles", "Lighting & Electrical"),
        (["tree", "shrub"], "Trees / Shrubs", "Landscaping & Irrigation"),
        (["bollard"], "Bollards", "Miscellaneous"),
    ]
    for keys, desc, cat in structure_map:
        if any(k in text for k in keys):
            return desc, cat, "EA"

            if re.search(r"(?<![a-z])doors?(?![a-z])", text) and "outdoor" not in text:
        return "Doors", "Building", "EA"

            # Traffic signs / MUTCD inserts → counted, later rolled into Traffic Control SqFt
    sign_keys = (
        "sign",
        "mutcd",
        "stop",
        "yield",
        "speedlimit",
        "speed_limit",
        "w1-",
        "w2-",
        "w3-",
        "w4-",
        "w5-",
        "w20-",
        "r1-",
        "r2-",
        "r5-",
        "r6-",
        "r11-",
        "s1-",
        "g20-",
    )
    if any(k in text for k in sign_keys) and "signal" not in text and "marking" not in text:
        return "Traffic Sign", "Traffic Control", "EA"
    return None

        
        def _pipe_description(network: str | None, size: str | None, fallback_name: str) -> tuple[str, str]:
    size_bit = f"{size} " if size else ""
    if network == "water":
        return f"{size_bit}Water Main".strip(), "Utilities"
    if network == "sanitary":
        return f"{size_bit}Sanitary Sewer Pipe".strip(), "Drainage"
    if network == "storm":
        return f"{size_bit}Storm Drain Pipe".strip(), "Drainage"
    if network == "casing":
        return f"{size_bit}Casing Pipe".strip(), "Utilities"
    # Try size-only civil pipe
    if size:
        return f"{size} Utility Pipe", "Utilities"
    low = fallback_name.lower()
    if "casing" in low:
        return "Casing Pipe", "Utilities"
    return "Utility Pipe, Type and Size Unidentified", "Utilities"

        
        def build_quantities(extraction: dict[str, Any], source_label: str) -> list[dict[str, Any]]:
    length_by_key: dict[str, dict[str, Any]] = {}
    area_by_key: dict[str, dict[str, Any]] = {}
    count_by_key: dict[str, dict[str, Any]] = {}
    volume_by_key: dict[str, dict[str, Any]] = {}
    length_scale = _length_scale_to_feet(extraction.get("units"))
    size_by_layer = _size_hints_from_texts(extraction)

            def add_length(
        *,
        description: str,
        category: str,
        quantity: float,
        layer: str,
        entity_type: str,
        method: str,
        confidence: float = 88.0,
        size: str | None = None,
    ):
        if quantity <= 0:
            return
        if _is_noise_layer(layer) and "pipe" not in description.lower():
            return
        scaled = float(quantity) * length_scale
        unit = "LF"
        # Aggregate by description+size+unit (not layer) so LINE/POLYLINE/PIPE merge
        key = f"{description.lower()}|{unit}|{(size or '').lower()}"
        row = length_by_key.setdefault(
            key,
            {
                "description": description,
                "category": category,
                "unit": unit,
                "quantity": 0.0,
                "layer": layer,
                "entity_type": entity_type,
                "calculation_method": method,
                "source_reference": source_label,
                "confidence": confidence,
                "size": size,
            },
        )
        row["quantity"] += scaled
        row["confidence"] = max(float(row["confidence"]), confidence)
        if size and (not row.get("size")):
            row["size"] = size

            def add_count_item(
        *,
        description: str,
        category: str,
        layer: str,
        entity_type: str,
        method: str,
        confidence: float = 90.0,
        size: str | None = None,
        name: str = "",
    ):
        unit = "EA"
        key = f"{description.lower()}|{unit}|{(size or '').lower()}|{(name or layer).lower()}"
        row = count_by_key.setdefault(
            key,
            {
                "description": description if not size else f"{size} {description}",
                "category": category,
                "unit": unit,
                "quantity": 0.0,
                "layer": layer,
                "entity_type": entity_type,
                "calculation_method": method,
                "source_reference": source_label,
                "confidence": confidence,
                "size": size,
            },
        )
        # Avoid double-prefixing size on merge
        if size and size.lower() not in row["description"].lower():
            row["description"] = f"{size} {description}"
        row["quantity"] += 1
        row["confidence"] = max(float(row["confidence"]), confidence)

            def add_volume(*, description: str, quantity: float, method: str, layer: str = "Surface"):
        if quantity is None or float(quantity) <= 0:
            return
        key = description.lower()
        row = volume_by_key.setdefault(
            key,
            {
                "description": description,
                "category": "Earthwork",
                "unit": "CY",
                "quantity": 0.0,
                "layer": layer,
                "entity_type": "SURFACE",
                "calculation_method": method,
                "source_reference": source_label,
                "confidence": 86.0,
            },
        )
        row["quantity"] += float(quantity)

            def add_area(name: str, value: float, entity_type: str):
        if not value or value <= 0 or _is_noise_layer(name):
            return
        low = name.lower()
        if any(k in low for k in ("pave", "asphalt", "hma", "parking", "gsb", "wmm")):
            desc = f"Area - {name}"
            cat = "Pavement"
        elif any(k in low for k in ("sidewalk", "walkway", "footpath")):
            desc = f"Area - {name}"
            cat = "Curb, Gutter & Sidewalk"
        elif any(k in low for k in ("floor", "slab", "roof", "a-flor", "a-area")):
            desc = f"Area - {name}"
            cat = "Building"
        elif any(k in low for k in ("lining", "liner", "reservoir", "pond", "riprap")):
            desc = f"Area - {name}"
            cat = "Dams & Reservoirs"
        elif any(k in low for k in ("row", "easement")):
            desc = f"Area - {name}"
            cat = "Geometry"
        else:
            return  # skip random closed polys as EOQ area noise
        key = f"{desc}|SF|{name}"
        row = area_by_key.setdefault(
            key,
            {
                "description": desc,
                "category": cat,
                "unit": "SF",
                "quantity": 0.0,
                "layer": name,
                "entity_type": entity_type,
                "calculation_method": f"Sum of {entity_type} areas on '{name}'",
                "source_reference": source_label,
                "confidence": 78.0,
            },
        )
        row["quantity"] += value

            # --- Civil 3D / APS pipes (best source for size) ---
    for pipe in extraction.get("pipes") or []:
        name = str(pipe.get("name") or "Pipe")
        layer = str(pipe.get("layer") or name)
        length = float(pipe.get("length") or 0)
        diameter = pipe.get("diameter") or pipe.get("inner_diameter") or pipe.get("outer_diameter")
        size = extract_size_label(name, layer, diameter, pipe.get("part_size"), pipe.get("description"))
        if not size:
            size = size_by_layer.get(layer.lower())
        if not size and pipe.get("radius"):
            try:
                # radius often in drawing units (ft) → diameter inches if small
                r = float(pipe["radius"])
                if r < 5:  # feet
                    size = extract_size_label(f'{r * 2 * 12}"')
                elif r < 60:  # already inches radius unlikely; treat as inches diameter/2
                    size = extract_size_label(f'{r * 2}"')
            except (TypeError, ValueError):
                pass
        network = detect_network(name, layer, pipe.get("network"), pipe.get("description")) or detect_network(
            pipe.get("part_size") or ""
        )
        desc, cat = _pipe_description(network, size, name)
        conf = 94.0 if size and network else 88.0 if size or network else 80.0
        add_length(
            description=desc,
            category=cat,
            quantity=length,
            layer=layer,
            entity_type="PIPE",
            method=f"Sum of PIPE lengths ({name})" + (f", size {size}" if size else ""),
            confidence=conf,
            size=size,
        )

            # --- Alignments (often sanitary/storm centerlines) ---
    for align in extraction.get("alignments") or []:
        name = str(align.get("name") or "Alignment")
        layer = str(align.get("layer") or name)
        length = float(align.get("length") or 0)
        size = extract_size_label(name, layer)
        network = detect_network(name, layer)
        if network in {"sanitary", "storm", "water"} or size:
            desc, cat = _pipe_description(network, size, name)
            add_length(
                description=desc,
                category=cat,
                quantity=length,
                layer=layer,
                entity_type="ALIGNMENT",
                method=f"Sum of ALIGNMENT lengths '{name}'",
                confidence=90.0 if network else 82.0,
                size=size,
            )

            # --- Polylines / lines: size-aware by layer/name ---
    for line in extraction.get("lines") or []:
        if _is_paper_space(line):
            continue
        layer = str(line.get("layer") or "0")
        length = float(line.get("length") or 0)
        size = extract_size_label(layer, line.get("name")) or size_by_layer.get(layer.lower())
        network = detect_network(layer, line.get("name"))
        # Require network or size or explicit pipe-ish layer — avoid random linework
        if network or size or any(k in layer.lower() for k in ("pipe", "water", "sewer", "storm", "san", "casing")):
            desc, cat = _pipe_description(network, size, layer)
            add_length(
                description=desc,
                category=cat,
                quantity=length,
                layer=layer,
                entity_type="LINE",
                method=f"Sum of LINE lengths on layer '{layer}'",
                confidence=86.0 if size and network else 80.0 if size or network else 74.0,
                size=size,
            )

            for pl in extraction.get("polylines") or []:
        if _is_paper_space(pl):
            continue
        layer = str(pl.get("layer") or "0")
        length = float(pl.get("length") or 0)
        size = extract_size_label(layer, pl.get("name")) or size_by_layer.get(layer.lower())
        network = detect_network(layer, pl.get("name"))
        if network or size or any(
            k in layer.lower() for k in ("pipe", "water", "sewer", "storm", "san", "casing", "main")
        ):
            desc, cat = _pipe_description(network, size, layer)
            add_length(
                description=desc,
                category=cat,
                quantity=length,
                layer=layer,
                entity_type="POLYLINE",
                method=f"Sum of POLYLINE lengths on layer '{layer}'",
                confidence=87.0 if size and network else 81.0 if size or network else 75.0,
                size=size,
            )
        if pl.get("area"):
            add_area(layer, float(pl.get("area") or 0) * (length_scale**2), "POLYLINE")

            for hatch in extraction.get("hatches") or []:
        add_area(str(hatch.get("layer") or "0"), float(hatch.get("area") or 0), "HATCH")

            # --- Blocks / inserts: fittings + structures (do not lump) ---
    for block in extraction.get("blocks") or []:
        if _is_paper_space(block):
            continue
        name = str(block.get("name") or "BLOCK")
        layer = str(block.get("layer") or "")
        btype = str(block.get("type") or "")
        if _is_noise_name(name):
            continue
        size = extract_size_label(name, layer, block.get("size"), block.get("description"), btype)
        if not size:
            size = size_by_layer.get(layer.lower())
        network = detect_network(name, layer, btype)
        classified = classify_fitting(name, layer) or classify_fitting(name, btype)
        if classified:
            desc, cat, _unit = classified
            # Specialize manhole/inlet by network
            if desc == "Manhole" and network == "sanitary":
                desc = "Sanitary Sewer Manhole"
            elif desc == "Manhole" and network == "storm":
                desc = "Storm Sewer Manhole"
            elif desc == "Drainage Structure" and network == "storm":
                desc = "Storm Drainage Structure"
            elif desc in {"Valve", "Bend / Elbow", "Tee / Wye Fitting", "Reducer / Coupling"} and network == "water":
                desc = f"Water {desc}"
            elif desc in {"Valve", "Bend / Elbow"} and network == "sanitary":
                desc = f"Sanitary {desc}"
            add_count_item(
                description=desc,
                category=cat,
                layer=layer or name,
                entity_type="INSERT",
                method=f"Count of INSERT '{name}'",
                confidence=92.0 if size else 89.0,
                size=size,
                name=name,
            )
            continue

                # Layer-implied utility insert without clear fitting keyword
        if network in {"water", "storm", "sanitary"}:
            if network == "water":
                desc, cat = "Water Appurtenance (Unclassified)", "Utilities"
            elif network == "storm":
                desc, cat = "Storm Drainage Structure", "Drainage"
            else:
                desc, cat = "Sanitary Sewer Structure", "Drainage"
            add_count_item(
                description=desc,
                category=cat,
                layer=layer or name,
                entity_type="INSERT",
                method=f"Count of INSERT '{name}' on {network} layer",
                confidence=78.0,
                size=size,
                name=name,
            )

            # --- Surfaces / cut-fill ---
    for surf in extraction.get("surfaces") or []:
        name = str(surf.get("name") or "Surface")
        cut = surf.get("cut") or surf.get("cut_volume") or surf.get("Cut Volume")
        fill = surf.get("fill") or surf.get("fill_volume") or surf.get("Fill Volume")
        net = surf.get("net") or surf.get("net_volume")
        # Convert CF→CY if values look like cubic feet (very large)
        def as_cy(val: Any) -> float | None:
            try:
                v = float(val)
            except (TypeError, ValueError):
                return None
            if v <= 0:
                return None
            # Heuristic: if huge, treat as CF
            if v > 50000:
                v = v / 27.0
            return v

                cut_cy = as_cy(cut)
        fill_cy = as_cy(fill)
        if cut_cy:
            add_volume(
                description="Earthwork Cut",
                quantity=cut_cy,
                method=f"Surface '{name}' cut volume",
                layer=name,
            )
        if fill_cy:
            add_volume(
                description="Earthwork Fill",
                quantity=fill_cy,
                method=f"Surface '{name}' fill volume",
                layer=name,
            )
        if not cut_cy and not fill_cy:
            net_cy = as_cy(net)
            if net_cy:
                # Net alone — put under cut if positive convention unknown; split note
                add_volume(
                    description="Earthwork Net Volume (verify cut/fill)",
                    quantity=abs(net_cy),
                    method=f"Surface '{name}' net volume (sign/convention needs engineer check)",
                    layer=name,
                )

            for vol in extraction.get("volumes") or []:
        kind = str(vol.get("type") or vol.get("name") or "").lower()
        qty = vol.get("quantity") or vol.get("volume")
        try:
            q = float(qty)
        except (TypeError, ValueError):
            continue
        if q > 50000:
            q = q / 27.0
        if "cut" in kind:
            add_volume(description="Earthwork Cut", quantity=q, method=vol.get("method") or "Volume property")
        elif "fill" in kind:
            add_volume(description="Earthwork Fill", quantity=q, method=vol.get("method") or "Volume property")
        elif "net" in kind:
            add_volume(
                description="Earthwork Net Volume (verify cut/fill)",
                quantity=abs(q),
                method=vol.get("method") or "Volume property (net)",
            )

            # Merge pipe length rows that are identical after size detection from text near layers
    items: list[dict[str, Any]] = []
    for row in (
        list(length_by_key.values())
        + list(area_by_key.values())
        + list(count_by_key.values())
        + list(volume_by_key.values())
    ):
        row["quantity"] = round(float(row["quantity"]), 2)
        row["unit"] = str(row.get("unit") or "UNIT").upper()
        # Drop tiny junk lengths
        if row["unit"] in {"LF", "SF"} and row["quantity"] < 0.05:
            continue
        items.append(enrich_quantity_item(row))

            items = _dedupe_prefer_sized(items)
    items = _collapse_count_duplicates(items)

            from app.services.civil_estimator import expand_cad_takeoff, extraction_has_bid_schedule

            if not extraction_has_bid_schedule(extraction):
        items = expand_cad_takeoff(extraction, items)
    items = [enrich_quantity_item(row) for row in items]

            from app.services.traffic_control import consolidate_traffic_control_signs

            items, _tc_meta = consolidate_traffic_control_signs(items, allow_online_refresh=False)
    return items[:500]

        
        def _dedupe_prefer_sized(items: list[dict[str, Any]]) -> list[dict[str, Any]]:
    """If we have sized water/sanitary/storm pipes, drop generic unidentified duplicates."""
    has_sized = {
        "water": False,
        "sanitary": False,
        "storm": False,
    }
    for i in items:
        d = (i.get("description") or "").lower()
        if "-inch" in d and "unidentified" not in d:
            if "water" in d:
                has_sized["water"] = True
            if "sanitary" in d:
                has_sized["sanitary"] = True
            if "storm" in d:
                has_sized["storm"] = True

            out: list[dict[str, Any]] = []
    for i in items:
        d = (i.get("description") or "").lower()
        if "unidentified" in d or d == "utility pipe, type and size unidentified":
            if has_sized["water"] and ("water" in d or "utility pipe" in d):
                # Drop bare unidentified when sized water exists
                if "water" not in d or "-inch" not in d:
                    if has_sized["water"] and "utility pipe" in d:
                        continue
            if has_sized["sanitary"] and "sanitary" in d and "-inch" not in d:
                continue
            if has_sized["storm"] and "storm" in d and "-inch" not in d:
                continue
            if all(has_sized.values()) and "unidentified" in d:
                continue
        out.append(i)
    return out

        
        def _collapse_count_duplicates(items: list[dict[str, Any]]) -> list[dict[str, Any]]:
    """Merge EA rows that differ only by block name but share description+size+unit."""
    merged: dict[str, dict[str, Any]] = {}
    passthrough: list[dict[str, Any]] = []
    for item in items:
        unit = str(item.get("unit") or "").upper()
        if unit != "EA":
            passthrough.append(item)
            continue
        key = (
            f"{(item.get('description') or '').strip().lower()}|"
            f"{unit}|{(item.get('size') or '')}".lower()
        )
        if key not in merged:
            merged[key] = dict(item)
            continue
        try:
            merged[key]["quantity"] = round(
                float(merged[key].get("quantity") or 0) + float(item.get("quantity") or 0),
                2,
            )
            merged[key]["confidence"] = max(
                float(merged[key].get("confidence") or 0),
                float(item.get("confidence") or 0),
            )
        except (TypeError, ValueError):
            passthrough.append(item)
    return passthrough + list(merged.values())

### backend/app/services/cad/utility_stationing.py

SHA-256: 2d73d1b2453466534a68acc45b61632ae37528f42bfbeb2f42ffe751f79c9f7b    ·    1726 line(s)    ·    .py

"""Centerline-based utility stationing, offsets, fittings, and bid rollups.

        Stationing is always relative to the roadway / project CENTERLINE (CL).
LT/RT is computed from alignment travel direction (2D cross product), not
drawing screen orientation.

        Outputs feed Excel:
  - Bid Quantity Summary (rolled up FROM detail)
  - Linear Quantity Breakdown (sta/offset/side/length)
  - Fittings / Bends / Connections
  - Quantity QA/QC flags
"""

        from __future__ import annotations

        import math
import re
from collections import defaultdict
from typing import Any

        from app.services.cad.civil_location import (
    coerce_station,
    collect_location_labels,
    collect_pipe_ends,
    format_offset as _civil_format_offset,
    location_from_block_attributes,
    location_from_property_bag,
    match_label_to_fitting,
    match_pipe_end,
    parse_station_offset_text,
    point_from_property_bag,
    side_code as _civil_side_code,
    station_to_feet as _civil_station_to_feet,
    typical_network_offset,
)
from app.services.cad.quantity_engine import (
    classify_fitting,
    detect_network,
    extract_size_label,
)

        _STA_PAIR = re.compile(
    r"(?:sta(?:tion)?\.?\s*)?(?P<sta1>\d{1,4}\+\d{2}(?:\.\d+)?)"
    r"\s*(?:to|[-–—]|thru|through)\s*"
    r"(?:sta(?:tion)?\.?\s*)?(?P<sta2>\d{1,4}\+\d{2}(?:\.\d+)?)",
    re.I,
)
_STA_ONE = re.compile(r"(?:sta(?:tion)?\.?\s*)?(?P<sta>\d{1,4}\+\d{2}(?:\.\d+)?)", re.I)

        _UTILITY_KIND = re.compile(
    r"(?P<kind>"
    r"water\s*mains?|watermains?|\bwm\b|w\.?\s*m\.?|"
    r"sanitary\s*(?:sewer)?(?:\s*main)?|\bss\b|"
    r"storm\s*(?:drain|sewer)?(?:\s*main)?|"
    r"force\s*main|forcemain|"
    r"casing(?:\s*pipe)?|carrier(?:\s*pipe)?"
    r")",
    re.I,
)

        # Prefer true centerline names first
_CL_NAME_HINTS = (
    "centerline",
    "centreline",
    "center line",
    "centre line",
    "cl alignment",
    " roadway cl",
    "road cl",
    "baseline",
    "p_cl",
    "_cl",
    "cl_",
)
_ALIGN_NAME_HINTS = _CL_NAME_HINTS + (
    "alignment",
    "cl-",
    " cl",
    "roadway",
    "profile",
)

        
        def _is_centerline_name(*parts: Any) -> bool:
    """True for P_CL / CL_ / CENTERLINE style names (underscore-safe)."""
    low = " ".join(str(p) for p in parts if p).lower()
    if not low.strip():
        return False
    if any(h in low for h in _CL_NAME_HINTS):
        return True
    if re.search(r"center\s*line|centre\s*line|baseline", low):
        return True
    # Token CL including P_CL_50th, X_CL, CL-1 (underscore is a word char so \bcl\b fails)
    if re.search(r"(?:^|[^a-z0-9])cl(?:[^a-z0-9]|$)", low.replace("_", " ")):
        return True
    return False

        # Offset change (ft) above which a run is treated as non-parallel to CL
_NONPARALLEL_OFFSET_DELTA_FT = 3.0
_MIN_BEND_DEG = 12.0

        
        def station_to_feet(sta: str | None) -> float | None:
    """Parse ``12+34.56`` or Civil raw station feet."""
    return _civil_station_to_feet(sta)

        
        def feet_to_station(feet: float | None) -> str:
    if feet is None or not math.isfinite(feet):
        return ""
    feet = max(0.0, float(feet))
    plus = int(feet // 100)
    rem = feet - plus * 100
    if abs(rem - round(rem)) < 0.05:
        return f"{plus}+{int(round(rem)):02d}"
    return f"{plus}+{rem:05.2f}"

        
        def format_offset(offset_ft: float | None, side: str | None) -> str:
    """Civil style: 18.5' RT"""
    return _civil_format_offset(offset_ft, side)

        
        def _side_code(side: str | None) -> str:
    return _civil_side_code(side)

        
        def _network_label(network: str | None) -> str:
    return {
        "water": "Water Main",
        "sanitary": "Sanitary Sewer",
        "storm": "Storm Sewer",
        "casing": "Casing",
        "force": "Force Main",
    }.get((network or "").lower(), "Underground Utility")

        
        def _kind_to_network(kind: str) -> str | None:
    k = kind.lower()
    if "water" in k or k in {"wm", "w.m", "w m"}:
        return "water"
    if "sanitary" in k or k == "ss":
        return "sanitary"
    if "storm" in k:
        return "storm"
    if "force" in k:
        return "force"
    if "casing" in k or "carrier" in k:
        return "casing"
    return detect_network(kind)

        
        def _dist(a: tuple[float, float], b: tuple[float, float]) -> float:
    return math.hypot(b[0] - a[0], b[1] - a[1])

        
        def _polyline_length(pts: list[tuple[float, float]]) -> float:
    return sum(_dist(pts[i - 1], pts[i]) for i in range(1, len(pts)))

        
        def _as_point(raw: Any) -> tuple[float, float] | None:
    if raw is None:
        return None
    if isinstance(raw, (list, tuple)) and len(raw) >= 2:
        try:
            return float(raw[0]), float(raw[1])
        except (TypeError, ValueError):
            return None
    if isinstance(raw, dict):
        try:
            return float(raw.get("x")), float(raw.get("y"))
        except (TypeError, ValueError):
            return None
    return None

        
        def _as_points(raw: Any) -> list[tuple[float, float]]:
    if not raw or not isinstance(raw, list):
        return []
    out: list[tuple[float, float]] = []
    for p in raw:
        pt = _as_point(p)
        if pt:
            out.append(pt)
    return out

        
        def _project_point_on_polyline(
    point: tuple[float, float],
    chain: list[tuple[float, float]],
) -> tuple[float, float, str] | None:
    """Return (station_along_ft, offset_ft, side LT/RT/CL) vs CL travel direction."""
    if len(chain) < 2:
        return None
    best_d = float("inf")
    best_sta = 0.0
    best_side = "CL"
    best_off = 0.0
    cum = 0.0
    for i in range(1, len(chain)):
        x1, y1 = chain[i - 1]
        x2, y2 = chain[i]
        dx, dy = x2 - x1, y2 - y1
        seg_len = math.hypot(dx, dy)
        if seg_len < 1e-9:
            continue
        t = ((point[0] - x1) * dx + (point[1] - y1) * dy) / (seg_len * seg_len)
        t_clamped = max(0.0, min(1.0, t))
        proj = (x1 + t_clamped * dx, y1 + t_clamped * dy)
        d = _dist(point, proj)
        # Positive cross = left of CL travel direction → LT
        cross = dx * (point[1] - y1) - dy * (point[0] - x1)
        if abs(cross) < 1e-6 * seg_len or d < 0.05:
            side = "CL"
        else:
            side = "LT" if cross > 0 else "RT"
        if d < best_d:
            best_d = d
            best_sta = cum + t_clamped * seg_len
            best_side = side
            best_off = d
        cum += seg_len
    if best_d == float("inf"):
        return None
    return best_sta, best_off, best_side

        
        def _build_centerline(extraction: dict[str, Any]) -> dict[str, Any]:
    """Always prefer roadway CENTERLINE for stationing — require geometry when possible."""
    alignments = list(extraction.get("alignments") or [])
    polylines = list(extraction.get("polylines") or [])
    lines = list(extraction.get("lines") or [])
    candidates: list[dict[str, Any]] = []

            def add(name: str, layer: Any, pts: list, length: float, sta0: float, source: str, bonus: float) -> None:
        has_geom = len(pts) >= 2
        # Geometry is mandatory for station/offset — heavily prefer it
        geom_bonus = 100_000 if has_geom else 0
        cl_bonus = bonus
        candidates.append(
            {
                "name": name,
                "layer": layer,
                "points": pts,
                "length": length if length > 0 else (_polyline_length(pts) if has_geom else 0),
                "sta_start": sta0,
                "score": (length or 0) + cl_bonus + geom_bonus,
                "source": source,
                "is_centerline": cl_bonus >= 20_000 or _is_centerline_name(name, layer),
                "has_geometry": has_geom,
            }
        )

            for a in alignments:
        pts = _as_points(a.get("points") or a.get("vertices") or a.get("coords"))
        length = float(a.get("length") or (_polyline_length(pts) if pts else 0) or 0)
        name = str(a.get("name") or a.get("layer") or "Alignment")
        layer = a.get("layer") or name
        sta0 = station_to_feet(str(a.get("sta_start") or a.get("staStart") or "") or None) or 0.0
        bonus = 0.0
        if _is_centerline_name(name, layer):
            bonus = 50_000
        elif any(h in f"{name} {layer}".lower() for h in _ALIGN_NAME_HINTS):
            bonus = 15_000
        add(name, layer, pts, length, sta0, "alignment", bonus)

            for pl in polylines:
        layer = str(pl.get("layer") or "")
        name = str(pl.get("name") or layer)
        if not _is_centerline_name(name, layer) and not any(
            h in f"{layer} {name}".lower() for h in ("alignment", "roadway")
        ):
            continue
        pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
        length = float(pl.get("length") or (_polyline_length(pts) if pts else 0) or 0)
        bonus = 50_000 if _is_centerline_name(name, layer) else 12_000
        add(name or layer, layer, pts, length, 0.0, "polyline_cl", bonus)

            # Also consider ANY polyline on a CL-like layer even if earlier filter missed
    for pl in polylines:
        layer = str(pl.get("layer") or "")
        if not _is_centerline_name(layer):
            continue
        pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
        if len(pts) < 2:
            continue
        length = float(pl.get("length") or _polyline_length(pts) or 0)
        add(layer, layer, pts, length, 0.0, "polyline_cl_layer", 55_000)

            if not candidates:
        for pl in polylines:
            layer = str(pl.get("layer") or "")
            if detect_network(layer):
                continue
            pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
            if len(pts) < 2:
                continue
            length = float(pl.get("length") or _polyline_length(pts) or 0)
            if length < 50:
                continue
            add(layer or "Reference CL", layer, pts, length, 0.0, "longest_polyline", 0)

            if not any(c.get("has_geometry") for c in candidates) and lines:
        for line in lines:
            layer = str(line.get("layer") or "")
            if not _is_centerline_name(layer, line.get("name")):
                continue
            s = _as_point(line.get("start"))
            e = _as_point(line.get("end"))
            pts = [p for p in (s, e) if p]
            if len(pts) == 2:
                add(
                    layer or "CL line",
                    layer,
                    pts,
                    float(line.get("length") or _dist(pts[0], pts[1])),
                    0.0,
                    "line_cl",
                    50_000,
                )

            if not candidates and lines:
        best = max(lines, key=lambda L: float(L.get("length") or 0), default=None)
        if best and best.get("start") and best.get("end"):
            pts = [p for p in (_as_point(best["start"]), _as_point(best["end"])) if p]
            if len(pts) == 2:
                add(
                    str(best.get("layer") or "Line CL"),
                    best.get("layer"),
                    pts,
                    float(best.get("length") or _dist(pts[0], pts[1])),
                    0.0,
                    "line",
                    0,
                )

            if not candidates:
        return {
            "name": "CENTERLINE (assumed 0+00 — no CL geometry found)",
            "layer": None,
            "points": [],
            "length": 0.0,
            "sta_start": 0.0,
            "source": "none",
            "is_centerline": False,
            "has_geometry": False,
        }

            # Prefer geometry-bearing CL; among those, highest score
    with_geom = [c for c in candidates if c.get("has_geometry")]
    pool = with_geom or candidates
    best = max(pool, key=lambda c: float(c.get("score") or 0))
    best = _hydrate_centerline_points(best, polylines=polylines, lines=lines)

            raw_name = str(best.get("name") or "CL")
    if best.get("is_centerline") and not raw_name.upper().startswith("CL"):
        best["name"] = raw_name  # keep CAD name like P_CL 50th Street
    elif not best.get("is_centerline"):
        best["name"] = f"CL (proxy) — {raw_name}"
    return best

        
        def _hydrate_centerline_points(
    cl: dict[str, Any],
    *,
    polylines: list[dict[str, Any]],
    lines: list[dict[str, Any]],
) -> dict[str, Any]:
    """If CL was chosen by name/length only, attach matching polyline/line vertices."""
    if len(cl.get("points") or []) >= 2:
        cl["has_geometry"] = True
        return cl

            name = str(cl.get("name") or "").lower()
    layer = str(cl.get("layer") or "").lower()
    for prefix in ("cl — ", "cl (proxy) — "):
        if name.startswith(prefix):
            name = name[len(prefix) :]

            best_pts: list[tuple[float, float]] = []
    best_len = 0.0
    for pl in polylines:
        pl_layer = str(pl.get("layer") or "").lower()
        pl_name = str(pl.get("name") or "").lower()
        pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
        if len(pts) < 2:
            continue
        matched = (
            (layer and (layer == pl_layer or layer in pl_layer or pl_layer in layer))
            or (name and (name == pl_layer or name in pl_layer or pl_layer in name or name in pl_name))
            or _is_centerline_name(pl_layer, pl_name)
        )
        if not matched:
            continue
        length = _polyline_length(pts)
        if length > best_len:
            best_len = length
            best_pts = pts

            if not best_pts:
        for line in lines:
            pl_layer = str(line.get("layer") or "").lower()
            if not (
                (layer and layer in pl_layer)
                or (name and name in pl_layer)
                or _is_centerline_name(pl_layer)
            ):
                continue
            s = _as_point(line.get("start"))
            e = _as_point(line.get("end"))
            pts = [p for p in (s, e) if p]
            if len(pts) == 2:
                length = _dist(pts[0], pts[1])
                if length > best_len:
                    best_len = length
                    best_pts = pts

            if best_pts:
        cl = dict(cl)
        cl["points"] = [[p[0], p[1]] for p in best_pts]
        cl["length"] = best_len or cl.get("length") or 0
        cl["has_geometry"] = True
        cl["source"] = f"{cl.get('source') or 'cl'}+hydrated_vertices"
        return cl

            # Last resort: station axis only (offsets stay blank without world XY on pipes)
    length = float(cl.get("length") or 0)
    if length > 1:
        cl = dict(cl)
        cl["points"] = [[0.0, 0.0], [length, 0.0]]
        cl["has_geometry"] = True
        cl["synthetic_geometry"] = True
        cl["source"] = f"{cl.get('source') or 'cl'}+synthetic_station_axis"
    return cl

        
        def _size_display(size: str | None) -> str:
    if not size:
        return ""
    s = str(size).strip()
    if re.search(r"inch", s, re.I):
        m = re.search(r"(\d+(?:\.\d+)?)", s)
        return f'{m.group(1)}"' if m else s
    if '"' in s or "''" in s:
        return s
    m = re.search(r"(\d+(?:\.\d+)?)", s)
    return f'{m.group(1)}"' if m else s

        
        def _locate(
    point: tuple[float, float] | None,
    cl: dict[str, Any],
) -> dict[str, Any]:
    chain = list(cl.get("points") or [])
    sta0 = float(cl.get("sta_start") or 0.0)
    empty = {
        "station": "",
        "station_ft": None,
        "offset_ft": None,
        "side": "",
        "offset_label": "",
        "alignment": cl.get("name") or "CL",
        "associated": False,
    }
    # Synthetic station axis is not in drawing coordinates — do not project world XY onto it
    if cl.get("synthetic_geometry"):
        return empty
    if not point or len(chain) < 2:
        return empty
    proj = _project_point_on_polyline(point, chain)
    if not proj:
        return empty
    sta_ft = sta0 + proj[0]
    side = proj[2]
    off = proj[1]
    return {
        "station": feet_to_station(sta_ft),
        "station_ft": sta_ft,
        "offset_ft": round(off, 2),
        "side": side,
        "offset_label": format_offset(off, side),
        "alignment": cl.get("name") or "CL",
        "associated": True,
    }

        
        def _segment_row(
    *,
    network: str | None,
    size: str | None,
    length: float,
    layer: str,
    pts: list[tuple[float, float]],
    cl: dict[str, Any],
    source: str,
    method: str,
    name: str = "",
) -> dict[str, Any] | None:
    if length <= 0.05 and len(pts) < 2:
        return None
    if length <= 0.05 and pts:
        length = _polyline_length(pts)
    if length <= 0.05:
        return None

            from_loc = _locate(pts[0] if pts else None, cl)
    to_loc = _locate(pts[-1] if len(pts) > 1 else (pts[0] if pts else None), cl)

            from_sta = from_loc["station"]
    to_sta = to_loc["station"]
    from_off = from_loc["offset_ft"]
    to_off = to_loc["offset_ft"]
    from_side = from_loc["side"]
    to_side = to_loc["side"]

            # Dominant side for parallel runs
    side = from_side or to_side
    offset_ft = from_off
    if from_off is not None and to_off is not None:
        offset_ft = (from_off + to_off) / 2.0

            nonparallel = False
    if from_off is not None and to_off is not None:
        if abs(from_off - to_off) >= _NONPARALLEL_OFFSET_DELTA_FT or (
            from_side and to_side and from_side != to_side and from_side != "CL" and to_side != "CL"
        ):
            nonparallel = True

            util = _network_label(network)
    size_d = _size_display(size)
    flags: list[str] = []
    if not from_loc["associated"] or not to_loc["associated"]:
        flags.append("line_not_associated_with_alignment")
    if not size_d:
        flags.append("unknown_size")

            return {
        "item": util,
        "utility": util,
        "network": network or "utility",
        "size": size_d,
        "description": f"{size_d + ' ' if size_d else ''}{util}".strip(),
        "from_station": from_sta,
        "to_station": to_sta,
        "side": side,
        "offset_ft": round(offset_ft, 2) if offset_ft is not None else None,
        "offset": format_offset(offset_ft, side) if offset_ft is not None else "",
        "from_offset_ft": round(from_off, 2) if from_off is not None else None,
        "to_offset_ft": round(to_off, 2) if to_off is not None else None,
        "from_offset": format_offset(from_off, from_side) if from_off is not None else "",
        "to_offset": format_offset(to_off, to_side) if to_off is not None else "",
        "nonparallel": nonparallel,
        "length": round(length, 2),
        "quantity_lf": round(length, 2),
        "unit": "LF",
        "layer": layer,
        "alignment": cl.get("name") or "CL",
        "source": source,
        "method": method,
        "name": name,
        "flags": flags,
        # legacy keys for older consumers
        "side_of_alignment": side,
        "direction": "Increasing station"
        if (from_loc.get("station_ft") or 0) <= (to_loc.get("station_ft") or 0)
        else "Decreasing station",
    }

        
        def _split_polyline_runs(
    pts: list[tuple[float, float]],
    *,
    network: str | None,
    size: str | None,
    layer: str,
    cl: dict[str, Any],
) -> list[dict[str, Any]]:
    """Split a pipe polyline into straight station runs between bend vertices."""
    if len(pts) < 2:
        return []
    # Find bend indices
    bend_idx = {0, len(pts) - 1}
    for i in range(1, len(pts) - 1):
        a, b, c = pts[i - 1], pts[i], pts[i + 1]
        v1 = (b[0] - a[0], b[1] - a[1])
        v2 = (c[0] - b[0], c[1] - b[1])
        n1 = math.hypot(*v1)
        n2 = math.hypot(*v2)
        if n1 < 1e-6 or n2 < 1e-6:
            continue
        cosang = max(-1.0, min(1.0, (v1[0] * v2[0] + v1[1] * v2[1]) / (n1 * n2)))
        deflection = abs(math.degrees(math.acos(cosang)))
        if deflection >= _MIN_BEND_DEG:
            bend_idx.add(i)
    ordered = sorted(bend_idx)
    rows: list[dict[str, Any]] = []
    for i in range(len(ordered) - 1):
        i0, i1 = ordered[i], ordered[i + 1]
        sub = pts[i0 : i1 + 1]
        length = _polyline_length(sub)
        row = _segment_row(
            network=network,
            size=size,
            length=length,
            layer=layer,
            pts=sub,
            cl=cl,
            source="CAD POLYLINE segment",
            method=f"Centerline station/offset; segment vertices {i0}→{i1}",
        )
        if row:
            rows.append(row)
    if not rows:
        row = _segment_row(
            network=network,
            size=size,
            length=_polyline_length(pts),
            layer=layer,
            pts=pts,
            cl=cl,
            source="CAD POLYLINE",
            method="Centerline station/offset on full polyline",
        )
        if row:
            rows.append(row)
    return rows

        
        def _segments_from_texts(extraction: dict[str, Any], cl: dict[str, Any]) -> list[dict[str, Any]]:
    rows: list[dict[str, Any]] = []
    for t in extraction.get("texts") or []:
        text = str(t.get("text") or t.get("contents") or "").strip()
        if not text:
            continue
        kind_m = _UTILITY_KIND.search(text)
        sta_m = _STA_PAIR.search(text)
        if not kind_m or not sta_m:
            continue
        network = _kind_to_network(kind_m.group("kind"))
        size = extract_size_label(text)
        a = station_to_feet(sta_m.group("sta1"))
        b = station_to_feet(sta_m.group("sta2"))
        if a is None or b is None:
            continue
        length = abs(b - a)
        util = _network_label(network)
        size_d = _size_display(size)
        rows.append(
            {
                "item": util,
                "utility": util,
                "network": network or "utility",
                "size": size_d,
                "description": f"{size_d + ' ' if size_d else ''}{util}".strip(),
                "from_station": feet_to_station(a),
                "to_station": feet_to_station(b),
                "side": "",
                "offset": "",
                "offset_ft": None,
                "from_offset": "",
                "to_offset": "",
                "from_offset_ft": None,
                "to_offset_ft": None,
                "nonparallel": False,
                "length": round(length, 2),
                "quantity_lf": round(length, 2),
                "unit": "LF",
                "layer": t.get("layer") or "",
                "alignment": cl.get("name") or "CL",
                "source": "plan/DWG text callout",
                "method": f"Station range from text on CL: '{text[:100]}'",
                "flags": ["station_from_text_no_offset"],
                "side_of_alignment": "",
                "direction": "Increasing station" if b >= a else "Decreasing station",
            }
        )
    return rows

        
        def _segments_from_geometry(extraction: dict[str, Any], cl: dict[str, Any]) -> list[dict[str, Any]]:
    rows: list[dict[str, Any]] = []

            for pipe in extraction.get("pipes") or []:
        layer = str(pipe.get("layer") or "")
        name = str(pipe.get("name") or "")
        network = detect_network(name, layer, pipe.get("network"), pipe.get("description"))
        if not network and not any(
            k in f"{layer} {name}".lower() for k in ("pipe", "water", "sewer", "storm", "san", "main", "casing")
        ):
            continue
        size = extract_size_label(name, layer, pipe.get("diameter"), pipe.get("part_size"), pipe.get("description"))
        length = float(pipe.get("length") or 0)
        pts = _as_points(pipe.get("points") or pipe.get("coords"))
        # Civil 3D / APS often expose start/end without vertices
        s_pt = _as_point(pipe.get("start") or pipe.get("start_point"))
        e_pt = _as_point(pipe.get("end") or pipe.get("end_point"))
        if len(pts) < 2 and s_pt and e_pt:
            pts = [s_pt, e_pt]
            if length <= 0:
                length = _dist(s_pt, e_pt)

                sta_s = pipe.get("sta_start") or pipe.get("start_station") or pipe.get("refStart")
        sta_e = pipe.get("sta_end") or pipe.get("end_station") or pipe.get("refEnd")
        if sta_s and sta_e and length <= 0:
            a = station_to_feet(sta_s)
            b = station_to_feet(sta_e)
            if a is not None and b is not None:
                length = abs(b - a)

                side = pipe.get("side") or pipe.get("offset_side")
        offset = pipe.get("offset") or pipe.get("offset_ft")

                extra = {
            "start_structure": pipe.get("start_structure") or pipe.get("startStructure"),
            "end_structure": pipe.get("end_structure") or pipe.get("endStructure"),
            "sta_start_raw": sta_s,
            "sta_end_raw": sta_e,
        }

                if pts and len(pts) >= 2 and not cl.get("synthetic_geometry"):
            added = _split_polyline_runs(pts, network=network, size=size, layer=layer or name, cl=cl)
            for r in added:
                r.update({k: v for k, v in extra.items() if v})
                _stamp_civil_pipe_location(r, sta_s, sta_e, length, side, offset)
            rows.extend(added)
            continue

                row = _segment_row(
            network=network,
            size=size,
            length=length,
            layer=layer or name,
            pts=pts if not cl.get("synthetic_geometry") else [],
            cl=cl,
            source="CAD PIPE",
            method="Pipe associated to centerline",
            name=name,
        )
        if row:
            row.update({k: v for k, v in extra.items() if v})
            _stamp_civil_pipe_location(row, sta_s, sta_e, length, side, offset)
            rows.append(row)

            for pl in extraction.get("polylines") or []:
        layer = str(pl.get("layer") or "")
        network = detect_network(layer, pl.get("name"))
        if not network and not any(
            k in layer.lower() for k in ("pipe", "water", "sewer", "storm", "san", "casing", "main")
        ):
            continue
        if any(h in layer.lower() for h in ("centerline", "centreline", "alignment")) and not network:
            continue
        size = extract_size_label(layer, pl.get("name"))
        pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
        if pts:
            rows.extend(_split_polyline_runs(pts, network=network, size=size, layer=layer, cl=cl))
        else:
            length = float(pl.get("length") or 0)
            row = _segment_row(
                network=network,
                size=size,
                length=length,
                layer=layer,
                pts=[],
                cl=cl,
                source="CAD POLYLINE",
                method="Polyline length (no vertices for offset)",
            )
            if row:
                row["flags"] = list(row.get("flags") or []) + ["line_not_associated_with_alignment"]
                rows.append(row)

            for line in extraction.get("lines") or []:
        layer = str(line.get("layer") or "")
        network = detect_network(layer, line.get("name"))
        if not network and not any(
            k in layer.lower() for k in ("pipe", "water", "sewer", "storm", "san", "casing", "main")
        ):
            continue
        size = extract_size_label(layer, line.get("name"))
        s = _as_point(line.get("start"))
        e = _as_point(line.get("end"))
        pts = [p for p in (s, e) if p]
        length = float(line.get("length") or (_dist(pts[0], pts[1]) if len(pts) == 2 else 0))
        row = _segment_row(
            network=network,
            size=size,
            length=length,
            layer=layer,
            pts=pts,
            cl=cl,
            source="CAD LINE",
            method="Line endpoints projected to centerline",
        )
        if row:
            rows.append(row)

            return rows

        
        def _stamp_civil_pipe_location(
    row: dict[str, Any],
    sta_s: Any,
    sta_e: Any,
    length: float,
    side: Any,
    offset: Any,
) -> None:
    """Apply Civil start/end station (one or both) and side/offset onto a linear row."""
    if not row.get("from_station") or not row.get("to_station"):
        a = station_to_feet(sta_s) if sta_s not in (None, "") else None
        b = station_to_feet(sta_e) if sta_e not in (None, "") else None
        if a is not None and b is not None:
            row["from_station"] = feet_to_station(a)
            row["to_station"] = feet_to_station(b)
            if float(row.get("quantity_lf") or 0) <= 0:
                row["quantity_lf"] = round(abs(b - a), 2)
                row["length"] = row["quantity_lf"]
            row["method"] = f"Pipe start/end station on CL ({row['from_station']} → {row['to_station']})"
        elif a is not None and length > 0:
            row["from_station"] = feet_to_station(a)
            row["to_station"] = feet_to_station(a + length)
            row["method"] = f"Pipe start station + 2D length ({row['from_station']} + {length:.1f} lf)"
        elif b is not None and length > 0:
            row["to_station"] = feet_to_station(b)
            row["from_station"] = feet_to_station(max(0.0, b - length))
            row["method"] = f"Pipe end station − 2D length ({row['to_station']} − {length:.1f} lf)"
        if row.get("from_station") and row.get("to_station"):
            row["flags"] = [f for f in (row.get("flags") or []) if f != "line_not_associated_with_alignment"]

            if side:
        row["side"] = _side_code(str(side)) or row.get("side")
        row["side_of_alignment"] = row.get("side")
    if offset is not None and offset != "":
        try:
            off_f = abs(float(offset))
            row["offset_ft"] = off_f
            row["offset"] = format_offset(off_f, row.get("side"))
        except (TypeError, ValueError):
            parsed_side = _side_code(str(offset))
            if parsed_side:
                row["side"] = row.get("side") or parsed_side
            row["offset"] = str(offset)

        
        def _norm_obj_name(name: Any) -> str:
    return re.sub(r"[^a-z0-9]+", "", str(name or "").lower())

        
        def _find_connection_by_name(name: Any, connections: list[dict[str, Any]]) -> dict[str, Any] | None:
    needle = _norm_obj_name(name)
    if len(needle) < 2:
        return None
    exact = [c for c in connections if _norm_obj_name(c.get("name")) == needle]
    if exact:
        return exact[0]
    for c in connections:
        other = _norm_obj_name(c.get("name"))
        if other and (needle in other or other in needle):
            return c
    return None

        
        def _linear_needs_station(row: dict[str, Any]) -> bool:
    return not str(row.get("from_station") or "").strip() or not str(row.get("to_station") or "").strip()

        
        def _linear_net_compat(seg: dict[str, Any], conn: dict[str, Any]) -> bool:
    sn = str(seg.get("network") or "").lower()
    cn = str(conn.get("network") or "").lower()
    if sn and cn and sn not in {"utility", ""} and cn not in {"utility", ""}:
        return sn == cn
    su = str(seg.get("utility") or "").lower()
    cu = str(conn.get("utility") or "").lower()
    if su and cu and su != "underground utility" and cu != "underground utility":
        return su == cu
    sl = str(seg.get("layer") or "").lower()
    clayer = str(conn.get("layer") or "").lower()
    if sl and sl not in {"0", "defpoints"} and clayer and sl == clayer:
        return True
    # Generic Civil pipes (layer 0) may connect any stationed structure
    if sn in {"", "utility"} or su in {"", "underground utility"}:
        return True
    return False

        
        def _apply_end_locations(row: dict[str, Any], a: dict[str, Any], b: dict[str, Any], method: str) -> None:
    sa = station_to_feet(a.get("station"))
    sb = station_to_feet(b.get("station"))
    if sa is None or sb is None:
        return
    if sa <= sb:
        row["from_station"] = feet_to_station(sa)
        row["to_station"] = feet_to_station(sb)
        from_c, to_c = a, b
    else:
        row["from_station"] = feet_to_station(sb)
        row["to_station"] = feet_to_station(sa)
        from_c, to_c = b, a
    side = _side_code(from_c.get("side") or to_c.get("side"))
    off = from_c.get("offset_ft")
    if off is None:
        off = to_c.get("offset_ft")
    if off is None:
        m = re.search(r"[-+]?\d+(?:\.\d+)?", str(from_c.get("offset") or to_c.get("offset") or ""))
        off = float(m.group(0)) if m else None
    if side:
        row["side"] = side
        row["side_of_alignment"] = side
    if off is not None:
        row["offset_ft"] = abs(float(off))
        row["offset"] = format_offset(row["offset_ft"], side)
        row["from_offset"] = from_c.get("offset") or format_offset(from_c.get("offset_ft"), from_c.get("side"))
        row["to_offset"] = to_c.get("offset") or format_offset(to_c.get("offset_ft"), to_c.get("side"))
        fo, to_ = from_c.get("offset_ft"), to_c.get("offset_ft")
        if fo is not None and to_ is not None and abs(float(fo) - float(to_)) >= _NONPARALLEL_OFFSET_DELTA_FT:
            row["nonparallel"] = True
    row["method"] = method
    row["flags"] = [f for f in (row.get("flags") or []) if f != "line_not_associated_with_alignment"]

        
        def _fill_linear_from_connections(
    segments: list[dict[str, Any]],
    connections: list[dict[str, Any]],
) -> None:
    """Civil pipe networks: station a run from its start/end structures or nearby fittings."""
    if not segments:
        return

            # 1) Named start/end structures on the pipe (Civil 3D pipe network)
    for seg in segments:
        if not _linear_needs_station(seg):
            continue
        a = _find_connection_by_name(seg.get("start_structure"), connections)
        b = _find_connection_by_name(seg.get("end_structure"), connections)
        if a and b and a.get("station") and b.get("station"):
            _apply_end_locations(
                seg, a, b, f"Pipe network structures '{a.get('name')}' → '{b.get('name')}'"
            )

            stationed = [c for c in connections if station_to_feet(c.get("station")) is not None]
    used: set[int] = set()

            # 2) Pair fittings whose station span ≈ pipe 2D length (parallel-to-CL networks)
    blanks = [s for s in segments if _linear_needs_station(s)]
    blanks.sort(key=lambda s: float(s.get("quantity_lf") or s.get("length") or 0), reverse=True)
    for seg in blanks:
        length = float(seg.get("quantity_lf") or seg.get("length") or 0)
        if length < 1:
            continue
        best: tuple[float, dict[str, Any], dict[str, Any]] | None = None
        for i, a in enumerate(stationed):
            if id(a) in used or not _linear_net_compat(seg, a):
                continue
            sa = station_to_feet(a.get("station"))
            if sa is None:
                continue
            for b in stationed[i + 1 :]:
                if id(b) in used or not _linear_net_compat(seg, b):
                    continue
                sb = station_to_feet(b.get("station"))
                if sb is None:
                    continue
                delta = abs(sb - sa)
                if delta < 1:
                    continue
                err = abs(delta - length)
                rel = err / max(length, delta)
                if rel > 0.35 and err > 25:
                    continue
                score = rel
                if seg.get("size") and (
                    str(a.get("size") or "") == str(seg.get("size"))
                    or str(b.get("size") or "") == str(seg.get("size"))
                ):
                    score -= 0.08
                if best is None or score < best[0]:
                    best = (score, a, b)
        if best:
            _, a, b = best
            used.add(id(a))
            used.add(id(b))
            _apply_end_locations(
                seg, a, b, f"Fitting stations {a.get('station')} → {b.get('station')} (span ≈ pipe length)"
            )

            # 3) Remaining blanks: walk station envelope of that utility (Civil draft)
    leftovers = [s for s in segments if _linear_needs_station(s)]
    if leftovers and stationed:
        by_key: dict[str, list[dict[str, Any]]] = defaultdict(list)
        for c in stationed:
            by_key[str(c.get("network") or c.get("utility") or "")].append(c)
        for seg in leftovers:
            length = float(seg.get("quantity_lf") or seg.get("length") or 0)
            if length < 1:
                continue
            pool = [
                c
                for c in stationed
                if _linear_net_compat(seg, c)
            ]
            fts = sorted(station_to_feet(c.get("station")) for c in pool if station_to_feet(c.get("station")) is not None)
            if not fts:
                continue
            cursor = fts[0]
            # If another leftover already placed on this network, continue from last to_station
            same = [
                s
                for s in segments
                if s is not seg
                and not _linear_needs_station(s)
                and _linear_net_compat(s, pool[0])
            ]
            if same:
                ends = [station_to_feet(s.get("to_station")) for s in same]
                ends = [e for e in ends if e is not None]
                if ends:
                    cursor = max(cursor, max(ends))
            seg["from_station"] = feet_to_station(cursor)
            seg["to_station"] = feet_to_station(cursor + length)
            typ_side, typ_off = typical_network_offset(segments, seg.get("network"))
            if not seg.get("side"):
                # Prefer a fitting's side at this utility
                for c in pool:
                    if c.get("side"):
                        seg["side"] = _side_code(c.get("side"))
                        break
                seg["side"] = seg.get("side") or typ_side
                seg["side_of_alignment"] = seg.get("side")
            if seg.get("offset_ft") is None:
                for c in pool:
                    if c.get("offset_ft") is not None:
                        seg["offset_ft"] = c.get("offset_ft")
                        break
                if seg.get("offset_ft") is None:
                    seg["offset_ft"] = typ_off
                if seg.get("offset_ft") is not None:
                    seg["offset"] = format_offset(float(seg["offset_ft"]), seg.get("side"))
            seg["method"] = (
                f"Stationed along {seg.get('utility') or 'utility'} using fitting envelope "
                f"from {seg['from_station']}"
            )
            seg["flags"] = [f for f in (seg.get("flags") or []) if f != "line_not_associated_with_alignment"]
            seg["flags"] = list(seg.get("flags") or []) + ["station_inferred_from_fittings"]

            # Side/offset only (stations already known)
    for seg in segments:
        if not seg.get("from_station"):
            continue
        if seg.get("side") and seg.get("offset"):
            continue
        typ_side, typ_off = typical_network_offset(
            [s for s in segments if s.get("side") and s.get("offset")],
            seg.get("network"),
        )
        if not typ_side:
            for c in stationed:
                if _linear_net_compat(seg, c) and c.get("side"):
                    typ_side = _side_code(c.get("side"))
                    if typ_off is None:
                        typ_off = c.get("offset_ft")
                    break
        if not seg.get("side") and typ_side:
            seg["side"] = typ_side
            seg["side_of_alignment"] = typ_side
        if not seg.get("offset") and typ_off is not None:
            seg["offset_ft"] = typ_off
            seg["offset"] = format_offset(float(typ_off), seg.get("side"))

        
        def _angle_from_name(*texts: Any) -> str:
    blob = " ".join(str(t) for t in texts if t)
    m = re.search(r"(\d{1,3}(?:\.\d+)?)\s*°", blob)
    if m:
        return f"{m.group(1)}°"
    m = re.search(r"\b(90|45|22\.5|11\.25|60|30)\b", blob)
    if m:
        return f"{m.group(1)}°"
    return ""

        
        def _fitting_insert_point(block: dict[str, Any]) -> tuple[float, float] | None:
    for key in ("insert", "position", "location", "point", "ref_point", "geometry_point", "center"):
        pt = _as_point(block.get(key))
        if pt:
            return pt
    x = block.get("easting") or block.get("x") or block.get("pos_x")
    y = block.get("northing") or block.get("y") or block.get("pos_y")
    try:
        if x is not None and y is not None:
            return (float(x), float(y))
    except (TypeError, ValueError):
        pass
    bag_pt = point_from_property_bag(block)
    return (float(bag_pt[0]), float(bag_pt[1])) if bag_pt else None

        
        def _infer_side_offset_from_segments(
    station: str,
    network: str | None,
    segments: list[dict[str, Any]],
) -> tuple[str, float | None, str]:
    """Copy side/offset from a linear run that covers this station."""
    sta = station_to_feet(station)
    if sta is None or not segments:
        return "", None, ""
    best: dict[str, Any] | None = None
    best_dist = float("inf")
    for s in segments:
        if network and s.get("network") and s.get("network") != network:
            continue
        a = station_to_feet(str(s.get("from_station") or ""))
        b = station_to_feet(str(s.get("to_station") or ""))
        if a is None or b is None:
            continue
        lo, hi = min(a, b), max(a, b)
        if lo - 2 <= sta <= hi + 2:
            dist = 0.0 if lo <= sta <= hi else min(abs(sta - lo), abs(sta - hi))
            if dist < best_dist:
                best_dist = dist
                best = s
    if not best:
        return "", None, ""
    side = _side_code(str(best.get("side") or best.get("side_of_alignment") or ""))
    off = best.get("offset_ft")
    if off is None and best.get("offset") is not None:
        m = re.search(r"[-+]?\d*\.?\d+", str(best.get("offset")))
        try:
            off = float(m.group(0)) if m else None
        except (TypeError, ValueError):
            off = None
    label = str(best.get("offset") or "") or format_offset(float(off) if off is not None else None, side)
    return side, float(off) if off is not None else None, label

        
        def _resolve_fitting_location(
    block: dict[str, Any],
    cl: dict[str, Any],
    *,
    network: str | None = None,
    segments: list[dict[str, Any]] | None = None,
    labels: list[dict[str, Any]] | None = None,
    pipe_ends: list[dict[str, Any]] | None = None,
    type_label: str = "",
) -> dict[str, Any]:
    """Civil 3D StationOffset, then properties, attributes, labels, pipe joints."""
    alignment = cl.get("name") or "CL"
    insert = _fitting_insert_point(block)
    loc = _locate(insert, cl)

            bag = location_from_property_bag(block)
    attrs = location_from_block_attributes(block)
    name_blob = " ".join(str(block.get(k) or "") for k in ("name", "description", "type", "layer"))
    name_parsed = parse_station_offset_text(name_blob)

            prop_sta = (
        bag.get("station")
        or attrs.get("station")
        or coerce_station(block.get("station") or block.get("sta") or block.get("ref_station") or block.get("rawStation"))
        or (name_parsed or {}).get("station")
        or ""
    )
    prop_side = _side_code(
        bag.get("side")
        or attrs.get("side")
        or block.get("side")
        or block.get("offset_side")
        or (name_parsed or {}).get("side")
    )
    prop_off_f = bag.get("offset_ft")
    if prop_off_f is None:
        prop_off_f = attrs.get("offset_ft")
    if prop_off_f is None:
        prop_off_f = (name_parsed or {}).get("offset_ft")

            station = loc["station"] or coerce_station(prop_sta) or str(prop_sta or "").strip()
    # Raw numbers like 548.11 → 5+48.11
    if station and "+" not in station:
        station = coerce_station(station) or station
    side = loc["side"] or prop_side
    offset_ft = loc["offset_ft"] if loc["offset_ft"] is not None else prop_off_f
    offset_label = loc["offset_label"] or format_offset(offset_ft, side)
    method_bits: list[str] = []
    if loc["associated"]:
        method_bits.append("Civil StationOffset (insert → CL)")
    elif prop_sta:
        method_bits.append("Civil station property / attribute")
    if prop_side or prop_off_f is not None:
        method_bits.append("Civil side/offset property")

            # Nearby plan-sheet / model callouts (STA + 15' LT)
    if labels and (not station or not side or offset_ft is None):
        hit = match_label_to_fitting(
            name=str(block.get("name") or ""),
            layer=str(block.get("layer") or ""),
            type_label=type_label or str(block.get("type") or ""),
            insert=insert,
            labels=labels,
        )
        if hit:
            station = station or hit.get("station") or ""
            side = side or hit.get("side") or ""
            if offset_ft is None:
                offset_ft = hit.get("offset_ft")
            method_bits.append("plan-sheet station/offset label")

            # Snap to pipe start/end (Civil structures sit on pipe joints)
    if pipe_ends and (not station or not side or offset_ft is None):
        hit = match_pipe_end(insert=insert, network=network, ends=pipe_ends)
        if hit:
            station = station or hit.get("station") or ""
            side = side or hit.get("side") or ""
            if offset_ft is None:
                offset_ft = hit.get("offset_ft")
            method_bits.append("pipe-network joint")

            if station and (not side or offset_ft is None) and segments:
        inf_side, inf_off, inf_label = _infer_side_offset_from_segments(station, network, segments)
        if not side and inf_side:
            side = inf_side
        if offset_ft is None and inf_off is not None:
            offset_ft = inf_off
            offset_label = inf_label or format_offset(offset_ft, side)
        elif not offset_label and inf_label:
            offset_label = inf_label
        if inf_side or inf_off is not None:
            method_bits.append("side/offset from pipe run at station")

            # Whole-network typical offset (water often a constant 15' LT, etc.)
    if station and (not side or offset_ft is None) and segments:
        typ_side, typ_off = typical_network_offset(segments, network)
        if not side and typ_side:
            side = typ_side
        if offset_ft is None and typ_off is not None:
            offset_ft = typ_off
        if typ_side or typ_off is not None:
            method_bits.append("typical network offset")

            if offset_ft is not None and not offset_label:
        offset_label = format_offset(offset_ft, side)
    elif offset_ft is not None and side and "CL" in offset_label and side != "CL":
        offset_label = format_offset(offset_ft, side)

            associated = bool(loc["associated"] or (station and (side or offset_ft is not None)))
    return {
        "station": station,
        "side": side,
        "offset_ft": round(float(offset_ft), 2) if offset_ft is not None else None,
        "offset_label": offset_label,
        "alignment": alignment,
        "associated": associated,
        "insert": insert,
        "method": ", ".join(method_bits) or "no CL association",
    }

        
        def _connections_from_blocks(
    extraction: dict[str, Any],
    cl: dict[str, Any],
    *,
    segments: list[dict[str, Any]] | None = None,
    labels: list[dict[str, Any]] | None = None,
    pipe_ends: list[dict[str, Any]] | None = None,
) -> list[dict[str, Any]]:
    rows: list[dict[str, Any]] = []
    segs = segments or []
    labs = labels or []
    ends = pipe_ends or []
    for block in extraction.get("blocks") or []:
        name = str(block.get("name") or "BLOCK")
        layer = str(block.get("layer") or "")
        btype = str(block.get("type") or "")
        classified = classify_fitting(name, layer) or classify_fitting(name, btype)
        blob = f"{name} {layer} {btype} {block.get('description') or ''}".lower()
        is_conn = bool(classified) or any(
            k in blob
            for k in (
                "bend",
                "elbow",
                "tee",
                "wye",
                "valve",
                "hydrant",
                "fitting",
                "reducer",
                "cross",
                "plug",
                "cap",
                "cleanout",
                "manhole",
                "inlet",
                "junction",
                "connection",
                "structure",
                "casing",
                "sleeve",
            )
        )
        if not is_conn:
            continue

                network = detect_network(name, layer, btype, block.get("description"))
        size = extract_size_label(name, layer, block.get("size"), block.get("description"), btype)
        type_label = classified[0] if classified else name
        if classified and classified[0] == "Manhole" and network == "sanitary":
            type_label = "Sanitary Sewer Manhole"
        elif classified and classified[0] == "Manhole" and network == "storm":
            type_label = "Storm Sewer Manhole"

                loc = _resolve_fitting_location(
            block,
            cl,
            network=network,
            segments=segs,
            labels=labs,
            pipe_ends=ends,
            type_label=type_label,
        )
        angle = _angle_from_name(name, btype, block.get("description"))
        connects = _network_label(network) if network else ""
        if size:
            connects = f"{_size_display(size)} {connects}".strip()

                flags: list[str] = []
        if not loc["associated"]:
            flags.append("line_not_associated_with_alignment")
        if not loc["station"]:
            flags.append("missing_station")
        if not loc["side"] or loc["offset_ft"] is None:
            flags.append("missing_side_offset")
        if type_label.lower() in {"block", name.lower()} and not classified:
            flags.append("unidentified_fitting")
        if not size:
            flags.append("unknown_size")

                rows.append(
            {
                "type": type_label,
                "connection_type": type_label,
                "size": _size_display(size),
                "station": loc["station"],
                "side": loc["side"],
                "offset_ft": loc["offset_ft"],
                "offset": loc["offset_label"],
                "direction_from_alignment": loc["side"],
                "angle": angle,
                "angle_type": angle or type_label,
                "connects_to": connects,
                "utility": _network_label(network),
                "network": network or "utility",
                "quantity": 1,
                "unit": "EA",
                "layer": layer,
                "alignment": loc["alignment"],
                "source": "CAD block / fitting",
                "method": (
                    f"{loc['method']}: '{name}' @ {loc['station'] or '—'} "
                    f"({loc['offset_label'] or 'no offset'})"
                ),
                "name": name,
                "flags": flags,
                "confidence": 90.0 if loc["associated"] and classified else 70.0,
            }
        )
    return rows

        
        def _bends_from_polylines(extraction: dict[str, Any], cl: dict[str, Any]) -> list[dict[str, Any]]:
    rows: list[dict[str, Any]] = []
    for pl in extraction.get("polylines") or []:
        layer = str(pl.get("layer") or "")
        network = detect_network(layer, pl.get("name"))
        if not network and not any(
            k in layer.lower() for k in ("pipe", "water", "sewer", "storm", "san", "casing", "main")
        ):
            continue
        pts = _as_points(pl.get("points") or pl.get("vertices") or pl.get("coords"))
        if len(pts) < 3:
            continue
        size = extract_size_label(layer, pl.get("name"))
        for i in range(1, len(pts) - 1):
            a, b, c = pts[i - 1], pts[i], pts[i + 1]
            v1 = (b[0] - a[0], b[1] - a[1])
            v2 = (c[0] - b[0], c[1] - b[1])
            n1 = math.hypot(*v1)
            n2 = math.hypot(*v2)
            if n1 < 1e-6 or n2 < 1e-6:
                continue
            cosang = max(-1.0, min(1.0, (v1[0] * v2[0] + v1[1] * v2[1]) / (n1 * n2)))
            deflection = abs(math.degrees(math.acos(cosang)))
            if deflection < _MIN_BEND_DEG:
                continue
            # Snap to common bend angles
            snapped = deflection
            for cand in (11.25, 22.5, 45.0, 60.0, 90.0):
                if abs(deflection - cand) <= 6:
                    snapped = cand
                    break
            loc = _locate(b, cl)
            util = _network_label(network)
            size_d = _size_display(size)
            rows.append(
                {
                    "type": "Bend",
                    "connection_type": f"Bend ({snapped:g}°)",
                    "size": size_d,
                    "station": loc["station"],
                    "side": loc["side"],
                    "offset_ft": loc["offset_ft"],
                    "offset": loc["offset_label"],
                    "direction_from_alignment": loc["side"],
                    "angle": f"{snapped:g}°",
                    "angle_type": f"{snapped:g}°",
                    "connects_to": f"{size_d} {util}".strip(),
                    "utility": util,
                    "network": network or "utility",
                    "quantity": 1,
                    "unit": "EA",
                    "layer": layer,
                    "alignment": loc["alignment"],
                    "source": "Polyline deflection",
                    "method": f"Vertex deflection {deflection:.1f}° → {snapped:g}° on CL",
                    "name": f"BEND@{loc['station'] or i}",
                    "flags": [] if loc["associated"] else ["line_not_associated_with_alignment"],
                    "confidence": 88.0 if loc["associated"] else 65.0,
                }
            )
    return rows

        
        def _merge_segments(rows: list[dict[str, Any]]) -> list[dict[str, Any]]:
    if not rows:
        return []
    text_rows = [r for r in rows if "text" in str(r.get("source") or "").lower()]
    geo_rows = [r for r in rows if r not in text_rows]
    out = list(text_rows)
    for g in geo_rows:
        dup = False
        for t in out:
            if t.get("network") != g.get("network"):
                continue
            if (t.get("size") or "") != (g.get("size") or ""):
                continue
            tq = float(t.get("quantity_lf") or 0)
            gq = float(g.get("quantity_lf") or 0)
            if tq > 0 and abs(tq - gq) / max(tq, 1) < 0.05:
                dup = True
                break
        if not dup:
            out.append(g)

            def sort_key(r: dict[str, Any]) -> tuple:
        ft = station_to_feet(str(r.get("from_station") or "")) or 0.0
        return (str(r.get("item") or r.get("utility") or ""), str(r.get("size") or ""), ft)

            out.sort(key=sort_key)
    return out

        
        def build_bid_summary_from_detail(
    segments: list[dict[str, Any]],
    connections: list[dict[str, Any]],
    *,
    eoq_items: list[dict[str, Any]] | None = None,
) -> list[dict[str, Any]]:
    """Roll Bid Quantity Summary FROM detail tables (source of truth)."""
    lf_groups: dict[tuple[str, str], float] = defaultdict(float)
    for s in segments:
        key = (str(s.get("item") or s.get("utility") or "Utility"), str(s.get("size") or ""))
        lf_groups[key] += float(s.get("quantity_lf") or s.get("length") or 0)

            ea_groups: dict[tuple[str, str, str], float] = defaultdict(float)
    for c in connections:
        typ = str(c.get("type") or c.get("connection_type") or "Fitting")
        # Group bends by angle when present
        angle = str(c.get("angle") or "")
        if "bend" in typ.lower() and angle:
            typ = f"Bend {angle}"
        key = (typ, str(c.get("size") or ""), str(c.get("utility") or ""))
        ea_groups[key] += float(c.get("quantity") or 1)

            # Optional bid item codes from EOQ lines
    code_by_desc: dict[str, str] = {}
    for item in eoq_items or []:
        desc = str(item.get("description") or "").strip().lower()
        code = str(item.get("item_code") or item.get("csi_code") or "").strip()
        if desc and code:
            code_by_desc[desc] = code

            summary: list[dict[str, Any]] = []
    for (util, size), qty in sorted(lf_groups.items(), key=lambda x: (x[0][0], x[0][1])):
        desc = f"{size + ' ' if size else ''}{util}".strip()
        code = ""
        for d, c in code_by_desc.items():
            if util.lower() in d and (not size or size.replace('"', "").split("-")[0] in d):
                code = c
                break
        summary.append(
            {
                "bid_item": code,
                "description": desc,
                "unit": "LF",
                "quantity": round(qty, 2),
                "detail_source": "Linear Quantity Breakdown",
                "rollup_note": f"Sum of station runs = {round(qty, 2)} LF",
            }
        )

            for (typ, size, util), qty in sorted(ea_groups.items(), key=lambda x: (x[0][0], x[0][1])):
        desc = f"{size + ' ' if size else ''}{typ}".strip()
        if util and util.lower() not in desc.lower():
            desc = f"{desc} ({util})"
        code = ""
        for d, c in code_by_desc.items():
            if typ.lower().split()[0] in d and (not size or size.replace('"', "")[:2] in d):
                code = c
                break
        summary.append(
            {
                "bid_item": code,
                "description": desc,
                "unit": "EA",
                "quantity": round(qty, 0) if abs(qty - round(qty)) < 0.01 else round(qty, 2),
                "detail_source": "Fittings / Bends / Connections",
                "rollup_note": f"Count of discrete components = {int(qty) if abs(qty-round(qty))<0.01 else qty} EA",
            }
        )
    return summary

        
        def build_qa_flags(
    segments: list[dict[str, Any]],
    connections: list[dict[str, Any]],
    cl: dict[str, Any],
) -> list[dict[str, Any]]:
    flags: list[dict[str, Any]] = []
    if not cl.get("points") or len(cl.get("points") or []) < 2:
        flags.append(
            {
                "severity": "high",
                "issue": "no_centerline_geometry",
                "message": "No centerline geometry found — stations/offsets may be incomplete. Verify CL in DWG.",
                "object": cl.get("name") or "CL",
                "station": "",
                "suggestion": "Ensure roadway centerline / alignment exists in CAD (named CL / CENTERLINE).",
            }
        )
    elif not cl.get("is_centerline"):
        flags.append(
            {
                "severity": "medium",
                "issue": "centerline_proxy",
                "message": f"Using proxy alignment '{cl.get('name')}' — confirm it is the project centerline.",
                "object": cl.get("name"),
                "station": "",
                "suggestion": "Rename/include true CL alignment for stationing.",
            }
        )

            for s in segments:
        for f in s.get("flags") or []:
            flags.append(
                {
                    "severity": "medium" if f != "unknown_size" else "low",
                    "issue": f,
                    "message": f"Linear run flagged: {f}",
                    "object": s.get("description") or s.get("item"),
                    "station": f"{s.get('from_station')}–{s.get('to_station')}",
                    "suggestion": "Review association to centerline / size callout.",
                }
            )
        # Overlap heuristic: same utility+size overlapping station ranges
    by_key: dict[tuple[str, str], list[dict[str, Any]]] = defaultdict(list)
    for s in segments:
        by_key[(str(s.get("network")), str(s.get("size")))].append(s)
    for key, group in by_key.items():
        for i, a in enumerate(group):
            af = station_to_feet(a.get("from_station"))
            at = station_to_feet(a.get("to_station"))
            if af is None or at is None:
                continue
            a0, a1 = min(af, at), max(af, at)
            for b in group[i + 1 :]:
                bf = station_to_feet(b.get("from_station"))
                bt = station_to_feet(b.get("to_station"))
                if bf is None or bt is None:
                    continue
                b0, b1 = min(bf, bt), max(bf, bt)
                overlap = max(0.0, min(a1, b1) - max(a0, b0))
                if overlap > 5:
                    flags.append(
                        {
                            "severity": "high",
                            "issue": "overlapping_pipe",
                            "message": f"Overlapping {key[1]} {key[0]} runs (~{overlap:.0f} ft station overlap)",
                            "object": a.get("description"),
                            "station": f"{a.get('from_station')} / {b.get('from_station')}",
                            "suggestion": "Check for duplicate geometry or double-counted segments.",
                        }
                    )

            for c in connections:
        for f in c.get("flags") or []:
            flags.append(
                {
                    "severity": "high" if f == "unidentified_fitting" else "medium",
                    "issue": f,
                    "message": f"Fitting flagged: {f}",
                    "object": f"{c.get('type')} {c.get('size')}".strip(),
                    "station": c.get("station") or "",
                    "suggestion": "Identify block / verify CL projection.",
                }
            )
        conf = float(c.get("confidence") or 100)
        if conf < 75:
            flags.append(
                {
                    "severity": "medium",
                    "issue": "low_ai_confidence",
                    "message": f"Low confidence ({conf:.0f}) on {c.get('type')}",
                    "object": c.get("name") or c.get("type"),
                    "station": c.get("station") or "",
                    "suggestion": "Engineer review recommended.",
                }
            )

            # Disconnected endpoints: segment ends with no nearby fitting within 5 ft station
    conn_stas = [station_to_feet(c.get("station")) for c in connections]
    conn_stas = [x for x in conn_stas if x is not None]
    for s in segments:
        for end_key in ("from_station", "to_station"):
            st = station_to_feet(s.get(end_key))
            if st is None or not conn_stas:
                continue
            if min(abs(st - cs) for cs in conn_stas) > 8:
                # Only flag if length is substantial mid-run ends are ok at stubs
                pass  # soft — skip noisy disconnected flags for now

            severity_rank = {"high": 0, "medium": 1, "low": 2}
    flags.sort(key=lambda f: (severity_rank.get(str(f.get("severity")), 9), str(f.get("issue"))))
    return flags

        
        def build_utilities_detail(
    extraction: dict[str, Any],
    *,
    eoq_items: list[dict[str, Any]] | None = None,
) -> dict[str, Any]:
    """Build CL-based stationing, fittings, bid summary rollup, and QA flags."""
    cl = _build_centerline(extraction if isinstance(extraction, dict) else {})
    text_segs = _segments_from_texts(extraction, cl)
    geo_segs = _segments_from_geometry(extraction, cl)
    segments = _merge_segments(text_segs + geo_segs)

            labels = collect_location_labels(list(extraction.get("texts") or []))
    pipe_ends = collect_pipe_ends(extraction)
    # After linear stationing, copy side/offset onto pipe ends that only had stations
    for end in pipe_ends:
        if end.get("station") and (not end.get("side") or end.get("offset_ft") is None):
            inf_side, inf_off, _ = _infer_side_offset_from_segments(
                str(end.get("station") or ""),
                str(end.get("network") or "") or None,
                segments,
            )
            if not end.get("side") and inf_side:
                end["side"] = inf_side
            if end.get("offset_ft") is None and inf_off is not None:
                end["offset_ft"] = inf_off

            connections = _connections_from_blocks(
        extraction, cl, segments=segments, labels=labels, pipe_ends=pipe_ends
    )
    connections.extend(_bends_from_polylines(extraction, cl))

            seen: set[str] = set()
    uniq_conn: list[dict[str, Any]] = []
    for c in connections:
        key = (
            f"{c.get('utility')}|{c.get('type')}|{c.get('station')}|"
            f"{c.get('size')}|{c.get('angle')}|{c.get('name')}"
        ).lower()
        if key in seen:
            continue
        seen.add(key)
        uniq_conn.append(c)

            def conn_key(c: dict[str, Any]) -> tuple:
        ft = station_to_feet(str(c.get("station") or "")) or 0.0
        return (str(c.get("utility") or ""), ft, str(c.get("type") or ""))

            uniq_conn.sort(key=conn_key)

            # Linear rows often have length only (APS pipes). Station them from Civil
    # start/end structures or fittings — same association Civil 3D uses.
    _fill_linear_from_connections(segments, uniq_conn)

            bid_summary = build_bid_summary_from_detail(segments, uniq_conn, eoq_items=eoq_items)
    qa_flags = build_qa_flags(segments, uniq_conn, cl)

            networks = sorted(
        {str(s.get("utility")) for s in segments} | {str(c.get("utility")) for c in uniq_conn}
    )
    return {
        "alignment": {
            "name": cl.get("name"),
            "layer": cl.get("layer"),
            "length": round(float(cl.get("length") or 0), 2),
            "sta_start": feet_to_station(float(cl.get("sta_start") or 0)),
            "source": cl.get("source"),
            "is_centerline": bool(cl.get("is_centerline")),
            "has_geometry": len(cl.get("points") or []) >= 2,
        },
        "segments": segments,
        "connections": uniq_conn,
        "bid_summary": bid_summary,
        "qa_flags": qa_flags,
        "summary": {
            "segment_count": len(segments),
            "connection_count": len(uniq_conn),
            "bid_item_count": len(bid_summary),
            "qa_flag_count": len(qa_flags),
            "utilities": networks,
            "total_lf": round(sum(float(s.get("quantity_lf") or 0) for s in segments), 2),
            "centerline": cl.get("name"),
        },
    }

### backend/app/services/cad/civil_location.py

SHA-256: 24bb4338e57186fefee720ca3e01b79dd4d3da2064706de7d6f3e4de8f23d9e3    ·    769 line(s)    ·    .py

"""Civil 3D station / offset / side — same formulas the product uses.

        Civil 3D ``Alignment.StationOffset(x, y, out station, out offset)``:

        * **Station** = alignment start station + distance along the alignment to the
  closest point (raw feet, displayed as ``12+34.56``).
* **Offset** is *signed*: **positive = RIGHT** of increasing station, negative = LEFT.
* **Side** on takeoff sheets is ``LT`` / ``RT`` / ``CL``.

        Plan sheets also store the same values as:

        * Civil object properties (``rawStation``, Easting/Northing, …)
* Block attributes (``STA``, ``STATION``, ``OFFSET``, ``SIDE``)
* Text / MTEXT callouts (``STA 5+48.11 8.2' RT``), often in paper space
"""

        from __future__ import annotations

        import math
import re
from typing import Any

        # Civil plan station: 0+00, 5+48.11, 12+00
_STA_TOKEN = re.compile(r"(?P<sta>\d{1,5}\+\d{2}(?:\.\d+)?)")
_STA_LABELED = re.compile(
    r"(?:sta(?:tion)?\.?\s*[:#\-]?\s*)?(?P<sta>\d{1,5}\+\d{2}(?:\.\d+)?)",
    re.I,
)
# 15.0' LT   |   LT 15'   |   15.00 FT RT   |   8.2' RIGHT
_OFFSET_SIDE = re.compile(
    r"(?P<off>\d+(?:\.\d+)?)\s*(?:'|ft|feet|m)?\s*['\"]?\s*(?P<side>L\.?T\.?|R\.?T\.?|LEFT|RIGHT|CL|CTR|CENTER)"
    r"|(?P<side2>L\.?T\.?|R\.?T\.?|LEFT|RIGHT)\s*(?P<off2>\d+(?:\.\d+)?)\s*(?:'|ft|feet)?",
    re.I,
)
_STA_THEN_OFFSET = re.compile(
    r"(?P<sta>\d{1,5}\+\d{2}(?:\.\d+)?)"
    r"[^\d+]{0,48}"
    r"(?:(?P<off>\d+(?:\.\d+)?)\s*(?:'|ft|feet)?\s*(?P<side>L\.?T\.?|R\.?T\.?|LEFT|RIGHT)"
    r"|(?P<side2>L\.?T\.?|R\.?T\.?|LEFT|RIGHT)\s*(?P<off2>\d+(?:\.\d+)?))",
    re.I,
)

        # Property-bag keys Civil / APS / block attributes actually use
_STA_KEYS = (
    "station",
    "raw station",
    "rawstation",
    "structure station",
    "pipe station",
    "refstation",
    "ref station",
    "sta",
    "sta.",
    "stn",
    "aligned station",
    "alignment station",
    "station along alignment",
    "station value",
    "start station",
    "starting station",
    "stastart",
)
_SIDE_KEYS = (
    "side",
    "offset side",
    "direction",
    "dir",
    "lt/rt",
    "l/r",
    "hand",
    "side of alignment",
    "offset direction",
)
_OFF_KEYS = (
    "offset",
    "2d offset",
    "horizontal offset",
    "offset from alignment",
    "rawoffset",
    "raw offset",
    "offset ft",
    "offset (ft)",
    "offset distance",
    "signed offset",
)
_XY_PAIRS = (
    ("easting", "northing"),
    ("east", "north"),
    ("start easting", "start northing"),
    ("end easting", "end northing"),
    ("position x", "position y"),
    ("pos x", "pos y"),
    ("insertion point x", "insertion point y"),
    ("insertionpointx", "insertionpointy"),
    ("insert x", "insert y"),
    ("location x", "location y"),
    ("refpt x", "refpt y"),
    ("ref pt x", "ref pt y"),
    ("center x", "center y"),
    ("origin x", "origin y"),
    ("geompositionx", "geompositiony"),
    ("x", "y"),
    ("start x", "start y"),
    ("start point x", "start point y"),
    ("end x", "end y"),
    ("end point x", "end point y"),
    ("min x", "min y"),  # bbox — refined below
)

        _ATTR_STA_TAGS = frozenset(
    {"STA", "STATION", "STN", "STA_NO", "STANO", "ALIGNSTA", "RAWSTA", "RAW_STATION"}
)
_ATTR_SIDE_TAGS = frozenset({"SIDE", "DIR", "HAND", "LT_RT", "LTRT", "OFFSETSIDE"})
_ATTR_OFF_TAGS = frozenset({"OFFSET", "OFF", "OS", "HOFFSET", "OFFSET_FT", "OFFS"})

        
        def _to_float(value: Any) -> float | None:
    if value is None or value == "":
        return None
    if isinstance(value, (int, float)) and not isinstance(value, bool):
        f = float(value)
        return f if math.isfinite(f) else None
    text = str(value).strip().replace(",", "").replace("'", "")
    m = re.search(r"[-+]?\d+(?:\.\d+)?(?:[eE][-+]?\d+)?", text)
    if not m:
        return None
    try:
        f = float(m.group(0))
    except ValueError:
        return None
    return f if math.isfinite(f) else None

        
        def station_to_feet(sta: Any) -> float | None:
    """Parse ``12+34.56`` or Civil raw station feet (``1234.56``)."""
    if sta is None or sta == "":
        return None
    if isinstance(sta, (int, float)) and not isinstance(sta, bool):
        f = float(sta)
        return f if math.isfinite(f) and 0 <= f < 1_000_000 else None
    s = str(sta).strip()
    m = re.match(r"(\d+)\+(\d+(?:\.\d+)?)$", s)
    if m:
        return float(m.group(1)) * 100.0 + float(m.group(2))
    # Allow leading STA
    m = _STA_TOKEN.search(s)
    if m and re.fullmatch(r"\s*(?:sta(?:tion)?\.?\s*)?\d+\+\d{2}(?:\.\d+)?\s*", s, re.I):
        return station_to_feet(m.group("sta"))
    try:
        f = float(s.replace(",", "").replace("'", ""))
    except ValueError:
        return None
    if math.isfinite(f) and 0 <= f < 1_000_000:
        return f
    return None

        
        def feet_to_station(feet: float | None) -> str:
    if feet is None or not math.isfinite(feet):
        return ""
    feet = max(0.0, float(feet))
    plus = int(feet // 100)
    rem = feet - plus * 100
    if abs(rem - round(rem)) < 0.05:
        return f"{plus}+{int(round(rem)):02d}"
    return f"{plus}+{rem:05.2f}"

        
        def coerce_station(value: Any) -> str:
    """Normalize any Civil station representation to ``12+34.56``."""
    ft = station_to_feet(value)
    if ft is None:
        m = _STA_TOKEN.search(str(value or ""))
        if m:
            ft = station_to_feet(m.group("sta"))
    return feet_to_station(ft) if ft is not None else ""

        
        def side_code(side: Any) -> str:
    s = str(side or "").strip().lower().replace(".", "")
    if not s:
        return ""
    if re.search(r"\bleft\b|\blt\b|^l$", s) or s.endswith(" lt") or " lt " in f" {s} ":
        return "LT"
    if re.search(r"\bright\b|\brt\b|^r$", s) or s.endswith(" rt") or " rt " in f" {s} ":
        return "RT"
    if re.search(r"\b(cl|center|centre|ctr|on cl|on-cl)\b", s):
        return "CL"
    if s in {"lt", "rt", "cl"}:
        return s.upper()
    return ""

        
        def civil_signed_offset(offset: float) -> tuple[str, float]:
    """Civil 3D signed offset → (side, absolute feet). Positive = RIGHT."""
    if abs(offset) < 0.05:
        return "CL", 0.0
    if offset > 0:
        return "RT", abs(offset)
    return "LT", abs(offset)

        
        def format_offset(offset_ft: float | None, side: str | None) -> str:
    if offset_ft is None:
        return ""
    code = side_code(side)
    if not code or code == "CL":
        return f"{float(offset_ft):.1f}' CL"
    return f"{float(offset_ft):.1f}' {code}"

        
        def _norm_key(key: Any) -> str:
    return re.sub(r"[^a-z0-9]+", " ", str(key or "").lower()).strip()

        
        def _lookup(props: dict[str, Any], keys: tuple[str, ...]) -> Any:
    if not props:
        return None
    exact = {_norm_key(k): v for k, v in props.items()}
    for key in keys:
        if key in exact and exact[key] not in (None, ""):
            return exact[key]
    # Substring only for longer keys so "sta" does not steal "start" / "state"
    long_keys = [k for k in keys if len(k) >= 5]
    if not long_keys:
        return None
    for nk, v in exact.items():
        if v in (None, ""):
            continue
        for key in long_keys:
            if key == nk or key in nk:
                return v
    return None

        
        def _xy_from_string(raw: Any) -> list[float] | None:
    if raw is None:
        return None
    nums = re.findall(r"[-+]?\d+(?:\.\d+)?(?:[eE][-+]?\d+)?", str(raw))
    if len(nums) < 2:
        return None
    try:
        x, y = float(nums[0]), float(nums[1])
    except ValueError:
        return None
    # Reject tiny "0, 1" style enums; world/state-plane coords are large
    if abs(x) < 1 and abs(y) < 1:
        return None
    return [x, y]

        
        def point_from_property_bag(props: dict[str, Any] | None) -> list[float] | None:
    """Best-effort world XY (Easting/Northing or insert) from a Civil/APS bag."""
    if not props:
        return None
    exact = {_norm_key(k): v for k, v in props.items()}

            for xk, yk in _XY_PAIRS:
        if xk == "min x":
            continue
        x = _to_float(exact.get(xk))
        y = _to_float(exact.get(yk))
        if x is not None and y is not None:
            return [float(x), float(y)]

            # Fuzzy: any *easting*/*x* paired with *northing*/*y*
    x_val = _lookup(props, ("easting", "position x", "insert x", "location x", "origin x", "center x"))
    y_val = _lookup(props, ("northing", "position y", "insert y", "location y", "origin y", "center y"))
    x = _to_float(x_val)
    y = _to_float(y_val)
    if x is not None and y is not None:
        return [float(x), float(y)]

            for key in (
        "location",
        "position",
        "insertion point",
        "geometry location",
        "insert",
        "refpt",
        "center",
        "origin",
        "geometry",
    ):
        pt = _xy_from_string(exact.get(key) or _lookup(props, (key,)))
        if pt:
            return pt

            # Bounding-box center (APS sometimes exposes only extents)
    minx = _to_float(exact.get("min x") or exact.get("minx") or exact.get("bbox min x"))
    maxx = _to_float(exact.get("max x") or exact.get("maxx") or exact.get("bbox max x"))
    miny = _to_float(exact.get("min y") or exact.get("miny") or exact.get("bbox min y"))
    maxy = _to_float(exact.get("max y") or exact.get("maxy") or exact.get("bbox max y"))
    if None not in (minx, maxx, miny, maxy):
        return [(minx + maxx) / 2.0, (miny + maxy) / 2.0]

            return None

        
        def location_from_property_bag(props: dict[str, Any] | None) -> dict[str, Any]:
    """Station / side / offset / insert from a flat Civil, APS, or attrib dict."""
    props = props or {}
    sta_raw = _lookup(props, _STA_KEYS)
    side_raw = _lookup(props, _SIDE_KEYS)
    off_raw = _lookup(props, _OFF_KEYS)

            station = coerce_station(sta_raw)
    side = side_code(side_raw)
    offset_ft: float | None = None

            off_f = _to_float(off_raw)
    if off_f is not None:
        # Combined "15.0' LT" in the offset field
        parsed_side = side_code(str(off_raw))
        if parsed_side:
            side = side or parsed_side
            offset_ft = abs(off_f)
        elif side:
            offset_ft = abs(off_f)
        else:
            # Bare Civil signed offset (positive = RT)
            side, offset_ft = civil_signed_offset(off_f)

            # Combined strings anywhere in the bag
    if not station or not side or offset_ft is None:
        for v in props.values():
            parsed = parse_station_offset_text(str(v or ""))
            if not parsed.get("station") and not parsed.get("side"):
                continue
            station = station or parsed.get("station") or ""
            side = side or parsed.get("side") or ""
            if offset_ft is None and parsed.get("offset_ft") is not None:
                offset_ft = parsed["offset_ft"]

            insert = point_from_property_bag(props)
    return {
        "station": station,
        "side": side,
        "offset_ft": offset_ft,
        "offset": format_offset(offset_ft, side) if offset_ft is not None else "",
        "insert": insert,
        "raw_station": sta_raw,
    }

        
        def parse_station_offset_text(text: str) -> dict[str, Any]:
    """Parse a civil callout: ``STA 5+48.11 8.2' RT``."""
    empty = {"station": "", "side": "", "offset_ft": None}
    if not text or not str(text).strip():
        return dict(empty)
    blob = str(text)

            m = _STA_THEN_OFFSET.search(blob)
    if m:
        side = side_code(m.group("side") or m.group("side2"))
        off = _to_float(m.group("off") or m.group("off2"))
        return {
            "station": coerce_station(m.group("sta")),
            "side": side,
            "offset_ft": abs(off) if off is not None else None,
        }

            sta_m = _STA_LABELED.search(blob)
    off_m = _OFFSET_SIDE.search(blob)
    station = coerce_station(sta_m.group("sta")) if sta_m else ""
    side = ""
    offset_ft = None
    if off_m:
        side = side_code(off_m.group("side") or off_m.group("side2"))
        offset_ft = _to_float(off_m.group("off") or off_m.group("off2"))
        if offset_ft is not None:
            offset_ft = abs(offset_ft)
    return {"station": station, "side": side, "offset_ft": offset_ft}

        
        def attributes_to_dict(raw: Any) -> dict[str, Any]:
    """Normalize INSERT attributes from DXF / plugin / APS."""
    out: dict[str, Any] = {}
    if not raw:
        return out
    if isinstance(raw, dict):
        for k, v in raw.items():
            if k is None or v is None:
                continue
            out[str(k).strip()] = v
            out[str(k).strip().upper()] = v
        return out
    if isinstance(raw, list):
        for item in raw:
            if not isinstance(item, dict):
                continue
            tag = item.get("tag") or item.get("name") or item.get("key")
            val = item.get("text") or item.get("value") or item.get("contents")
            if tag is not None and val is not None:
                out[str(tag).strip()] = val
                out[str(tag).strip().upper()] = val
    return out

        
        def location_from_block_attributes(block: dict[str, Any]) -> dict[str, Any]:
    attrs = attributes_to_dict(
        block.get("attributes") or block.get("attribs") or block.get("atts")
    )
    if not attrs:
        return {"station": "", "side": "", "offset_ft": None, "insert": None}

            sta = None
    side = None
    off = None
    for k, v in attrs.items():
        ku = str(k).upper().replace(" ", "")
        if ku in _ATTR_STA_TAGS or ku.endswith("STATION") or ku == "STA":
            sta = v
        elif ku in _ATTR_SIDE_TAGS:
            side = v
        elif ku in _ATTR_OFF_TAGS:
            off = v

            bag = dict(attrs)
    if sta is not None:
        bag["station"] = sta
    if side is not None:
        bag["side"] = side
    if off is not None:
        bag["offset"] = off
    loc = location_from_property_bag(bag)
    # Attribute strings like "5+48.11 8.2' RT" on a single STA tag
    if (not loc["station"] or loc["offset_ft"] is None) and sta is not None:
        parsed = parse_station_offset_text(str(sta))
        loc["station"] = loc["station"] or parsed["station"]
        loc["side"] = loc["side"] or parsed["side"]
        if loc["offset_ft"] is None:
            loc["offset_ft"] = parsed["offset_ft"]
    return loc

        
        def collect_location_labels(texts: list[dict[str, Any]]) -> list[dict[str, Any]]:
    """Index drawing/sheet callouts that carry station and/or offset."""
    labels: list[dict[str, Any]] = []
    for t in texts or []:
        blob = str(t.get("text") or t.get("contents") or t.get("value") or "")
        parsed = parse_station_offset_text(blob)
        if not parsed["station"] and parsed["offset_ft"] is None:
            continue
        insert = None
        if str(t.get("space") or "").lower() != "paper":
            raw = t.get("insert") or t.get("position") or t.get("location")
            if isinstance(raw, (list, tuple)) and len(raw) >= 2:
                try:
                    insert = (float(raw[0]), float(raw[1]))
                except (TypeError, ValueError):
                    insert = None
        labels.append(
            {
                "text": blob,
                "station": parsed["station"],
                "side": parsed["side"],
                "offset_ft": parsed["offset_ft"],
                "insert": insert,
                "layer": str(t.get("layer") or ""),
                "space": str(t.get("space") or "model"),
                "used": False,
            }
        )
    return labels

        
        def _text_kind_hints(text: str) -> set[str]:
    low = text.lower()
    hints: set[str] = set()
    mapping = (
        ("valve", "valve"),
        ("gv", "valve"),
        ("hydrant", "hydrant"),
        ("tee", "tee"),
        ("wye", "wye"),
        ("bend", "bend"),
        ("elbow", "bend"),
        ("reducer", "reducer"),
        ("manhole", "manhole"),
        (" mh", "manhole"),
        ("inlet", "inlet"),
        ("catch", "inlet"),
        ("fitting", "fitting"),
        ("structure", "structure"),
    )
    for key, hint in mapping:
        if key in low:
            hints.add(hint)
    return hints

        
        def match_label_to_fitting(
    *,
    name: str,
    layer: str,
    type_label: str,
    insert: tuple[float, float] | None,
    labels: list[dict[str, Any]],
    snap_ft: float = 60.0,
) -> dict[str, Any] | None:
    """Pick the best unused station/offset callout for this fitting."""
    if not labels:
        return None
    blob = f"{name} {layer} {type_label}".lower()
    hints = _text_kind_hints(blob)
    best: dict[str, Any] | None = None
    best_score = -1.0

            for lab in labels:
        if lab.get("used"):
            continue
        score = 0.0
        lab_hints = _text_kind_hints(str(lab.get("text") or ""))
        if hints and lab_hints and hints & lab_hints:
            score += 8
        elif hints and lab_hints:
            score -= 2
        if insert and lab.get("insert"):
            d = math.hypot(insert[0] - lab["insert"][0], insert[1] - lab["insert"][1])
            if d <= snap_ft:
                score += max(0.0, 20.0 - d / 4.0)
            elif lab.get("space") != "paper":
                continue
        elif insert and lab.get("space") != "paper" and lab.get("insert"):
            continue
        if lab.get("station"):
            score += 3
        if lab.get("side") or lab.get("offset_ft") is not None:
            score += 2
        if score > best_score:
            best_score = score
            best = lab

            if best is None or best_score < 8:
        # Paper-space / lexical only: type hint match + station
        lexical: dict[str, Any] | None = None
        lex_score = -1.0
        for lab in labels:
            if lab.get("used") or not lab.get("station"):
                continue
            lab_hints = _text_kind_hints(str(lab.get("text") or ""))
            score = 4 if (hints and lab_hints and hints & lab_hints) else 0
            if score > lex_score:
                lex_score = score
                lexical = lab
        if lexical and lex_score >= 4:
            lexical["used"] = True
            return lexical
        return None

            best["used"] = True
    return best

        
        def civil3d_station_offset(
    point: tuple[float, float],
    chain: list[tuple[float, float]],
    sta_start: float = 0.0,
) -> dict[str, Any] | None:
    """Mirror ``Alignment.StationOffset`` on a 2D alignment polyline.

            Returns station string, signed Civil offset (RT+), takeoff side, and abs feet.
    """
    if not point or len(chain) < 2:
        return None
    best_d = float("inf")
    best_sta = 0.0
    best_signed = 0.0
    cum = 0.0
    for i in range(1, len(chain)):
        x1, y1 = chain[i - 1]
        x2, y2 = chain[i]
        dx, dy = x2 - x1, y2 - y1
        seg_len = math.hypot(dx, dy)
        if seg_len < 1e-9:
            continue
        t = ((point[0] - x1) * dx + (point[1] - y1) * dy) / (seg_len * seg_len)
        t_clamped = max(0.0, min(1.0, t))
        proj = (x1 + t_clamped * dx, y1 + t_clamped * dy)
        d = math.hypot(point[0] - proj[0], point[1] - proj[1])
        # Cross > 0 → left of travel (LT). Civil signed offset is opposite (RT+).
        cross = dx * (point[1] - y1) - dy * (point[0] - x1)
        signed = d if cross < 0 else -d
        if abs(cross) < 1e-6 * seg_len or d < 0.05:
            signed = 0.0
        if d < best_d:
            best_d = d
            best_sta = cum + t_clamped * seg_len
            best_signed = signed
        cum += seg_len
    if best_d == float("inf"):
        return None
    side, off_abs = civil_signed_offset(best_signed) if abs(best_signed) >= 0.05 else ("CL", 0.0)
    # Prefer geometric LT/RT from cross (already encoded in signed)
    sta_ft = sta_start + best_sta
    return {
        "station": feet_to_station(sta_ft),
        "station_ft": sta_ft,
        "offset_ft": round(off_abs, 2),
        "side": side,
        "offset": format_offset(off_abs, side),
        "civil_signed_offset": round(best_signed, 2),
    }

        
        def collect_pipe_ends(extraction: dict[str, Any]) -> list[dict[str, Any]]:
    """Pipe start/end stations and XY — fittings usually sit on these joints."""
    ends: list[dict[str, Any]] = []
    for pipe in extraction.get("pipes") or []:
        network = pipe.get("network")
        size = pipe.get("diameter") or pipe.get("size") or pipe.get("part_size")
        side = side_code(pipe.get("side") or pipe.get("offset_side"))
        off = _to_float(pipe.get("offset_ft") if pipe.get("offset_ft") is not None else pipe.get("offset"))
        if off is not None and not side:
            side, off = civil_signed_offset(off)
        elif off is not None:
            off = abs(off)
        pairs = (
            (pipe.get("sta_start") or pipe.get("start_station"), pipe.get("start") or pipe.get("start_point")),
            (pipe.get("sta_end") or pipe.get("end_station"), pipe.get("end") or pipe.get("end_point")),
        )
        for sta, pt in pairs:
            station = coerce_station(sta)
            insert = None
            if isinstance(pt, (list, tuple)) and len(pt) >= 2:
                try:
                    insert = (float(pt[0]), float(pt[1]))
                except (TypeError, ValueError):
                    insert = None
            if not station and not insert:
                continue
            ends.append(
                {
                    "station": station,
                    "side": side,
                    "offset_ft": off,
                    "insert": insert,
                    "network": network,
                    "size": size,
                    "used": False,
                }
            )
    return ends

        
        def match_pipe_end(
    *,
    insert: tuple[float, float] | None,
    network: str | None,
    ends: list[dict[str, Any]],
    snap_ft: float = 8.0,
) -> dict[str, Any] | None:
    best = None
    best_d = snap_ft
    for end in ends:
        if end.get("used") or not end.get("insert") or not insert:
            continue
        if network and end.get("network") and str(end["network"]).lower() != str(network).lower():
            continue
        d = math.hypot(insert[0] - end["insert"][0], insert[1] - end["insert"][1])
        if d <= best_d:
            best_d = d
            best = end
    if best:
        best["used"] = True
    return best

        
        def typical_network_offset(
    segments: list[dict[str, Any]],
    network: str | None,
) -> tuple[str, float | None]:
    """Most common side/offset for a utility — Civil networks sit on a constant offset."""
    sides: dict[str, int] = {}
    offs: list[float] = []
    for s in segments or []:
        if network and s.get("network") and s.get("network") != network:
            continue
        side = side_code(s.get("side") or s.get("side_of_alignment"))
        if side:
            sides[side] = sides.get(side, 0) + 1
        off = s.get("offset_ft")
        if off is None and s.get("offset") is not None:
            off = _to_float(s.get("offset"))
        if off is not None:
            offs.append(abs(float(off)))
    side = max(sides, key=sides.get) if sides else ""
    offset_ft = None
    if offs:
        offs.sort()
        offset_ft = offs[len(offs) // 2]
    return side, offset_ft

        
        def useful_property_attr(label: str) -> bool:
    """Whether an APS properties.db attribute should be loaded for stationing."""
    low = (label or "").lower()
    keys = (
        "layer",
        "length",
        "area",
        "type",
        "name",
        "handle",
        "radius",
        "perimeter",
        "measurement",
        "contents",
        "textcontent",
        "text",
        "pipe",
        "structure",
        "alignment",
        "parcel",
        "network",
        "part size",
        "diameter",
        "inner",
        "outer",
        "material",
        "cut",
        "fill",
        "volume",
        "surface",
        "slope",
        "description",
        "family",
        "style",
        "station",
        "sta",
        "stn",
        "start",
        "end",
        "refstart",
        "refend",
        "easting",
        "northing",
        "east",
        "north",
        "offset",
        "side",
        "direction",
        "location",
        "position",
        "insert",
        "insertion",
        "origin",
        "center",
        "coord",
        "bbox",
        "min x",
        "max x",
        "min y",
        "max y",
        "minx",
        "maxx",
        "raw",
        "align",
        "refpt",
        "geom",
    )
    if any(k in low for k in keys):
        return True
    # Lone X / Y / Z geometry attributes
    token = re.sub(r"[^a-z0-9]+", " ", low).strip()
    if token in {"x", "y", "z", "x y", "xy"}:
        return True
    return False

### backend/app/services/cad/engine.py

SHA-256: f08bd346bd5c79cea007770d4c67b1b21fc488a7e3ea2d2bfe15e4e955b759d2    ·    202 line(s)    ·    .py

"""CAD & Civil 3D Intelligence Engine orchestrator."""

        from __future__ import annotations

        import json
from pathlib import Path

        from sqlalchemy import delete, select
from sqlalchemy.orm import Session, selectinload

        from app.models.cad import CadJobStatus, CadModel, CadQuantity, CadSourceFormat
from app.models.document import Document, DocumentType, ProcessingStatus
from app.services.cad.autodesk_aps import parse_civil3d_package, parse_dwg
from app.services.cad.dxf_parser import parse_dxf
from app.services.cad.landxml_parser import parse_landxml
from app.services.cad.quantity_engine import build_quantities
from app.services.openai_client import enrich_cad_quantities_with_openai, openai_configured
from app.services.storage import storage_service
from app.config import get_settings

        
        def detect_cad_format(document: Document) -> CadSourceFormat | None:
    name = document.original_filename.lower()
    ext = Path(name).suffix.lower().lstrip(".")
    if document.document_type == DocumentType.DXF or ext == "dxf":
        return CadSourceFormat.DXF
    if document.document_type == DocumentType.DWG or ext == "dwg":
        return CadSourceFormat.DWG
    if document.document_type == DocumentType.LANDXML or ext in {"landxml"} or (
        ext == "xml" and "land" in name
    ):
        return CadSourceFormat.LANDXML
    if document.document_type == DocumentType.CIVIL3D or ext == "json" or "civil" in name:
        return CadSourceFormat.CIVIL3D
    if ext == "xml":
        return CadSourceFormat.LANDXML
    return None

        
        def process_cad_document(db: Session, document: Document) -> CadModel:
    fmt = detect_cad_format(document)
    if not fmt:
        raise ValueError("Document is not a supported CAD/Civil format")

            model = db.scalar(select(CadModel).where(CadModel.document_id == document.id))
    if not model:
        model = CadModel(
            project_id=document.project_id,
            document_id=document.id,
            source_format=fmt,
        )
        db.add(model)

            model.status = CadJobStatus.PARSING
    model.error_message = None
    document.processing_status = ProcessingStatus.PROCESSING
    db.commit()

            try:
        path = storage_service.resolve_local_path(document.storage_key)
        if not path.exists():
            raise FileNotFoundError("CAD file missing from storage")

                if fmt == CadSourceFormat.DXF:
            extraction = parse_dxf(path)
        elif fmt == CadSourceFormat.LANDXML:
            extraction = parse_landxml(path)
        elif fmt == CadSourceFormat.CIVIL3D:
            extraction = parse_civil3d_package(path)
        else:
            extraction = parse_dwg(path)

                model.engine = extraction.get("engine") or "autovad_cad"
        model.units = str(extraction.get("units")) if extraction.get("units") is not None else None
        model.summary = extraction.get("summary")
        model.layers_json = json.dumps(extraction.get("layers") or [], ensure_ascii=True)
        model.entities_json = json.dumps(
            {
                "lines": (extraction.get("lines") or [])[:200],
                "polylines": (extraction.get("polylines") or [])[:800],
                "circles": (extraction.get("circles") or [])[:100],
                "hatches": (extraction.get("hatches") or [])[:100],
                "alignments": extraction.get("alignments") or [],
                "pipes": extraction.get("pipes") or [],
                "surfaces": extraction.get("surfaces") or [],
                "volumes": extraction.get("volumes") or [],
                "cross_sections": extraction.get("cross_sections") or [],
            },
            ensure_ascii=True,
        )
        model.blocks_json = json.dumps(extraction.get("blocks") or [], ensure_ascii=True)
        model.dimensions_json = json.dumps(extraction.get("dimensions") or [], ensure_ascii=True)
        model.texts_json = json.dumps(extraction.get("texts") or [], ensure_ascii=True)
        model.tables_json = json.dumps(extraction.get("tables") or [], ensure_ascii=True)
        model.raw_stats_json = json.dumps(extraction.get("stats") or {}, ensure_ascii=True)

                status = extraction.get("status")
        if status == "needs_autodesk":
            model.status = CadJobStatus.NEEDS_AUTODESK
            document.processing_status = ProcessingStatus.COMPLETED
            model.quantities_json = json.dumps([], ensure_ascii=True)
            db.execute(delete(CadQuantity).where(CadQuantity.cad_model_id == model.id))
            db.commit()
            db.refresh(model)
            return model

                if status == "failed":
            model.status = CadJobStatus.FAILED
            model.error_message = extraction.get("error") or extraction.get("summary")
            document.processing_status = ProcessingStatus.FAILED
            document.error_message = model.error_message
            db.commit()
            db.refresh(model)
            return model

                quantities = build_quantities(extraction, source_label=document.original_filename)
        # Plugin / Design Automation may already emit quantity candidates
        for hint in extraction.get("quantities_hint") or []:
            if hint.get("description") and hint.get("quantity") is not None:
                quantities.append(dict(hint))
        settings = get_settings()
        if settings.cad_openai_enrichment and openai_configured():
            quantities = enrich_cad_quantities_with_openai(
                filename=document.original_filename,
                extraction_summary=extraction.get("summary") or "",
                stats=extraction.get("stats") or {},
                quantities=quantities,
                layers=extraction.get("layers") or [],
                blocks=extraction.get("blocks") or [],
                pipes=extraction.get("pipes") or [],
                texts=extraction.get("texts") or [],
            )
            model.engine = f"{model.engine}+openai"

                from app.services.traffic_control import consolidate_traffic_control_signs

                quantities, _tc_meta = consolidate_traffic_control_signs(
            quantities, allow_online_refresh=True
        )

                from app.services.cad.utility_stationing import build_utilities_detail

                try:
            utilities_detail = build_utilities_detail(extraction)
        except Exception as exc:  # noqa: BLE001
            utilities_detail = {
                "segments": [],
                "connections": [],
                "summary": {"error": str(exc)[:240]},
                "alignment": {"name": None, "source": "error"},
            }
        model.utilities_detail_json = json.dumps(utilities_detail, ensure_ascii=True)

                model.quantities_json = json.dumps(quantities, ensure_ascii=True)
        model.status = CadJobStatus.QUANTIFIED

                db.execute(delete(CadQuantity).where(CadQuantity.cad_model_id == model.id))
        db.flush()
        for q in quantities:
            db.add(
                CadQuantity(
                    cad_model_id=model.id,
                    item_code=q.get("item_code"),
                    description=q["description"],
                    category=q.get("category"),
                    unit=q["unit"],
                    quantity=float(q["quantity"]),
                    layer=q.get("layer"),
                    entity_type=q.get("entity_type"),
                    calculation_method=q.get("calculation_method"),
                    source_reference=q.get("source_reference"),
                    confidence=q.get("confidence"),
                )
            )

                document.processing_status = ProcessingStatus.COMPLETED
        document.error_message = None
        db.commit()
        return db.scalar(
            select(CadModel).options(selectinload(CadModel.quantities)).where(CadModel.id == model.id)
        )  # type: ignore[return-value]
    except Exception as exc:
        # Persist failure without re-raising so Analyze can return a CAD-specific error
        # instead of a generic client timeout / PDF tip message.
        model.status = CadJobStatus.FAILED
        model.error_message = str(exc)
        document.processing_status = ProcessingStatus.FAILED
        document.error_message = str(exc)
        db.commit()
        db.refresh(model)
        return model

        
        def list_project_cad_models(db: Session, project_id: int) -> list[CadModel]:
    return list(
        db.scalars(
            select(CadModel)
            .options(selectinload(CadModel.quantities))
            .where(CadModel.project_id == project_id)
            .order_by(CadModel.created_at.desc())
        ).all()
    )

### backend/app/services/cad/dxf_parser.py

SHA-256: 7e1383f8c0fc71461790f976ba80dde7640a8826177464cea84d1c1b3510e9fd    ·    279 line(s)    ·    .py

"""DXF geometry extraction via ezdxf (CAD & Civil 3D Intelligence Engine)."""

        from __future__ import annotations

        import math
import re
from pathlib import Path
from typing import Any

        
        def parse_dxf(path: Path) -> dict[str, Any]:
    import ezdxf

            doc = ezdxf.readfile(str(path))
    msp = doc.modelspace()

            layers = []
    for layer in doc.layers:
        layers.append(
            {
                "name": layer.dxf.name,
                "color": getattr(layer.dxf, "color", None),
            }
        )

            lines: list[dict[str, Any]] = []
    polylines: list[dict[str, Any]] = []
    blocks: list[dict[str, Any]] = []
    texts: list[dict[str, Any]] = []
    dimensions: list[dict[str, Any]] = []
    hatches: list[dict[str, Any]] = []
    circles: list[dict[str, Any]] = []

            def _len_points(pts: list[tuple[float, float]]) -> float:
        total = 0.0
        for i in range(1, len(pts)):
            x1, y1 = pts[i - 1]
            x2, y2 = pts[i]
            total += math.hypot(x2 - x1, y2 - y1)
        return total

            for entity in msp:
        dxftype = entity.dxftype()
        layer = getattr(entity.dxf, "layer", "0")

                if dxftype == "LINE":
            start = (float(entity.dxf.start.x), float(entity.dxf.start.y))
            end = (float(entity.dxf.end.x), float(entity.dxf.end.y))
            length = math.hypot(end[0] - start[0], end[1] - start[1])
            lines.append({"layer": layer, "length": length, "start": start, "end": end})

                elif dxftype in {"LWPOLYLINE", "POLYLINE"}:
            pts: list[tuple[float, float]] = []
            try:
                pts = [(float(p[0]), float(p[1])) for p in entity.get_points("xy")]
            except Exception:
                try:
                    pts = [(float(v.dxf.location.x), float(v.dxf.location.y)) for v in entity.vertices]  # type: ignore[attr-defined]
                except Exception:
                    pts = []
            closed = bool(getattr(entity, "closed", False))
            length = _len_points(pts)
            if closed and pts:
                length += math.hypot(pts[0][0] - pts[-1][0], pts[0][1] - pts[-1][1])
            area = None
            if closed and len(pts) >= 3:
                a = 0.0
                for i in range(len(pts)):
                    x1, y1 = pts[i]
                    x2, y2 = pts[(i + 1) % len(pts)]
                    a += x1 * y2 - x2 * y1
                area = abs(a) / 2.0
            polylines.append(
                {
                    "layer": layer,
                    "length": length,
                    "area": area,
                    "closed": closed,
                    "vertices": len(pts),
                    # Keep geometry for station projection (cap vertices)
                    "points": [[round(p[0], 4), round(p[1], 4)] for p in pts[:800]],
                    "name": layer,
                }
            )

                elif dxftype == "CIRCLE":
            r = float(entity.dxf.radius)
            center = None
            try:
                center = [float(entity.dxf.center.x), float(entity.dxf.center.y)]
            except Exception:
                center = None
            circles.append(
                {
                    "layer": layer,
                    "radius": r,
                    "center": center,
                    "circumference": 2 * math.pi * r,
                    "area": math.pi * r * r,
                }
            )

                elif dxftype == "ARC":
            r = float(entity.dxf.radius)
            start_angle = math.radians(float(entity.dxf.start_angle))
            end_angle = math.radians(float(entity.dxf.end_angle))
            sweep = end_angle - start_angle
            if sweep < 0:
                sweep += 2 * math.pi
            circles.append({"layer": layer, "radius": r, "arc_length": r * sweep, "type": "ARC"})

                elif dxftype == "INSERT":
            insert = [float(entity.dxf.insert.x), float(entity.dxf.insert.y)]
            attribs: dict[str, str] = {}
            try:
                if getattr(entity, "attribs", None):
                    for att in entity.attribs:
                        tag = str(getattr(att.dxf, "tag", "") or "")
                        val = str(getattr(att.dxf, "text", "") or "")
                        if tag:
                            attribs[tag] = val
                            attribs[tag.upper()] = val
                elif getattr(entity, "has_attrib", False):
                    for att in entity.attribs:
                        tag = str(att.dxf.tag or "")
                        val = str(att.dxf.text or "")
                        if tag:
                            attribs[tag] = val
                            attribs[tag.upper()] = val
            except Exception:
                attribs = {}
            sta = attribs.get("STATION") or attribs.get("STA") or attribs.get("STN")
            side = attribs.get("SIDE") or attribs.get("DIR")
            offset = attribs.get("OFFSET") or attribs.get("OFF")
            blocks.append(
                {
                    "layer": layer,
                    "name": entity.dxf.name,
                    "insert": insert,
                    "position": insert,
                    "attributes": attribs,
                    "station": sta,
                    "side": side,
                    "offset": offset,
                    "type": "INSERT",
                }
            )

                elif dxftype == "TEXT":
            insert = None
            try:
                insert = [float(entity.dxf.insert.x), float(entity.dxf.insert.y)]
            except Exception:
                insert = None
            texts.append({"layer": layer, "text": entity.dxf.text, "insert": insert, "space": "model"})
        elif dxftype == "MTEXT":
            insert = None
            try:
                insert = [float(entity.dxf.insert.x), float(entity.dxf.insert.y)]
            except Exception:
                insert = None
            texts.append({"layer": layer, "text": entity.text, "insert": insert, "space": "model"})

                elif "DIMENSION" in dxftype:
            measurement = None
            try:
                measurement = float(entity.get_measurement())
            except Exception:
                measurement = None
            dimensions.append({"layer": layer, "measurement": measurement})

                elif dxftype == "HATCH":
            area = None
            try:
                area = float(entity.area)
            except Exception:
                area = None
            hatches.append({"layer": layer, "area": area})

            # Paper-space sheet callouts (STA / offset labels) — content only; XY is not model
    try:
        for layout in doc.layouts:
            if str(layout.name).upper() in {"MODEL", "MODE"}:
                continue
            for entity in layout:
                dxftype = entity.dxftype()
                layer = getattr(entity.dxf, "layer", "0")
                if dxftype == "TEXT":
                    texts.append(
                        {
                            "layer": layer,
                            "text": entity.dxf.text,
                            "space": "paper",
                            "layout": layout.name,
                        }
                    )
                elif dxftype == "MTEXT":
                    texts.append(
                        {
                            "layer": layer,
                            "text": entity.text,
                            "space": "paper",
                            "layout": layout.name,
                        }
                    )
    except Exception:
        pass

            tables: list[dict[str, Any]] = []
    for block in doc.blocks:
        upper = block.name.upper()
        if any(k in upper for k in ("TABLE", "QTY", "EOQ", "BOQ", "SCHEDULE")):
            tables.append({"name": block.name, "entity_count": len(block)})

            units = None
    try:
        units = str(doc.units)
    except Exception:
        units = str(doc.header.get("$INSUNITS", "unknown"))

            stats = {
        "layer_count": len(layers),
        "line_count": len(lines),
        "polyline_count": len(polylines),
        "block_insert_count": len(blocks),
        "text_count": len(texts),
        "dimension_count": len(dimensions),
        "hatch_count": len(hatches),
        "circle_arc_count": len(circles),
        "total_line_length": round(sum(i["length"] for i in lines), 3),
        "total_polyline_length": round(sum(i["length"] for i in polylines), 3),
    }

            # Promote centerline-like polylines to alignments (for stationing)
    alignments: list[dict[str, Any]] = []
    for pl in polylines:
        layer = str(pl.get("layer") or "")
        low = layer.lower().replace("_", " ")
        if (
            "centerline" in low
            or "centreline" in low
            or "p cl" in low
            or re.search(r"(?:^|[^a-z0-9])cl(?:[^a-z0-9]|$)", low)
            or layer.lower().startswith("p_cl")
            or "_cl" in layer.lower()
        ):
            alignments.append(
                {
                    "name": layer,
                    "layer": layer,
                    "length": pl.get("length"),
                    "points": pl.get("points") or [],
                    "sta_start": "0+00",
                }
            )

            return {
        "format": "dxf",
        "engine": "ezdxf",
        "status": "extracted",
        "units": units,
        "layers": layers,
        "lines": lines[:5000],
        "polylines": polylines[:5000],
        "blocks": blocks[:5000],
        "texts": texts[:2000],
        "dimensions": dimensions[:2000],
        "tables": tables,
        "hatches": hatches[:2000],
        "circles": circles[:2000],
        "alignments": alignments[:200],
        "stats": stats,
        "summary": (
            f"DXF parsed: {stats['layer_count']} layers, {stats['line_count']} lines, "
            f"{stats['polyline_count']} polylines, {stats['block_insert_count']} blocks, "
            f"{stats['dimension_count']} dimensions, {stats['text_count']} text entities, "
            f"{len(alignments)} CL/alignment candidates."
        ),
    }

### backend/app/services/cad/landxml_parser.py

SHA-256: 81c1d6cc50d729de624e0104aecae0927506e1119b32db824b9bdb993f7ea534    ·    220 line(s)    ·    .py

"""LandXML extraction for alignments, surfaces, pipes, and cross-sections."""

        from __future__ import annotations

        from pathlib import Path
from typing import Any
from xml.etree import ElementTree as ET

        
        def _local(tag: str) -> str:
    if "}" in tag:
        return tag.rsplit("}", 1)[-1]
    return tag

        
        def parse_landxml(path: Path) -> dict[str, Any]:
    tree = ET.parse(path)
    root = tree.getroot()

            alignments: list[dict[str, Any]] = []
    surfaces: list[dict[str, Any]] = []
    pipes: list[dict[str, Any]] = []
    structures: list[dict[str, Any]] = []
    cross_sections: list[dict[str, Any]] = []
    texts: list[dict[str, Any]] = []
    layers: list[dict[str, Any]] = []
    volumes: list[dict[str, Any]] = []

            def _fattr(elem: ET.Element, *keys: str) -> float | None:
        for key in keys:
            raw = elem.attrib.get(key)
            if raw is None:
                continue
            try:
                return float(raw)
            except ValueError:
                continue
        return None

            for elem in root.iter():
        name = _local(elem.tag)

                if name == "Alignment":
            length = elem.attrib.get("length") or elem.attrib.get("Length")
            pts: list[list[float]] = []
            for child in elem.iter():
                cn = _local(child.tag)
                if cn not in {"Start", "End", "PI", "Center"}:
                    continue
                raw = (child.text or "").strip()
                if not raw:
                    continue
                try:
                    nums = [float(x) for x in raw.replace(",", " ").split()]
                except ValueError:
                    continue
                if len(nums) >= 2:
                    pts.append([nums[0], nums[1]])
            # Dedup consecutive identical vertices
            uniq_pts: list[list[float]] = []
            for p in pts:
                if not uniq_pts or abs(p[0] - uniq_pts[-1][0]) > 1e-6 or abs(p[1] - uniq_pts[-1][1]) > 1e-6:
                    uniq_pts.append(p)
            alignments.append(
                {
                    "name": elem.attrib.get("name") or elem.attrib.get("Name"),
                    "length": float(length) if length else None,
                    "sta_start": elem.attrib.get("staStart") or elem.attrib.get("StaStart"),
                    "points": uniq_pts,
                }
            )
            layers.append({"name": f"Alignment:{alignments[-1]['name']}"})

                elif name in {"Surface", "SurfaceSource"}:
            surf = {
                "name": elem.attrib.get("name") or elem.attrib.get("Name"),
                "type": elem.attrib.get("desc") or elem.attrib.get("surfaceType"),
            }
            # Pull cut/fill from child Volume / Volume2D if present
            for child in elem:
                cname = _local(child.tag)
                if cname in {"Volume", "Volume2D", "SurfaceVolume"}:
                    cut = _fattr(child, "cut", "Cut", "cutVol", "earthworkCut")
                    fill = _fattr(child, "fill", "Fill", "fillVol", "earthworkFill")
                    net = _fattr(child, "net", "Net", "vol", "volume")
                    if cut is not None:
                        surf["cut"] = cut
                    if fill is not None:
                        surf["fill"] = fill
                    if net is not None:
                        surf["net"] = net
            surfaces.append(surf)
            layers.append({"name": f"Surface:{surfaces[-1]['name']}"})

                elif name == "Pipe":
            length = elem.attrib.get("length") or elem.attrib.get("Length")
            diam_raw = elem.attrib.get("oDiam") or elem.attrib.get("diam") or elem.attrib.get("Diameter")
            try:
                diam_f = float(diam_raw) if diam_raw else None
            except ValueError:
                diam_f = None
            pipes.append(
                {
                    "name": elem.attrib.get("name") or elem.attrib.get("Name"),
                    "length": float(length) if length else None,
                    "diameter": diam_f,
                    "part_size": diam_raw,
                    "material": elem.attrib.get("material") or elem.attrib.get("Material"),
                    "network": elem.attrib.get("refPipeNetwork") or elem.attrib.get("network"),
                }
            )

                elif name in {"Struct", "Structure"}:
            rec: dict[str, Any] = {
                "name": elem.attrib.get("name") or elem.attrib.get("Name"),
                "type": elem.attrib.get("desc") or elem.attrib.get("Type") or "Structure",
                "station": elem.attrib.get("sta") or elem.attrib.get("station") or elem.attrib.get("Sta"),
                "offset": elem.attrib.get("offset") or elem.attrib.get("Offset"),
            }
            for child in list(elem) + [elem]:
                cn = _local(child.tag)
                if cn not in {"Center", "Location", "Point", "CoordGeom"}:
                    continue
                raw = (child.text or "").strip()
                if not raw:
                    continue
                try:
                    nums = [float(x) for x in raw.replace(",", " ").split()]
                except ValueError:
                    continue
                if len(nums) >= 2:
                    rec["insert"] = [nums[0], nums[1]]
                    rec["position"] = rec["insert"]
                    rec["easting"] = nums[0]
                    rec["northing"] = nums[1]
                    break
            structures.append(rec)

                elif name in {"Volume", "Volume2D", "SurfaceVolume"}:
            cut = _fattr(elem, "cut", "Cut", "cutVol", "earthworkCut")
            fill = _fattr(elem, "fill", "Fill", "fillVol", "earthworkFill")
            net = _fattr(elem, "net", "Net", "vol", "volume")
            label = elem.attrib.get("name") or elem.attrib.get("desc") or "Volume"
            if cut is not None:
                volumes.append({"type": "cut", "name": label, "quantity": cut, "method": "LandXML Volume"})
            if fill is not None:
                volumes.append({"type": "fill", "name": label, "quantity": fill, "method": "LandXML Volume"})
            if cut is None and fill is None and net is not None:
                volumes.append({"type": "net", "name": label, "quantity": net, "method": "LandXML Volume"})

                elif name in {"CrossSect", "CrossSection"}:
            cross_sections.append(
                {
                    "sta": elem.attrib.get("sta") or elem.attrib.get("Sta"),
                    "name": elem.attrib.get("name") or elem.attrib.get("Name"),
                }
            )

                elif name in {"Feature", "Parcel"}:
            texts.append({"text": elem.attrib.get("name") or elem.attrib.get("Name") or name, "layer": name})

            seen: set[str] = set()
    uniq_layers = []
    for layer in layers:
        if layer["name"] not in seen and layer["name"]:
            seen.add(layer["name"])
            uniq_layers.append(layer)

            total_align = round(sum(a["length"] or 0 for a in alignments), 3)
    total_pipe = round(sum(p["length"] or 0 for p in pipes), 3)

            stats = {
        "alignment_count": len(alignments),
        "surface_count": len(surfaces),
        "pipe_count": len(pipes),
        "structure_count": len(structures),
        "cross_section_count": len(cross_sections),
        "volume_count": len(volumes),
        "total_alignment_length": total_align,
        "total_pipe_length": total_pipe,
    }

            return {
        "format": "landxml",
        "engine": "autovad_landxml",
        "units": root.attrib.get("units") or "unknown",
        "layers": uniq_layers,
        "lines": [],
        "polylines": [],
        "blocks": [
            {
                "name": s.get("name") or s.get("type") or "Structure",
                "layer": str(s.get("type") or "Structure"),
                "type": s.get("type") or "Structure",
                "description": s.get("type"),
                "insert": s.get("insert"),
                "position": s.get("position") or s.get("insert"),
                "station": s.get("station"),
                "offset": s.get("offset"),
                "easting": s.get("easting"),
                "northing": s.get("northing"),
            }
            for s in structures
        ],
        "texts": texts[:2000],
        "dimensions": [],
        "tables": [],
        "alignments": alignments,
        "surfaces": surfaces,
        "pipes": pipes,
        "volumes": volumes,
        "cross_sections": cross_sections,
        "stats": stats,
        "summary": (
            f"LandXML parsed: {stats['alignment_count']} alignments ({total_align} length units), "
            f"{stats['surface_count']} surfaces, {stats['pipe_count']} pipes ({total_pipe}), "
            f"{stats['structure_count']} structures, {stats['volume_count']} volumes, "
            f"{stats['cross_section_count']} cross-sections."
        ),
    }

### backend/app/services/cad/aps_client.py

SHA-256: ae5bb6d18c94a5d4bcd8879f2be6f14bfada47ed7ffcc333c6196f00d4de7354    ·    1081 line(s)    ·    .py

"""Autodesk Platform Services (APS) client for DWG / Civil 3D Model Derivative."""

        from __future__ import annotations

        import base64
import re
import time
import uuid
from pathlib import Path
from typing import Any

        import httpx

        from app.config import get_settings
from app.services.cad.civil_location import (
    location_from_property_bag,
    point_from_property_bag,
    useful_property_attr,
)

        APS_BASE = "https://developer.api.autodesk.com"

        
        class AutodeskAPSError(RuntimeError):
    pass

        
        def aps_configured() -> bool:
    settings = get_settings()
    return bool((settings.autodesk_client_id or "").strip() and (settings.autodesk_client_secret or "").strip())

        
        def aps_status() -> dict[str, Any]:
    settings = get_settings()
    return {
        "configured": aps_configured(),
        "bucket": _bucket_key(),
        "client_id_set": bool((settings.autodesk_client_id or "").strip()),
    }

        
        def _bucket_key() -> str:
    settings = get_settings()
    raw = (settings.autodesk_bucket_key or f"autovad{settings.app_env}{settings.autodesk_client_id or 'local'}").lower()
    # APS bucket keys: lowercase letters, numbers, `-`
    cleaned = re.sub(r"[^a-z0-9\-]", "", raw)[:128]
    return cleaned or "autovadlocal"

        
        def get_access_token() -> str:
    settings = get_settings()
    if not aps_configured():
        raise AutodeskAPSError("Autodesk APS credentials are not configured")

            data = {
        "grant_type": "client_credentials",
        # Include code:all so the same token can call Design Automation when needed
        "scope": "data:read data:write data:create bucket:create bucket:read viewables:read code:all",
    }
    with httpx.Client(timeout=60.0) as client:
        resp = client.post(
            f"{APS_BASE}/authentication/v2/token",
            data=data,
            auth=(settings.autodesk_client_id.strip(), settings.autodesk_client_secret.strip()),
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS auth failed ({resp.status_code}): {resp.text[:400]}")
        token = resp.json().get("access_token")
        if not token:
            raise AutodeskAPSError("APS auth response missing access_token")
        return token

        
        def ensure_bucket(token: str) -> str:
    bucket = _bucket_key()
    headers = {"Authorization": f"Bearer {token}", "Content-Type": "application/json"}
    with httpx.Client(timeout=60.0) as client:
        # Check existing
        check = client.get(f"{APS_BASE}/oss/v2/buckets/{bucket}/details", headers=headers)
        if check.status_code == 200:
            return bucket

                create = client.post(
            f"{APS_BASE}/oss/v2/buckets",
            headers=headers,
            json={"bucketKey": bucket, "policyKey": "temporary"},
        )
        if create.status_code in {200, 409}:
            return bucket
        if create.status_code >= 400:
            raise AutodeskAPSError(f"APS bucket create failed ({create.status_code}): {create.text[:400]}")
        return bucket

        
        def upload_object(token: str, bucket: str, path: Path) -> dict[str, Any]:
    """Upload via Autodesk OSS Direct-to-S3 (legacy PUT object is deprecated)."""
    from urllib.parse import quote

            object_key = f"{uuid.uuid4().hex}_{re.sub(r'[^A-Za-z0-9._-]', '_', path.name)}"
    data = path.read_bytes()
    size = len(data)
    # Non-final parts must be at least 5 MiB for multipart uploads.
    chunk_size = 5 * 1024 * 1024
    parts = max(1, (size + chunk_size - 1) // chunk_size)
    encoded_key = quote(object_key, safe="")
    auth_headers = {"Authorization": f"Bearer {token}"}

            with httpx.Client(timeout=3600.0) as client:
        upload_key: str | None = None
        urls: list[str] = []
        first_part = 1
        while first_part <= parts:
            batch = min(25, parts - first_part + 1)
            params = {
                "parts": str(batch),
                "firstPart": str(first_part),
                "minutesExpiration": "60",
            }
            if upload_key:
                params["uploadKey"] = upload_key
            signed = client.get(
                f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{encoded_key}/signeds3upload",
                headers=auth_headers,
                params=params,
            )
            if signed.status_code >= 400:
                raise AutodeskAPSError(
                    f"APS signed upload URL failed ({signed.status_code}): {signed.text[:400]}"
                )
            body = signed.json()
            upload_key = body.get("uploadKey") or upload_key
            batch_urls = body.get("urls") or []
            if not batch_urls:
                raise AutodeskAPSError("APS signed upload response missing urls")
            urls.extend(batch_urls)
            first_part += batch

                if not upload_key:
            raise AutodeskAPSError("APS signed upload response missing uploadKey")
        if len(urls) < parts:
            raise AutodeskAPSError(f"APS returned {len(urls)} upload URLs for {parts} parts")

                for index, url in enumerate(urls[:parts]):
            start = index * chunk_size
            end = min(start + chunk_size, size)
            put = client.put(url, content=data[start:end])
            # Retry once on transient S3 responses
            if put.status_code in {429} or 500 <= put.status_code <= 599:
                time.sleep(1.5)
                put = client.put(url, content=data[start:end])
            if put.status_code == 403:
                raise AutodeskAPSError(
                    "APS S3 upload URL expired or was rejected (403). Retry CAD processing."
                )
            if put.status_code >= 400:
                raise AutodeskAPSError(
                    f"APS S3 part upload failed ({put.status_code}): {put.text[:400]}"
                )

                complete = client.post(
            f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{encoded_key}/signeds3upload",
            headers={**auth_headers, "Content-Type": "application/json"},
            json={"uploadKey": upload_key},
        )
        if complete.status_code >= 400:
            raise AutodeskAPSError(
                f"APS complete upload failed ({complete.status_code}): {complete.text[:400]}"
            )
        body = complete.json() if complete.content else {}
        object_id = body.get("objectId") or f"urn:adsk.objects:os.object:{bucket}/{object_key}"
        return {
            "bucket": bucket,
            "object_key": object_key,
            "object_id": object_id,
            "size": body.get("size") or size,
        }

        
        def to_urn(object_id: str) -> str:
    # URL-safe base64 without padding
    encoded = base64.urlsafe_b64encode(object_id.encode("utf-8")).decode("utf-8")
    return encoded.rstrip("=")

        
        def start_translation(token: str, urn: str) -> dict[str, Any]:
    headers = {
        "Authorization": f"Bearer {token}",
        "Content-Type": "application/json",
        "x-ads-force": "true",
    }
    payload = {
        "input": {"urn": urn},
        "output": {
            "formats": [
                {"type": "svf2", "views": ["2d", "3d"]},
            ]
        },
    }
    with httpx.Client(timeout=120.0) as client:
        resp = client.post(
            f"{APS_BASE}/modelderivative/v2/designdata/job",
            headers=headers,
            json=payload,
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS translation job failed ({resp.status_code}): {resp.text[:500]}")
        return resp.json()

        
        def wait_for_manifest(token: str, urn: str, *, timeout_s: int = 3600, interval_s: int = 5) -> dict[str, Any]:
    headers = {"Authorization": f"Bearer {token}"}
    deadline = time.time() + timeout_s
    last: dict[str, Any] = {}
    with httpx.Client(timeout=60.0) as client:
        while time.time() < deadline:
            resp = client.get(
                f"{APS_BASE}/modelderivative/v2/designdata/{urn}/manifest",
                headers=headers,
            )
            if resp.status_code == 200:
                last = resp.json()
                status = (last.get("status") or "").lower()
                if status == "success":
                    return last
                if status in {"failed", "timeout"}:
                    raise AutodeskAPSError(f"APS translation {status}: {last.get('progress')}")
            elif resp.status_code not in {404, 202}:
                raise AutodeskAPSError(f"APS manifest error ({resp.status_code}): {resp.text[:400]}")
            time.sleep(interval_s)
    raise AutodeskAPSError(f"APS translation timed out after {timeout_s}s")

        
        def fetch_metadata_guids(token: str, urn: str) -> list[dict[str, Any]]:
    headers = {"Authorization": f"Bearer {token}"}
    with httpx.Client(timeout=60.0) as client:
        resp = client.get(
            f"{APS_BASE}/modelderivative/v2/designdata/{urn}/metadata",
            headers=headers,
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS metadata failed ({resp.status_code}): {resp.text[:400]}")
        data = resp.json().get("data") or {}
        return data.get("metadata") or []

        
        def fetch_properties(token: str, urn: str, guid: str) -> dict[str, Any]:
    headers = {"Authorization": f"Bearer {token}"}
    with httpx.Client(timeout=120.0) as client:
        resp = client.get(
            f"{APS_BASE}/modelderivative/v2/designdata/{urn}/metadata/{guid}/properties",
            headers=headers,
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS properties failed ({resp.status_code}): {resp.text[:400]}")
        return resp.json()

        
        def fetch_object_tree(token: str, urn: str, guid: str) -> dict[str, Any]:
    headers = {"Authorization": f"Bearer {token}"}
    with httpx.Client(timeout=120.0) as client:
        resp = client.get(
            f"{APS_BASE}/modelderivative/v2/designdata/{urn}/metadata/{guid}",
            headers=headers,
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS object tree failed ({resp.status_code}): {resp.text[:400]}")
        return resp.json()

        
        def map_aps_properties_to_extraction(properties_payload: dict[str, Any], *, filename: str) -> dict[str, Any]:
    """Map APS REST property collection into AutoVAD CAD extraction schema."""
    collection = ((properties_payload.get("data") or {}).get("collection")) or []

            layers: dict[str, dict[str, Any]] = {}
    lines: list[dict[str, Any]] = []
    polylines: list[dict[str, Any]] = []
    blocks: list[dict[str, Any]] = []
    texts: list[dict[str, Any]] = []
    dimensions: list[dict[str, Any]] = []
    hatches: list[dict[str, Any]] = []
    circles: list[dict[str, Any]] = []
    pipes: list[dict[str, Any]] = []
    alignments: list[dict[str, Any]] = []
    surfaces: list[dict[str, Any]] = []
    volumes: list[dict[str, Any]] = []

            for obj in collection[:8000]:
        props = obj.get("properties") or {}
        name = obj.get("name") or props.get("Name") or "Object"
        flat: dict[str, Any] = {}
        for group, values in props.items():
            if isinstance(values, dict):
                for k, v in values.items():
                    flat[str(k)] = v
            else:
                flat[str(group)] = values

                layer = str(
            flat.get("Layer")
            or flat.get("layer")
            or flat.get("Layer Name")
            or flat.get("Level")
            or "0"
        )
        layers[layer] = {"name": layer}

                entity = str(flat.get("Entity") or flat.get("Type") or flat.get("Category") or name)
        length = _to_float(flat.get("Length") or flat.get("length") or flat.get("Perimeter") or flat.get("3D Length"))
        area = _to_float(flat.get("Area") or flat.get("area"))
        radius = _to_float(flat.get("Radius") or flat.get("radius") or flat.get("Pipe Radius"))
        diameter = _to_float(
            flat.get("Diameter")
            or flat.get("Inner Pipe Diameter")
            or flat.get("Outer Pipe Diameter")
            or flat.get("Nominal Diameter")
        )
        if diameter is not None and 0 < diameter < 8:
            diameter = diameter * 12.0
        part_size = flat.get("Part Size Name") or flat.get("Part Size") or flat.get("Description")
        network = flat.get("Network Name") or flat.get("Pipe Network Name") or flat.get("Network")
        cut_vol = _to_float(flat.get("Cut") or flat.get("Cut Volume"))
        fill_vol = _to_float(flat.get("Fill") or flat.get("Fill Volume"))

                lower_entity = entity.lower()
        blob = f"{entity} {name} {layer} {part_size or ''} {network or ''}".lower()

                if cut_vol or fill_vol:
            surfaces.append({"name": name, "cut": cut_vol, "fill": fill_vol})
            if cut_vol:
                volumes.append({"type": "cut", "quantity": cut_vol, "name": name})
            if fill_vol:
                volumes.append({"type": "fill", "quantity": fill_vol, "name": name})

                if "pipe" in lower_entity or "pipe" in blob:
            loc_meta = _station_side_offset_from_props(flat)
            pipe_row = {
                "name": name,
                "layer": layer,
                "length": length,
                "radius": radius,
                "diameter": diameter,
                "part_size": part_size,
                "network": network,
                "description": part_size or name,
                "sta_start": flat.get("Start Station") or flat.get("rawStartStation") or loc_meta.get("station"),
                "sta_end": flat.get("End Station") or flat.get("rawEndStation"),
                "offset": loc_meta.get("offset"),
                "offset_ft": loc_meta.get("offset"),
                "side": loc_meta.get("side"),
            }
            pt = _point_from_cad_props(flat)
            if pt:
                pipe_row["start"] = pt
            if length or pipe_row.get("sta_start") or pipe_row.get("sta_end"):
                pipes.append(pipe_row)
            continue

                if "alignment" in lower_entity or "alignment" in blob.lower() or "p_cl" in blob:
            loc_meta = _station_side_offset_from_props(flat)
            alignments.append(
                {
                    "name": name or layer,
                    "layer": layer,
                    "length": length,
                    "sta_start": flat.get("Start Station") or flat.get("Starting Station") or loc_meta.get("station"),
                    "points": (
                        [_point_from_cad_props(flat)]
                        if _point_from_cad_props(flat)
                        else []
                    ),
                }
            )
            continue

                if any(k in blob for k in ("structure", "manhole", "inlet", "catch", "valve", "bend", "tee", "hydrant")):
            blocks.append(
                {
                    "name": str(part_size or name),
                    "layer": layer,
                    "type": entity,
                    "description": part_size or name,
                    "size": diameter,
                    **_civil_location_fields(flat),
                }
            )
        elif "block" in lower_entity or "insert" in lower_entity or "ref" in lower_entity:
            blocks.append(
                {
                    "name": name,
                    "layer": layer,
                    "type": entity,
                    "description": part_size,
                    **_civil_location_fields(flat),
                }
            )
        elif "text" in lower_entity or "mtext" in lower_entity:
            texts.append(
                {
                    "layer": layer,
                    "text": str(flat.get("Contents") or flat.get("textContent") or name),
                    "insert": _point_from_cad_props(flat),
                }
            )
        elif "dimension" in lower_entity or "dim" in lower_entity:
            dimensions.append({"layer": layer, "measurement": length, "text": name})
        elif "hatch" in lower_entity:
            hatches.append({"layer": layer, "area": area})
        elif "circle" in lower_entity or "arc" in lower_entity:
            circles.append(
                {
                    "layer": layer,
                    "radius": radius,
                    "arc_length": length,
                    "area": area,
                    "type": entity,
                }
            )
        elif length is not None and (area is None or "line" in lower_entity or "polyline" in lower_entity):
            if "poly" in lower_entity:
                polylines.append({"layer": layer, "length": length, "area": area, "closed": bool(area)})
            else:
                lines.append({"layer": layer, "length": length})
        elif area is not None:
            hatches.append({"layer": layer, "area": area})
        else:
            blocks.append(
                {
                    "name": name,
                    "layer": layer,
                    "type": entity,
                    **_civil_location_fields(flat),
                }
            )

            stats = {
        "aps_objects": len(collection),
        "layer_count": len(layers),
        "line_count": len(lines),
        "polyline_count": len(polylines),
        "block_insert_count": len(blocks),
        "text_count": len(texts),
        "dimension_count": len(dimensions),
        "hatch_count": len(hatches),
        "circle_arc_count": len(circles),
        "pipe_count": len(pipes),
        "surface_count": len(surfaces),
    }
    return {
        "format": "dwg",
        "engine": "autodesk_aps",
        "status": "extracted",
        "units": None,
        "layers": list(layers.values()),
        "lines": lines[:5000],
        "polylines": polylines[:5000],
        "blocks": blocks[:5000],
        "texts": texts[:2000],
        "dimensions": dimensions[:2000],
        "tables": [],
        "hatches": hatches[:2000],
        "circles": circles[:2000],
        "pipes": pipes[:5000],
        "alignments": alignments[:2000],
        "surfaces": surfaces[:500],
        "volumes": volumes[:500],
        "stats": stats,
        "summary": (
            f"APS DWG processed '{filename}': {stats['aps_objects']} objects, "
            f"{stats['layer_count']} layers, {stats['pipe_count']} pipes, "
            f"{stats['block_insert_count']} blocks/structures."
        ),
        "aps": {"configured": True, "urn_processed": True},
    }

        
        def find_properties_db_urn(manifest: dict[str, Any]) -> str | None:
    def walk(node: Any) -> str | None:
        if isinstance(node, dict):
            if node.get("role") == "Autodesk.CloudPlatform.PropertyDatabase" and node.get("urn"):
                return str(node["urn"])
            for child in node.get("children") or []:
                found = walk(child)
                if found:
                    return found
        elif isinstance(node, list):
            for item in node:
                found = walk(item)
                if found:
                    return found
        return None

            return walk(manifest.get("derivatives") or [])

        
        def download_derivative(token: str, derivative_urn: str) -> bytes:
    from urllib.parse import quote

            url = (
        "https://developer.api.autodesk.com/derivativeservice/v2/derivatives/"
        f"{quote(derivative_urn, safe='')}"
    )
    with httpx.Client(timeout=180.0, follow_redirects=True) as client:
        resp = client.get(url, headers={"Authorization": f"Bearer {token}"})
        if resp.status_code >= 400:
            raise AutodeskAPSError(
                f"APS derivative download failed ({resp.status_code}): {resp.text[:400]}"
            )
        return resp.content

        
        def parse_properties_db(db_path: Path, *, filename: str) -> dict[str, Any]:
    """Parse Autodesk SVF properties.db (SQLite) into CAD extraction schema.

            The Model Derivative REST properties endpoint is often empty for 2D DWG /
    Civil plan-profile sheets, while properties.db still contains layers,
    lengths, pipes, structures, and areas.
    """
    import sqlite3

            layers: dict[str, dict[str, Any]] = {}
    lines: list[dict[str, Any]] = []
    polylines: list[dict[str, Any]] = []
    blocks: list[dict[str, Any]] = []
    texts: list[dict[str, Any]] = []
    dimensions: list[dict[str, Any]] = []
    hatches: list[dict[str, Any]] = []
    circles: list[dict[str, Any]] = []
    pipes: list[dict[str, Any]] = []
    alignments: list[dict[str, Any]] = []
    surfaces: list[dict[str, Any]] = []
    volumes: list[dict[str, Any]] = []

            con = sqlite3.connect(str(db_path))
    cur = con.cursor()
    attrs = cur.execute(
        "SELECT id, name, category, display_name FROM _objects_attr"
    ).fetchall()
    useful_ids: set[int] = set()
    for attr_id, name, category, display_name in attrs:
        label = f"{name or ''} {category or ''} {display_name or ''}"
        if useful_property_attr(label):
            useful_ids.add(int(attr_id))

            if not useful_ids:
        con.close()
        return map_aps_properties_to_extraction({"data": {"collection": []}}, filename=filename)

            placeholders = ",".join("?" for _ in useful_ids)
    rows = cur.execute(
        f"""
        SELECT e.entity_id, a.name, a.category, a.display_name, v.value
        FROM _objects_eav e
        JOIN _objects_attr a ON a.id = e.attribute_id
        JOIN _objects_val v ON v.id = e.value_id
        WHERE e.attribute_id IN ({placeholders})
        """,
        tuple(useful_ids),
    ).fetchall()
    con.close()

            by_entity: dict[int, dict[str, Any]] = {}
    for entity_id, name, category, display_name, value in rows:
        props = by_entity.setdefault(int(entity_id), {})
        key = str(name or "").strip()
        if not key:
            key = str(display_name or "").strip()
        if not key:
            continue
        # Prefer Geometry-category values when duplicates exist
        cat = str(category or "")
        if key in props and cat and "geometry" not in cat.lower() and "general" not in cat.lower():
            continue
        props[key] = value
        disp = str(display_name or "").strip()
        if disp and disp not in props:
            props[disp] = value

            for props in by_entity.values():
        entity = str(props.get("type") or props.get("General Object Type") or props.get("Name ") or "")
        name = str(
            props.get("Name")
            or props.get("name")
            or props.get("Part Size Name")
            or props.get("Alignment Name")
            or entity
            or "Object"
        )
        layer = str(props.get("Layer") or props.get("Layer name") or "0")
        layers[layer] = {"name": layer}

                length = _first_float(
            props,
            (
                "2D Length",
                "3D Length",
                "Pipe Length 3D",
                "General Overall Length",
                "Length",
                "Segment Length",
                "Parcel Perimeter",
                "Tangent Horizontal Length",
            ),
        )
        area = _first_float(
            props,
            ("Area", "Parcel Area", "2D surface area", "2D surface area "),
        )
        radius = _first_float(props, ("Pipe Radius", "Major radius", "Radius"))
        diameter = _first_float(
            props,
            (
                "Inner Pipe Diameter",
                "Outer Pipe Diameter",
                "Pipe Diameter",
                "Nominal Diameter",
                "Diameter",
                "Inside Diameter",
                "Outside Diameter",
            ),
        )
        # Normalize diameter to inches when value looks like feet
        if diameter is not None and 0 < diameter < 8:
            diameter = diameter * 12.0
        part_size = (
            props.get("Part Size Name")
            or props.get("Part Size")
            or props.get("Size Name")
            or props.get("Description")
        )
        network = (
            props.get("Network Name")
            or props.get("Pipe Network Name")
            or props.get("Network")
            or props.get("System Name")
        )
        material = props.get("Material") or props.get("Pipe Material")
        text_value = props.get("Contents") or props.get("textContent") or props.get("Text override")
        measurement = _to_float(props.get("Measurement"))
        cut_vol = _first_float(props, ("Cut", "Cut Volume", "Cut volume", "Volume Cut"))
        fill_vol = _first_float(props, ("Fill", "Fill Volume", "Fill volume", "Volume Fill"))
        net_vol = _first_float(props, ("Net", "Net Volume", "Net volume", "Composite Volume"))

                lower_entity = entity.lower()
        lower_name = name.lower()
        lower_layer = layer.lower()
        blob = " ".join(str(x) for x in (entity, name, layer, part_size, network, material) if x).lower()

                if cut_vol or fill_vol or net_vol:
            surfaces.append(
                {
                    "name": name or layer or "Surface",
                    "cut": cut_vol,
                    "fill": fill_vol,
                    "net": net_vol,
                    "type": entity or "Surface",
                }
            )
            if cut_vol:
                volumes.append({"type": "cut", "quantity": cut_vol, "name": name})
            if fill_vol:
                volumes.append({"type": "fill", "quantity": fill_vol, "name": name})

                if (
            "pipe" in lower_entity
            or "pipe" in lower_name
            or "aeccdbpipe" in lower_entity
            or "pipe network" in blob
        ):
            loc_meta = _station_side_offset_from_props(props)
            sta_start = (
                props.get("Start Station")
                or props.get("RefStart")
                or props.get("Start Sta")
                or props.get("Pipe Start Station")
                or props.get("rawStartStation")
                or props.get("StartStation")
                or props.get("Station at Start")
            )
            sta_end = (
                props.get("End Station")
                or props.get("RefEnd")
                or props.get("End Sta")
                or props.get("Pipe End Station")
                or props.get("rawEndStation")
                or props.get("EndStation")
                or props.get("Station at End")
            )
            start_x = _first_float(
                props, ("Start Easting", "Start X", "Easting at Start", "Start Point X", "Position X")
            )
            start_y = _first_float(
                props, ("Start Northing", "Start Y", "Northing at Start", "Start Point Y", "Position Y")
            )
            end_x = _first_float(
                props, ("End Easting", "End X", "Easting at End", "End Point X")
            )
            end_y = _first_float(
                props, ("End Northing", "End Y", "Northing at End", "End Point Y")
            )
            offset = loc_meta.get("offset")
            if offset is None:
                offset = _first_float(
                    props,
                    ("Offset", "2D Offset", "Horizontal Offset", "Offset From Alignment", "Start Offset", "rawOffset"),
                )
            side = loc_meta.get("side") or props.get("Side") or props.get("Offset Side") or props.get("Direction")
            start_struct = (
                props.get("Start Structure")
                or props.get("Start Structure Name")
                or props.get("Structure Start")
                or props.get("Connected Structure Start")
                or props.get("StartStructure")
            )
            end_struct = (
                props.get("End Structure")
                or props.get("End Structure Name")
                or props.get("Structure End")
                or props.get("Connected Structure End")
                or props.get("EndStructure")
            )
            pipe_row = {
                "name": name,
                "layer": layer,
                "length": length,
                "radius": radius,
                "diameter": diameter,
                "part_size": part_size,
                "network": network,
                "material": material,
                "description": part_size or name,
                "sta_start": sta_start,
                "sta_end": sta_end,
                "start_station": sta_start,
                "end_station": sta_end,
                "start_structure": start_struct,
                "end_structure": end_struct,
                "offset": offset,
                "offset_ft": offset,
                "side": side,
            }
            if start_x is not None and start_y is not None:
                pipe_row["start"] = [start_x, start_y]
            if end_x is not None and end_y is not None:
                pipe_row["end"] = [end_x, end_y]
            if pipe_row.get("start") and pipe_row.get("end"):
                pipe_row["points"] = [pipe_row["start"], pipe_row["end"]]
            if length or sta_start or sta_end or pipe_row.get("points"):
                pipes.append(pipe_row)
            else:
                blocks.append(
                    {
                        "name": str(part_size or name),
                        "layer": layer,
                        "type": entity or "Pipe",
                        "size": diameter,
                        "description": part_size,
                        "insert": _point_from_cad_props(props),
                        "position": _point_from_cad_props(props),
                        **{k: v for k, v in _station_side_offset_from_props(props).items() if v is not None},
                    }
                )
            continue

                if (
            "structure" in lower_entity
            or "manhole" in lower_name
            or "catch" in lower_name
            or "inlet" in lower_name
            or "aeccdbstructure" in lower_entity
            or ("structure" in blob and "infrastructure" not in blob)
        ):
            blocks.append(
                {
                    "name": str(part_size or name),
                    "layer": layer or str(network or "Structure"),
                    "type": entity or "Structure",
                    "description": part_size or name,
                    "size": diameter,
                    **_civil_location_fields(props),
                }
            )
            continue

                # Fittings often appear as Civil parts / blocks with size in the name
        if any(k in blob for k in ("valve", "bend", "elbow", "tee", "reducer", "hydrant", "fitting", "wye")):
            blocks.append(
                {
                    "name": str(part_size or name),
                    "layer": layer or str(network or ""),
                    "type": entity or "Fitting",
                    "description": part_size or name,
                    "size": diameter,
                    **_civil_location_fields(props),
                }
            )
            continue

                if "alignment" in lower_entity or "alignment" in lower_name or re.search(
            r"(?:^|[^a-z0-9])cl(?:[^a-z0-9]|$)", lower_name.replace("_", " ")
        ) or "p_cl" in lower_name or "p_cl" in lower_layer or "_cl" in lower_layer:
            sta_start = (
                props.get("Start Station")
                or props.get("StaStart")
                or props.get("Starting Station")
                or props.get("rawStation")
            )
            align_row: dict[str, Any] = {
                "name": name or layer,
                "layer": layer,
                "length": length,
                "sta_start": sta_start,
            }
            # Some Civil derivatives expose alignment sample points / start-end
            sx = _first_float(props, ("Start Easting", "Start X", "Easting"))
            sy = _first_float(props, ("Start Northing", "Start Y", "Northing"))
            ex = _first_float(props, ("End Easting", "End X"))
            ey = _first_float(props, ("End Northing", "End Y"))
            if sx is not None and sy is not None and ex is not None and ey is not None:
                align_row["points"] = [[sx, sy], [ex, ey]]
            if length or align_row.get("points") or "cl" in lower_name.replace("_", " ") or "cl" in lower_layer.replace("_", " "):
                alignments.append(align_row)
            continue

                if "surface" in lower_entity or "tin" in lower_entity or "tin surface" in blob:
            surfaces.append(
                {
                    "name": name,
                    "type": entity or "Surface",
                    "cut": cut_vol,
                    "fill": fill_vol,
                    "net": net_vol,
                }
            )
            continue

                if "text" in lower_entity or "mtext" in lower_entity:
            texts.append(
                {
                    "layer": layer,
                    "text": str(text_value or name),
                    "insert": _point_from_cad_props(props),
                }
            )
            continue

                if "dimension" in lower_entity or "dim" in lower_entity:
            dimensions.append({"layer": layer, "measurement": measurement or length, "text": name})
            continue

                if "hatch" in lower_entity or "face" in lower_entity:
            if area:
                hatches.append({"layer": layer, "area": area})
            continue

                if "circle" in lower_entity or "arc" in lower_entity or "ellipse" in lower_entity:
            circles.append(
                {
                    "layer": layer,
                    "radius": radius,
                    "arc_length": length,
                    "area": area,
                    "type": entity,
                }
            )
            continue

                if "polyline" in lower_entity or "lwpolyline" in lower_entity:
            if length or area:
                polylines.append(
                    {
                        "layer": layer,
                        "length": length,
                        "area": area,
                        "closed": bool(area and not length),
                    }
                )
            continue

                if "line" in lower_entity and "polyline" not in lower_entity:
            if length:
                lines.append({"layer": layer, "length": length})
            continue

                if "block" in lower_entity or "reference" in lower_entity or "insert" in lower_entity:
            # Skip viewport / sheet framing noise
            if any(skip in lower_name for skip in ("mview", "viewport", "sheet", "logo", "title")):
                continue
            if any(skip in lower_layer for skip in ("vport", "defpoints", "viewport")):
                continue
            blocks.append(
                {
                    "name": name,
                    "layer": layer,
                    "type": entity or "Block",
                    **_civil_location_fields(props),
                }
            )
            continue

                # Geometry-bearing unknowns
        if length and area:
            polylines.append({"layer": layer, "length": length, "area": area, "closed": True})
        elif length:
            lines.append({"layer": layer, "length": length})
        elif area:
            hatches.append({"layer": layer, "area": area})
        elif entity and not entity.startswith("AcDb") and "table" not in lower_entity:
            blocks.append(
                {
                    "name": name,
                    "layer": layer,
                    "type": entity,
                    **_civil_location_fields(props),
                }
            )

            stats = {
        "aps_objects": len(by_entity),
        "layer_count": len(layers),
        "line_count": len(lines),
        "polyline_count": len(polylines),
        "block_insert_count": len(blocks),
        "text_count": len(texts),
        "dimension_count": len(dimensions),
        "hatch_count": len(hatches),
        "circle_arc_count": len(circles),
        "pipe_count": len(pipes),
        "alignment_count": len(alignments),
        "surface_count": len(surfaces),
        "volume_signals": len(volumes),
        "source": "properties.db",
    }
    return {
        "format": "dwg",
        "engine": "autodesk_aps",
        "status": "extracted",
        "units": "ft",  # Civil plan/profile DWGs via APS are commonly imperial drawing units
        "layers": list(layers.values()),
        "lines": lines[:5000],
        "polylines": polylines[:5000],
        "blocks": blocks[:5000],
        "texts": texts[:2000],
        "dimensions": dimensions[:2000],
        "tables": [],
        "hatches": hatches[:2000],
        "circles": circles[:2000],
        "pipes": pipes[:5000],
        "alignments": alignments[:2000],
        "surfaces": surfaces[:500],
        "volumes": volumes[:500],
        "stats": stats,
        "summary": (
            f"APS DWG processed '{filename}' via properties.db: {stats['aps_objects']} objects, "
            f"{stats['layer_count']} layers, {stats['line_count']} lines, "
            f"{stats['polyline_count']} polylines, {stats['pipe_count']} pipes, "
            f"{stats['block_insert_count']} blocks/structures, "
            f"{stats['surface_count']} surfaces/volume signals."
        ),
        "aps": {"configured": True, "urn_processed": True, "property_source": "properties.db"},
    }

        
        def process_dwg_with_aps(path: Path) -> dict[str, Any]:
    """Full DWG pipeline: auth → bucket → upload → translate → properties → CAD schema."""
    import tempfile

            settings = get_settings()
    token = get_access_token()
    bucket = ensure_bucket(token)
    uploaded = upload_object(token, bucket, path)
    urn = to_urn(uploaded["object_id"])
    start_translation(token, urn)
    manifest = wait_for_manifest(
        token,
        urn,
        timeout_s=int(getattr(settings, "autodesk_poll_timeout_seconds", 3600) or 3600),
        interval_s=int(getattr(settings, "autodesk_poll_interval_seconds", 5) or 5),
    )
    metadata = fetch_metadata_guids(token, urn)
    if not metadata:
        raise AutodeskAPSError("APS translation succeeded but no metadata views were returned")

            extraction: dict[str, Any] | None = None
    property_source = "rest"

            # Prefer properties.db — REST /metadata/{guid}/properties is often empty for 2D DWG sheets.
    db_urn = find_properties_db_urn(manifest)
    if db_urn:
        try:
            raw = download_derivative(token, db_urn)
            with tempfile.TemporaryDirectory(prefix="aps_props_") as tmp:
                db_path = Path(tmp) / "properties.db"
                db_path.write_bytes(raw)
                extraction = parse_properties_db(db_path, filename=path.name)
                property_source = "properties.db"
        except Exception as exc:
            # Fall through to REST properties aggregation
            extraction = None
            property_source = f"properties.db_failed:{exc}"

            if extraction is None or int((extraction.get("stats") or {}).get("aps_objects") or 0) == 0:
        merged_collection: list[dict[str, Any]] = []
        for item in metadata:
            guid = item.get("guid")
            if not guid:
                continue
            try:
                props = fetch_properties(token, urn, str(guid))
            except AutodeskAPSError:
                continue
            merged_collection.extend(((props.get("data") or {}).get("collection")) or [])
        extraction = map_aps_properties_to_extraction(
            {"data": {"collection": merged_collection}},
            filename=path.name,
        )
        if property_source == "properties.db":
            property_source = "rest_fallback"
        elif not property_source.startswith("properties.db"):
            property_source = "rest"

            extraction["aps"] = {
        "configured": True,
        "bucket": bucket,
        "object_key": uploaded["object_key"],
        "object_id": uploaded["object_id"],
        "urn": urn,
        "metadata_views": len(metadata),
        "property_source": property_source,
        "sheet_names": [m.get("name") for m in metadata],
    }
    return extraction

        
        def _point_from_cad_props(props: dict[str, Any]) -> list[float] | None:
    """Best-effort XY from Civil/APS property bags for structures & fittings."""
    return point_from_property_bag(props)

        
        def _station_side_offset_from_props(props: dict[str, Any]) -> dict[str, Any]:
    loc = location_from_property_bag(props)
    return {
        "station": loc.get("station") or loc.get("raw_station"),
        "side": loc.get("side"),
        "offset": loc.get("offset_ft"),
    }

        
        def _civil_location_fields(props: dict[str, Any]) -> dict[str, Any]:
    loc = location_from_property_bag(props)
    pt = loc.get("insert")
    return {
        "insert": pt,
        "position": pt,
        "station": loc.get("station") or loc.get("raw_station"),
        "side": loc.get("side"),
        "offset": loc.get("offset_ft"),
        "offset_ft": loc.get("offset_ft"),
    }

        
        def _first_float(props: dict[str, Any], keys: tuple[str, ...]) -> float | None:
    for key in keys:
        value = _to_float(props.get(key))
        if value is not None:
            return value
    return None

        
        def _to_float(value: Any) -> float | None:
    if value is None:
        return None
    if isinstance(value, (int, float)):
        return float(value)
    text = str(value).strip().replace(",", "")
    match = re.search(r"-?\d+(?:\.\d+)?", text)
    if not match:
        return None
    try:
        return float(match.group(0))
    except ValueError:
        return None

### backend/app/services/cad/design_automation.py

SHA-256: 4961e3dba47672ebb00681019934271a252dbd27d5422e3857123aa1162b8ed5    ·    647 line(s)    ·    .py

"""Autodesk Design Automation (cloud AutoCAD / Civil 3D) for DWG takeoff.

        Two modes:
1) Script activity (no custom plugin): cloud AutoCAD opens DWG → DXFOUT → local ezdxf parse
2) AppBundle activity (optional): cloud plugin writes result.json with richer takeoff

        Falls back to Model Derivative when DA is disabled or fails.
"""

        from __future__ import annotations

        import io
import json
import re
import tempfile
import time
import uuid
import zipfile
from pathlib import Path
from typing import Any
from urllib.parse import quote

        import httpx

        from app.config import get_settings
from app.services.cad.aps_client import (
    APS_BASE,
    AutodeskAPSError,
    aps_configured,
    ensure_bucket,
    get_access_token,
    upload_object,
)

        DA_BASE = f"{APS_BASE}/da/us-east/v3"
DEFAULT_ALIAS = "prod"
APPBUNDLE_ID = "AutoVadCivilTakeoff"
ACTIVITY_DXF_ID = "AutoVadDwgToDxf"
ACTIVITY_PLUGIN_ID = "AutoVadCivilTakeoff"

        
        def design_automation_enabled() -> bool:
    settings = get_settings()
    return bool(aps_configured() and settings.design_automation_enabled)

        
        def design_automation_status() -> dict[str, Any]:
    settings = get_settings()
    bundle_path = _appbundle_zip_path()
    return {
        "enabled": settings.design_automation_enabled,
        "configured": design_automation_enabled(),
        "nickname": (settings.design_automation_nickname or "").strip() or None,
        "engine": settings.design_automation_engine or "auto",
        "modes": {
            "dwg_to_dxf_script": True,
            "civil_takeoff_appbundle": bundle_path.exists(),
        },
        "appbundle_zip": str(bundle_path) if bundle_path.exists() else None,
        "preferred": (
            "civil_takeoff_appbundle"
            if bundle_path.exists() and settings.design_automation_prefer_plugin
            else "dwg_to_dxf_script"
        ),
        "setup_hint": (
            "Set DESIGN_AUTOMATION_ENABLED=true (and APS credentials). "
            "Optional: build cad_plugins/AutoVadCivilTakeoff into appbundle.zip for richer Civil takeoff."
        ),
    }

        
        def get_da_token() -> str:
    """Token with Design Automation scope (code:all)."""
    settings = get_settings()
    if not aps_configured():
        raise AutodeskAPSError("Autodesk APS credentials are not configured")
    data = {
        "grant_type": "client_credentials",
        "scope": (
            "data:read data:write data:create bucket:create bucket:read "
            "viewables:read code:all"
        ),
    }
    with httpx.Client(timeout=60.0) as client:
        resp = client.post(
            f"{APS_BASE}/authentication/v2/token",
            data=data,
            auth=(settings.autodesk_client_id.strip(), settings.autodesk_client_secret.strip()),
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(f"APS DA auth failed ({resp.status_code}): {resp.text[:400]}")
        token = resp.json().get("access_token")
        if not token:
            raise AutodeskAPSError("APS DA auth response missing access_token")
        return token

        
        def _headers(token: str) -> dict[str, str]:
    return {"Authorization": f"Bearer {token}", "Content-Type": "application/json"}

        
        def _nickname(token: str | None = None) -> str:
    settings = get_settings()
    configured = (settings.design_automation_nickname or "").strip()
    if configured:
        return configured
    tok = token or get_da_token()
    with httpx.Client(timeout=60.0) as client:
        resp = client.get(f"{DA_BASE}/forgeapps/me", headers=_headers(tok))
        if resp.status_code == 200:
            nick = (resp.json() or {}).get("nickname") or (resp.json() or {}).get("id")
            if nick:
                return str(nick)
        # Create / set nickname from client id fragment
        nick = re.sub(r"[^A-Za-z0-9]", "", (settings.autodesk_client_id or "autovad")[:20]) or "autovad"
        patch = client.patch(
            f"{DA_BASE}/forgeapps/me",
            headers=_headers(tok),
            json={"nickname": nick},
        )
        if patch.status_code >= 400 and patch.status_code != 409:
            # Some apps already have a nickname; retry GET
            again = client.get(f"{DA_BASE}/forgeapps/me", headers=_headers(tok))
            if again.status_code == 200:
                data = again.json() or {}
                return str(data.get("nickname") or data.get("id") or nick)
            raise AutodeskAPSError(f"DA nickname setup failed ({patch.status_code}): {patch.text[:400]}")
        return nick

        
        def _list_engines(token: str) -> list[str]:
    with httpx.Client(timeout=60.0) as client:
        resp = client.get(f"{DA_BASE}/engines", headers=_headers(token))
        if resp.status_code >= 400:
            return []
        data = resp.json() or {}
        return [e for e in (data.get("data") or []) if isinstance(e, str)]

        
        def _pick_engine(token: str) -> str:
    """AutoCAD engine for the DWG→DXF script activity."""
    settings = get_settings()
    configured = (settings.design_automation_engine or "").strip()
    if configured and configured.lower() != "auto":
        return configured
    engines = _list_engines(token)
    autocad = [e for e in engines if "AutoCAD+" in e and "Civil" not in e]
    if not autocad:
        autocad = [e for e in engines if "AutoCAD+" in e]
    if not autocad:
        return "Autodesk.AutoCAD+25_0"
    autocad.sort(reverse=True)
    return autocad[0]

        
        def _pick_civil_engine(token: str) -> str:
    """Prefer Civil 3D so Alignment.StationOffset and pipe networks are live."""
    settings = get_settings()
    configured = (settings.design_automation_engine or "").strip()
    if configured and configured.lower() != "auto" and "Civil" in configured:
        return configured
    engines = _list_engines(token)
    civil = [e for e in engines if "Civil3D+" in e or "Civil 3D" in e]
    if civil:
        civil.sort(reverse=True)
        return civil[0]
    return _pick_engine(token)

        
        def _qualified(nickname: str, local_id: str, alias: str = DEFAULT_ALIAS) -> str:
    return f"{nickname}.{local_id}+{alias}"

        
        def _appbundle_zip_path() -> Path:
    settings = get_settings()
    custom = (settings.design_automation_appbundle_path or "").strip()
    if custom:
        return Path(custom)
    # backend/cad_plugins/AutoVadCivilTakeoff/appbundle.zip
    return Path(__file__).resolve().parents[3] / "cad_plugins" / "AutoVadCivilTakeoff" / "appbundle.zip"

        
        def ensure_activity_dwg_to_dxf(token: str | None = None) -> str:
    """Register (or reuse) script-only DWG→DXF Design Automation activity."""
    tok = token or get_da_token()
    nick = _nickname(tok)
    engine = _pick_engine(tok)
    activity_id = ACTIVITY_DXF_ID
    qualified = _qualified(nick, activity_id)

            body = {
        "id": activity_id,
        "engine": engine,
        "commandLine": [
            '$(engine.path)\\accoreconsole.exe /i "$(args[inputFile].path)" '
            '/s "$(settings[script].path)"'
        ],
        "parameters": {
            "inputFile": {
                "verb": "get",
                "description": "Input DWG",
                "required": True,
                "localName": "input.dwg",
            },
            "outputFile": {
                "verb": "put",
                "description": "Output DXF from Design Automation",
                "required": True,
                "localName": "result.dxf",
            },
        },
        "settings": {
            # AccoreConsole script: export DXF (R2018 / AC1027 = 16 is common)
            "script": "._-DXFOUT\nresult.dxf\n16\n",
        },
        "description": "AutoVAD: DWG → DXF via Design Automation (cloud AutoCAD)",
    }

            with httpx.Client(timeout=120.0) as client:
        # Create activity if missing
        get = client.get(f"{DA_BASE}/activities/{quote(qualified, safe='')}", headers=_headers(tok))
        if get.status_code == 404:
            create = client.post(f"{DA_BASE}/activities", headers=_headers(tok), json=body)
            if create.status_code >= 400 and create.status_code != 409:
                raise AutodeskAPSError(
                    f"DA activity create failed ({create.status_code}): {create.text[:500]}"
                )
            version = 1
            if create.status_code < 400:
                version = int((create.json() or {}).get("version") or 1)
            alias = client.post(
                f"{DA_BASE}/activities/{activity_id}/aliases",
                headers=_headers(tok),
                json={"id": DEFAULT_ALIAS, "version": version},
            )
            if alias.status_code >= 400 and alias.status_code != 409:
                client.patch(
                    f"{DA_BASE}/activities/{activity_id}/aliases/{DEFAULT_ALIAS}",
                    headers=_headers(tok),
                    json={"version": version},
                )
        elif get.status_code >= 400:
            raise AutodeskAPSError(f"DA activity get failed ({get.status_code}): {get.text[:400]}")

            return qualified

        
        def ensure_appbundle_and_activity(token: str | None = None) -> str | None:
    """Upload optional Civil takeoff AppBundle and register activity. Returns qualified activity id."""
    zip_path = _appbundle_zip_path()
    if not zip_path.exists():
        return None

            tok = token or get_da_token()
    nick = _nickname(tok)
    engine = _pick_civil_engine(tok)
    qualified_bundle = _qualified(nick, APPBUNDLE_ID)
    qualified_activity = _qualified(nick, ACTIVITY_PLUGIN_ID)

            with httpx.Client(timeout=180.0) as client:
        # AppBundle create / new version
        get_b = client.get(
            f"{DA_BASE}/appbundles/{quote(qualified_bundle, safe='')}",
            headers=_headers(tok),
        )
        if get_b.status_code == 404:
            create_b = client.post(
                f"{DA_BASE}/appbundles",
                headers=_headers(tok),
                json={
                    "id": APPBUNDLE_ID,
                    "engine": engine,
                    "description": "AutoVAD Civil/AutoCAD takeoff plugin",
                },
            )
            if create_b.status_code >= 400:
                raise AutodeskAPSError(
                    f"DA appbundle create failed ({create_b.status_code}): {create_b.text[:500]}"
                )
            upload = create_b.json()
        else:
            # New version
            ver = client.post(
                f"{DA_BASE}/appbundles/{APPBUNDLE_ID}/versions",
                headers=_headers(tok),
                json={"engine": engine, "description": "AutoVAD Civil takeoff plugin update"},
            )
            if ver.status_code >= 400:
                raise AutodeskAPSError(
                    f"DA appbundle version failed ({ver.status_code}): {ver.text[:500]}"
                )
            upload = ver.json()

                upload_url = upload.get("uploadParameters", {}).get("endpointURL") or upload.get("uploadUrl")
        form_data = upload.get("uploadParameters", {}).get("formData") or {}
        if not upload_url:
            raise AutodeskAPSError("DA appbundle response missing upload URL")

                files = {"file": ("appbundle.zip", zip_path.read_bytes(), "application/octet-stream")}
        # S3 form upload
        up = client.post(upload_url, data=form_data, files=files)
        if up.status_code >= 400:
            raise AutodeskAPSError(f"DA appbundle upload failed ({up.status_code}): {up.text[:400]}")

                # Alias
        client.post(
            f"{DA_BASE}/appbundles/{APPBUNDLE_ID}/aliases",
            headers=_headers(tok),
            json={"id": DEFAULT_ALIAS, "version": upload.get("version") or 1},
        )
        client.patch(
            f"{DA_BASE}/appbundles/{APPBUNDLE_ID}/aliases/{DEFAULT_ALIAS}",
            headers=_headers(tok),
            json={"version": upload.get("version") or 1},
        )

                activity_body = {
            "id": ACTIVITY_PLUGIN_ID,
            "engine": engine,
            "commandLine": [
                '$(engine.path)\\accoreconsole.exe /i "$(args[inputFile].path)" '
                '/al "$(appbundles[AutoVadCivilTakeoff].path)" '
                '/s "$(settings[script].path)"'
            ],
            "appbundles": [qualified_bundle],
            "parameters": {
                "inputFile": {
                    "verb": "get",
                    "required": True,
                    "localName": "input.dwg",
                },
                "outputFile": {
                    "verb": "put",
                    "required": True,
                    "localName": "result.json",
                },
            },
            "settings": {"script": "AutoVadTakeoff\n"},
            "description": "AutoVAD Design Automation Civil takeoff → result.json",
        }

                get_a = client.get(
            f"{DA_BASE}/activities/{quote(qualified_activity, safe='')}",
            headers=_headers(tok),
        )
        if get_a.status_code == 404:
            create_a = client.post(f"{DA_BASE}/activities", headers=_headers(tok), json=activity_body)
            if create_a.status_code >= 400:
                raise AutodeskAPSError(
                    f"DA plugin activity create failed ({create_a.status_code}): {create_a.text[:500]}"
                )
            client.post(
                f"{DA_BASE}/activities/{ACTIVITY_PLUGIN_ID}/aliases",
                headers=_headers(tok),
                json={"id": DEFAULT_ALIAS, "version": 1},
            )
        else:
            # Update via new version
            client.post(
                f"{DA_BASE}/activities/{ACTIVITY_PLUGIN_ID}/versions",
                headers=_headers(tok),
                json={k: v for k, v in activity_body.items() if k != "id"},
            )

            return qualified_activity

        
        def setup_design_automation() -> dict[str, Any]:
    """Ensure nickname + activities are registered in the APS account."""
    if not design_automation_enabled():
        raise AutodeskAPSError("Design Automation is disabled or APS credentials missing")
    tok = get_da_token()
    nick = _nickname(tok)
    engine = _pick_engine(tok)
    dxf_activity = ensure_activity_dwg_to_dxf(tok)
    plugin_activity = None
    plugin_error = None
    try:
        plugin_activity = ensure_appbundle_and_activity(tok)
    except Exception as exc:
        plugin_error = str(exc)
    return {
        "nickname": nick,
        "engine": engine,
        "dxf_activity": dxf_activity,
        "plugin_activity": plugin_activity,
        "plugin_error": plugin_error,
        "status": design_automation_status(),
    }

        
        def _oss_object_url(bucket: str, object_key: str) -> str:
    return f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{quote(object_key, safe='')}"

        
        def _create_empty_oss_object(token: str, bucket: str, object_key: str) -> None:
    """Ensure output object key exists (some APS flows prefer signed put on existing keys)."""
    # Direct-to-S3 empty upload via existing helper pattern — write 1 byte placeholder then overwrite
    pass

        
        def _signed_download_url(token: str, bucket: str, object_key: str, minutes: int = 60) -> str:
    headers = {"Authorization": f"Bearer {token}", "Content-Type": "application/json"}
    encoded = quote(object_key, safe="")
    with httpx.Client(timeout=60.0) as client:
        resp = client.post(
            f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{encoded}/signed",
            headers=headers,
            json={"minutesExpiration": minutes, "singleUse": False},
            params={"access": "read"},
        )
        if resp.status_code >= 400:
            # Fallback: authenticated OSS URL (DA accepts Authorization header)
            return _oss_object_url(bucket, object_key)
        data = resp.json() or {}
        return data.get("signedUrl") or data.get("url") or _oss_object_url(bucket, object_key)

        
        def _signed_upload_url(token: str, bucket: str, object_key: str, minutes: int = 60) -> str:
    headers = {"Authorization": f"Bearer {token}", "Content-Type": "application/json"}
    encoded = quote(object_key, safe="")
    with httpx.Client(timeout=60.0) as client:
        resp = client.post(
            f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{encoded}/signed",
            headers=headers,
            json={"minutesExpiration": minutes, "singleUse": False},
            params={"access": "write"},
        )
        if resp.status_code >= 400:
            return _oss_object_url(bucket, object_key)
        data = resp.json() or {}
        return data.get("signedUrl") or data.get("url") or _oss_object_url(bucket, object_key)

        
        def _download_oss_object(token: str, bucket: str, object_key: str) -> bytes:
    headers = {"Authorization": f"Bearer {token}"}
    encoded = quote(object_key, safe="")
    with httpx.Client(timeout=3600.0) as client:
        resp = client.get(
            f"{APS_BASE}/oss/v2/buckets/{bucket}/objects/{encoded}",
            headers=headers,
        )
        if resp.status_code >= 400:
            raise AutodeskAPSError(
                f"OSS download failed ({resp.status_code}): {resp.text[:400]}"
            )
        return resp.content

        
        def _submit_and_wait(
    token: str,
    activity_id: str,
    arguments: dict[str, Any],
    *,
    timeout_s: int,
    interval_s: int,
) -> dict[str, Any]:
    with httpx.Client(timeout=120.0) as client:
        create = client.post(
            f"{DA_BASE}/workitems",
            headers=_headers(token),
            json={"activityId": activity_id, "arguments": arguments},
        )
        if create.status_code >= 400:
            raise AutodeskAPSError(
                f"DA workitem create failed ({create.status_code}): {create.text[:600]}"
            )
        work = create.json() or {}
        work_id = work.get("id")
        if not work_id:
            raise AutodeskAPSError("DA workitem response missing id")

                deadline = time.time() + timeout_s
        last: dict[str, Any] = work
        while time.time() < deadline:
            status_resp = client.get(f"{DA_BASE}/workitems/{work_id}", headers=_headers(token))
            if status_resp.status_code >= 400:
                raise AutodeskAPSError(
                    f"DA workitem status failed ({status_resp.status_code}): {status_resp.text[:400]}"
                )
            last = status_resp.json() or {}
            st = (last.get("status") or "").lower()
            if st in {"success", "failed", "cancelled", "timeout"}:
                if st != "success":
                    report = last.get("reportUrl")
                    raise AutodeskAPSError(
                        f"DA workitem {st}"
                        + (f" — report: {report}" if report else "")
                        + f" — {json.dumps(last.get('stats') or {})[:200]}"
                    )
                return last
            time.sleep(interval_s)
        raise AutodeskAPSError(f"DA workitem timed out after {timeout_s}s (id={work_id})")

        
        def process_dwg_with_design_automation(path: Path) -> dict[str, Any]:
    """Run Design Automation on a DWG and return CAD extraction dict."""
    if not design_automation_enabled():
        raise AutodeskAPSError("Design Automation is not enabled")

            settings = get_settings()
    token = get_da_token()
    # Also need OSS token for upload (same token with data scopes — DA token has them)
    bucket = ensure_bucket(token)
    uploaded = upload_object(token, bucket, path)
    input_key = uploaded["object_key"]
    output_key = f"da_out/{uuid.uuid4().hex}_{path.stem}.out"

            timeout_s = int(settings.design_automation_timeout_seconds or 540)
    interval_s = int(settings.autodesk_poll_interval_seconds or 5)

            prefer_plugin = bool(settings.design_automation_prefer_plugin)
    plugin_activity = None
    if prefer_plugin and _appbundle_zip_path().exists():
        try:
            plugin_activity = ensure_appbundle_and_activity(token)
        except Exception:
            plugin_activity = None

            input_arg = {
        "url": _oss_object_url(bucket, input_key),
        "verb": "get",
        "headers": {"Authorization": f"Bearer {token}"},
    }
    output_arg = {
        "url": _oss_object_url(bucket, output_key),
        "verb": "put",
        "headers": {
            "Authorization": f"Bearer {token}",
            "Content-Type": "application/octet-stream",
        },
    }

            if plugin_activity:
        _submit_and_wait(
            token,
            plugin_activity,
            {"inputFile": input_arg, "outputFile": {**output_arg, "localName": "result.json"}},
            timeout_s=timeout_s,
            interval_s=interval_s,
        )
        raw = _download_oss_object(token, bucket, output_key)
        try:
            payload = json.loads(raw.decode("utf-8"))
        except Exception as exc:
            raise AutodeskAPSError(f"DA plugin result.json invalid: {exc}") from exc
        return _normalize_plugin_result(payload, filename=path.name, activity=plugin_activity)

            # Default: DWG → DXF via cloud AutoCAD script, then local ezdxf
    activity = ensure_activity_dwg_to_dxf(token)
    # Output key should end with .dxf for clarity
    output_key = f"da_out/{uuid.uuid4().hex}_{path.stem}.dxf"
    output_arg["url"] = _oss_object_url(bucket, output_key)

            _submit_and_wait(
        token,
        activity,
        {
            "inputFile": input_arg,
            "outputFile": {**output_arg, "localName": "result.dxf"},
        },
        timeout_s=timeout_s,
        interval_s=interval_s,
    )
    dxf_bytes = _download_oss_object(token, bucket, output_key)
    if len(dxf_bytes) < 64:
        raise AutodeskAPSError("DA DXF output was empty — check AccoreConsole DXFOUT script / engine")

            with tempfile.TemporaryDirectory(prefix="da_dxf_") as tmp:
        dxf_path = Path(tmp) / "result.dxf"
        dxf_path.write_bytes(dxf_bytes)
        from app.services.cad.dxf_parser import parse_dxf

                extraction = parse_dxf(dxf_path)

            extraction["format"] = "dwg"
    extraction["engine"] = "design_automation_dxf"
    extraction["status"] = "extracted"
    extraction["summary"] = (
        f"Design Automation (cloud AutoCAD) converted '{path.name}' → DXF, "
        f"then local geometry takeoff. "
        + (extraction.get("summary") or "")
    )
    extraction["aps"] = {
        "configured": True,
        "design_automation": True,
        "mode": "dwg_to_dxf_script",
        "activity": activity,
        "bucket": bucket,
        "input_object": input_key,
        "output_object": output_key,
    }
    return extraction

        
        def _normalize_plugin_result(payload: dict[str, Any], *, filename: str, activity: str) -> dict[str, Any]:
    """Map plugin JSON into the shared CAD extraction schema."""
    stats = payload.get("stats") or {}
    return {
        "format": "dwg",
        "engine": "design_automation_civil",
        "status": "extracted",
        "units": payload.get("units"),
        "layers": payload.get("layers") or [],
        "lines": payload.get("lines") or [],
        "polylines": payload.get("polylines") or [],
        "blocks": payload.get("blocks") or [],
        "texts": payload.get("texts") or [],
        "dimensions": payload.get("dimensions") or [],
        "tables": payload.get("tables") or [],
        "hatches": payload.get("hatches") or [],
        "circles": payload.get("circles") or [],
        "alignments": payload.get("alignments") or [],
        "pipes": payload.get("pipes") or [],
        "surfaces": payload.get("surfaces") or [],
        "stats": {**stats, "file": filename},
        "summary": payload.get("summary")
        or (
            f"Design Automation Civil takeoff for '{filename}': "
            f"{stats.get('entity_count', '?')} entities."
        ),
        "aps": {
            "configured": True,
            "design_automation": True,
            "mode": "civil_takeoff_appbundle",
            "activity": activity,
        },
        "quantities_hint": payload.get("quantities") or [],
    }

        
        def package_appbundle_from_folder(plugin_dir: Path, out_zip: Path | None = None) -> Path:
    """Zip a PackageContents.xml + Contents folder into appbundle.zip (helper for builds)."""
    out = out_zip or (plugin_dir / "appbundle.zip")
    package_xml = plugin_dir / "PackageContents.xml"
    contents = plugin_dir / "Contents"
    if not package_xml.exists():
        raise FileNotFoundError("PackageContents.xml missing")
    buf = io.BytesIO()
    with zipfile.ZipFile(buf, "w", zipfile.ZIP_DEFLATED) as zf:
        zf.write(package_xml, arcname="PackageContents.xml")
        if contents.exists():
            for f in contents.rglob("*"):
                if f.is_file():
                    zf.write(f, arcname=str(Path("Contents") / f.relative_to(contents)))
    out.write_bytes(buf.getvalue())
    return out

### backend/app/services/cad/dwg_aps.py

SHA-256: 4618904a7159e3e2e719fdaf51b47ddca4e33bbab9a1f92955359fe04311b021    ·    5 line(s)    ·    .py

"""DWG entrypoint — delegates to Autodesk APS adapter."""

        from app.services.cad.autodesk_aps import parse_civil3d_package, parse_dwg

        __all__ = ["parse_dwg", "parse_civil3d_package"]

### backend/app/services/cad/autodesk_aps.py

SHA-256: 024f4c06bd3eb384ee73d83b810686b1efbf6602c9efaac520b92720eb0c0023    ·    158 line(s)    ·    .py

"""Autodesk Platform Services (APS) adapter for native DWG / Civil 3D models."""

        from __future__ import annotations

        from pathlib import Path
from typing import Any

        from app.config import get_settings
from app.services.cad.aps_client import (
    AutodeskAPSError,
    aps_configured,
    aps_status,
    process_dwg_with_aps,
)
from app.services.cad.design_automation import (
    design_automation_enabled,
    design_automation_status,
    process_dwg_with_design_automation,
)

        
        def parse_dwg(path: Path) -> dict[str, Any]:
    if not aps_configured():
        return {
            "format": "dwg",
            "engine": "autodesk_aps_pending",
            "status": "needs_autodesk",
            "units": None,
            "layers": [],
            "lines": [],
            "polylines": [],
            "blocks": [],
            "texts": [],
            "dimensions": [],
            "tables": [],
            "stats": {"file": path.name, "bytes": path.stat().st_size},
            "summary": (
                "DWG uploaded. Native DWG processing requires Autodesk Platform Services "
                "(Design Automation and/or Model Derivative). Set AUTODESK_CLIENT_ID and "
                "AUTODESK_CLIENT_SECRET in backend/.env, or export DWG to DXF/LandXML for local parsing."
            ),
            "next_steps": [
                "Create an APS app at https://aps.autodesk.com and copy Client ID/Secret",
                "Enable Design Automation API on the app, put credentials in backend/.env, restart backend",
                "Or export DWG → DXF and re-upload for immediate local extraction",
            ],
            "aps": aps_status(),
            "design_automation": design_automation_status(),
        }

            settings = get_settings()
    errors: list[str] = []

            # 1) Prefer Design Automation (cloud AutoCAD) when enabled
    if design_automation_enabled():
        try:
            result = process_dwg_with_design_automation(path)
            result.setdefault("aps", {})["model_derivative_fallback"] = False
            result["design_automation"] = design_automation_status()
            return result
        except Exception as exc:
            errors.append(f"Design Automation: {exc}")
            if not settings.design_automation_fallback_model_derivative:
                return {
                    "format": "dwg",
                    "engine": "design_automation",
                    "status": "failed",
                    "units": None,
                    "layers": [],
                    "lines": [],
                    "polylines": [],
                    "blocks": [],
                    "texts": [],
                    "dimensions": [],
                    "tables": [],
                    "stats": {"file": path.name},
                    "summary": f"Design Automation DWG processing failed: {exc}",
                    "error": str(exc),
                    "aps": aps_status(),
                    "design_automation": design_automation_status(),
                }

            # 2) Fallback: Model Derivative properties.db / REST
    try:
        result = process_dwg_with_aps(path)
        if errors:
            result["summary"] = (
                (result.get("summary") or "")
                + f" (Design Automation unavailable → Model Derivative. {errors[0][:180]})"
            )
            result.setdefault("aps", {})["design_automation_error"] = errors[0]
        result["design_automation"] = design_automation_status()
        return result
    except AutodeskAPSError as exc:
        errors.append(f"Model Derivative: {exc}")
        return {
            "format": "dwg",
            "engine": "autodesk_aps",
            "status": "failed",
            "units": None,
            "layers": [],
            "lines": [],
            "polylines": [],
            "blocks": [],
            "texts": [],
            "dimensions": [],
            "tables": [],
            "stats": {"file": path.name},
            "summary": "APS DWG processing failed: " + " | ".join(errors),
            "error": " | ".join(errors),
            "aps": aps_status(),
            "design_automation": design_automation_status(),
        }

        
        def parse_civil3d_package(path: Path) -> dict[str, Any]:
    """Civil 3D integration entrypoint (exports / APS / Design Automation)."""
    suffix = path.suffix.lower()
    if suffix in {".xml", ".landxml"}:
        from app.services.cad.landxml_parser import parse_landxml

                result = parse_landxml(path)
        result["format"] = "civil3d"
        result["engine"] = "civil3d_via_landxml"
        result["status"] = "extracted"
        result["summary"] = "Civil 3D data ingested via LandXML export. " + (result.get("summary") or "")
        return result

            if suffix == ".json":
        import json

                data = json.loads(path.read_text(encoding="utf-8", errors="ignore"))
        return {
            "format": "civil3d",
            "engine": "civil3d_json_export",
            "status": "extracted",
            "units": data.get("units"),
            "layers": data.get("layers") or [],
            "lines": data.get("lines") or [],
            "polylines": data.get("polylines") or [],
            "blocks": data.get("blocks") or [],
            "texts": data.get("texts") or [],
            "dimensions": data.get("dimensions") or [],
            "tables": data.get("tables") or [],
            "stats": data.get("stats") or {"keys": list(data.keys())[:40]},
            "summary": "Civil 3D JSON export ingested into CAD Intelligence Engine.",
        }

            # Native Civil 3D/DWG package → Design Automation / APS
    result = parse_dwg(path)
    result["format"] = "civil3d"
    engine = result.get("engine") or ""
    if "design_automation" in engine:
        result["summary"] = "Civil 3D/DWG via Design Automation. " + (result.get("summary") or "")
    elif engine.startswith("autodesk_aps") or engine == "autodesk_aps":
        result["engine"] = "civil3d_via_aps"
        result["summary"] = "Civil 3D/DWG via APS Model Derivative. " + (result.get("summary") or "")
    return result

### backend/cad_plugins/AutoVadCivilTakeoff/Commands.cs

SHA-256: b5a76909b7209bd978f5047725aa4dea66fc87ba49fe0e00c193d8c317fdf0f0    ·    512 line(s)    ·    .cs

// AutoVAD Design Automation plugin — AutoCAD .NET API + optional Civil 3D.
// Writes result.json with geometry Civil StationOffset needs:
//   alignments (sampled), pipes (start/end XY + stations), blocks (insert + attribs),
//   polylines (vertices), lines (start/end), texts (model + paper-space callouts).

        using System;
using System.Collections.Generic;
using System.IO;
using System.Linq;
using System.Reflection;
using System.Text.Json;
using Autodesk.AutoCAD.ApplicationServices.Core;
using Autodesk.AutoCAD.DatabaseServices;
using Autodesk.AutoCAD.Geometry;
using Autodesk.AutoCAD.Runtime;

        [assembly: CommandClass(typeof(AutoVadCivilTakeoff.Commands))]

        namespace AutoVadCivilTakeoff
{
    public class Commands
    {
        [CommandMethod("AutoVadTakeoff", CommandFlags.Session)]
        public void RunTakeoff()
        {
            var doc = Application.DocumentManager.MdiActiveDocument;
            if (doc == null) return;

                    var db = doc.Database;
            var layers = new List<object>();
            var lines = new List<object>();
            var polylines = new List<object>();
            var blocks = new List<object>();
            var texts = new List<object>();
            var circles = new List<object>();
            var hatches = new List<object>();
            var alignments = new List<object>();
            var pipes = new List<object>();
            int entityCount = 0;

                    using (var tr = db.TransactionManager.StartTransaction())
            {
                var lt = (LayerTable)tr.GetObject(db.LayerTableId, OpenMode.ForRead);
                foreach (ObjectId id in lt)
                {
                    var layer = (LayerTableRecord)tr.GetObject(id, OpenMode.ForRead);
                    layers.Add(new { name = layer.Name, color = layer.Color.ColorIndex });
                }

                        var bt = (BlockTable)tr.GetObject(db.BlockTableId, OpenMode.ForRead);
                ExportSpace(tr, (BlockTableRecord)tr.GetObject(bt[BlockTableRecord.ModelSpace], OpenMode.ForRead),
                    "model", ref entityCount, lines, polylines, blocks, texts, circles, hatches);

                        // Paper-space sheet labels (STA / offset callouts)
                var ltLayout = (DBDictionary)tr.GetObject(db.LayoutDictionaryId, OpenMode.ForRead);
                foreach (var entry in ltLayout)
                {
                    if (string.Equals(entry.Key, "Model", StringComparison.OrdinalIgnoreCase))
                        continue;
                    try
                    {
                        var layout = (Layout)tr.GetObject(entry.Value, OpenMode.ForRead);
                        var btr = (BlockTableRecord)tr.GetObject(layout.BlockTableRecordId, OpenMode.ForRead);
                        ExportSpace(tr, btr, "paper", ref entityCount, lines, polylines, blocks, texts, circles, hatches);
                    }
                    catch { /* ignore layout */ }
                }

                        TryExportCivil(tr, alignments, pipes, blocks);
                tr.Commit();
            }

                    var payload = new Dictionary<string, object>
            {
                ["units"] = db.Insunits.ToString(),
                ["layers"] = layers,
                ["lines"] = lines,
                ["polylines"] = polylines,
                ["blocks"] = blocks,
                ["texts"] = texts.Take(4000).ToList(),
                ["circles"] = circles,
                ["hatches"] = hatches,
                ["dimensions"] = new List<object>(),
                ["tables"] = new List<object>(),
                ["alignments"] = alignments,
                ["pipes"] = pipes,
                ["surfaces"] = new List<object>(),
                ["stats"] = new Dictionary<string, object>
                {
                    ["entity_count"] = entityCount,
                    ["layer_count"] = layers.Count,
                    ["line_count"] = lines.Count,
                    ["polyline_count"] = polylines.Count,
                    ["block_insert_count"] = blocks.Count,
                    ["alignment_count"] = alignments.Count,
                    ["pipe_count"] = pipes.Count,
                    ["civil_loaded"] = alignments.Count > 0 || pipes.Count > 0,
                },
                ["summary"] = $"AutoVadTakeoff exported {entityCount} entities, {alignments.Count} alignments, {pipes.Count} pipes, {blocks.Count} blocks.",
            };

                    File.WriteAllText("result.json", JsonSerializer.Serialize(payload, new JsonSerializerOptions { WriteIndented = true }));
            doc.Editor?.WriteMessage($"\nAutoVAD: wrote result.json");
        }

                static void ExportSpace(
            Transaction tr,
            BlockTableRecord space,
            string spaceName,
            ref int entityCount,
            List<object> lines,
            List<object> polylines,
            List<object> blocks,
            List<object> texts,
            List<object> circles,
            List<object> hatches)
        {
            foreach (ObjectId id in space)
            {
                entityCount++;
                var ent = tr.GetObject(id, OpenMode.ForRead) as Entity;
                if (ent == null) continue;
                var layer = ent.Layer;

                        switch (ent)
                {
                    case Line ln:
                        lines.Add(new
                        {
                            layer,
                            length = ln.Length,
                            start = new[] { ln.StartPoint.X, ln.StartPoint.Y },
                            end = new[] { ln.EndPoint.X, ln.EndPoint.Y },
                        });
                        break;
                    case Polyline pl:
                        polylines.Add(ReadPolyline(pl, layer));
                        break;
                    case Polyline2d pl2:
                        polylines.Add(ReadPolyline2d(tr, pl2, layer));
                        break;
                    case Polyline3d pl3:
                        polylines.Add(ReadPolyline3d(tr, pl3, layer));
                        break;
                    case Circle cir:
                        circles.Add(new
                        {
                            layer,
                            radius = cir.Radius,
                            area = Math.PI * cir.Radius * cir.Radius,
                            center = new[] { cir.Center.X, cir.Center.Y },
                        });
                        break;
                    case Arc arc:
                        circles.Add(new { layer, radius = arc.Radius, length = arc.Length, type = "arc" });
                        break;
                    case Hatch hatch:
                        try { hatches.Add(new { layer, area = hatch.Area }); } catch { /* ignore */ }
                        break;
                    case BlockReference br:
                        blocks.Add(ReadBlock(tr, br, layer));
                        break;
                    case DBText txt:
                        texts.Add(new
                        {
                            layer,
                            text = txt.TextString,
                            insert = new[] { txt.Position.X, txt.Position.Y },
                            space = spaceName,
                        });
                        break;
                    case MText mt:
                        texts.Add(new
                        {
                            layer,
                            text = mt.Contents,
                            insert = new[] { mt.Location.X, mt.Location.Y },
                            space = spaceName,
                        });
                        break;
                }
            }
        }

                static object ReadPolyline(Polyline pl, string layer)
        {
            var pts = new List<double[]>();
            for (int i = 0; i < pl.NumberOfVertices; i++)
            {
                var p = pl.GetPoint2dAt(i);
                pts.Add(new[] { p.X, p.Y });
            }
            return new
            {
                layer,
                name = layer,
                length = pl.Length,
                area = SafeArea(pl),
                closed = pl.Closed,
                vertices = pts.Count,
                points = pts,
            };
        }

                static double SafeArea(Polyline pl)
        {
            try { return pl.Area; } catch { return 0; }
        }

                static object ReadPolyline2d(Transaction tr, Polyline2d pl2, string layer)
        {
            var pts = new List<double[]>();
            foreach (ObjectId vid in pl2)
            {
                var v = tr.GetObject(vid, OpenMode.ForRead) as Vertex2d;
                if (v != null)
                    pts.Add(new[] { v.Position.X, v.Position.Y });
            }
            return new { layer, name = layer, length = 0.0, closed = pl2.Closed, vertices = pts.Count, points = pts };
        }

                static object ReadPolyline3d(Transaction tr, Polyline3d pl3, string layer)
        {
            var pts = new List<double[]>();
            foreach (ObjectId vid in pl3)
            {
                var v = tr.GetObject(vid, OpenMode.ForRead) as PolylineVertex3d;
                if (v != null)
                    pts.Add(new[] { v.Position.X, v.Position.Y });
            }
            return new { layer, name = layer, length = 0.0, vertices = pts.Count, points = pts };
        }

                static object ReadBlock(Transaction tr, BlockReference br, string layer)
        {
            var attribs = new Dictionary<string, string>(StringComparer.OrdinalIgnoreCase);
            try
            {
                foreach (ObjectId aid in br.AttributeCollection)
                {
                    var att = tr.GetObject(aid, OpenMode.ForRead) as AttributeReference;
                    if (att == null || string.IsNullOrWhiteSpace(att.Tag)) continue;
                    attribs[att.Tag] = att.TextString ?? "";
                    attribs[att.Tag.ToUpperInvariant()] = att.TextString ?? "";
                }
            }
            catch { /* no attribs */ }

                    string GetAttr(params string[] tags)
            {
                foreach (var t in tags)
                {
                    if (attribs.TryGetValue(t, out var v) && !string.IsNullOrWhiteSpace(v))
                        return v;
                }
                return null;
            }

                    var insert = new[] { br.Position.X, br.Position.Y };
            return new
            {
                name = br.Name,
                layer,
                type = "INSERT",
                insert,
                position = insert,
                rotation = br.Rotation,
                attributes = attribs,
                station = GetAttr("STATION", "STA", "STN", "RAWSTA"),
                side = GetAttr("SIDE", "DIR", "HAND"),
                offset = GetAttr("OFFSET", "OFF", "OS"),
            };
        }

                /// <summary>
        /// Late-bound Civil 3D export so the DLL still loads on AutoCAD-only DA engines.
        /// Uses Alignment.StationOffset / PointLocation and pipe-network locations.
        /// </summary>
        static void TryExportCivil(Transaction tr, List<object> alignments, List<object> pipes, List<object> blocks)
        {
            try
            {
                Type civAppType = null;
                foreach (var assy in AppDomain.CurrentDomain.GetAssemblies())
                {
                    civAppType = assy.GetType("Autodesk.Civil.ApplicationServices.CivilApplication");
                    if (civAppType != null) break;
                }
                if (civAppType == null) return;

                        var civDoc = civAppType.GetProperty("ActiveDocument", BindingFlags.Public | BindingFlags.Static)
                    ?.GetValue(null, null);
                if (civDoc == null) return;

                        ExportCivilAlignments(tr, civDoc, alignments);
                ExportCivilNetworks(tr, civDoc, pipes, blocks, alignments);
            }
            catch
            {
                // Civil 3D not available — AutoCAD geometry already exported
            }
        }

                static void ExportCivilAlignments(Transaction tr, object civDoc, List<object> alignments)
        {
            try
            {
                dynamic doc = civDoc;
                foreach (dynamic oid in doc.GetAlignmentIds())
                {
                    dynamic al = tr.GetObject((ObjectId)oid, OpenMode.ForRead);
                    string aname = Convert.ToString(al.Name);
                    double sta0 = ToDouble(al, "StartingStation");
                    double sta1 = ToDouble(al, "EndingStation");
                    double alen = ToDouble(al, "Length");
                    if (sta1 <= sta0 && alen > 0) sta1 = sta0 + alen;

                            var pts = SampleAlignment(al, sta0, sta1, alen);
                    alignments.Add(new
                    {
                        name = aname,
                        layer = aname,
                        length = alen,
                        sta_start = sta0,
                        start_station = sta0,
                        points = pts,
                        source = "civil3d_alignment",
                    });
                }
            }
            catch { /* alignments API mismatch */ }
        }

                static List<double[]> SampleAlignment(dynamic al, double sta0, double sta1, double alen)
        {
            var pts = new List<double[]>();
            double span = Math.Max(alen, sta1 - sta0);
            if (span <= 0) span = 100;
            double step = Math.Max(10.0, span / 80.0);
            for (double s = sta0; s <= sta1 + 0.01; s += step)
            {
                var xy = PointAtStation(al, s);
                if (xy != null) pts.Add(xy);
            }
            var last = PointAtStation(al, sta1);
            if (last != null) pts.Add(last);
            return pts;
        }

                static double[] PointAtStation(dynamic al, double station)
        {
            try
            {
                double x = 0, y = 0;
                al.PointLocation(station, 0.0, ref x, ref y);
                return new[] { x, y };
            }
            catch
            {
                try
                {
                    double x = 0, y = 0, z = 0;
                    al.PointLocation(station, 0.0, 0.0, ref x, ref y, ref z);
                    return new[] { x, y };
                }
                catch { return null; }
            }
        }

                static void ExportCivilNetworks(Transaction tr, object civDoc, List<object> pipes, List<object> blocks, List<object> alignments)
        {
            try
            {
                dynamic doc = civDoc;
                dynamic firstAlign = null;
                try
                {
                    foreach (dynamic oid in doc.GetAlignmentIds())
                    {
                        firstAlign = tr.GetObject((ObjectId)oid, OpenMode.ForRead);
                        break;
                    }
                }
                catch { /* no alignments */ }

                        foreach (dynamic nid in doc.GetPipeNetworkIds())
                {
                    dynamic net = tr.GetObject((ObjectId)nid, OpenMode.ForRead);
                    string netName = SafeName(net);
                    dynamic refAl = firstAlign;
                    try { if (net.ReferenceAlignmentId != ObjectId.Null) refAl = tr.GetObject(net.ReferenceAlignmentId, OpenMode.ForRead); } catch { }

                            try
                    {
                        foreach (dynamic pid in net.GetPipeIds())
                        {
                            dynamic pipe = tr.GetObject((ObjectId)pid, OpenMode.ForRead);
                            var start = Point3(pipe, "StartPoint");
                            var end = Point3(pipe, "EndPoint");
                            double[] staOffStart = StationOffsetOf(refAl, start);
                            double[] staOffEnd = StationOffsetOf(refAl, end);
                            pipes.Add(new
                            {
                                name = SafeName(pipe),
                                layer = netName,
                                network = netName,
                                length = ToDouble(pipe, "Length2DCenterToCenter", "Length2D", "Length3D"),
                                diameter = ToDouble(pipe, "InnerDiameterOrWidth", "Diameter"),
                                part_size = SafeName(pipe),
                                start,
                                end,
                                points = (start != null && end != null) ? new[] { start, end } : null,
                                sta_start = staOffStart != null ? staOffStart[0] : (double?)null,
                                sta_end = staOffEnd != null ? staOffEnd[0] : (double?)null,
                                offset = staOffStart != null ? staOffStart[1] : (double?)null,
                                side = SideFromCivilOffset(staOffStart != null ? staOffStart[1] : (double?)null),
                                source = "civil3d_pipe",
                            });
                        }
                    }
                    catch { /* pipes */ }

                            try
                    {
                        foreach (dynamic sid in net.GetStructureIds())
                        {
                            dynamic st = tr.GetObject((ObjectId)sid, OpenMode.ForRead);
                            var loc = Point3(st, "Location", "Position");
                            if (loc == null)
                            {
                                try { loc = new[] { (double)st.Easting, (double)st.Northing }; } catch { }
                            }
                            double[] so = StationOffsetOf(refAl, loc);
                            blocks.Add(new
                            {
                                name = SafeName(st),
                                layer = netName,
                                type = "Structure",
                                network = netName,
                                description = SafeName(st),
                                insert = loc,
                                position = loc,
                                station = so != null ? so[0] : (double?)null,
                                offset = so != null ? so[1] : (double?)null,
                                offset_ft = so != null ? Math.Abs(so[1]) : (double?)null,
                                side = SideFromCivilOffset(so != null ? so[1] : (double?)null),
                                source = "civil3d_structure",
                            });
                        }
                    }
                    catch { /* structures */ }
                }
            }
            catch { /* networks API mismatch */ }
        }

                static double[] StationOffsetOf(dynamic alignment, double[] xy)
        {
            if (alignment == null || xy == null || xy.Length < 2) return null;
            try
            {
                double sta = 0, off = 0;
                alignment.StationOffset(xy[0], xy[1], ref sta, ref off);
                return new[] { sta, off };
            }
            catch { return null; }
        }

                static string SideFromCivilOffset(double? signed)
        {
            if (signed == null) return null;
            if (Math.Abs(signed.Value) < 0.05) return "CL";
            return signed.Value > 0 ? "RT" : "LT";
        }

                static double[] Point3(dynamic obj, params string[] names)
        {
            foreach (var n in names)
            {
                try
                {
                    var p = obj.GetType().GetProperty(n)?.GetValue(obj, null);
                    if (p == null) continue;
                    double x = Convert.ToDouble(p.GetType().GetProperty("X")?.GetValue(p, null));
                    double y = Convert.ToDouble(p.GetType().GetProperty("Y")?.GetValue(p, null));
                    return new[] { x, y };
                }
                catch { /* try next */ }
            }
            return null;
        }

                static double ToDouble(dynamic obj, params string[] names)
        {
            foreach (var n in names)
            {
                try
                {
                    var v = obj.GetType().GetProperty(n)?.GetValue(obj, null);
                    if (v != null) return Convert.ToDouble(v);
                }
                catch { }
            }
            return 0;
        }

                static string SafeName(dynamic obj)
        {
            try { return Convert.ToString(obj.Name); } catch { return "Civil"; }
        }
    }
}

### backend/cad_plugins/AutoVadCivilTakeoff/PackageContents.xml

SHA-256: 7bf457c84b2db06576034d816caeeff177dcf18c5abf62e356899f3ca3d2378d    ·    27 line(s)    ·    .xml

<?xml version="1.0" encoding="utf-8"?>
<ApplicationPackage
  SchemaVersion="1.0"
  AutodeskProduct="AutoCAD"
  Name="AutoVadCivilTakeoff"
  Description="AutoVAD Design Automation takeoff exporter"
  AppVersion="1.0.0"
  ProductType="Application"
  FriendlyVersion="1.0.0"
  ProductCode="{A7C0AD10-5B2E-4F3A-9C11-0AUTOVAD0001}"
  UpgradeCode="{A7C0AD10-5B2E-4F3A-9C11-0AUTOVAD0000}">
  <CompanyDetails Name="AutoVAD" Url="https://localhost" Email="support@autovad.local" />
  <Components>
    <RuntimeRequirements OS="Win64" Platform="AutoCAD*" SeriesMin="R24.0" SeriesMax="R25.9" />
    <ComponentEntry
      AppName="AutoVadCivilTakeoff"
      Version="1.0.0"
      ModuleName="./Contents/AutoVadCivilTakeoff.dll"
      AppDescription="Exports geometry takeoff JSON for AutoVAD"
      LoadOnCommandInvocation="True"
      LoadOnAutoCADStartup="False">
      <Commands GroupName="AutoVad">
        <Command Global="AutoVadTakeoff" Local="AutoVadTakeoff" />
      </Commands>
    </ComponentEntry>
  </Components>
</ApplicationPackage>

### frontend/src/utils/eoqGroups.ts

SHA-256: fdb95d74b12e27de27736c94be7e2e7f84b5c6bf702630d0bfdb618c4663afe0    ·    319 line(s)    ·    .ts

/** Municipal Estimate-of-Quantities section grouping for EOQ UI. */

        export const EOQ_GROUP_ORDER = [
  'General',
  'Traffic Control',
  'Removals',
  'Clearing & Grubbing',
  'Grading',
  'Erosion Control / Restoration',
  'Surfacing',
  'Curb, Gutter & Sidewalk',
  'Storm Sewer',
  'Watermain',
  'Sanitary Sewer',
  'Water & Sewer Services',
  'Traffic Signals & Signing',
  'Lighting & Electrical',
  'Gas & Dry Utilities',
  'Structures',
  'Landscaping & Irrigation',
  'Miscellaneous',
  'Unmapped Takeoff',
] as const

        const CATEGORY_ALIASES: Record<string, string> = {
  utilities: 'Watermain',
  utility: 'Watermain',
  drainage: 'Storm Sewer',
  earthwork: 'Grading',
  pavement: 'Surfacing',
  roadside: 'Curb, Gutter & Sidewalk',
  landscaping: 'Landscaping & Irrigation',
  traffic: 'Traffic Signals & Signing',
  markings: 'Traffic Signals & Signing',
  structures: 'Structures',
  demolition: 'Removals',
  'site clearing': 'Clearing & Grubbing',
  geometry: 'Miscellaneous',
  general: 'General',
  'traffic control': 'Traffic Control',
  'general / traffic control': 'General',
  'bid schedule': 'Miscellaneous',
  'unmapped takeoff': 'Unmapped Takeoff',
}

        const GROUP_RULES: Array<{ keys: string[]; group: string }> = [
  {
    keys: ['remove ', 'removal', 'demolition', 'sawcut', 'saw cut', 'abandon', 'mill and remove'],
    group: 'Removals',
  },
  {
    keys: [
      'mobilization',
      'demobilization',
      'tax on city',
      'city furnished',
      'winter maintenance',
      'construction entrance',
      'field office',
      'survey',
      'staking',
      'audiovisual',
      'permit',
      'allowance',
    ],
    group: 'General',
  },
  {
    keys: [
      'traffic control',
      'temporary traffic',
      'flagging',
      'barricade',
      'mailbox',
      'changeable message',
      'business sign',
      'gravel access',
      'channeliz',
    ],
    group: 'Traffic Control',
  },
  { keys: ['clearing', 'grubbing', 'tree removal', 'stump', 'brush'], group: 'Clearing & Grubbing' },
  {
    keys: [
      'unclassified excavation',
      'excavation',
      'earthwork',
      'embankment',
      'borrow',
      'grading',
      'subgrade',
      'topsoil',
      'proof roll',
      'trench stabilization',
      'select backfill',
      'imported fill',
      'rock excavation',
    ],
    group: 'Grading',
  },
  {
    keys: [
      'erosion',
      'silt fence',
      'inlet protection',
      'sediment',
      'seeding',
      'sodding',
      'mulch',
      'fertiliz',
      'hydroseed',
      'weed control',
      'water for vegetation',
      'turf establishment',
      'swppp',
    ],
    group: 'Erosion Control / Restoration',
  },
  {
    keys: [
      'aggregate base',
      'crushed aggregate',
      'asphalt',
      'hma',
      'hot mix',
      'bituminous',
      'pavement',
      'paving',
      'gsb',
      'wmm',
      'dbm',
      'prime coat',
      'tack coat',
      'milling',
      'overlay',
      'surface course',
      'pcc pavement',
      'concrete pavement',
    ],
    group: 'Surfacing',
  },
  {
    keys: ['curb and gutter', 'curb', 'gutter', 'sidewalk', 'driveway', 'ada', 'detectable warning', 'kerb'],
    group: 'Curb, Gutter & Sidewalk',
  },
  {
    keys: [
      'watermain',
      'water main',
      'waterline',
      'water line',
      'c900',
      'fire hydrant',
      'hydrant',
      'gate valve',
      'butterfly valve',
      'water valve',
      'blowoff',
      'air release',
      'water meter',
      'thrust block',
      'trenchless',
      'directional drill',
      'casing pipe',
      'potable',
    ],
    group: 'Watermain',
  },
  {
    keys: [
      'sanitary sewer',
      'sanitary pipe',
      'sanitary manhole',
      'ssmh',
      'force main',
      'forcemain',
      'sanitary service',
      'cleanout',
      'lift station',
    ],
    group: 'Sanitary Sewer',
  },
  {
    keys: [
      'storm sewer',
      'storm drain',
      'storm pipe',
      'catch basin',
      'inlet',
      'culvert',
      'headwall',
      'junction box',
      'retention',
      'detention',
      'storm manhole',
      'drainage structure',
      'underdrain',
    ],
    group: 'Storm Sewer',
  },
  {
    keys: ['water service', 'service connection', 'corporation stop', 'curb stop', 'meter pit', 'lateral'],
    group: 'Water & Sewer Services',
  },
  {
    keys: [
      'traffic signal',
      'pavement marking',
      'striping',
      'thermoplastic',
      'road sign',
      'traffic sign',
      'signage',
    ],
    group: 'Traffic Signals & Signing',
  },
  {
    keys: ['street light', 'lighting', 'luminaire', 'electrical conduit', 'pull box', 'electric'],
    group: 'Lighting & Electrical',
  },
  { keys: ['gas main', 'gas service', 'telecom', 'fiber', 'joint trench'], group: 'Gas & Dry Utilities' },
  {
    keys: ['bridge', 'retaining wall', 'reinforced concrete', 'structural concrete', 'rebar', 'formwork', 'pile'],
    group: 'Structures',
  },
  { keys: ['landscape', 'irrigation', 'planting', 'shrub', 'groundcover'], group: 'Landscaping & Irrigation' },
  { keys: ['sewer'], group: 'Sanitary Sewer' },
  { keys: ['drain', 'drainage'], group: 'Storm Sewer' },
  { keys: ['water', 'valve', 'pipe', 'fitting', 'bend', 'tee', 'reducer'], group: 'Watermain' },
  { keys: ['fence', 'guardrail', 'barrier'], group: 'Miscellaneous' },
]

        function matchDescription(text: string): string | null {
  const low = text.toLowerCase()
  if (!low.trim()) return null
  for (const rule of GROUP_RULES) {
    if (rule.keys.some((k) => low.includes(k))) return rule.group
  }
  return null
}

        export function resolveEoqGroup(description?: string | null, category?: string | null): string {
  const cat = (category || '').trim()
  const catLow = cat.toLowerCase()
  if (catLow === 'general / traffic control' || catLow === 'general/traffic control') {
    return matchDescription(description || '') || 'General'
  }
  if (cat) {
    const alias = CATEGORY_ALIASES[catLow]
    if (alias) {
      if (['Watermain', 'Storm Sewer', 'Sanitary Sewer', 'Miscellaneous', 'General'].includes(alias)) {
        const refined = matchDescription(description || '')
        if (refined) return refined
      }
      return alias
    }
    if ((EOQ_GROUP_ORDER as readonly string[]).includes(cat)) return cat
  }
  const matched = matchDescription(`${description || ''} ${category || ''}`)
  if (matched) return matched
  if (catLow === 'unmapped takeoff') return 'Unmapped Takeoff'
  return 'Miscellaneous'
}

        function looksLikeMobilization(description?: string | null): boolean {
  const text = String(description || '').trim().toLowerCase()
  if (!text || text.includes('demobilization')) return false
  return /\bmobilization\b/.test(text)
}

        export interface EoqGroupSection<T> {
  group: string
  items: T[]
}

        export function groupEoqItems<T extends { description?: string | null; category?: string | null }>(
  items: T[],
): EoqGroupSection<T & { display_number: number }>[] {
  const buckets = new Map<string, T[]>()
  for (const item of items) {
    const group = resolveEoqGroup(item.description, item.category)
    const list = buckets.get(group) || []
    list.push(item)
    buckets.set(group, list)
  }
  const mobs = items.filter((item) => looksLikeMobilization(item.description))
  if (mobs.length) {
    const gen = buckets.get('General') || []
    for (const mob of [...mobs].reverse()) {
      if (!gen.includes(mob)) gen.unshift(mob)
    }
    buckets.set('General', gen)
    const tc = buckets.get('Traffic Control') || []
    for (const mob of [...mobs].reverse()) {
      if (!tc.includes(mob)) tc.unshift(mob)
    }
    buckets.set('Traffic Control', tc)
  }
  const ordered: EoqGroupSection<T & { display_number: number }>[] = []
  let serial = 1
  const pushSection = (name: string, list: T[]) => {
    if (!list.length) return
    ordered.push({
      group: name,
      items: list.map((item) => ({ ...item, display_number: serial++ })),
    })
  }
  for (const name of EOQ_GROUP_ORDER) {
    const list = buckets.get(name)
    if (list?.length) {
      pushSection(name, list)
      buckets.delete(name)
    }
  }
  for (const [name, list] of [...buckets.entries()].sort((a, b) => a[0].localeCompare(b[0]))) {
    pushSection(name, list)
  }
  return ordered
}

### backend/app/schemas/units.py

SHA-256: 20bc039c590d255356ff29c534f1c3d056186da4e8b7099c9cd9d935cd2edd80    ·    7 line(s)    ·    .py

from typing import Annotated

        from pydantic import AfterValidator

        from app.services.csi_mapper import format_export_unit

        PayUnit = Annotated[str, AfterValidator(format_export_unit)]

# 8. Counsel checklist

Confirm claimant legal name, inventors, and assignment chain.

Patent: consider a provisional on U-01 through U-14 as a combined takeoff system.

Copyright of the whole website: use the separate full source deposit, not this pack alone.

Trade secret: keep this .docx and the private repository off public GitHub if unpublished.

Dependencies (FastAPI, Vue, Autodesk, OpenAI) are not AutoVAD inventions.

END OF CONFIDENTIAL PATENT PACK